2.3.4  FORMAL DEFINITIONS OF MESSAGES

2.3.4.1  Encoding/Decoding Rules

2.3.4.1.1	A CPDLC-air-ASE shall be capable of encoding AircraftPDUs APDUs and decoding

GroundPDUs APDUs.

2.3.4.1.2	A CPDLC-ground-ASE shall be capable of encoding GroundPDUs APDUs and

decoding AircraftPDUs APDUs.

2.3.4.2  CPDLC ASN.1 Abstract Syntax

2.3.4.2.1	The abstract syntax of the CPDLC protocol data units shall comply with the description

contained in the ASN.1 module CPDLCMessageSetVersion1 conforming to ISO/IEC 8824, as defined

in this section.

CPDLCMessageSetVersion1 DEFINITIONS::=

BEGIN

-- ----------------------------------------------------------------------------------

-- Ground Generated Messages - Top level

-- ----------------------------------------------------------------------------------

GroundPDUs ::= CHOICE

{

abortUser		[0]	CPDLCUserAbortReason,

abortProvider		[1]	CPDLCProviderAbortReason,

startup			[2]	UplinkMessage,

send			[3]	ATCUplinkMessage,

forward		[4]	ATCForwardMessage,

forwardresponse	[5]	ATCForwardResponse,

...

}

UplinkMessage ::= CHOICE

{

noMessage		[0]	NULL,

aTCUplinkMessage	[1]	ATCUplinkMessage

}

ATCUplinkMessage ::= SEQUENCE

{

header			ATCMessageHeader,

elementIds		SEQUENCE SIZE (1..5) OF ATCUplinkMsgElementId

}

ATCForwardMessage ::= SEQUENCE

{

forwardHeader		ForwardHeader,

forwardMessage	ForwardMessage

}

ForwardHeader ::= SEQUENCE

{

dateTime		DateTimeGroup,

aircraftID		AircraftFlightIdentification,

aircraftAddress		AircraftAddress

}

ForwardMessage ::= CHOICE

{

upElementIDs		[0]	SEQUENCE SIZE (1..5) OF ATCUplinkMsgElementId,

downElementIDs	[1]	SEQUENCE SIZE (1..5) OF ATCDownlinkMsgElementId

}

ATCForwardResponse ::= ENUMERATED

{

success			(0),

service-not-supported		(1),

version-not-equal		(2),

...

}

-- ----------------------------------------------------------------------------------

-- Aircraft Generated Messages - Top level

-- ----------------------------------------------------------------------------------

AircraftPDUs::= CHOICE

{

abortUser	[0]	CPDLCUserAbortReason,

abortProvider	[1]	CPDLCProviderAbortReason,

startdown	[2]	StartDownMessage,

send		[3]	ATCDownlinkMessage,

...

}

StartDownMessage ::= SEQUENCE

{

mode			Mode DEFAULT cpdlc,

startDownlinkMessage 	DownlinkMessage

}

Mode ::= ENUMERATED

{

cpdlc		(0),

dsc		(1)

}

DownlinkMessage ::= CHOICE

{

noMessage			[0]	NULL,

aTCDownlinkMessage		[1]	ATCDownlinkMessage

}

ATCDownlinkMessage ::= SEQUENCE

{

header			ATCMessageHeader,

elementIds		SEQUENCE SIZE (1..5) OF ATCDownlinkMsgElementId

}

-- ----------------------------------------------------------------------------------

-- Uplink and Downlink messages - Common Elements

-- ----------------------------------------------------------------------------------

ATCMessageHeader ::= SEQUENCE

{

messageIdNumber		[0]	MsgIdentificationNumber,

messageRefNumber		[1]	MsgReferenceNumber		OPTIONAL,

dateTime			[2]	DateTimeGroup,

logicalAck			[3]	LogicalAck			DEFAULT

notRequired

}

MsgIdentificationNumber ::= INTEGER (0..63)

MsgReferenceNumber ::= INTEGER (0..63)

CPDLCUserAbortReason ::= ENUMERATED

{

undefined					(0),

no-message-identification-numbers-available	(1),

duplicate-message-identification-numbers	(2),

no-longer-next-data-authority			(3),

current-data-authority-abort			(4),

commanded-termination			(5),

invalid-response				(6),

...

}

CPDLCProviderAbortReason ::= ENUMERATED

{

timer-expired					(0),

undefined-error					(1),

invalid-PDU					(2),

not-permitted-PDU				(3),

communication-service-error			(4),

communication-service-failure			(5),

invalid-QOS-parameter				(6),

expected-PDU-missing				(7),

...

}

LogicalAck ::= ENUMERATED

{

required			(0),

notRequired			(1)

}

-- ----------------------------------------------------------------------------------

-- Uplink message element

-- ----------------------------------------------------------------------------------

ATCUplinkMsgElementId ::= CHOICE

{

--	UNABLE						Urg(N)/Alr(L)/Resp(N)

uM0NULL						[0] NULL,

--	STANDBY						Urg(N)/Alr(L)/Resp(N)

uM1NULL						[1] NULL,

--	REQUEST DEFERRED				Urg(N)/Alr(L)/Resp(N)

uM2NULL						[2] NULL,

--	ROGER						Urg(N)/Alr(L)/Resp(N )

uM3NULL						[3] NULL,

--	AFFIRM 						Urg(N)/Alr(L)/Resp(N )

uM4NULL						[4] NULL,

--	NEGATIVE						Urg(N)/Alr(L)/Resp(N )

uM5NULL						[5] NULL,

--	EXPECT [level]					Urg(L)/Alr(L)/Resp(R)

uM6Level						[6] Level,

--	EXPECT CLIMB AT [time]				Urg(L)/Alr(L)/Resp(R)

uM7Time						[7] Time,

--	EXPECT CLIMB AT [position]				Urg(L)/Alr(L)/Resp(R)

uM8Position						[8] Position,

--	EXPECT DESCENT AT [time]				Urg(L)/Alr(L)/Resp(R)

uM9Time						[9] Time,

--	EXPECT DESCENT AT [position]			Urg(L)/Alr(L)/Resp(R)

uM10Position						[10] Position,

--	EXPECT CRUISE CLIMB AT [time]			Urg(L)/Alr(L)/Resp(R)

uM11Time						[11] Time,

--	EXPECT CRUISE CLIMB AT [position]		Urg(L)/Alr(L)/Resp(R)

uM12Position						[12] Position,

--	AT [time] EXPECT CLIMB TO [level]		Urg(L)/Alr(L)/Resp(R)

uM13TimeLevel					[13] TimeLevel,

--	AT [position] EXPECT CLIMB TO [level]		Urg(L)/Alr(L)/Resp(R)

uM14PositionLevel					[14] PositionLevel,

--	AT [time] EXPECT DESCENT TO [level]		Urg(L)/Alr(L)/Resp(R)

uM15TimeLevel					[15] TimeLevel,

--	AT [position] EXPECT DESCENT TO [level]		Urg(L)/Alr(L)/Resp(R)

uM16PositionLevel					[16] PositionLevel,

--	AT [time] EXPECT CRUISE CLIMB TO [level]	Urg(L)/Alr(L)/Resp(R)

uM17TimeLevel					[17] TimeLevel,

--	AT [position] EXPECT CRUISE CLIMB TO [level]	Urg(L)/Alr(L)/Resp(R)

uM18PositionLevel					[18] PositionLevel,

--	MAINTAIN [level]					Urg(N)/Alr(M)/Resp(W/U)

uM19Level						[19] Level,

--	CLIMB TO [level]					Urg(N)/Alr(M)/Resp(W/U)

uM20Level						[20] Level,

--	AT [time] CLIMB TO [level]				Urg(N)/Alr(M)/Resp(W/U)

uM21TimeLevel					[21] TimeLevel,

--	AT [position] CLIMB TO [level]			Urg(N)/Alr(M)/Resp(W/U)

uM22PositionLevel					[22] PositionLevel,

--	DESCEND TO [level]					Urg(N)/Alr(M)/Resp(W/U)

uM23Level 						[23] Level,

--	AT [time] DESCEND TO [level] 			Urg(N)/Alr(M)/Resp(W/U)

uM24TimeLevel 					[24] TimeLevel,

--	AT [position] DESCEND TO [level] 			Urg(N)/Alr(M)/Resp(W/U)

uM25PositionLevel 					[25] PositionLevel,

--	CLIMB TO REACH [level] BY [time]			Urg(N)/Alr(M)/Resp(W/U)

uM26LevelTime 					[26] LevelTime,

--	CLIMB TO REACH [level] BY [position]		Urg(N)/Alr(M)/Resp(W/U)

uM27LevelPosition 					[27] LevelPosition,

--	DESCEND TO REACH [level] BY [time]		Urg(N)/Alr(M)/Resp(W/U)

uM28LevelTime 					[28] LevelTime,

--	DESCEND TO REACH [level] BY [position]		Urg(N)/Alr(M)/Resp(W/U)

uM29LevelPosition 					[29] LevelPosition,

--	MAINTAIN BLOCK [level] TO [level]		Urg(N)/Alr(M)/Resp(W/U)

uM30LevelLevel 					[30] LevelLevel,

--	CLIMB TO AND MAINTAIN BLOCK [level] TO [level]

--								Urg(N)/Alr(M)/Resp(W/U)

uM31LevelLevel 					[31] LevelLevel,

--	DESCEND TO AND MAINTAIN BLOCK [level] TO [level]

--								Urg(N)/Alr(M)/Resp(W/U)

uM32LevelLevel 					[32] LevelLevel,

--	Reserved						Urg(L)/Alr(L)/Resp(Y)

uM33NULL						[33] NULL,

--	CRUISE CLIMB TO [level]				Urg(N)/Alr(M)/Resp(W/U)

uM34Level 						[34] Level,

--	CRUISE CLIMB ABOVE [level]			Urg(N)/Alr(M)/Resp(W/U)

uM35Level 						[35] Level,

--	EXPEDITE CLIMB TO [level]			Urg(U)/Alr(M)/Resp(W/U)

uM36Level 						[36] Level,

--	EXPEDITE DESCENT TO [level]			Urg(U)/Alr(M)/Resp(W/U)

uM37Level 						[37] Level,

--	IMMEDIATELY CLIMB TO [level]			Urg(D)/Alr(H)/Resp(W/U)

uM38Level 						[38] Level,

--	IMMEDIATELY DESCEND TO [level]		Urg(D)/Alr(H)/Resp(W/U)

uM39Level 						[39] Level,

Reserved						Urg(L)/Alr(L)/Resp(Y)

uM40NULL						[40]NULL,

--	Reserved						Urg(L)/Alr(L)/Resp(Y)

uM40NULL						[40]NULL,

--	EXPECT TO CROSS [position] AT [level] 		Urg(L)/Alr(L)/Resp(R)

uM42PositionLevel					[42] PositionLevel,

--	EXPECT TO CROSS [position] AT OR ABOVE [level]

--								Urg(L)/Alr(L)/Resp(R)

uM43PositionLevel					[43] PositionLevel,

--	EXPECT TO CROSS [position] AT OR BELOW [level]

--								Urg(L)/Alr(L)/Resp(R)

uM44PositionLevel 					[44] PositionLevel,

--	EXPECT TO CROSS [position] AT AND MAINTAIN [level]

--								Urg(L)/Alr(L)/Resp(R)

uM45PositionLevel 					[45] PositionLevel,

--	CROSS [position] AT [level]				Urg(N)/Alr(M)/Resp(W/U)

uM46PositionLevel 					[46] PositionLevel,

--	CROSS [position] AT OR ABOVE [level]		Urg(N)/Alr(M)/Resp(W/U)

uM47PositionLevel 					[47] PositionLevel,

--	CROSS [position] AT OR BELOW [level]		Urg(N)/Alr(M)/Resp(W/U)

uM48PositionLevel 					[48]PositionLevel,

--	CROSS [position] AT AND MAINTAIN [level]	Urg(N)/Alr(M)/Resp(W/U)

uM49PositionLevel 					[49] PositionLevel,

--	CROSS [position] BETWEEN [level] AND [level]	Urg(N)/Alr(M)/Resp(W/U)

uM50PositionLevelLevel				[50] PositionLevelLevel,

--	CROSS [position] AT [time]				Urg(N)/Alr(M)/Resp(W/U)

uM51PositionTime 					[51] PositionTime,

--	CROSS [position] AT OR BEFORE [time]		Urg(N)/Alr(M)/Resp(W/U)

uM52PositionTime 					[52] PositionTime,

--	CROSS [position] AT OR AFTER [time]		Urg(N)/Alr(M)/Resp(W/U)

uM53PositionTime 					[53] PositionTime,

--	CROSS [position] BETWEEN [time] AND [time]	Urg(N)/Alr(M)/Resp(W/U)

uM54PositionTimeTime				[54] PositionTimeTime,

--	CROSS [position] AT [speed]				Urg(N)/Alr(M)/Resp(W/U)

uM55PositionSpeed 					[55] PositionSpeed,

--	CROSS [position] AT OR LESS THAN [speed]		Urg(N)/Alr(M)/Resp(W/U)

uM56PositionSpeed 					[56] PositionSpeed,

--	CROSS [position] AT OR GREATER THAN [speed]

--								Urg(N)/Alr(M)/Resp(W/U)

uM57PositionSpeed 					[57] PositionSpeed,

--	CROSS [position] AT [time] AT [level]			Urg(N)/Alr(M)/Resp(W/U)

uM58PositionTimeLevel				[58] PositionTimeLevel,

--	CROSS [position] AT OR BEFORE [time] AT [level]	Urg(N)/Alr(M)/Resp(W/U)

uM59PositionTimeLevel 				[59] PositionTimeLevel,

--	CROSS [position] AT OR AFTER [time] AT [level]	Urg(N)/Alr(M)/Resp(W/U)

uM60PositionTimeLevel				[60] PositionTimeLevel,

--	CROSS [position] AT AND MAINTAIN [level] AT [speed]

--								Urg(N)/Alr(M)/Resp(W/U)

uM61PositionLevelSpeed 				[61] PositionLevelSpeed,

--	AT [time] CROSS [position] AT AND MAINTAIN [level]

--								Urg(N)/Alr(M)/Resp(W/U)

uM62TimePositionLevel				[62] TimePositionLevel,

--	AT [time] CROSS [position] AT AND MAINTAIN [level] AT [speed]

--								Urg(N)/Alr(M)/Resp(W/U)

uM63TimePositionLevelSpeed						[63]

TimePositionLevelSpeed,

--	OFFSET [distanceSpecifiedspecifiedDistance] [direction] OF ROUTE

Urg(N)/Alr(M)/Resp(W/U)

uM64DistanceSpecified Direction			[64] DistanceSpecified Direction,

--	AT [position] OFFSET [distanceSpecifiedspecifiedDistance] OF ROUTE

--								Urg(N)/Alr(M)/Resp(W/U)

uM65PositionDistanceSpecified Direction		[65] PositionDistanceSpecified

Direction,

--	AT [time] OFFSET [distanceSpecifiedspecifiedDistance] [direction] OF ROUTE

--								Urg(N)/Alr(M)/Resp(W/U)

uM66TimeDistanceSpecified Direction			[66] TimeDistanceSpecified Direction,

--	PROCEED BACK ON ROUTE				Urg(N)/Alr(M)/Resp(W/U)

uM67NULL 						[67] NULL,

--	REJOIN ROUTE BY [position]				Urg(N)/Alr(M)/Resp(W/U)

uM68Position 						[68] Position,

--	REJOIN ROUTE BY [time]				Urg(N)/Alr(M)/Resp(W/U)

uM69Time 						[69] Time,

--	EXPECT BACK ON ROUTE BY [position]		Urg(L)/Alr(L)/Resp(R)

uM70Position 						[70] Position,

--	EXPECT BACK ON ROUTE BY [time]		Urg(L)/Alr(L)/Resp(R)

uM71Time 						[71] Time,

--	RESUME OWN NAVIGATION			Urg(N)/Alr(M)/Resp(W/U)

uM72NULL 						[72] NULL,

--	[DepartureClearance]					Urg(N)/Alr(M)/Resp(W/U)

uM73DepartureClearance 				[73] DepartureClearance,

--	PROCEED DIRECT TO [position]			Urg(N)/Alr(M)/Resp(W/U)

uM74Position 						[74] Position,

--	WHEN ABLE PROCEED DIRECT TO [position]	Urg(N)/Alr(M)/Resp(W/U)

uM75Position 						[75] Position,

--	AT [time] PROCEED DIRECT TO [position]		Urg(N)/Alr(M)/Resp(W/U)

uM76TimePosition 					[76] TimePosition,

--	AT [position] PROCEED DIRECT TO [position]	Urg(N)/Alr(M)/Resp(W/U)

uM77PositionPosition 					[77] PositionPosition,

--	AT [level] PROCEED DIRECT TO [position]		Urg(N)/Alr(M)/Resp(W/U)

uM78LevelPosition 					[78] LevelPosition,

--	CLEARED TO [position] VIA [routeClearance]	Urg(N)/Alr(M)/Resp(W/U)

uM79PositionRouteClearance				[79] PositionRouteClearance,

--	CLEARED [routeClearance]				Urg(N)/Alr(M)/Resp(W/U)

uM80RouteClearance 					[80] RouteClearance,

--	CLEARED [procedureName]				Urg(N)/Alr(M)/Resp(W/U)

uM81ProcedureName 					[81] ProcedureName,

--	CLEARED TO DEVIATE UP TO [distanceSpecifiedspecifiedDistance] [direction] OF ROUTE

--								Urg(N)/Alr(M)/Resp(W/U)

uM82DistanceSpecifiedDirection			[82] DistanceSpecifiedDirection,

--	AT [position] CLEARED [routeClearance]		Urg(N)/Alr(M)/Resp(W/U)

uM83PositionRouteClearance				[83] PositionRouteClearance,

--	AT [position] CLEARED [procedureName]		Urg(N)/Alr(M)/Resp(W/U)

uM84PositionProcedureName 				[84] PositionProcedureName,

--	EXPECT [routeClearance]				Urg(L)/Alr(L)/Resp(R)

uM85RouteClearance 					[85] RouteClearance,

--	AT [position] EXPECT [routeClearance] 		Urg(L)/Alr(L)/Resp(R)

uM86PositionRouteClearance				[86] PositionRouteClearance,

--	EXPECT DIRECT TO [position]			Urg(L)/Alr(L)/Resp(R)

uM87Position 						[87] Position,

--	AT [position] EXPECT DIRECT TO [position]		Urg(L)/Alr(L)/Resp(R)

uM88PositionPosition 					[88]PositionPosition,

--	AT [time] EXPECT DIRECT TO [position]		Urg(L)/Alr(L)/Resp(R)

uM89TimePosition 					[89] TimePosition,

--	AT [level] EXPECT DIRECT TO [position]		Urg(L)/Alr(L)/Resp(R)

uM90LevelPosition 					[90] LevelPosition,

--	HOLD AT [position] MAINTAIN [level] INBOUND TRACK [degrees][direction]

--	TURNS [legtype]					Urg(N)/Alr(M)/Resp(W/U)

uM91HoldClearance 					[91] HoldClearance,

--	HOLD AT [position] AS PUBLISHED MAINTAIN [level]

--								Urg(N)/Alr(M)/Resp(W/U)

uM92PositionLevel 					[92] PositionLevel,

--	EXPECT FURTHER CLEARANCE AT [time]		Urg(L)/Alr(L)/Resp(R)

uM93Time 						[93] Time,

--	TURN [direction] HEADING [degrees]			Urg(N)/Alr(M)/Resp(W/U)

uM94DirectionDegrees 				[94] DirectionDegrees,

--	TURN [direction] GROUND TRACK [degrees]		Urg(N)/Alr(M)/Resp(W/U)

uM95DirectionDegrees					[95] DirectionDegrees,

--	CONTINUE PRESENT HEADING			Urg(N)/Alr(M)/Resp(W/U)

uM96NULL 						[96] NULL,

--	AT [position] FLY HEADING [degrees]		Urg(N)/Alr(M)/Resp(W/U)

uM97PositionDegrees 					[97] PositionDegrees,

--	IMMEDIATELY TURN [direction] HEADING [degrees]

--								Urg(D)/Alr(H)/Resp(W/U)

uM98DirectionDegrees 				[98] DirectionDegrees,

--	EXPECT [procedureName]				Urg(L)/Alr(L)/Resp(R)

uM99ProcedureName 					[99] ProcedureName,

--	AT [time] EXPECT [speed]				Urg(L)/Alr(L)/Resp(R)

uM100TimeSpeed 					[100] TimeSpeed,

--	AT [position] EXPECT [speed]				Urg(L)/Alr(L)/Resp(R)

uM101PositionSpeed 					[101] PositionSpeed,

--	AT [level] EXPECT [speed]				Urg(L)/Alr(L)/Resp(R)

uM102LevelSpeed 					[102] LevelSpeed,

--	AT [time] EXPECT [speed] TO [speed] 		Urg(L)/Alr(L)/Resp(R)

uM103TimeSpeedSpeed 				[103] TimeSpeedSpeed,

--	AT [position] EXPECT [speed] TO [speed]		Urg(L)/Alr(L)/Resp(R)

uM104PositionSpeedSpeed 				[104] PositionSpeedSpeed,

--	AT [level] EXPECT [speed] TO [speed]			Urg(L)/Alr(L)/Resp(R)

uM105LevelSpeedSpeed 				[105] LevelSpeedSpeed,

--	MAINTAIN [speed]					Urg(N)/Alr(M)/Resp(W/U)

uM106Speed 						[106] Speed,

--	MAINTAIN PRESENT SPEED			Urg(N)/Alr(M)/Resp(W/U)

uM107NULL 						[107] NULL,

--	MAINTAIN [speed] OR GREATER			Urg(N)/Alr(M)/Resp(W/U)

uM108Speed 						[108] Speed,

--	MAINTAIN [speed] OR LESS				Urg(N)/Alr(M)/Resp(W/U)

uM109Speed 						[109] Speed,

--	MAINTAIN [speed] TO [speed]			Urg(N)/Alr(M)/Resp(W/U)

uM110SpeedSpeed 					[110] SpeedSpeed,

--	INCREASE SPEED TO [speed]			Urg(N)/Alr(M)/Resp(W/U)

uM111Speed 						[111] Speed,

--	INCREASE SPEED TO [speed] OR GREATER  	Urg(N)/Alr(M)/Resp(W/U)

uM112Speed 						[112] Speed,

--	REDUCE SPEED TO [speed]				Urg(N)/Alr(M)/Resp(W/U)

uM113Speed 						[113] Speed,

--	REDUCE SPEED TO [speed] OR LESS		Urg(N)/Alr(M)/Resp(W/U)

uM114Speed 						[114] Speed,

--	DO NOT EXCEED [speed]				Urg(N)/Alr(M)/Resp(W/U)

uM115Speed 						[115] Speed,

--	RESUME NORMAL SPEED				Urg(N)/Alr(M)/Resp(W/U)

uM116NULL 						[116] NULL,

--	CONTACT [unitname] [frequency]			Urg(N)/Alr(M)/Resp(W/U)

uM117UnitNameFrequency 				[117] UnitNameFrequency,

--	AT [position] CONTACT [unitname] [frequency]	Urg(N)/Alr(M)/Resp(W/U)

uM118PositionUnitNameFrequency			[118] PositionUnitNameFrequency,

--	AT [time] CONTACT [unitname] [frequency]		Urg(N)/Alr(M)/Resp(W/U)

uM119TimeUnitNameFrequency			[119] TimeUnitNameFrequency,

--	MONITOR [unitname] [frequency]			Urg(N)/Alr(M)/Resp(W/U)

uM120UnitNameFrequency				[120] UnitNameFrequency,

--	AT [position] MONITOR [unitname] [frequency]	Urg(N)/Alr(M)/Resp(W/U)

uM121PositionUnitNameFrequency			[121] PositionUnitNameFrequency,

--	AT [time] MONITOR [unitname] [frequency]		Urg(N)/Alr(M)/Resp(W/U)

uM122TimeUnitNameFrequency 			[122] TimeUnitNameFrequency,

--	SQUAWK [code]					Urg(N)/Alr(M)/Resp(W/U)

uM123Code 						[123] Code,

--	STOP SQUAWK					Urg(N)/Alr(M)/Resp(W/U)

uM124NULL 						[124] NULL,

--	SQUAWK MODE CHARLIE				Urg(N)/Alr(M)/Resp(W/U)

uM125NULL						[125] NULL,

--	STOP SQUAWK MODE CHARLIE			Urg(N)/Alr(M)/Resp(W/U)

uM126NULL						[126] NULL,

--	REPORT BACK ON ROUTE				Urg(N)/Alr(M)/Resp(W/U)

uM127NULL 						[127] NULL,

--	REPORT LEAVING [level]				Urg(N)/Alr(M)/Resp(W/U)

uM128Level 						[128] Level,

--	REPORT MAINTAINING [level]			Urg(N)/Alr(M)/Resp(W/U)

uM129Level						[129] Level,

--	REPORT PASSING [position]				Urg(N)/Alr(M)/Resp(W/U)

uM130Position 					[130] Position,

--	REPORT REMAINING FUEL AND PERSONS ON BOARD

--								Urg(N)/Alr(M)/Resp(Y)

uM131NULL 						[131] NULL,

--	REPORT POSITION					Urg(N)/Alr(M)/Resp(Y)

uM132NULL						[132] NULL,

--	REPORT PRESENT LEVEL				Urg(N)/Alr(M)/Resp(Y)

uM133NULL						[133] NULL,

--	REPORT [speedtype] [speedtype] [speedtype]SPEED	Urg(N)/Alr(M)/Resp(Y)

uM134SpeedTypeSpeedTypeSpeedType		[134] SpeedTypeSpeedTypeSpeedType,

--	CONFIRM ASSIGNED LEVEL			Urg(N)/Alr(M)/Resp(Y)

uM135NULL 						[135] NULL,

--	CONFIRM ASSIGNED SPEED			Urg(N)/Alr(M)/Resp(Y)

uM136NULL 						[136] NULL,

--	CONFIRM ASSIGNED ROUTE			Urg(N)/Alr(M)/Resp(Y)

uM137NULL 						[137] NULL,

--	CONFIRM TIME OVER REPORTED WAYPOINT 	Urg(N)/Alr(M)/Resp(Y)

uM138NULL 						[138] NULL,

--	CONFIRM REPORTED WAYPOINT			Urg(N)/Alr(M)/Resp(Y)

uM139NULL 						[139] NULL,

--	CONFIRM NEXT WAYPOINT			Urg(N)/Alr(M)/Resp(Y)

uM140NULL 						[140] NULL,

--	CONFIRM NEXT WAYPOINT ETA			Urg(N)/Alr(M)/Resp(Y)

uM141NULL 						[141] NULL,

--	CONFIRM ENSUING WAYPOINT			Urg(N)/Alr(M)/Resp(Y)

uM142NULL 						[142] NULL,

--	CONFIRM REQUEST					Urg(N)/Alr(M)/Resp(Y)

uM143NULL 						[143] NULL,

--	CONFIRM SQUAWK					Urg(N)/Alr(M)/Resp(Y)

uM144NULL 						[144] NULL,

--	REPORT HEADING					Urg(N)/Alr(M)/Resp(Y)

uM145NULL 						[145] NULL,

--	REPORT GROUND TRACK				Urg(N)/Alr(M)/Resp(Y)

uM146NULL 						[146] NULL,

--	REQUEST POSITION REPORT			Urg(N)/Alr(M)/Resp(Y )

uM147NULL 						[147] NULL,

--	WHEN CAN YOU ACCEPT [level]			Urg(N)/Alr(M)/Resp(Y)

uM148Level 						[148] Level,

--	CAN YOU ACCEPT [level] AT [position]		Urg(N)/Alr(M)/Resp(A/N)

uM149LevelPosition 					[149] LevelPosition,

--	CAN YOU ACCEPT [level] AT [time]			Urg(N)/Alr(M)/Resp(A/N)

uM150LevelTime 					[150] LevelTime,

--	WHEN CAN YOU ACCEPT [speed]			Urg(N)/Alr(M)/Resp(Y)

uM151Speed 						[151] Speed,

--	WHEN CAN YOU ACCEPT [distanceSpecifiedspecifiedDistance] [direction] OFFSET

--								Urg(N)/Alr(M)/Resp(Y)

uM152DistanceSpecifiedDirection			[152] DistanceSpecifiedDirection,

--	ALTIMETER [altimeter]				Urg(N)/Alr(M)/Resp(R)

uM153Altimeter 					[153] Altimeter,

--	RADAR SERVICE TERMINATED			Urg(N)/Alr(M)/Resp(R)

uM154NULL						[154] NULL,

--	RADAR CONTACT [position]				Urg(N)/Alr(M)/Resp(R)

uM155Position 						[155] Position,

--	RADAR CONTACT LOST				Urg(N)/Alr(M)/Resp(R)

uM156NULL 						[156] NULL,

--	CHECK STUCK MICROPHONE [frequency]		Urg(U)/Alr(M)/Resp(N)

uM157Frequency 					[157] Frequency,

--	ATIS [atiscode]						Urg(N)/Alr(M)/Resp(R)

uM158AtisCode 					[158] ATISCode,

--	ERROR [errorInformation]				Urg(U)/Alr(M)/Resp(N)

uM159ErrorInformation 				[159] ErrorInformation,

--	NEXT DATA AUTHORITY [facility] 			Urg(L)/Alr(N)/Resp(N)

uM160Facility 						[160] Facility,

--	END SERVICE					Urg(L)/Alr(N)/Resp(N)

uM161NULL 						[161] NULL,

--	SERVICE UNAVAILABLE				Urg(L)/Alr(L)/Resp(N )

uM162NULL 						[162] NULL,

--	[facilitydesignation]					Urg(L)/Alr(N)/Resp(N)

uM163FacilityDesignation				[163] FacilityDesignation,

--	WHEN READY					Urg(L)/Alr(N)/Resp(N)

uM164NULL 						[164] NULL,

--	THEN							Urg(L)/Alr(N)/Resp(N)

uM165NULL 						[165] NULL,

--	DUE TO [traffictype]TRAFFIC				Urg(L)/Alr(N)/Resp(N)

uM166TrafficType 					[166] TrafficType,

--	DUE TO AIRSPACE RESTRICTION			Urg(L)/Alr(N)/Resp(N)

uM167NULL 						[167] NULL,

--	DISREGARD						Urg(N)/Alr(M)/Resp(R)

uM168NULL 						[168] NULL,

--	[freetext]						Urg(N)/Alr(L)/Resp(R)

uM169FreeText					[169] FreeText,

--	[freetext]						Urg(D)/Alr(H)/Resp(R)

uM170FreeText 					[170] FreeText,

--	CLIMB AT [verticalRate] MINIMUM			Urg(N)/Alr(M)/Resp(W/U)

uM171VerticalRate					[171] VerticalRate,

--	CLIMB AT [verticalRate] MAXIMUM			Urg(N)/Alr(M)/Resp(W/U)

uM172VerticalRate					[172] VerticalRate,

--	DESCEND AT [verticalRate] MINIMUM		Urg(N)/Alr(M)/Resp(W/U)

uM173VerticalRate					[173] VerticalRate,

--	DESCEND AT [verticalRate] MAXIMUM		Urg(N)/Alr(M)/Resp(W/U)

uM174VerticalRate					[174] VerticalRate,

--	REPORT REACHING [level]				Urg(N)/Alr(M)/Resp(RW/U)

uM175Level						[175] Level,

--	MAINTAIN OWN SEPARATION AND VMC	Urg(N)/Alr(M)/Resp(W/U)

uM176NULL						[176] NULL,

--	AT PILOTS DISCRETION				Urg(L)/Alr(L)/Resp(N)

uM177NULL						[177] NULL,

--	Reserved						Urg(L)/Alr(L)/Resp(Y)

uM178NULL						[178] NULL,

--	SQUAWK IDENT					Urg(N)/Alr(M)/Resp(W/U)

uM179NULL						[179] NULL,

--	REPORT REACHING BLOCK [level] TO [level]	Urg(N)/Alr(M)/Resp(W/U)

uM180LevelLevel					[180] LevelLevel,

--	REPORT DISTANCE [tofrom] [position]		Urg(N)/Alr(M)/Resp(Y)

uM181ToFromPosition					[181] ToFromPosition,

--	CONFIRM ATIS CODE				Urg(N)/Alr(M)/Resp(Y)

uM182NULL						[182] NULL,

--	[freetext]						Urg(N)/Alr(M)/Resp(N)

uM183FreeText					[183] FreeText,

--	AT [time] REPORT DISTANCE [tofrom] [position]	Urg(N)/Alr(M)/Resp(Y)

uM184TimeToFromPosition				[184] TimeToFromPosition,

--	AFTER PASSING [position] CLIMB TO [level]		Urg(N)/Alr(M)/Resp(W/U)

uM185PositionLevel					[185] PositionLevel,

--	AFTER PASSING [position] DESCEND TO [level]	Urg(N)/Alr(M)/Resp(W/U)

uM186PositionLevel					[186] PositionLevel,

--	[freetext]						Urg(L)/Alr(N)/Resp(N)

uM187FreeText					[187] FreeText,

--	AFTER PASSING [position] MAINTAIN [speed]	Urg(N)/Alr(M)/Resp(W/U)

uM188PositionSpeed					[188] PositionSpeed,

--	ADJUST SPEED TO [speed]				Urg(N)/Alr(M)/Resp(W/U)

uM189Speed						[189] Speed,

--	FLY HEADING [degrees]				Urg(N)/Alr(M)/Resp(W/U)

uM190Degrees						[190] Degrees,

--	ALL ATS TERMINATED				Urg(N)/Alr(M)/Resp(R)

uM191NULL						[191] NULL,

--	REACH [level] BY [time]				Urg(N)/Alr(M)/Resp(W/U)

uM192LevelTime					[192] LevelTime,

--	IDENTIFICATION LOST				Urg(N)/Alr(M)/Resp(R)

uM193NULL						[193] NULL,

--	[freetext]						Urg(N)/Alr(L)/Resp(Y)

uM194FreeText					[194] FreeText,

--	[freetext]						Urg(L)/Alr(L)/Resp(R)

uM195FreeText					[195] FreeText,

--	[freetext]						Urg(N)/Alr(M)/Resp(W/U)

uM196FreeText					[196] FreeText,

--	[freetext]						Urg(U)/Alr(M)/Resp(W/U)

uM197FreeText					[197] FreeText,

--	[freetext]						Urg(D)/Alr(H)/Resp(W/U)

uM198FreeText					[198] FreeText,

--	[freetext]						Urg(N)/Alr(L)/Resp(N)

uM199FreeText					[199] FreeText,

--	REPORT REACHING					Urg(N)/Alr(M)/Resp(R)

uM200NULL						[200] NULL,

--	Not Used						Urg(L)/Alr(L)/Resp(N)

uM201NULL						[201] NULL,

--	Not Used						Urg(L)/Alr(L)/Resp(N)

uM202NULL						[202] NULL,

--	[freetext]						Urg(N)/Alr(M)/Resp(R)

uM203FreeText					[203] FreeText,

--	[freetext]						Urg(N)/Alr(M)/Resp(Y)

uM204FreeText					[204] FreeText,

--	[freetext]						Urg(N)/Alr(M)/Resp(A/N)

uM205FreeText					[205] FreeText,

--	[freetext]						Urg(L)/Alr(N)/Resp(Y)

uM206FreeText					[206] FreeText,

--	[freetext]						Urg(L)/Alr(L)/Resp(Y)

uM207FreeText					[207] FreeText,

--	[freetext]						Urg(L)/Alr(L)/Resp(N)

uM208FreeText					[208] FreeText,

--	REACH [level] BY [position]				Urg(N)/Alr(M)/Resp(W/U)

uM209LevelPosition					[209] LevelPosition,

--	IDENTIFIED [position]				Urg(N)/Alr(M)/Resp(R)

uM210Position						[210] Position,

--	REQUEST FORWARDED				Urg(N)/Alr(L)/Resp(N)

uM211NULL						[211] NULL,

--	[facilitydesignation] ATIS [atiscode] CURRENT	Urg(N)/Alr(M)/Resp(R)

uM212FacilityDesignationATISCode			[212] FacilityDesignationATISCode,

--	[facilitydesignation] ALTIMETER [altimeter]		Urg(N)/Alr(M)/Resp(R)

uM213FacilityDesignationAltimeter			[213] FacilityDesignationAltimeter,

--	RVR RUNWAY [runway] VISUAL RANGE [rvr]	Urg(N)/Alr(M)/Resp(R)

uM214RunwayRVR					[214] RunwayRVR,

--	TURN [direction][degrees][direction]				Urg(N)/Alr(M)/Resp(W/U)

uM215 DirectionDegreesDirection				[215] DirectionDegrees,

--	REQUEST FLIGHT PLAN				Urg(N)/Alr(M)/Resp(Y)

uM216NULL						[216] NULL,

--	REPORT ARRIVAL					Urg(N)/Alr(M)/Resp(Y)

uM217NULL						[217] NULL,

--	REQUEST ALREADY RECEIVED			Urg(L)/Alr(N)/Resp(N)

uM218NULL						[218] NULL,

--	STOP CLIMB AT [level]				Urg(U)/Alr(M)/Resp(W/U)

uM219Level						[219] Level,

--	STOP DESCENT AT [level]				Urg(U)/Alr(M)/Resp(W/U)

uM220Level						[220] Level,

--	STOP TURN HEADING [degrees]			Urg(U)/Alr(M)/Resp(W/U)

uM221Degrees						[221] Degrees,

--	NO SPEED RESTRICTION				Urg(L)/Alr(L)/Resp(R)

uM222NULL						[222] NULL,

--	REDUCE TO MINIMUM APPROACH SPEED	Urg(N)/Alr(M)/Resp(W/U)

uM223NULL						[223] NULL,

--	NO DELAY EXPECTED				Urg(N)/Alr(L)/Resp(R)

uM224NULL						[224] NULL,

--	DELAY NOT DETERMINED				Urg(N)/Alr(L)/Resp(R)

uM225NULL						[225] NULL,

--	EXPECTED APPROACH TIME [time]			Urg(N)/Alr(L)/Resp(R)

uM226Time						[226] Time,

--	LOGICAL ACKNOWLEDGMENT			Urg(N)/Alr(M)/Resp(N)

uM227NULL						[227] NULL,

--	REPORT ETA [position]				Urg(L)/Alr(L)/Resp(Y)

uM228Position						[228] Position,

--	REPORT ALTERNATE AERODROME		Urg(L)/Alr(L)/Resp(Y)

uM229NULL						[229] NULL,

--	IMMEDIATELY					Urg(D)/Alr(H)/Resp(N)

uM230NULL						[230] NULL,

--	STATE PREFERRED LEVEL				Urg(L)/Alr(L)/Resp(Y)

uM231NULL						[231] NULL,

--	STATE-TOP-OF-DESCENT				Urg(L)/Alr(L)/Resp(Y)

uM232NULL						[232] NULL,

--	USE OF LOGICAL ACKNOWLEDGMENT PROHIBITED

--								Urg(N)/Alr(M)/Resp(N)

uM233NULL						[233] NULL,

--	FLIGHT PLAN NOT HELD				Urg(L)/Alr(L)/Resp(N)

uM234NULL						[234] NULL,

--	ROGER 7500						Urg(U)/Alr(H)/Resp(N)

uM235NULL						[235] NULL,

--	LEAVE CONTROLLED AIRSPACE			Urg(N)/Alr(M)/Resp(W/U)

uM236NULL						[236] NULL,

...

}

------------------------------------------------------------------------------------

-- Downlink message element

-- ----------------------------------------------------------------------------------

ATCDownlinkMsgElementId ::= CHOICE

{

--	WILCO						Urg(N)/Alr(M)/Resp(N)

dM0NULL 						[0] NULL,

--	UNABLE						Urg(N)/Alr(M)/Resp(N)

dM1NULL 						[1] NULL,

--	STANDBY						Urg(N)/Alr(M)/Resp(N)

dM2NULL 						[2] NULL,

--	ROGER						Urg(N)/Alr(M)/Resp(N)

dM3NULL 						[3] NULL,

--	AFFIRM						Urg(N)/Alr(M)/Resp(N)

dM4NULL 						[4] NULL,

--	NEGATIVE						Urg(N)/Alr(M)/Resp(N)

dM5NULL 						[5] NULL,

--	REQUEST [level]					Urg(N)/Alr(L)/Resp(Y)

dM6Level 						[6] Level,

--	REQUEST BLOCK [level] TO [level]			Urg(N)/Alr(L)/Resp(Y)

dM7LevelLevel 					[7] LevelLevel,

--	REQUEST CRUISE CLIMB TO [level]			Urg(N)/Alr(L)/Resp(Y)

dM8Level 						[8] Level,

--	REQUEST CLIMB TO [level]				Urg(N)/Alr(L)/Resp(Y)

dM9Level						[9] Level,

--	REQUEST DESCENT TO [level]			Urg(N)/Alr(L)/Resp(Y)

dM10Level 						[10] Level,

--	AT [position] REQUEST CLIMB TO [level] 		Urg(N)/Alr(L)/Resp(Y)

dM11PositionLevel 					[11] PositionLevel,

--	AT [position] REQUEST DESCENT TO [level]		Urg(N)/Alr(L)/Resp(Y)

dM12PositionLevel 					[12] PositionLevel,

--	AT [time] REQUEST CLIMB TO [level]		Urg(N)/Alr(L)/Resp(Y)

dM13TimeLevel 					[13] TimeLevel,

--	AT [time] REQUEST DESCENT TO [level]		Urg(N)/Alr(L)/Resp(Y)

dM14TimeLevel					[14] TimeLevel,

--	REQUEST OFFSET [distanceSpecifiedspecifiedDistance] [direction] OF ROUTE

--								Urg(N)/Alr(L)/Resp(Y)

dM15DistanceSpecifiedDirection			[15] DistanceSpecifiedDirection,

--	AT [position] REQUEST OFFSET [distanceSpecifiedspecifiedDistance] [direction] OF ROUTE

--								Urg(N)/Alr(L)/Resp(Y)

dM16PositionDistanceSpecifiedDirection						[16]

PositionDistanceSpecifiedDirection,

--	AT [time] REQUEST OFFSET [distanceSpecifiedspecifiedDistance] [direction] OF ROUTE

--								Urg(N)/Alr(L)/Resp(Y)

dM17TimeDistanceSpecifiedDirection			[17] TimeDistanceSpecifiedDirection,

--	REQUEST [speed]					Urg(N)/Alr(L)/Resp(Y)

dM18Speed 						[18] Speed,

--	REQUEST [speed] TO [speed]				Urg(N)/Alr(L)/Resp(Y)

dM19SpeedSpeed 					[19] SpeedSpeed,

--	REQUEST VOICE CONTACT				Urg(N)/Alr(L)/Resp(Y)

dM20NULL 						[20] NULL,

--	REQUEST VOICE CONTACT [frequency]		Urg(N)/Alr(L)/Resp(Y)

dM21Frequency 					[21] Frequency,

--	REQUEST DIRECT TO [position]			Urg(N)/Alr(L)/Resp(Y)

dM22Position 						[22] Position,

--	REQUEST [procedureName]				Urg(N)/Alr(L)/Resp(Y)

dM23ProcedureName 					[23] ProcedureName,

--	REQUEST [routeClearance]				Urg(N)/Alr(L)/Resp(Y)

dM24RouteClearance 					[24] RouteClearance,

--	REQUEST [clearanceType] CLEARANCE		Urg(N)/Alr(L)/Resp(Y)

dM25ClearanceType 					[25] ClearanceType,

--	REQUEST WEATHER DEVIATION TO  [position] VIA [routeClearance]

--								Urg(N)/Alr(L)/Resp(Y)

dM26PositionRouteClearance 				[26] PositionRouteClearance,

--	REQUEST WEATHER DEVIATION UP  TO [distanceSpecifiedspecifiedDistance] [direction]

OF ROUTE

--								Urg(N)/Alr(L)/Resp(Y)

dM27DistanceSpecifiedDirection			[27] DistanceSpecifiedDirection,

--	LEAVING [level]					Urg(N)/Alr(L)/Resp(N)

dM28Level 						[28] Level,

--	CLIMBING TO [level]					Urg(N)/Alr(M)/Resp(N)

dM29Level 						[29]Level,

--	DESCENDING TO [level]				Urg(N)/Alr(M)/Resp(N)

dM30Level 						[30] Level,

--	PASSING [position]					Urg(N)/Alr(M)/Resp(N)

dM31Position 						[31] Position,

--	PRESENT LEVEL [level]				Urg(N)/Alr(M)/Resp(N)

dM32Level 						[32] Level,

--	PRESENT POSITION [position]			Urg(N)/Alr(M)/Resp(N)

dM33Position 						[33] Position,

--	PRESENT SPEED [speed]				Urg(N)/Alr(M)/Resp(N)

dM34Speed 						[34] Speed,

--	PRESENT HEADING [degrees]			Urg(N)/Alr(M)/Resp(N)

dM35Degrees 						[35] Degrees,

--	PRESENT GROUND TRACK [degrees]		Urg(N)/Alr(M)/Resp(N)

dM36Degrees 						[36] Degrees,

--	MAINTAINING [level]					Urg(N)/Alr(M)/Resp(N)

dM37Level 						[37] Level,

--	ASSIGNED LEVEL [level]				Urg(N)/Alr(M)/Resp(N)

dM38Level 						[38] Level,

--	ASSIGNED SPEED [speed]				Urg(N)/Alr(M)/Resp(N)

dM39Speed 						[39] Speed,

--	ASSIGNED ROUTE [routeClearance]			Urg(N)/Alr(M)/Resp(N)

dM40RouteClearance 					[40] RouteClearance,

--	BACK ON ROUTE					Urg(N)/Alr(M)/Resp(N)

dM41NULL 						[41] NULL,

--	NEXT WAYPOINT [position]				Urg(N)/Alr(M)/Resp(N)

dM42Position 						[42] Position,

--	NEXT WAYPOINT ETA [time]			Urg(N)/Alr(M)/Resp(N)

dM43Time 						[43] Time,

--	ENSUING WAYPOINT [position]			Urg(N)/Alr(M)/Resp(N)

dM44Position 						[44] Position,

--	REPORTED WAYPOINT [position]			Urg(N)/Alr(M)/Resp(N)

dM45Position 						[45] Position,

--	REPORTED WAYPOINT [time]			Urg(N)/Alr(M)/Resp(N)

dM46Time 						[46] Time,

--	SQUAWKING [code]					Urg(N)/Alr(M)/Resp(N)

dM47Code 						[47] Code,

--	POSITION REPORT [positionreport]			Urg(N)/Alr(M)/Resp(N)

dM48PositionReport 					[48] PositionReport,

--	WHEN CAN WE EXPECT [speed]			Urg(L)/Alr(L)/Resp(Y)

dM49Speed 						[49] Speed,

--	WHEN CAN WE EXPECT [speed] TO [speed]		Urg(L)/Alr(L)/Resp(Y)

dM50SpeedSpeed 					[50] SpeedSpeed,

--	WHEN CAN WE EXPECT BACK ON ROUTE	Urg(L)/Alr(L)/Resp(Y)

dM51NULL 						[51] NULL,

--	WHEN CAN WE EXPECT LOWER LEVEL		Urg(L)/Alr(L)/Resp(Y)

dM52NULL 						[52] NULL,

--	WHEN CAN WE EXPECT HIGHER LEVEL		Urg(L)/Alr(L)/Resp(Y)

dM53NULL 						[53] NULL,

--	WHEN CAN WE EXPECT CRUISE CLIMB TO [level]

--								Urg(L)/Alr(L)/Resp(Y)

dM54Level 						[54] Level,

--	PAN PAN PAN					Urg(U)/Alr(H)/Resp(Y)

dM55NULL 						[55] NULL,

--	MAYDAY MAYDAY MAYDAY			Urg(D)/Alr(H)/Resp(Y)

dM56NULL 						[56] NULL,

--	[remainingFuel] OF FUEL REMAINING AND [personsonboard] PERSONS ON BOARD

--								Urg(U)/Alr(H)/Resp(N)

dM57RemainingFuelPersonsOnBoard 			[57] RemainingFuelPersonsOnBoard,

--	CANCEL EMERGENCY				Urg(U)/Alr(M)/Resp(Y)

dM58NULL 						[58] NULL,

--	DIVERTING TO [position] VIA [routeClearance]	Urg(U)/Alr(H)/Resp(Y)

dM59PositionRouteClearance 				[59] PositionRouteClearance,

--	OFFSETTING [distanceSpecifiedspecifiedDistance] [direction] OF ROUTE

Urg(U)/Alr(H)/Resp(Y)

dM60DistanceSpecifiedDirection 			[60] DistanceSpecifiedDirection,

--	DESCENDING TO [level]				Urg(U)/Alr(H)/Resp(Y)

dM61Level 						[61] Level,

--	ERROR [errorInformation]				Urg(U)/Alr(L)/Resp(N)

dM62ErrorInformation 					[62] ErrorInformation,

--	NOT CURRENT DATA AUTHORITY			Urg(L)/Alr(L)/Resp(N)

dM63NULL 						[63] NULL,

--	[facilitydesignation]					Urg(L)/Alr(L)/Resp(N)

dM64FacilityDesignation 				[64] FacilityDesignation,

--	DUE TO WEATHER					Urg(L)/Alr(L)/Resp(N)

dM65NULL 						[65] NULL,

--	DUE TO AIRCRAFT PERFORMANCE		Urg(L)/Alr(L)/Resp(N)

dM66NULL 						[66] NULL,

--	[freetext]						Urg(N)/Alr(L)/Resp(N)

dM67FreeText 						[67] FreeText,

--	[freetext]						Urg(D)/Alr(H)/Resp(Y)

dM68FreeText 						[68] FreeText,

--	REQUEST VMC DESCENT				Urg(N)/Alr(L)/Resp(Y)

dM69NULL						[69] NULL,

--	REQUEST HEADING [degrees]			Urg(N)/Alr(L)/Resp(Y)

dM70Degrees 						[70] Degrees,

--	REQUEST GROUND TRACK [degrees]		Urg(N)/Alr(L)/Resp(Y)

dM71Degrees						[71] Degrees,

--	REACHING [level]					Urg(N)/Alr(M)/Resp(N)

dM72Level						[72] Level,

--	[versionnumber]					Urg(L)/Alr(L)/Resp(N)

dM73Versionnumber					[73] VersionNumber,

--	REQUEST TO MAINTAIN OWN SEPARATION AND VMC

--								Urg(L)/Alr(L)/Resp(Y)

dM74NULL						[74] NULL,

--	AT PILOTS DISCRETION				Urg(L)/Alr(L)/Resp(N)

dM75NULL						[75] NULL,

--	REACHING BLOCK [level] TO [level]			Urg(N)/Alr(N)/Resp(N)

dM76LevelLevel					[76] LevelLevel,

--	ASSIGNED BLOCK [level] TO [level]			Urg(N)/Alr(N)/Resp(N)

dM77LevelLevel					[77] LevelLevel,

--	AT [time] [distance] [tofrom] [position]			Urg(N)/Alr(N)/Resp(N)

dM78TimeDistanceToFromPosition			[78] TimeDistanceToFromPosition,

--	ATIS [atiscode]						Urg(N)/Alr(M)/Resp(N)

dM79AtisCode						[79] ATISCode,

--	DEVIATING [distanceSpecifiedspecifiedDistance] [direction] OF ROUTE

Urg(N)/Alr(M)/Resp(Y)

dM80DistanceSpecifiedDirection			[80] DistanceSpecifiedDirection,

--	WE CAN ACCEPT [level] AT [time]			Urg(N)/Alr(L)/Resp(N)

dM81LevelTime					[81] LevelTime,

--	WE CANNNOT ACCEPT [level]			Urg(N)/Alr(L)/Resp(N)

dM82Level						[82] Level,

--	WE CAN ACCEPT [speed] AT [time]			Urg(N)/Alr(L)/Resp(N)

dM83SpeedTime					[83] SpeedTime,

--	WE CANNNOT ACCEPT [speed]			Urg(N)/Alr(L)/Resp(N)

dM84Speed						[84] Speed,

--	WE CAN ACCEPT [distanceSpecifiedspecifiedDistance] [direction] AT [time]

--								Urg(N)/Alr(L)/Resp(N)

dM85DistanceSpecifiedDirectionTime			[85] DistanceSpecifiedDirectionTime,

--	WE CANNNOT ACCEPT [distanceSpecifiedspecifiedDistance] [direction]

Urg(N)/Alr(L)/Resp(N)

dM86DistanceSpecifiedDirection				[86]

DistanceSpecifiedDirection,

--	WHEN CAN WE EXPECT CLIMB TO [level]		Urg(L)/Alr(L)/Resp(Y)

dM87Level						[87] Level,

--	WHEN CAN WE EXPECT DESCENT TO [level]	Urg(L)/Alr(L)/Resp(Y)

dM88Level						[88] Level,

--	MONITORING [unitname] [frequency]			Urg(L)/Alr(M)/Resp(N)

dM89UnitnameFrequency				[89] UnitNameFrequency,

--	[freetext]						Urg(N)/Alr(M)/Resp(N)

dM90FreeText 						[90] FreeText,

--	[freetext]						Urg(N)/Alr(L)/Resp(Y)

dM91FreeText 						[91] FreeText,

--	[freetext]						Urg(L)/Alr(L)/Resp(Y)

dM92FreeText 						[92] FreeText,

--	[freetext]						Urg(U)/Alr(H)/Resp(N)

dM93FreeText 						[93] FreeText,

--	[freetext]						Urg(D)/Alr(H)/Resp(N)

dM94FreeText 						[94] FreeText,

--	[freetext]						Urg(U)/Alr(M)/Resp(N)

dM95FreeText 						[95] FreeText,

--	[freetext]						Urg(U)/Alr(L)/Resp(N)

dM96FreeText 						[96] FreeText,

--	[freetext]						Urg(L)/Alr(L)/Resp(N)

dM97FreeText 						[97] FreeText,

--	[freetext]						Urg(N)/Alr(N)/Resp(N)

dM98FreeText 						[98] FreeText,

--	CURRENT DATA AUTHORITY			Urg(L)/Alr(L)/Resp(N)

dM99NULL 						[99] NULL,

--	LOGICAL ACKNOWLEDGMENT			Urg(N)/Alr(M)/Resp(N)

dM100NULL 						[100] NULL,

--	REQUEST END OF SERVICE				Urg(L)/Alr(L)/Resp(Y)

dM101NULL 						[101] NULL,

--	LANDING REPORT					Urg(N)/Alr(N)/Resp(N)

dM102NULL 						[102] NULL,

--	CANCELLING IFR					Urg(N)/Alr(N)/Resp(Y)

dM103NULL 						[103] NULL,

--	ETA[position][time]					Urg(L)/Alr(L)/Resp(N)

dM104PositionTime 					[104] PositionTime,

--	ALTERNATE AERODROME[airport]			Urg(L)/Alr(L)/Resp(N)

dM105Airport 						[105] Airport,

--	PREFERRED LEVEL[level]				Urg(L)/Alr(L)/Resp(N)

dM106Level 						[106] Level,

--	NOT AUTHORIZED NEXT DATA AUTHORITY	Urg(L)/Alr(L)/Resp(N)

dM107NULL						[107] NULL,

--	DE-ICING COMPLETE				Urg(L)/Alr(L)/Resp(N)

dM108NULL 						[108] NULL,

--	TOP OF DESCENT [time]				Urg(L)/Alr(L)/Resp(N)

dM109Time						[109] Time,

--	TOP OF DESCENT [position]				Urg(L)/Alr(L)/Resp(N)

dM110Position						[110] Position,

--	TOP OF DESCENT [time] [position]			Urg(L)/Alr(L)/Resp(N)

dM111TimePosition					[111] TimePosition,

--	SQUAWKING 7500					Urg(U)/Alr(H)/Resp(N)

dM112NULL						[112] NULL,

--	[speedType] [speedType] [speedType] SPEED [speed]	Urg(N)/Alr(L)/Resp(N)

dM113SpeedTypeSpeedTypeSpeedTypeSpeed		[113]

SpeedTypeSpeedTypeSpeedTypeSpeed,

...

}

AircraftAddress ::= BIT STRING (SIZE(24))

AircraftFlightIdentification ::= IA5String (SIZE (2..8))

Airport ::= IA5String (SIZE (4))

Altimeter ::=  CHOICE

{

altimeterEnglish	[0]	AltimeterEnglish,

altimeterMetric		[1]	AltimeterMetric

}

AltimeterEnglish ::= INTEGER (2200..3200)

-- unit = Inches Mercury, Range (22.00 .. 32.00), resolution = 0.01

AltimeterMetric ::= INTEGER  (7500..12500)

-- unit = Hectopascal, Range (750.0..1250.0), resolution = 0.1

ATISCode ::= IA5String (SIZE (1))

ATSRouteDesignator ::= IA5String (SIZE (2..7))

ATWAlongTrackWaypoint ::= SEQUENCE

{

position 		[0]	Position,

aTWDistance 		[1]	ATWDistance,

speed 			[2]	Speed				OPTIONAL,

aTWLevels		[3]	ATWLevelSequence		OPTIONAL

}

ATWLevel ::= SEQUENCE

{

atw		ATWLevelTolerance,

level		Level

}

ATWLevelSequence ::= SEQUENCE SIZE (1..2) OF ATWLevel

ATWLevelTolerance ::= ENUMERATED

{

at		(0),

atorabove	(1),

atorbelow	(2)

}

ATWDistance ::= SEQUENCE

{

atwDistanceTolerance		ATWDistanceTolerance,

distance			Distance

}

ATWDistanceTolerance ::= ENUMERATED

{

plus		(0),

minus		(1)

}

ClearanceType ::= ENUMERATED

{

noneSpecified	(0),

approach	(1),

departure	(2),

further		(3),

start-up		(4),

pushback	(5),

taxi		(6),

take-off	(7),

landing		(8),

oceanic		(9),

en-route	(10),

downstream	(11),

...

}

Code ::=  SEQUENCE SIZE (4) OF CodeOctalDigit

CodeOctalDigit ::= INTEGER (0..7)

ControlledTime ::= SEQUENCE

{

time		Time,

timeTolerance	TimeTolerance

}

Date ::= SEQUENCE

{

year		Year,

month		Month,

day		Day

}

DateTimeGroup ::= SEQUENCE

{

date		Date,

timehhmmss	Timehhmmss

}

Day ::= INTEGER (1..31)

--unit = Day, Range (1..31), resolution = 1

DegreeIncrement ::= INTEGER (1..20)

--unit = Degree, Range (1..20), resolution = 1

Degrees ::=  CHOICE

{

degreesMagnetic	[0]	DegreesMagnetic,

degreesTrue 		[1]	DegreesTrue

}

DegreesMagnetic ::= INTEGER (1..360)

--unit = degree, Range (1..360), resolution = 1

DegreesTrue ::= INTEGER (1..360)

--unit = degree, Range (1..360), resolution = 1

DepartureClearance ::= SEQUENCE

{

aircraftFlightIdentification	[0]	AircraftFlightIdentification,

clearanceLimit			[1]	Position,

flightInformation		[2]	FlightInformation	OPTIONAL,

furtherInstructions		[3]	FurtherInstructions	OPTIONAL

}

DepartureMinimumInterval ::= INTEGER (1..150)

--unit = Minute, Range (0.1..15.0), resolution = 0.1

Direction ::= ENUMERATED

{

left			(0),

right			(1),

eitherSide		(2),

north			(3),

south			(4),

east			(5),

west			(6),

northEast  		(7),

northWest		(8),

southEast 		(9),

southWest		(10)

}

DirectionDegrees ::=  SEQUENCE

{

direction		Direction,

degrees		Degrees

}

Distance ::= CHOICE

{

distanceNm 	[0]	DistanceNm,

distanceKm 	[1]	DistanceKm

}

DistanceKm ::= INTEGER (0..8000)

--	unit = Kilometer, Range (0..2000), resolution = 0.25

DistanceNm ::= INTEGER (0..9999)

--	unit = Nautical Mile, Range (0..999.9), resolution = 0.1

DistanceSpecified ::= CHOICE

{

distanceSpecifiedNm	[0]	DistanceSpecifiedNm,

distanceSpecifiedKm 	[1]	DistanceSpecifiedKm

}

DistanceSpecifiedDirection ::=  SEQUENCE

{

distanceSpecified		DistanceSpecified,

direction		Direction

}

DistanceSpecifiedDirectionTime ::=  SEQUENCE

{

distanceSpecifiedDirection	DistanceSpecifiedDirection,

time				Time

}

DistanceSpecifiedKm ::= INTEGER (1..500)

--	unit = Kilometer, Range (1..500), resolution = 1

DistanceSpecifiedNm ::= INTEGER (1..250)

--	unit = Nautical Mile, Range (1..250), resolution = 1

ErrorInformation ::=  ENUMERATED

{

unrecognizedMsgReferenceNumber		(0),

logicalAcknowledgmentNotAccepted		(1),

insufficientResources				(2),

invalidMessageElementCombination		(3),

invalidMessageElement				(4),

...

}

Facility ::= CHOICE

{

noFacility		[0]	NULL,

facilityDesignation	[1]	FacilityDesignation

}

FacilityDesignation ::=  IA5String (SIZE (4..8))

FacilityFunction ::= ENUMERATED

{

center			(0),

approach 		(1),

tower			(2),

final			(3),

groundControl		(4),

clearanceDelivery	(5),

departure		(6),

control			(7),

radio			(8),

...

}

FacilityDesignationAltimeter ::= SEQUENCE

{

facilityDesignation		FacilityDesignation,

altimeter			Altimeter

}

FacilityDesignationATISCode ::= SEQUENCE

{

facilityDesignation		FacilityDesignation,

aTISCode			ATISCode

}

FacilityIdentification ::=  CHOICE

{

facilityDesignation	[0]	FacilityDesignation,

facilityName		[1]	FacilityName

}

FacilityName ::= IA5String (SIZE (3..18))

Fix ::= IA5String (SIZE (1..5))

FixName ::=  SEQUENCE

{

name			[0]	Fix,

latlon			[1]	LatitudeLongitude	OPTIONAL

}

FlightInformation ::= CHOICE

{

routeOfFlight		[0]	RouteInformation,

levelsOfFlight		[1]	LevelsOfFlight,

routeAndLevels	[2]	RouteAndLevels

}

FreeText ::=  IA5String (SIZE (1..256))

Frequency ::=  CHOICE

{

frequencyhf		[0]	Frequencyhf,

frequencyvhf		[1]	Frequencyvhf,

frequencyuhf		[2]	Frequencyuhf,

frequencysatchannel	[3]	Frequencysatchannel

}

Frequencyhf ::= INTEGER (2850..28000)

--	unit = Kilohertz, Range (2850..28000), resolution = 1

Frequencysatchannel ::= NumericString (SIZE (12))

--	Frequencysatchannel corresponds to a 12 digit telephone number

Frequencyuhf ::= INTEGER  (9000..15999)

--	unit = Megahertz, Range (225.000..399.975), resolution = 0.025

Frequencyvhf ::= INTEGER (23600..27398)

--	unit = Megahertz, Range (118.000..136.990), resolution = 0.005

FurtherInstructions ::= SEQUENCE

{

code			[0]	Code				OPTIONAL,

frequencyDeparture	[1]	UnitNameFrequency		OPTIONAL,

clearanceExpiryTime	[2]	Time				OPTIONAL,

airportDeparture	[3]	Airport				OPTIONAL,

airportDestination	[4]	Airport				OPTIONAL,

timeDeparture		[5]	TimeDeparture			OPTIONAL,

runwayDeparture	[6]	Runway			OPTIONAL,

revisionNumber	[7]	RevisionNumber		OPTIONAL,

aTISCode		[8]	ATISCode			OPTIONAL

}

Holdatwaypoint ::= SEQUENCE

{

position				[0]	Position,

holdatwaypointspeedlow		[1]	Speed		OPTIONAL,

aTWlevel				[2]	ATWLevel	OPTIONAL,

holdatwaypointspeedhigh		[3]	Speed		OPTIONAL,

direction				[4]	Direction	OPTIONAL,

degrees				[5]	Degrees	OPTIONAL,

eFCtime				[6]	Time		OPTIONAL,

legtype					[7]	LegType	OPTIONAL

}

HoldClearance ::=  SEQUENCE

{

position 			[0]	Position,

level				[1]	Level,

degrees			[2]	Degrees,

direction			[3]	Direction,

legType			[4]	LegType		OPTIONAL

}

Humidity ::= INTEGER (0..100)

-- unit = Percent Humidity, Range (0..100), resolution = 1

InterceptCourseFrom ::= SEQUENCE

{

fromSelection		InterceptCourseFromSelection,

degrees		Degrees

}

InterceptCourseFromSelection ::= CHOICE

{

publishedIdentifier 		[0]	PublishedIdentifier,

latitudeLongitude 		[1]	LatitudeLongitude,

placeBearingPlaceBearing 	[2]	PlaceBearingPlaceBearing,

placeBearingDistance 		[3]	PlaceBearingDistance

}

Icing ::= ENUMERATED

{

trace		(0),

light		(1),

moderate	(2),

severe		(3)

}

Latitude ::= SEQUENCE

{

latitudeType			LatitudeType,

latitudeDirection		LatitudeDirection

}

LatitudeDegrees  ::= INTEGER (0..90000)

-- unit = Degree, Range (0..90), resolution = 0.001

LatitudeDegreesMinutes ::= SEQUENCE

{

latitudeWholeDegrees		LatitudeWholeDegrees,

minutesLatLon			MinutesLatLon

}

LatitudeDegreesMinutesSeconds ::= SEQUENCE

{

latitudeWholeDegrees		LatitudeWholeDegrees,

latlonWholeMinutes		LatLonWholeMinutes,

secondsLatLon			SecondsLatLon

}

LatitudeDirection ::= ENUMERATED

{

north		(0),

south		(1)

}

LatitudeWholeDegrees ::= INTEGER (0..89)

-- unit = Degree, Range (0..89), resolution = 1

LatitudeLongitude ::= SEQUENCE

{

latitude		[0]		Latitude,	OPTIONAL,

longitude	[1]		Longitude	OPTIONAL

}

LatitudeReportingPoints ::= SEQUENCE

{

latitudeDirection	LatitudeDirection,

latitudeDegrees		LatitudeDegrees

}

LatitudeType ::= CHOICE

{

latitudeDegrees			[0]	LatitudeDegrees,

latitudeDegreesMinutes		[1]	LatitudeDegreesMinutes,

latitudeDMS			[2]	LatitudeDegreesMinutesSeconds

}

LatLonWholeMinutes ::= INTEGER (0..59)

-- unit = Minute, Range (0..59), resolution = 1

LatLonReportingPoints ::= CHOICE

{

latitudeReportingPoints		[0]	LatitudeReportingPoints,

longitudeReportingPoints	[1]	LongitudeReportingPoints

}

LegDistance ::= CHOICE

{

legDistanceEnglish		[0]	LegDistanceEnglish,

legDistanceMetric		[1]	LegDistanceMetric

}

LegDistanceEnglish ::= INTEGER (0..50)

-- unit = Nautical Mile, Range (0..50), resolution = 1

LegDistanceMetric ::= INTEGER (1..128)

-- unit = Kilometer, Range (1..128), resolution = 1

LegTime ::= INTEGER (0..10)

--unit = Minute, Range (0..10), resolution = 1

LegType ::= CHOICE

{

legDistance	[0]	LegDistance,

legTime	[1]	LegTime

}

Level ::= CHOICE

{

singleLevel	[0]	LevelType,

blockLevel	[1]	SEQUENCE SIZE (2) OF LevelType

}

LevelFeet ::=  INTEGER (-60..7000)

--unit = Feet, Range (-600..70000), resolution = 10

LevelFlightLevel ::= INTEGER (30..700)

--unit = Level (100 Feet), Range (030..700), resolution = 1

LevelFlightLevelMetric ::= INTEGER (100..2500)

--unit = Level (10 Meters), Range (100..2500), resolution = 1

LevelLevel ::=  SEQUENCE SIZE (2) OF Level

LevelMeters ::= INTEGER (-30..25000)

--unit = Meter, Range (-30..25000), resolution = 1

LevelPosition ::= SEQUENCE

{

level				Level,

position			Position

}

LevelProcedureName ::= SEQUENCE

{

level				Level,

procedureName		ProcedureName

}

LevelsOfFlight ::= CHOICE

{

level			[0]	Level,

procedureName	[1]	ProcedureName,

levelProcedureName	[2]	LevelProcedureName

}

LevelSpeed ::= SEQUENCE

{

level			Level,

speed			Speed

}

LevelSpeedSpeed ::= SEQUENCE

{

level			Level,

speeds			SpeedSpeed

}

LevelTime ::= SEQUENCE

{

level			Level,

time			Time

}

LevelType ::=  CHOICE

{

levelFeet			[0]	LevelFeet,

levelMeters			[1]	LevelMeters,

levelFlightLevel		[2]	LevelFlightLevel,

levelFlightLevelMetric		[3]	LevelFlightLevelMetric

}

Longitude ::= SEQUENCE

{

longitudeType				LongitudeType,

longitudeDirection			LongitudeDirection

}

LongitudeDegrees ::= INTEGER (0..180000)

--unit = Degree, Range (0..180), resolution = 0.001

LongitudeDegreesMinutes ::= SEQUENCE

{

longitudeWholeDegrees	LongitudeWholeDegrees,

minutesLatLon			MinutesLatLon

}

LongitudeDegreesMinutesSeconds ::= SEQUENCE

{

longitudeWholeDegrees	LongitudeWholeDegrees,

latonWholeMinutes		LatLonWholeMinutes,

secondsLatLon			SecondsLatLon

}

LongitudeDirection ::= ENUMERATED

{

east		(0),

west		(1)

}

LongitudeWholeDegrees ::= INTEGER (0..179)

-- unit = Degree, Range (0..179), resolution = 1

LongitudeReportingPoints ::= SEQUENCE

{

longitudeDirection		LongitudeDirection,

longitudeDegrees		LongitudeDegrees

}

LongitudeType ::= CHOICE

{

longitudeDegrees		[0]	LontitudeDegrees,

longitudeDegreesMinutes	[1]	LongitudeDegreesMinutes,

longitudeDMS			[2]	LongitudeDegreesMinutesSeconds

}

MinutesLatLon ::= INTEGER (0..5999)

--unit = Minute, Range (0..59.99), resolution = 0.01

Month ::= INTEGER (1..12)

--unit = 1 Month, Range (1..12), resolution = 1

Navaid ::= SEQUENCE

{

name			[0]		NavaidName,

latlon			[1]		LatitudeLongitude		OPTIONAL

}

NavaidName ::= IA5String (SIZE (1..4))

PersonsOnBoard ::= INTEGER (1..1024)

PlaceBearing ::= SEQUENCE

{

publishedIdentifier			PublishedIdentifier,

degrees				Degrees

}

PlaceBearingDistance ::= SEQUENCE

{

publishedIdentifier			PublishedIdentifier,

degrees				Degrees,

distance				Distance

}

PlaceBearingPlaceBearing ::= SEQUENCE SIZE (2) OF PlaceBearing

Position ::= CHOICE

{

fixName			[0]	FixName,

navaid				[1]	Navaid,

airport				[2]	Airport,

latitudeLongitude		[3]	LatitudeLongitude,

placeBearingDistance		[4]	PlaceBearingDistance

}

PositionDegrees ::= SEQUENCE

{

position			Position,

degrees			Degrees

}

PositionDistanceSpecifiedDirection ::= SEQUENCE

{

position			Position,

distanceSpecifiedDirection	DistanceSpecifiedDirection

}

PositionLevel ::= SEQUENCE

{

position		Position,

level			Level

}

PositionLevelLevel ::= SEQUENCE

{

position		Position,

levels			LevelLevel

}

PositionLevelSpeed ::= SEQUENCE

{

positionlevel		PositionLevel,

speed			Speed

}

PositionPosition ::=  SEQUENCE SIZE (2) OF Position

PositionProcedureName ::= SEQUENCE

{

position			Position,

procedureName		ProcedureName

}

PositionReport ::= SEQUENCE

{

positioncurrent		 	[0]	Position,

timeatpositioncurrent 		[1]	Time,

level				[2]	Level,

fixnext				[3]	Position		OPTIONAL,

timeetaatfixnext		[4]	Time			OPTIONAL,

fixnextplusone			[5]	Position		OPTIONAL,

timeetaatdestination		[6]	Time			OPTIONAL,

remainingFuel			[7]	RemainingFuel		OPTIONAL,

temperature			[8]	Temperature		OPTIONAL,

winds	 			[9]	Winds			OPTIONAL,

turbulence			[10]	Turbulence		OPTIONAL,

icing				[11]	Icing			OPTIONAL,

speed				[12]	Speed			OPTIONAL,

speedground			[13]	SpeedGround		OPTIONAL,

verticalChange			[14]	VerticalChange		OPTIONAL,

trackAngle			[15]	Degrees		OPTIONAL,

heading			[16]	Degrees		OPTIONAL,

distance			[17]	Distance		OPTIONAL,

humidity			[18]	Humidity		OPTIONAL,

reportedWaypointPosition	[19]	Position		OPTIONAL,

reportedWaypointTime		[20]	Time			OPTIONAL,

reportedWaypointLevel	[21]	Level			OPTIONAL

}

PositionRouteClearance ::= SEQUENCE

{

position		Position,

routeClearance		RouteClearance

}

PositionSpeed ::= SEQUENCE

{

position		Position,

speed			Speed

}

PositionSpeedSpeed ::= SEQUENCE

{

position		Position,

speeds			SpeedSpeed

}

PositionTime ::= SEQUENCE

{

position		Position,

time			Time

}

PositionTimeLevel ::= SEQUENCE

{

positionTime		PositionTime,

level			Level

}

PositionTimeTime ::= SEQUENCE

{

position		Position,

times			TimeTime

}

PositionUnitNameFrequency ::= SEQUENCE

{

position		Position,

unitname		UnitName,

frequency		Frequency

}

Procedure ::= IA5String (SIZE (1..20))

ProcedureName ::= SEQUENCE

{

type			[0]	ProcedureType,

procedure		[1]	Procedure,

transition		[2]	ProcedureTransition	OPTIONAL

}

ProcedureTransition ::= IA5String (SIZE (1..5))

ProcedureType ::= ENUMERATED

{

arrival			(0),

approach		(1),

departure		(2)

}

PublishedIdentifier ::= CHOICE

{

fixName		[0]	FixName,

navaid			[1]	Navaid

}

RemainingFuel ::= Time

RemainingFuelPersonsOnBoard ::=  SEQUENCE

{

remainingFuel		RemainingFuel,

personsOnBoard	PersonsOnBoard

}

ReportingPoints ::= SEQUENCE

{

latLonReportingPoints		[0]	LatLonReportingPoints,

degreeIncrement		[1]	DegreeIncrement		OPTIONAL

}

RevisionNumber ::= INTEGER (1..16)

RouteAndLevels ::= SEQUENCE

{

routeOfFlight		RouteInformation,

levelsOfFlight		LevelsOfFlight

}

RouteClearance ::= SEQUENCE

{

airportDeparture		[0]	Airport					OPTIONAL,

airportDestination		[1]	Airport					OPTIONAL,

runwayDeparture		[2]	Runway				OPTIONAL,

procedureDeparture		[3]	ProcedureName			OPTIONAL,

runwayArrival			[4]	Runway				OPTIONAL,

procedureApproach		[5]	ProcedureName			OPTIONAL,

procedureArrival		[6]	ProcedureName			OPTIONAL,

routeInformations		[7]	SEQUENCE SIZE (1..128)

OF RouteInformation			OPTIONAL,

routeInformationAdditional	[8]	RouteInformationAdditional		OPTIONAL

}

RouteInformation ::= CHOICE

{

publishedIdentifier 		[0]	PublishedIdentifier,

latitudeLongitude 		[1]	LatitudeLongitude,

placeBearingPlaceBearing 	[2]	PlaceBearingPlaceBearing,

placeBearingDistance 		[3]	PlaceBearingDistance,

aTSRouteDesignator 		[4]	ATSRouteDesignator

}

RouteInformationAdditional ::= SEQUENCE

{

aTWAlongTrackWaypoints	[0]	SEQUENCE SIZE (1..8) OF

ATWAlongTrackWaypoint  						OPTIONAL,

reportingpoints			[1]	ReportingPoints

OPTIONAL,

interceptCourseFroms		[2]	SEQUENCE SIZE (1..4) OF InterceptCourseFrom

OPTIONAL,

holdAtWaypoints		[3]	SEQUENCE SIZE (1..8) OF Holdatwaypoint

OPTIONAL,

waypointSpeedLevels		[4]	SEQUENCE SIZE (1..32) OF WaypointSpeedLevel

OPTIONAL,

rTARequiredTimeArrivals	[5]	SEQUENCE SIZE (1..32) OF

RTARequiredTimeArrival

OPTIONAL

}

RTARequiredTimeArrival ::= SEQUENCE

{

position		[0]	Position,

rTATime		[1]	RTATime,

rTATolerance		[2]	RTATolerance		OPTIONAL

}

RTATime ::= SEQUENCE

{

time			Time,

timeTolerance		TimeTolerance

}

RTATolerance ::= INTEGER (1..150)

--unit= Minute, Range (0.1..15.0), resolution = 0.1

Runway ::= SEQUENCE

{

direction		RunwayDirection,

configuration		RunwayConfiguration

}

RunwayDirection ::= INTEGER (1..36)

RunwayConfiguration ::= ENUMERATED

{

left		(0),

right		(1),

center		(2),

none		(3)

}

RunwayRVR ::= SEQUENCE

{

runway		Runway,

rVR			RVR

}

RVR ::= CHOICE

{

rVRFeet 	[0]	RVRFeet,

rVRMeters 	[1]	RVRMeters

}

RVRFeet ::= INTEGER (0..6100)

-- unit = Feet, Range (0..6100), resolution = 1

RVRMeters ::= INTEGER (0..1500)

-- unit = Meters (0..1500), resolution = 1

SecondsLatLon ::= INTEGER (0..59)

--unit = Second, Range (0.. 59), resolution = 1

Speed ::=  CHOICE

{

speedIndicated 		[0]	SpeedIndicated,

speedIndicatedMetric 		[1]	SpeedIndicatedMetric,

speedTrue 			[2]	SpeedTrue,

speedTrueMetric 		[3]	SpeedTrueMetric,

speedGround 			[4]	SpeedGround,

speedGroundMetric 		[5]	SpeedGroundMetric,

speedMach 			[6]	SpeedMach

}

SpeedIndicated ::= INTEGER (0..400)

-- unit = Knots, Range (0..400), resolution = 1

SpeedIndicatedMetric ::= INTEGER (0..800)

-- unit = Kilometers/Hour, Range (0..800), resolution = 1

SpeedGround ::= INTEGER (-50..2000)

-- unit = Knots, Range (-50..2000), resolution = 1

SpeedGroundMetric ::= INTEGER (-100..4000)

-- unit = Kilometers/Hour, Range (-100..4000), resolution = 1

SpeedMach ::= INTEGER (500..4000)

-- unit = Mach Range (0.5 to 4.0), resolution = 0.001

SpeedSpeed ::=  SEQUENCE SIZE (2) OF Speed

SpeedTime ::= SEQUENCE

{

speed			Speed,

time			Time

}

SpeedTrue ::= INTEGER (0..2000)

-- unit = Knots, Range (0..2000), resolution = 1

SpeedTrueMetric ::= INTEGER (0..4000)

-- unit = Kilometers/Hour, Range (0..4000), resolution = 1

SpeedType ::= ENUMERATED

{

noneSpecified	(0),

indicated	(1),

true		(2),

ground		(3),

mach		(4),

approach	(5),

cruise		(6),

minimum	(7),

maximum	(8),

...

}

SpeedTypeSpeedTypeSpeedType ::= SEQUENCE SIZE (3) OF SpeedType

SpeedTypeSpeedTypeSpeedTypeSpeed ::= SEQUENCE

{

speedTypes	SpeedTypeSpeedTypeSpeedType,

speed		Speed

}

Temperature ::= INTEGER (-100..100)

-- unit = Degree Celsius, Range (-100..100), resolution = 1

Time ::= SEQUENCE

{

hours		TimeHours,

minutes	TimeMinutes

}

TimeLevel ::= SEQUENCE

{

time		Time,

level		Level

}

TimeDeparture ::= SEQUENCE

{

timeDepartureAllocated		[0]	Time				OPTIONAL,

timeDepartureControlled		[1]	ControlledTime		OPTIONAL,

timeDepartureClearanceExpected	[2]	Time				OPTIONAL,

departureMinimumInterval		[3]	DepartureMinimumInterval	OPTIONAL

}

TimeDistanceSpecifiedDirection ::= SEQUENCE

{

time					Time,

distanceSpecifiedDirection		DistanceSpecifiedDirection

}

TimeDistanceToFromPosition ::=  SEQUENCE

{

time			Time,

distance		Distance,

tofrom			ToFrom,

position		Position

}

Timehhmmss ::= SEQUENCE

{

hoursminutes		Time,

seconds		TimeSeconds

}

TimeHours ::= INTEGER (0..23)

-- unit = Hour, Range (0..23), resolution = 1

TimeUnitNameFrequency ::=  SEQUENCE

{

time			Time,

unitName		UnitName,

frequency		Frequency

}

TimeMinutes ::= INTEGER (0..59)

-- unit = Minute, Range (0..59), resolution = 1

TimePosition ::=  SEQUENCE

{

time			Time,

position		Position

}

TimePositionLevel ::=    SEQUENCE

{

timeposition		TimePosition,

level			Level

}

TimePositionLevelSpeed ::= SEQUENCE

{

timeposition		TimePosition,

levelspeed		LevelSpeed

}

TimeSeconds ::= INTEGER (0..59)

-- unit = Second, Range (0..59), resolution = 1

TimeSpeed ::=  SEQUENCE

{

time			Time,

speed			Speed

}

TimeSpeedSpeed ::=  SEQUENCE

{

time			Time,

speedspeed	 	SpeedSpeed

}

TimeTime ::= SEQUENCE SIZE (2) OF Time

TimeToFromPosition ::= SEQUENCE

{

time			Time,

tofrom			ToFrom,

position		Position

}

TimeTolerance ::= ENUMERATED

{

at		(0),

atorafter	(1),

atorbefore	(2)

}

ToFrom ::= ENUMERATED

{

to		(0),

from		(1)

}

ToFromPosition ::=  SEQUENCE

{

toFrom			ToFrom,

position		Position

}

TrafficType ::= ENUMERATED

{

noneSpecified		(0),

oppositeDirection	(1),

sameDirection		(2),

converging		(3),

crossing		(4),

diverging		(5),

...0

}

Turbulence ::= ENUMERATED

{

light		(0),

moderate	(1),

severe		(2)

}

UnitName ::=   SEQUENCE

{

facilityDesignation	[0]	FacilityDesignation,

facilityName		[1]	FacilityName		OPTIONAL,

facilityFunction		[2]	FacilityFunction

}

UnitNameFrequency ::=  SEQUENCE

{

unitName		UnitName,

frequency		Frequency

}

VersionNumber ::=  INTEGER (0..15)

VerticalChange ::= SEQUENCE

{

direction		VerticalDirection,

rate			VerticalRate

}

VerticalDirection ::= ENUMERATED

{

up	(0),

down	(1)

}

VerticalRate ::=  CHOICE

{

verticalRateEnglish 		[0]	VerticalRateEnglish,

verticalRateMetric 		[1]	VerticalRateMetric

}

VerticalRateEnglish ::= INTEGER (0..3000)

-- unit = Feet/Minute, Range (0..30000), resolution = 10

VerticalRateMetric ::= INTEGER (0..1000)

-- unit = Meters/Minute, Range (0..10000), resolution = 10

WaypointSpeedLevel ::= SEQUENCE

{

position 		[0]	Position,

speed			[1]	Speed				OPTIONAL,

aTWLevels		[2]	ATWLevelSequence		OPTIONAL

}

WindDirection ::= INTEGER (1..360)

-- unit = Degree, Range (1..360), resolution = 1

Winds ::= SEQUENCE

{

direction		WindDirection,

speed			WindSpeed

}

WindSpeed ::= CHOICE

{

windSpeedEnglish	[0]	WindSpeedEnglish,

windSpeedMetric	[1]	WindSpeedMetric

}

WindSpeedEnglish ::= INTEGER (0..255)

-- unit = Knot, Range (0..255), resolution = 1

WindSpeedMetric ::= INTEGER (0..511)

-- unit = Kilometer/Hour, Range (0..511), resolution = 1

Year ::= INTEGER (1996..2095)

-- unit = Year, Range (1996..2095), resolution = 1

END