Proposed SARPs Amendment in response to PDR 98060006

(Correlation of ATSC Class with a/g subnetwork type in Airborne Router)

The following text provides a proposed SARPs amendment for the Proposed Defect Report (PDR) 98060006 along the outline solution agreed by WG 2 during its Bordeaux meeting in October 1998 (action 16/8).

A)	In Chapter 5.3:

A1)	Add the following new note to paragraph 5.3.5.1:

“Note 6.— The ATSC Class assigned to an Air/Ground Subnetwork and the traffic type(s) allowed to pass over this Air/Ground Subnetwork are known a priori to the Air/Ground Router attached to each such subnetwork. They are communicated to an Airborne Router using the options part of an ISO/IEC 9542 ISH PDU which is uplinked to the Airborne Router as part of the route initiation procedure as described in 5.3.5.2.”

A2)	Add the following new paragraph 5.3.5.2.6.5:

”5.3.5.2.6.5	An ATN Air/Ground Router shall include the Mobile Subnetwork Capabilities Parameter, as defined in 5.8.2.1.3, in the options part of the uplinked ISH PDU. The Mobile Subnetwork Capabilities Parameter shall indicate any restrictions on traffic types permitted to pass over the Mobile Subnetwork and the ATSC Class of the Mobile Subnetwork, if the ATN Operational Communications traffic type – Air Traffic Service Communications traffic category is among the permissible traffic types for this Mobile Subnetwork.

Note 1.— The ATSC Class assigned to a Mobile Subnetwork and the traffic type(s) allowed to pass over this Mobile Subnetwork are uplinked to the Airborne Router to enable this router to make the appropriate routing decision when downlinking packets over an air/ground adjacency which is made up of more than one Mobile Subnetwork.

Note 2.— The ISH PDU is only ever sent in the context of a single Mobile Subnetwork between the Air/Ground and Airborne Router. Thus the capability information carried in the Mobile Subnetwork Capabilities Parameter is unambiguously associated with this subnetwork.”

A3)	In the existing paragraph 5.3.5.2.6.7 delete "... to the indicated NET ..."

A4)	Renumber the existing paragraphs 5.3.5.2.6.5 through 5.3.5.2.6.6 to become paragraphs 5.3.5.2.6.6 through 5.3.5.2.6.7.

A5)	Add the following new paragraphs 5.3.5.2.6.8 and 5.3.5.2.6.9:

”5.3.5.2.6.8	Whenever an ISO/IEC 9542 ISH PDU is received by an Airborne Router, this router shall evaluate the Mobile Subnetwork Capabilities Parameter contained in the options part of the received ISH PDU.

5.3.5.2.6.9	The Airborne Router shall use the received subnetwork capability information to update its local configuration data concerning the permissible traffic type(s) and the supported ATSC Class of the Mobile Subnetwork over which the ISH PDU was received.”

A6)	Renumber the existing paragraphs 5.3.5.2.6.7 and 5.3.5.2.6.8 to become paragraphs 5.3.5.2.6.10 and 5.3.5.2.6.11.

A7)	Add the following new paragraph 5.3.5.2.10.5:

“5.3.5.2.10.5	If a BIS-BIS connection was already established with the remote ATN Air/Ground Router, then the IS-SME of the Airborne Router shall cause the IDRP Routing Decision Function to be invoked in order to rebuild the FIB, the Loc_RIB and relevant Adj-RIB-Out(s) taking into account the additional subnetwork connectivity.”

A8)	Renumber the existing paragraphs 5.3.5.2.10.5 through 5.3.5.2.10.9 to become paragraphs 5.3.5.2.10.6 through 5.3.5.2.10.10.

A9)	In the existing paragraph 5.3.5.2.12.3 bullets a) and b) replace at the end of the paragraph "... determined from locally known information" by "... determined from the Mobile Subnetwork Capabilities Parameter contained in the options part of the received ISH PDU, or from locally known information if such a parameter is not present in the received ISH PDU."

B)	In Chapter 5.8:

B1)	Add the following new section 5.8.2.1.3:

“5.8.2.1.3	The Mobile Subnetwork Capabilities Parameter

5.8.2.1.3.1	ATN Air/Ground and Airborne Routers shall support the Mobile Subnetwork Capabilities Parameter in the options part of an ISO/IEC 9542 ISH PDU.

5.8.2.1.3.2	The Mobile Subnetwork Capabilities Parameter shall be used in the ATN to convey information about the ATSC Class and the traffic type(s) supported by an ATN Mobile Subnetwork.

5.8.2.1.3.3	The Mobile Subnetwork Capabilities Parameter shall consist of three fields, as illustrated in Figure 5.8-1, and shall not occur more than once in the options part of an ISO/IEC 9542 ISH PDU.

# Figure 5.8-1: The Mobile Subnetwork Capabilities Parameter

5.8.2.1.3.4	Encoding of the Mobile Subnetwork Capabilities Parameter

5.8.2.1.3.4.1	The Mobile Subnetwork Capabilities Parameter code field shall be one octet in length and shall always be encoded as binary [1000 0001] to indicate the Mobile Subnetwork Capabilities Parameter.

Note.— The above parameter code and its associated semantics are defined by this specification for the ATN in addition to the parameter codes specified by ISO/IEC 9542. ISO/IEC 9542 only uses eight bit parameter codes with bits 8 and 7 set to one and has reserved a parameter code of 255 for possible future extensions. The future use of the above ATN parameter code by an ISO standard cannot be ruled out but is highly unlikely.

5.8.2.1.3.4.2	The Mobile Subnetwork Capabilities Parameter length field shall be one octet long and shall define the length in octets of the Mobile Subnetwork Capabilities Parameter value field.

5.8.2.1.3.4.3	Mobile Subnetwork Capabilities Parameter Value Field

5.8.2.1.3.4.3.1	The first octet of this field shall indicate the traffic type(s) allowed to pass over the Air/Ground Subnetwork over which the ISO/IEC 9542 ISH PDU is exchanged.

5.8.2.1.3.4.3.2	This octet shall comprise a bit map, where each bit corresponds to a different traffic type.

5.8.2.1.3.4.3.3	The assignment of bits to traffic types shall be according to Table 5.8-4, where bit 0 is the low order bit.

5.8.2.1.3.4.3.4	Setting a bit to one shall indicate that the corresponding traffic type is allowed to pass over the air/ground subnetwork.

5.8.2.1.3.4.3.5	The semantics of bits 5 to 7 shall be reserved for future use and shall always be set to one.

Note 1.— A value of FFh is used to imply no restrictions.

Note 2.— The first octet of the Mobile Subnetwork Capabilities Parameter Value field has the same encoding and semantics as the second octet of the Air/Ground Subnetwork Type Security Tag Set of the IDRP Security Path Attribute which is defined in 5.8.3.2.3.2.3.

5.8.2.1.3.4.3.5	If bit 0 of the first octet of the Mobile Subnetwork Capabilities Parameter Value field is set to one, then this field shall contain a second octet which defines the ATSC Class supported by that Air/Ground Subnetwork.

Note.— Bit 0 of the first octet set to one indicates that the Air/Ground Subnetwork is available to the ATN Operational Communications traffic type – Air Traffic Service Communications traffic category.

5.8.2.1.3.4.3.6	If present, the second octet of the Mobile Subnetwork Capabilities Parameter Value field shall be encoded according to Table 5.8-1.

Table 5.8-1: Encoding of Supported ATSC Class

| Value | ATSC Class |
| --- | --- |
| 0000 0001 | A |
| 0000 0010 | B |
| 0000 0100 | C |
| 0000 1000 | D |
| 0001 0000 | E |
| 0010 0000 | F |
| 0100 0000 | G |
| 1000 0000 | H |

Note.— ATSC Class ”H” is the lowest and Class ”A” is the highest class.

B2)	Renumber the existing paragraph 5.8.2.1.3 to become paragraph 5.8.2.1.4.

B3)	In the existing Table 5.8-1 add a new line following the ”SNMk-s” line with the following entries:

Item = MSNC-s

Protocol Function = <s> Mobile Subnetwork Capabilities

Clauses = ATN SARPs Ref: 5.8.2.1.3, 5.3.5.2.6.5

ISO Status = --

ATN Support = ISH-s: M

B4)	In the existing Table 5.8-1 add another new line following the above new line with the following entries:

Item = MSNC-r

Protocol Function = <r> Mobile Subnetwork Capabilities

Clauses = ATN SARPs Ref: 5.8.2.1.3, 5.3.5.2.6.9

ISO Status = --

ATN Support = ISH-r: M

B5)	Renumber the Tables 5.8-1 through 5.8-7 to become Tables 5.8-2 through 5.8-8.

B6)	In the existing paragraph 5.8.2.2.1 replace “Table 5.8-1” by “Table 5.8-2”.

B7)	In the existing paragraph 5.8.3.2.3.2.2 replace “Table 5.8-2” by “Table 5.8-3”.

B8)	In the existing paragraph 5.8.3.2.3.2.5 replace “Table 5.8-3” by “Table 5.8-4”.

B9)	In the existing paragraph 5.8.3.2.3.3.6 replace “Table 5.8-4” by “Table 5.8-5”.

B10)	In the existing paragraph 5.8.3.2.8.1 replace “Table 5.8-5” by “Table 5.8-6”.

B11)	In the existing paragraph 5.8.3.3.2.1.1 replace “Table 5.8-6” by “Table 5.8-7”.