Title:                                  IDRP Traffic Typing

PDR Reference:                          98060008

Originator Reference:                   IDRP-traffic

SARPs Document Reference:               V2.2 – 5.3.2.2.3.5 and

5.8.3.2.14.1

Status:                                 RESOLVED

PDR Revision Date:

PDR Submission Date:                    15/06/98

Submitting State/Organisation:          Open Network Solutions, Inc.

Submitting Author Name:                 James Moulton

Submitting Author E-mail Address:       moulton@ons.com

Submitting Author Supplemental

Contact Information:

SARPs Date:                             ICAO Version 2.2

SARPs Language:                         English

Summary of Defect:

The referenced paragraph (5.8.3.2.14.1) specifies that BISPDUs will be

transmitted by CLNP with a traffic type of „Systems Management".

The referenced paragraph on routing policy (5.3.2.2.3.5.1) specifies

that a CLNP pdu with a traffic type of „Systems Management" may only be

sent on subnetworks with „ATN Systems Management Communications" set in

the permissible traffic types.

The specification of permitted traffic type of „ATSC only" results in

the prohibition of sending IDRP traffic on that link.

Assigned SME:                           Sub-Volume V SME (K.-P. Graf)

Discussion:

There seem to be two potential solutions to this problem:

1. Re-define IDRP pdus to a different traffic type

2. Change the policy so that System Management traffic may be sent on

any link.

The proposed solution 1 is needed if the type of system management data

includes exchanges other than IDRP.  That is, if system management needs

to send data over a link then it may be appropriate to segregate IDRP

traffic from system management traffic.

The proposed solution 2 is needed to allow IDRP traffic on ATSC-only

links.  (Of course, it can also be stated that all ATSC-only links must

also have the System Mangament traffic type set.)  It may be appropriate

to establish a traffic type hierarchy where some traffic types like IDRP

traffic can go on any link except higher „priority" links.

SME comment:

Resolve reported problem by additional SARPs note stating the need for

appropriate configuration of ATN routers concerning the routing

restrictions applicable to the router's adjacencies.

Proposed SARPs amendment:

The following note should be included at the end of para 5.8.3.2.14.1:

"Note.-  To ensure the exchange of ISO/IEC 10747 BISPDUs over an

air/ground adjacency under the above traffic type classification, the

air/ground router or airborne router respectively must be configured in

a way that includes „ATN Systems Management Communications" in the set

of permissible traffic types allowed to pass over the air/ground

subnetwork(s) forming the air/ground adjacency. Otherwise, an IDRP

connection may not be established over the air/ground adjacency;

consequently no CLNP PDUs will ever flow over it and the adjacency will

be unusable."

SME Recommendation to CCB:

CCB Decision:  Proposed SARPs amendment agreed