Title:  Incomplete specification for use of V.42bis by Mobile SNDCF

PDR Reference: 97100001

Originator Reference:

SARPs Document Reference:  ICS SARPs, Section 5.7.6.2.1.5

PDR Status:  RESOLVED

PDR Revision Date:    04/12/97

PDR Submission Date:  06/10/97

Submitting State/Organization:  Eurocontrol/MWA

Submitting Author Name:  Tony Whyman

Submitting Author E-mail Address:  whyman@mwassocs.demon.co.uk

Submitting Author Supplemental Contact Information:

21 Orchard Close, Alresford, Hants, SO24 9PY, England

Tel: +44 1962 735580

FAX: +44 1962 735581

SARPs Date:  ICAO Version 2.0 (27 June 1997)

SARPs Language:  English

Summary of Defect:

The Mobile SNDCF currently makes the use of ITU-T recommendation V.42bis

optional stream compression algorithm. However, the specification is

incomplete in the following areas:

1. V.42bis specifies a stream mode compression algorithm for use in

modems.

In an ATN implementation it needs to be adapted to work in a packet mode

with the compressor flushed after each packet is compressed. This is to

ensure that each received packet can be compressed in its entirety

without needing information from the next packet. This adaptation has

not been specified.

2. Under V.42bis, both compressor and decompressor maintain state

information which is kept synchronised by the compressed data stream.

Any errors introduced into this data stream will affect not just one

packet,but all subsequent packets. It is thus highly desireable to

append a checksum on the uncompressed data to each packet. Any failures

in the algorithm or errors introduced into the compressed data can

therefore be detected and the compressor/decompressor state reset.

Furthermore, V.42bis also has IPR problems due to a patent owned by

Unisys Inc. This may limit its deployment in the aeronautical community

and compromise its specification in SARPs. Validation work done by

Eurocontrol has shown (WP2/WP407) that alternative (the "Deflate"

algorithm) is often superior to V.42bis. It is also not encumbered by

any IPR issue. Its specification also includes packet mode operations,

and it has been implemented in the Eurocontrol TAR. It is thus

recommended as an alternative to V.42bis.

Assigned SME: Subvolume V SME (K.-P. Graf)

Proposed SARPs Amendment:

1.) In Figure 5.7-2 "Format for Call Request User Data" and Figure 5.7-3

"Format for Call Accept User Data", change "V.42bis" to "Deflate".

2.) In Table 5.7-2 "Compression Options Offered Parameter", change the

definition of bit 6 to "Deflate".

3.) In 5.7.6.1.1, Note 2, replace the existing bulleted list by the

following three bullets:

"Local Reference (LREF) Compression as specified in 5.7.6.3;

The ICAO Address Compression Algorithm (ACA) as specified in 5.7.6.4;

Data Stream Mode Compression as specified in 5.7.6.5."

4.) In 5.7.6.1.1 add the following new Note 3

"Note 3.- The Data Stream Mode Compression uses the Deflate algorithm

which was originally specified in IETF RFC 1951."

and renumber the existing Notes 3 through 6.

5.) In 5.7.6.2.1.5.9 Note, 5.7.6.2.1.7.3.4, 5.7.6.2.1.5.10,

5.7.6.2.2.2.2, 5.7.6.2.3.2(b), 5.7.6.2.3.3(b), Table 5-7.3, and

5.7.7.8.1 change „V.42bis" to "Deflate".

6.) Add the new section 5.7.6.5 contained in the file

atnp/ccb/sme5/deflate1.zip to the SARPs.

Please note that the changes contained in the file delate1.zip may

appear more extensive than those SARPs for V.42bis. This is because the

use of V.42bis was not adequately described, and it is not possible to

formally reference an IETF RFC from an ICAO SARPs.

Please note that the text contained in the file deflate1.zip is an

integral part of the proposed SARPs amendment. It could not be included

in this PDR form due to the limitations of the atnp-wg2-sdm exploder

with respect to the size of distributed messages.

SME Recommendation to CCB: Accept proposed SARPs amendment

CCB Decision: RESOLVED