## ATNP WG1/WP11-7

9 March 1998

ATNP WG1 – Eleventh Meeting

Rio de Janeiro, Brazil

11-12 March 1998

Proposed Communiqué to IATA & ICCAIA

Presented by Ron Jones, FAA

Summary

The ATN as well a number of current and future avionics system employ the ICAO 24-bit aircraft address.    Current airframe configurations provide no means for certain existing systems, such as the Flight Management System, to obtain the aircraft address.    This paper proposes a communiqué to IATA and ICCAIA to request they initiate action to have AEEC define the means for generating and distributing the aircraft address on the airframe.

Attachment:    Draft Communiqué to IATA & ICCAIA

1.	Introduction

The ATN, as well as number of current and future avionics systems, employ the ICAO 24-bit aircraft address.    Current airframe configurations provide no means for certain existing systems, such as the Flight Management System (FMS), to obtain the aircraft address.    One suggestion, in the case of the FMS, has been to allow for the flight crew to enter the aircraft address. This could seriously compromise the use of the ATN as the manual entry process can be error prone. This paper proposes a communiqué to IATA to request they initiate action to have AEEC define the means for generating and distributing the aircraft address.

2.	Proposal

It is proposed that ATNP WG1 develop a communiqué to IATA & ICCAIA, based on the attached draft, requesting that IATA & ICCAIA coordinate with AEEC to define a means by which the aircraft address can be generated and distributed on the airframe.

ATTACHMENT

Draft Communiqué to IATA & ICCAIA

ICAO Aeronautical Telecommunication Network Panel (ATNP)

Systems Planning and Concept Working Group – WG1

11th Meeting, 11-12 March 1998

Background:		ICAO has developed, or is in the processing of developing Standards and Recommended Practices (SARPs) for a number of data communications systems, or subsystems, that support the overall Future Air Navigation System (FANS) concept.    As one element of the FANS concept, ICAO embarked a decade ago on the development of a global telecommunications network that would serve both airborne and ground users.    This effort has resulted in the development of the ATN Standards and Recommended Practices (SARPs) that are expected to be published later this year.    ATN, as well as other ICAO defined systems involving data communications, require each airframe to have a unique data address.    The ICAO 24-bit aircraft address was created for this purpose and was codified within ICAO Annex 10 about 10 years ago. Existing airborne systems already using the aircraft address include Mode S and TCAS.    Several systems becoming available over the next few years, including ATN, VDL, HFDL, AMSS, and ADS-B are also required to use the ICAO aircraft address.    ICAO has recently agreed to modify the ICAO flight plan to add the aircraft address.

Also the ongoing work of the ICAO ATN Panel includes the definition of security enhancements for the ATN.    While these security provisions are not expected to be implemented in the initial operational ATN capabilities, consideration should be given to how security related information could be managed onboard the aircraft.

AEEC has included provisions within the Characteristics for Mode S and TCAS avionics to set the aircraft address through physical provisions within the wiring harness.    This allows for the replacement of avionics without any manual reconfiguration for the specific airframe.    More recent activities within AEEC have included discussions of an Airplane Personality Module (APM) that would include the ICAO 24-bit aircraft address along with other information specific to a given aircraft configuration (reference: Supplement 2 to ARINC Report 607).    The APM would be a memory module that could be used in conjunction with various future ARINC 700-series avionics.

Issue:		Neither of the two solutions considered by AEEC for entering the ICAO 24-bit aircraft address (i.e., use of wiring harness connections or the use of an APM), provide a general solution for existing avionics units to obtain the aircraft address.    This becomes a significant problem were the ATN is concerned as the aircraft address is incorporated into the network addresses of end systems and intermediate systems.    Specifically, if existing airborne systems, such as an existing FMS, were to be used as an ATN end system (ES) then it must have the aircraft address in order to participate within the network.    Also the airborne intermediate system (IS) (e.g., CMU) and the air-ground subnetworks that are to be used by the ATN need to have knowledge of the aircraft address.    The alternative of having the flight crew manually enter the aircraft address into the FMS, which has been proposed by some, is viewed by ATNP WG1 as being unacceptably prone to error.    A general technical mechanism needs to urgently be defined and implemented to allow for off-line setup of the value of the aircraft address and subsequent generation and distribution to all avionics units needing this information and for which no other means of obtaining the aircraft address has already been implemented.    If existing avionics are to be accommodated, that do not support use of an APM, this might require the definition of a new or modified LRU that would output the aircraft address onto an aircraft data buss.

The ICAO ATN Panel is currently in the process of defining security enhancements for the ATN.    The current proposal calls for the use authentication services based on the use of X.509 certificates. With this approach the airborne IS and ESs supporting ATS applications will need access to certain of the information contained in the X.509 certificate (e.g., the aircraft’s private key and the cryptographic algorithm version number).    One approach could be to include such information in the APM, although this would have the limitation of not accommodating existing ESs that do support use of an APM.    The ATN security provisions are still under development and are not expected to be implemented in the initial operational ATN capabilities.    However consideration should be given to how security related information could be managed onboard the aircraft.

Request:	IATA and ICCAIA are requested to engage in discussions with AEEC to identify an urgent need to define a general solution for the generation and distribution of the ICAO 24-bit aircraft address information on the airframe.      Also the AEEC should be engaged to identify an appropriate means to manage ATN security information onboard the aircraft.