## ATNP WG1/WP11-5

ATNP WG1 – Eleventh Meeting

Rio de Janeiro, Brazil

11-12 March 1998

Agenda Item 3-N

End-to-End Performance for ATS Services over the ATN

Presented by Ron Jones, FAA

Summary

This paper summarizes the results of investigations into the ATN end-to-end transit delay values when AMSS, Mode S and VDL Mode 2 are employed as the air-ground subnetworks.      A summary of the RTCA/EUROCAE activities to define Require Communications Performance (RCP) is also provided.

Appendix A - Aircraft Subnetwork Transit Delay using Williamsburg Protocol (provided by Boeing)

Appendix B –RTCA/EUROCAE SG3, Position Paper 3 on "Methodology for Developing RCP-types", Feb. 1998 draft.

References:

1.	“Evaluation of ATN/AMSS End-to-End Performance by Computer Simulation,” ICAO ATNP WG1/IP3-2, October 1995 (Banff).    Subsequent updates provided in March 1996 (Brussels) and October 1996 (Alexandria) ATNP WG2 meetings.

2.	“ATN Time Performance Measurements Over Mode S,” ICAO ATNP WG1/IP4-5, January 1996 (Brisbane).

3.	FAA/ARINC Cooperative Research and Development Agreement, "VDL Mode 2 Simulation Report", November 28, 1998

4.	"Comparative End-to-End AMSS Data Message Transfer Delays", ICAO AMCP WP437, 22 January 1997.

5.	"Further information on ADS message transmission times for 10.5 Kb/s satellite channels," ICAO AMCP WP480, June 1997.

1.    Introduction

The ATN SARPs include as the seventh system-level requirement the following:

“The ATN shall offer ATSC classes in accordance with the criteria in Table 3-1.”

Table 3-1 of the ATN SARPs defines the following ATSC traffic classes and corresponding one-way ATN end-to-end transit delay at 95% probability.

| One way ATN End-to-End Transit Delay at 95% probability (seconds) | ATSC Class |
| --- | --- |
| Reserved | A |
| 4.5 | B |
| 7.2 | C |
| 13.5 | D |
| 18 | E |
| 27 | F |
| 50 | G |
| 100 | H |
| No value specified | no preference |

ATS providers are expected to specify the required ATS traffic class for each application (e.g., CPDLC, D-ATIS, etc.) for each operational domain (i.e., terminal, en route, tower and oceanic).    The ATN routers will select the path to be used for forwarding a data packet based on the specified traffic class.

WG1 was tasked to develop end-to-end performance requirements by ATNP/2.    RTCA and EUROCAE have undertaken the definition of "Required Communications Performance" (RCP) applicable air-ground data communication systems, including ATN.    Also the ICAO ADSP has a work program item to define end-to-end RCP.    Unlike the past work of ICAO and RTCA/EUROCAE in the definition of "Required Navigation Performance" (RNP), the RCP definition is expected to involve the definition of perhaps 3 relatively independent variables.    The challenge will be to start from an overall end-to-end RCP and to be able to allocate the contributions of individual subsystems against this overall RCP.    This will become critical as we begin to certify future communication systems as well as the individual subsystems.

The remainder of this working paper focuses on providing information on the expected end-to-end performance of ATN services when a variety of air-ground subnetworks are used.    Appendix B to this working paper includes draft material from the RTCA/EURCAE subgroup activities defining RCP.

2.    Background

The values specified in the ATN SARPs for the ATN end-to-end transit delay were derived from values specified in the draft ICAO Manual on ATS Data Link Applications that was prepared by the ICAO ADS Panel.    The operational requirements defined by the ADSP was for 'user-to-user' delivery delay.    The scope of the ATN SARPs is for the ATN communications service and does not include the application processing necessary to prepare a message for output to an ATC controller or flight crew or for the processing of inputs from the controller or flight crew.    Generally the ATNP, in defining the values associated with the ATSC traffic classes, has used a value for the ATN end-to-end transit delay that is 90% of that specified by the ADSP for the user-to-user delivery delay.    Thus, 10% of the time specified by the ADSP has been allocated to the application processing functions that are outside the scope of the ATN SARPs.    This working paper is an extension of materials presented to an accepted by the ATNP for the purpose of validating the range of defined ATSC traffic class values.    However, for the purpose of RTCA/EUROCAE considerations the scope of the paper has been broadened to capture the anticipated performance range of application processing and ATN end-systems (i.e., hosts) and avionics subnetworks.

3.    Discussion

3.1	ATN End-to-End Performance

The results presented in the working paper are based on simulations, limited test bed evaluations and measured performance of existing systems.    The ability of the ATN to satisfy the system level requirement for ATN end-to-end transit delay consistent with a specified traffic class is largely dependent on the performance provided by the available mobile subnetworks.    However, the performance of the end system application processing and the airborne subnetwork can become a limiting factor for the most demanding ATSC traffic classes.    Mobile subnetworks implemented in conformance with the applicable ICAO subnetwork SARPs and the associated MOPS may exhibit varying performance because of two primary factors:

a)    optional features and/or performance levels allowed by the subnetwork SARPs/MOPS

b)    the operating environment in which the subnetwork is used, such as the number of aircraft served on a given channel

Specifically, ICAO and RTCA/EUROCAE have previously developed SARPs and MOPS for AMSS, VDL Modes 1 & 2 and Mode S mobile subnetworks    Standards for HFDL and VDL Modes 3 & 4 are currently under development and are not addressed by this paper.    The variations in end-to-end subnetwork performance allowed within the SARPs/MOPS for each of these mobile subnetworks and the relationship to the operating environment are summarized in the paragraphs below.    For the purpose of this analysis the following network topology was used.    The ATN end-to-end transit delay is from the Application Entity (AE) in one end system (ES) to the AE in the peer end system.    The sources of delays that contribute to the end-to-end value are depicted in the following figure.    This network topology assumes that the two ground intermediate systems and three subnetworks exist between the ground mobile subnetwork DCE (i.e., ground subnetwork processor) and the ground ES.    The ground ES in this case would typically be an ATC automation system.

### Figure 1 – Network elements that contribute to end-to-end transit delay

Within the ES the ATN communications stack is a source of a transit delay.    Airborne and ground subnetworks that connect an ES to intermediate system (IS) or an IS to an IS are another source of transit delay.    The IS contributes a delay in the forwarding of CLNP packets.    The airborne or ground subnetworks that connect the IS to the air-ground mobile subnetwork processor (i.e., DCE) also introduces delays.    Finally the air-ground mobile subnetwork represents the most significant contributor of transit delays.    The following observations, assumptions and conclusions describe the delay contributions of the network elements.

- End System – The ES will include an ATN protocol stack that supports the internetwork communications service and the upper layer communications service as well as the Application Entity for the desired application(s) (e.g., CPDLC).    While no explicit test measurements have been made for an implementation of the ATN communications stack within an ES, analysis of the test data collected from the ATN internet communication service (SV-5) validation and from the ATN air-ground application validation leads one to conclude that the ES contribution to transit delay will be quite short (e.g., <10 milliseconds) where a the ES employs a modern 32-bit processor.    For this analysis a conservative approach was taken for estimating the transit delay for a packet going through the ES ATN communication services (including ES physical and data link layer processing) within the ES is assumed to be 50 milliseconds.

- Airborne subnetworks – This will vary by aircraft type.    Appendix A provides information on the transit delays associated with the use of the existing and planned data busses and data link layer protocols.    It appears that the transit delay performance of the existing ARINC 429/Williamsburg connection oriented protocol is not well suited for use where short overall end-to-end transit delay is required, such as for ATSC class C traffic type.    The planned connectionless version of the Williamsburg protocol however appears to be applicable to all of currently defined ATSC traffic classes. Implementations not constrained to using Williamsburg protocol, such as would be true for other than air carrier aircraft, could perhaps attain still lower transit delays.    Two cases are included in this analysis.    For the first case Subnet A and Subnet B are assumed to be the preferred configuration providing a transit delay of 100 milliseconds, closely corresponding to the worst case value predicted when using a high speed connectionless Williamsburg protocol (version 3) to deliver a 256 byte message unit (see Appendix A).    The second case considered is the other extreme where ATN is being supported by an aircraft that uses the connection oriented Williamsburg protocol and low speed ARINC 429.

- Airborne Intermediate System – As part of the U.S. FAA’s validation of the ICAO SARPs, Mitre Corp. performed performance tests on the ATN routers they had developed.    For the performance test they used the lowest performance hardware platform used in their ATN test bed (i.e., 80486, 50 MHz based PCs).    Three routers were used in the test configuration and the router under test was routing based on the traffic class specified by the security parameter of the CLNP packet.    Packet rates were varied up to greater than 1000 CLNP packets per second.    Tests were conducted where the specified traffic type exactly matched an available path as well cases where no path was available of the specified traffic type and the router had to find the best available path according to the rules as specified in sub-volume 5 of the ATN SARPs.    The worst case scenario used was when Traffic Class H was specified in the CLNP packet and the only available path supported Class B.    The worst case transit delays through the IS were under 1 millisecond.    Since other implementations could result in lower performance, for the purposes of this analysis the transit delay for a packet to be routed through the IS is assumed to be 5 milliseconds.

- Air-ground subnetwork -    The air-ground (i.e., mobile) subnetworks is by far the largest contributor to transit delay across the ATN.    The following paragraphs summarize the results of simulations and test bed evaluations when the ICAO specified air-ground subnetworks are used.

- Ground IS – As with airborne IS, the transit delay of the ground IS is expected be less than 1 millisecond.    Since other implementation could result in lower performance than measured for the FAA/Mitre test bed router, for this analysis the transit delay for a packet to be routed through the IS is assumed to be 5 milliseconds.

- Ground subnetworks – Ground subnetworks should be sized to satisfy the needs for ATSC.    They may include dedicated point-to-point circuits, wide area networks and local area networks.    For this analysis ground Subnet C, Subnet D and Subnet E were assumed to use wide area networks to provide the connectivity.    A transit delay of 200 milliseconds is assumed for each of these subnetworks.    This is a conservative value since a review of the performance of the U.S. FAA operated NADIN II, X.25 network shows a 147 millisecond transit delay with a 4 node hop.    This network uses relatively slow 56 Kbps truck circuits.    The planned upgrade of NADIN II to use T1 circuits for the backbone is expected to reduce the transit delay to 17 milliseconds for a 4 node hop.

- Ground ES – The same approach has been used as for the airborne ES to account for transit delay.    For this analysis the transit delay for a packet going through the ATN communication services within the ES is assumed to be 50 milliseconds.

Note that estimates for the time required for the application processing, that is outside the scope of the ATN SARPs, is discussed later in this paper.

Table 1 below summarizes the transit times described above.    When the contribution of the air-ground mobile subnetwork is excluded the total ATN end-to-end transit delay is approximately 0.9 seconds for the preferred airborne configuration.    This value is used in the following material to estimate the total ATN end-to-end transit delay when each of the ICAO air-ground mobile subnetworks are used.

Table 1    - ATN End-to-End Transit Delay Contributions (Per Figure 1)

| Network Element | Preferred Configuration - Transit Delay at 95% probability (connectionless Williamsburg & high speed 429) | Retrofit Configuration - Transit Delay at 95% probability (connection oriented Williamburg & low speed 429) |
| --- | --- | --- |
| Airborne ES | 0.050 seconds | 0.050 seconds |
| Airborne Subnetwork A | 0.100 seconds | 1.206 seconds |
| Airborne IS | 0.005 seconds | 0.005 seconds |
| Airborne Subnetwork B | 0.100 seconds | 1.206 seconds |
| Air-Ground Subnetwork | Varies by Subnet | Varies by Subnet |
| Ground Subnetwork C | 0.200 seconds | 0.200 seconds |
| Ground IS | 0.005 seconds | 0.005 seconds |
| Ground subnetwork D | 0.200 seconds | 0.200 seconds |
| Ground IS | 0.005 seconds | 0.005 seconds |
| Ground Subnetwork E | 0.200 seconds | 0.200 seconds |
| Ground ES | 0.050 seconds | 0.050 seconds |
| # TOTAL ATN Transit Delay # (less A-G Subnet Contribution) | # 0.915 seconds | # 3.127 seconds |

3.1.1	AMSS subnetwork currently supports two airborne configurations providing channel rates of 600 bps and 10.5 Kbps respectively.    The end-to-end performance achieved is substantially influenced by the channel rate supported by the given avionics.    Future upgrades to the space segment are expected to enable substantially higher channel rates which can be expected to improve the AMSS end-to-end performance over what can be achieved today.

The relationship between the operating environment and the end-to-end performance offered by AMSS centers around the number of aircraft on a given AMSS channel and the characteristics of the communications traffic (i.e., messages rates, message lengths, message priority, etc.).

3.1.2	The SARPs for the Mode S subnetwork allows variations in both the ground interrogator and the airborne transponder characteristics.    Ground interrogators may vary in the number of aircraft they can serve and the data link capacity offered.    One major variation is the rotation rate of the ground interrogator’s antenna.    Since the primary role of Mode S is to support surveillance, the antenna’s rotation rate is typically selected based on the surveillance requirements in the given airspace.    The extremes for possible implementations of Mode S ground interrogators vary from electronically-scanned antennas with potentially very short (i.e., less than one second) access delay up to en route interrogators with mechanically rotating antennas with rotation periods of    10 to 12 seconds.    On the airborne end the Mode S SARPs and MOPS define 3 levels of transponders with data link capabilities potentially usable with ATN (i.e., level 3, 4 and 5 transponders).    The end-to-end performance can be effected by the transponder level on a given aircraft.

The relationship between the operating environment and the end-to-end performance offered by the Mode S subnetwork centers around the characteristics of the communications traffic (i.e., messages rates, message lengths, message priority, etc.) and the number of aircraft simultaneously within the beam of the ground interrogator’s antenna.    For existing Mode S ground interrogator implementations the throughput and delivery times are relatively constant until the aircraft density increases to a high level.      Beyond this traffic density a gradual increase the transit delay occurs.    Generally, the communications loading for the air-ground applications included in the current ATN SARPs is quite modest compared to the capacity offered by Mode S ground interrogators that have been implemented in the U.S. or are to expected to be implemented elsewhere.    Because of this situation the performance that be provided to a given user is determined, to a large extend, by the level of the Mode S transponder installed and the rotation rate of the ground interrogator’s antenna.    As a result the performance data collected for a single user would be representative of the performance that can be provided to many users receiving the ATS applications via ATN.

3.1.3	The VDL subnetwork currently supports two airborne and ground configurations operating with signaling rates of 2400 bps and 31.5 Kbps respectively. The end-to-end performance achieved is substantially influenced by the signaling rate supported by the given avionics/ground station.    Mode 1 VDL uses the 2400 bps signaling rate and is viewed as having limited applicability to ATSC in the context of ATN air-ground applications.    Mode 2 and the future Mode 3 VDL use the higher 31.5 Kbps.    Mode 2 VDL employes a carrier sense multiple access (CSMA) channel scheme.    The proposed Mode 3 VDL supports both voice and data employing a time division multiple access (TDMA) scheme while the proposed Mode 4 VDL uses an 19.2 Kbps signaling rate and a TDMA access scheme.    Modes 1 and 2 do not support priority handling while future modes are expected to support this feature.    As a result, the end-to-end performance offered by VDL for support of flight safety communications will depend both on the signaling rate as well as the VDL Mode of operation. The AMCP has approved SARPs covering Mode 1 and Mode 2 VDL while Mode 3 and Mode 4 SARPs is still to be finalized.    Therefore, this analysis focuses on Mode 2 VDL as the only mature version of VDL that has the potential capacity to generally support ATS applications.

The relationship between the operating environment and the end-to-end performance offered by the VDL subnetwork centers around the characteristics of the communications traffic (i.e., messages rates, message lengths, message priority, etc.) and the number of aircraft operating on a given VDL channel.    Furthermore, Mode 2 VDL does not support an internal priority mechanism which impacts the ability of the system to segregate one priority of traffic from another on the same channel.    ICAO and some administrations have rules requiring segregation of safety related communications (e.g., CPDLC) from lower priority communications.    One approach to directly addressing priority handling with VDL Mode 2 is with the use of channel(s) dedicated for ATS safety communications.    End-to-end transit delays over Mode 2 VDL will have the general characteristic, exhibited by all CSMA based systems, of showing moderate increases in transit delay as the number of users increases until some critical threshold is reached beyond which transit delay exponentially increase toward infinity.    This occurs at the point where message queues develop.    In order to maintain acceptable performance, VDL Mode 2 will need to be sized to operate at a level of loading such that the peak demands on the system do not result in the onset of message queues.

3.2 Validation Results

The results presented in this working paper summarize the related materials in the referenced documents.    Because of the variations allowed under the existing mobile subnetwork SARPs and the reported performance applies to specific subnetwork configurations and alternative configurations may provide superior or poorer performance. The following overall assumptions have been made for the purposes of this report:

a)	The focus of the report is on CPDLC since this is the most time critical of the initial ATN applications;

b)	The ATS ground end systems (i.e., ATC automation systems) sending/receiving CPDLC messages are only two routers away from the subnetwork access point for the air-ground mobile subnetwork;

c)	Efficient implementations of routers, ground subnetworks, airborne subnetworks, upper layers and ACSE’s will be used;

d)	The number of aircraft operating on the given mobile subnetwork is within the range that subnetwork is designed to support.

3.2.1    Performance with Mode S as the Mobile subnetwork

The end-to-end performance when Mode S is used as a subnetwork is reported in reference 2.    This evaluation employed a U.S. FAA production Mode S ground interrogator with an antenna rotation period of 4.8 seconds.    The airborne Mode S transponder was a level 3 unit from Rockwell Collins and represents the minimum level applicable to ATS communications via the ATN.    An experimental ATN router was operated onboard the test aircraft and a second experimental router was used on the ground.    The airborne and ground end systems included ATN communications software up through transport layer (i.e., TP4), but did not include support for the upper layer communication services.    The airborne router and end system were implemented on a 80486 based laptop computer.    The ground router and end system were each hosted on 80486 based desktop PC’s.    Experimental CPDLC application software, based on RTCA DO-219 messages, generated and received CPDLC messages.    Test software supported data collection for the measurement of achieved performance.    In this scenario a total of 10 CPDLC messages were selected (5 in the uplink direction and 5 in the downlink direction).    Each type of message was sent approximately 10 times.

If we average the times for all uplinks messages we obtain 5.4 seconds as the average end-to-end transit delay.    If we average the times for all downlink messages we obtain 4.8 seconds.      Unfortunately the data from this experiment was not collected in a manner that would make it possible to subsequently determine the transit delay at a 95% probability.

Subsequent measurements of Mode S subnetwork performance were conducted by the FAA Technical Center in September and October 1996. For these tests a commercial level 4 Mode S transponder was procured from Honeywell, a production Mode S interrogator with a 4.8 second antenna rotation rate was used, and experimental Mode S subnetwork processors were used (ADLP and GDLP).    Note that a level 4 Mode S transponder supports downlink extended length messages, while a level 3 transponder does not.    However, the performance for delivering CPDLC message is not effected since all downlink CPDLC messages in this experiment used standard length downlink messages.    The level 4 transponder performance advantage would become evident during IDRP exchanges, TP4 connection establishment and other operations involving relatively long downlink messages.    The avionics was installed on a Convair 580, twin turboprop aircraft for the flight data collection.    Data was also collection with the avionics equipment located in a ground laboratory using a roof mounted antenna with r.f. line-of-sight to the Mode S interrogator.

These experiments used a single CPDLC test message consisting of 7 octets of application data. This was believed to be representative of a typical short CPDLC message.    To this was added a 9 octet TP4 header and 6 octet compressed CLNP header.    The Mode S subnetwork added the compressed ISO 8208 header and data link layer header as defined by the Mode S subnetwork SARPs. The subnetwork performance data was collected over two test runs with a total sample size of greater than 600 messages. Unfortunately, the test configuration used for these tests allowed only for the collection of meaningful data for uplink messages.    Based on the earlier test results, described above, that showed shorter transit delays for downlink messages than for uplink messages, the conservative assumption is made here that downlink transit delays over the Mode S subnetwork are the same as measured for uplink messages.    The specific results from the most recent flight tests produced the following uplink transit delays at 95% probability:

Test run 1 – 335 sample messages - transit delay = 6.18 seconds

Test run 2 – 333 sample messages – transit delay = 5.93 seconds

The larger of these values is used for the following analysis.    Table 2 presents the measured Mode S subnetwork transit delay, at 95% probability, and indicates the estimated additional contribution of the end systems, intermediate systems, ground subnetworks and airborne subnetworks, using the network topology previously shown in Figure 1, and as previously listed in Table 1, to produce the estimated overall end-to-end transit delay over the ATN when Mode S is used as the subnetwork.

Table 2    - ATN End-to-End Transit Delay with Mode S at the A-G Subnet

| Network Element(s) | Preferred Configuration - Transit Delay at 95% probability | Retrofit Configuration – Transit Delay at 95% probability |
| --- | --- | --- |
| Mode S Subnetwork | 6.180 seconds | 6.180 seconds |
| Contribution of Other Network Elements (from Table 1) | 0.915 seconds | 3.127 seconds |
| # TOTAL ATN Transit Delay | # 7.095 seconds | # 9.307 seconds |

With a Mode S ground interrogator having a rotation rate of 4.8 seconds, traffic class C, as defined by the ATN SARPs, is supported for the preferred airborne configuration (i.e., connectionless Williamsburg protocol and high speed ARINC 429).    However, when the retrofit airborne configuration (i.e., connection oriented Williamsburg protocol and low speed ARINC 429) is used the performance drops to supporting traffic class D.      For traffic classes demanding shorter transit delays (e.g., traffic class B) the use of Mode S ground interrogators employing faster rotation rates or electronically scanned antennas would be required.

3.2.2    Performance with AMSS as the Mobile subnetwork

The end-to-end performance when AMSS is used as a subnetwork was reported in Reference 1.    This evaluation employed Opnet simulation models of the AMSS subnetwork and ATN end systems developed by Mayflower Communications, integrated with an Opnet simulation model of the ATN internetwork developed by Mitre Corp. The ATN/AMSS computer model uses the traffic model that was developed by RTCA SC-165 WG-3.    Initial simulation results were presented in October 1995 and updates were later provided to ATNP WG2 in Brussels (March 1996) and Alexandria (October 1996).    Tables 3 and 4 below present the developed traffic model.    For AMSS, one P channel, one T channel, and a variable number of R channels are selected to maintain a desired loading level.

Table 3 - Downlink Traffic Model - Oceanic Flight, Per Aircraft

| Name | Type [Note] | Description | Application Message Length (octets) | Interarrival Time (minutes, mean) | AMSS Priority or % mix | ATN Priority or % mix |
| --- | --- | --- | --- | --- | --- | --- |
| DN1 | ATC[1] | Request | 23 | 54.5 | Q = 11 | 3, 4 |
| DN2 | ATC[2] | Wilco | 3 | 54.5 | Q = 11 | 3, 4 |
| DN3 | ATC[3] | ADS = Basic + Earth Ref. | 17 | 15 | Q = 11 | 3, 4 |
| DN4 | ATC[3] | ADS = Basic + Earth Ref.+Met. | 22 | 60 | Q = 11 | 3, 4 |
| DN5 | AOC[4,5] | Various | mean = 50 | 30 | 0/50/50/0 | 0/50/50/0 |
| DN6 | AAC[5] | Various | mean = 50 | 300 | Q = 5 | 9 |
| DN7 | APC[6] | C-ch signal (air-orig) | N/A | 60 | Q = 9 | N/A |
| DN8 | ALL[7,8] | Background Traffic Exponential Distribution | mean = 160 Truncated [1-600] | 72 | 2/18/60/20 | 2/18/60/20 |

Table 4 - Uplink Traffic Model - Oceanic Flight, Per Aircraft

| Name | Type [Note] | Description | Application Message Length (octets) | Interarrival Time (minutes, mean) | AMSS Priority or % mix | ATN Priority or % mix |
| --- | --- | --- | --- | --- | --- | --- |
| UP1 | ATC[1,2] | ALT Assignment + Xing Restriction + Report Reaching | 16 | 60 | Q = 11 | 3, 4 |
| UP2 | ATC[1,2] | DARPS Route Clearance, 15 WPS | 194 | 480 | Q = 11 | 3, 4 |
| UP3 | AOC[4,9] | Various | mean = 128 | 30 | 0/50/50/0 | 0/50/50/0 |
| UP4 | AAC[9] | Various | mean = 128 | 300 | Q = 5 | 9 |
| UP5 | APC[6] | C-Ch Signal (air original) | N/A | 60 | Q = 9 | N/A |
| UP6 | APC[6] | C-Ch Signal (gnd original) | N/A | 120 | Q = 9 | N/A |
| UP7 | ALL[8,10] | Background Traffic Exponential Distribution | mean = 160 Truncated [1-600] | 81 | 2/18/60/20 | 2/18/60/20 |

Notes:

1.	DN1 precedes UP1 and UP2 by 30 second mean, exponential distribution.

2.	DN2 follows UP1 and UP2 by 30 second mean, exponential distribution.

3.	Every fifth ADS report is a DN4; others are DN3's.

4.	UP3 follows a DN5 by 60 second mean, exponential distribution.

5.	Data length for AOC & AAC downlink messages are generated using a Gaussian process having a mean of 50 octets, s= 283, then truncated for a minimum of 1 octet.    The resultant distribution has a mean of 245 octets.

6.	Call traffic offered load is per 100 passengers for both speech and fax calls, and should be scaled for various passenger loads.

7.	DN8 traffic adjusted by scaling interarrival time to be 20% of the total DN1-DN7 traffic.

8.	Priority mix to be read as percentages, Urgent/Flight Safety/Other Safety/Normal AAC & APC.

9.	Data length for AOC & AAC uplink messages are generated using a Gaussian process having a mean of 128 octets, s=242, then truncated for a minimum of 1 octet.    The resultant distribution has a mean of 248 octets.

10.	UP7 traffic adjusted by scaling interarrival time to be 20% of the total UP1 - UP6 traffic.

The results of the simulations indicate that as expected the end-to-end transit delay for the 10.5 Kbps AMSS service differs substantially from that obtained with the 600 bps AMSS service.    Also the transit delay increases with longer message lengths and for lower priority messages. The following figures present the end-to-end transit delays for 600 bps AMSS service and for the 10.5 Kbps service.    The left figure of each pair is for downlink messages and the right figure is for uplink messages.

Figure 2 -    95 % end-to-end transit delay in seconds, from aircraft (left) and to-aircraft (right) direction for 600 bps AMSS channel rate.

Figure 3 -    95 % end-to-end transit delay in seconds, from aircraft (left) and to-aircraft (right) direction for 10,500 bps AMSS channel rate.

The results of this analysis, when applied to the CPDLC class of messages shows that the downlink transit delays will be much shorter than for the uplink messages.    For the 600 bps AMSS service the uplink plus downlink (i.e., round trip) transit delay will be approximately 105 seconds.    Since the operational significance of the transit delays when applied to CPDLC is the two-way times (e.g., time from when an uplink clearance is issued until the wilco is received), the uplink plus downlink transit delays (at 95% probability could be expressed as an averaged one-way transit delay of approximately 53 seconds.

In a similar fashion, the results of the AMSS simulations predict the downlink plus uplink transit delays for the 10.5 Kbps AMSS service will be approximately 17 seconds which can be expressed as an averaged one-way transit delay (at 95% probability) of 8.5 seconds.

ADS Europe project conducted flight trials using the AMSS.    Initial results from the ADS Europe trials were presented at a January 1997, meeting of WG-A of the ICAO AMC Panel (reference 4).    and subsequently updated for the 10.5 Kbps service (reference 5).    Data was collected for several different cases under the ADS Europe Trials project.    The results varied between these individual exercises.    The reported two-way round trip transit delays for ADS periodic contract (i.e., flight safety priority) messages are summarized in Table 5 below and is typical of the data collected for a specific test configuration.    The values for the 10.5 Kbps channel are from the updated report (reference 5) that was presented to the AMCP.

Table 5    - AMSS Transit Delay at 95% Probability from ADS Europe Trials

| Category | 600 bps Channel Transit Delays (Seconds) | 10.5 Kbps Channel Transit Delays (Seconds |
| --- | --- | --- |
| Round Trip – R channel | 39 | 10.0 |
| Round Trip – T channel | 35 | 12.5 |

These round trip times would equate to an averaged one-way transit delay of approximately 19 seconds for the 600 bps AMSS channel and 10.9 seconds for the 10.5 Kbps service.    In the case of the 600 bps service the ADS Europe Trials transit delays are significantly less than the those presented above from the simulation efforts.    For the 10.5 Kbps service however, the measured transit delays were generally somewhat greater than predicted by the simulations.      However AMSS channel loading will influence the transit delays via AMSS and the channel loading during the ADS Europe trial is unknown.

For the purposes of this working paper we will use the poorer of the simulation or ADS Europe Trails results for the estimation of the AMSS transit delay.    Table 6 presents the total ATN end-to-end transit delay for a network as shown in Figure 1 when AMSS is used as the mobile air-ground subnetwork.    The transit delay contribution of the network elements is as listed in Table 1.

Table 6    -	ATN End-to-End Transit Delay with AMSS at the A-G Subnet for high priority traffic

(A) 600 bps AMSS service

| Network Element(s) | Preferred Configuration - 600 bps AMSS Transit Delay @ 95% probability | Retrofit Configuration - 600 bps AMSS Transit Delay @ 95% probability |
| --- | --- | --- |
| AMSS Subnetwork | 53.0 seconds | 53.0 seconds |
| Contribution of Other Network Elements (from Table 1) | 0.915 seconds | 3.127 seconds |
| # TOTAL ATN Transit Delay | # 53.915 seconds | # 56.127 seconds |

The 600 bps AMSS service using either the the preferred or retofit airborne configuration supports an ATN end-to-end service corresponding to ATSC traffic class H.

(B) 10.5 Kbps AMSS service

| Network Element(s) | Preferred Configuration - 10.5 Kbps AMSS Transit Delay @ 95% probability | Retrofit Configuration - 10.5 Kbps AMSS Transit Delay @ 95% probability |
| --- | --- | --- |
| AMSS Subnetwork | 12.5 seconds | 12.5 seconds |
| Contribution of Other Network Elements (from Table 1) | 0.915 seconds | 3.127 seconds |
| # TOTAL ATN Transit Delay | # 13.415 seconds | # 15.627 seconds |

The 10.5 Kbps AMSS service using the preferred airborne configuration (i.e., connectionless Williamsburg protocol and high speed ARINC 429) supports an ATN end-to-end service corresponding to ATSC traffic class D.    However the retrofit airborne configuration (i.e., connection oriented Williamsburg protocol and low speed ARINC 429) supports an ATN end-to-end service corresponding to ATSC traffic class E.

3.2.3    Performance with VDL Mode 2 as the Mobile subnetwork

Aeronautical Radio (ARINC) and the FAA entered into a cooperative research and development agreement to investigate the performance of VDL when used as a subnetwork of the ATN.    ARINC has developed an Opnet simulation model for VDL Mode 2.    As part of this model the operational environment of the New York area has been modeled.    Step 1 of this study has simulated a single ground station providing VDL coverage to aircraft on the airport surface.    Step 2 of the study covered a New York terminal ATC environment where VDL ground stations are envisioned which would operate co-channel.    Step 3 increased the scope and fidelity of the simulations to include 25 VDL ground stations serving terminal and en route airspace in the U.S. northeast corridor (i.e., North Carolina to Boston and west to Pittsburgh). The aircraft locations were random (i.e., did not model actual traffic patterns).    The communications traffic model employed for the simulation exercises included the ICAO/RTCA defined ATS applications as well as AOC applications.    The ATS message traffic observed during previous FAA data link operational simulations using an air traffic controller team and fight crews was used in terms of message rates.

The simulation model was used to determine the number of aircraft that could be served on a given VDL channel while providing the desired level of performance of 3.0 seconds transit delay, at 95% probability, across the VDL subnetwork in terminal airspace and 5.0 seconds transit delay in en route airspace.    The communications traffic on a per aircraft basis was held constant.    The number of ATN equipped aircraft was varied and the probability of maintaining the transit delay across the VDL subnetwork was determined.    Reference 3 describes the VDL simulation efforts in some detail and presents the results.

3.2.3.1    Airport Surface Domain

As presented in Reference 3, the results of the VDL Mode 2 simulations indicate that on the airport surface 160 aircraft can be served on a single VHF channel while providing a transit delay of 3.0 seconds across the VDL subnetwork at 95% probability.

Table 7    -	ATN End-to-End Transit Delay with VDL Mode 2 Subnetwork on the AIRPORT SURFACE with 150 Aircraft on a VDL Channel

| Network Element(s) | Preferred Configuration - Transit Delay @ 95% probability | Retrofit Configuration - Transit Delay @ 95% probability |
| --- | --- | --- |
| VDL Mode 2 Subnetwork | 3.0 seconds | 3.0 seconds |
| Contribution of Other Network Elements (from Table 1) | 0.915 seconds | 3.127 seconds |
| # TOTAL ATN Transit Delay | # 3.915 seconds | # 6.127 seconds |

With the preferred avionics configuration an ATN end-to-end transit delay of 3.915 seconds is anticipated when accounting for all of the network elements shown in Figure 1.    This corresponds to ATSC traffic class B.    However with the retrofit avionics configuration the ATN end-to-end delay increases to 6.127 seconds which corresponds to a traffic class of C.

3.2.3.2    Terminal Domain

This part of the study covered a terminal ATC environment centered around New York where six VDL ground stations are envisioned which would operate co-channel.      The results of the simulation indicate that 150 aircraft can be supported on a single VDL channel with a 3.0 second transit delay for the case where AOC and ATS both share the same channel.    This increases to 250 aircraft when the ATS safety communications is provided over a dedicated channel.    As noted earlier it may be necessary to segregate ATS safety communications on it own VDL channel(s) in order to comply with ICAO and State requirements for use of the spectrum.    Note these numbers are for the total of all terminal aircraft within all terminal airspace over a relatively large geographic area, not just for a single terminal.

Table 8    -	ATN End-to-End Transit Delay with VDL Mode 2 Subnetwork in Terminal airspace with 250 aircraft on a VDL channel dedicated to ATS Safety communication

| Network Element(s) | Preferred Configuration - Transit Delay @ 95% probability | Retrofit Configuration - Transit Delay @ 95% probability |
| --- | --- | --- |
| VDL Mode 2 Subnetwork | 3.0 seconds | 3.0 seconds |
| Contribution of Other Network Elements (from Table 1) | 0.915 seconds | 3.127 seconds |
| # TOTAL ATN Transit Delay | # 3.915 seconds | # 6.127 seconds |

With the preferred avionics configuration an ATN end-to-end transit delay of 3.915 seconds is anticipated when accounting for all of the network elements shown in Figure 1.    This corresponds to ATSC traffic class B.    However with the retrofit avionics configuration the ATN end-to-end delay increases to 6.127 seconds which corresponds to a traffic class of C.

3.2.3.3 En Route Domain

This part of the study covered the en route ATC environment for the U.S. northeast corridor where 25 VDL ground stations are envisioned which would operate co-channel.      The results of the simulation indicate that 150 aircraft can be supported on a single VDL channel with a 5.0 second transit delay for the case where AOC and ATS both share the same channel.    This increases to 600 aircraft when the ATS safety communications is provided over a dedicated channel.    As noted earlier it may be necessary to segregate ATS safety communications on it own VDL channel(s) in order to comply with ICAO and State requirements for use of the spectrum.    Note these numbers are for the total of all en route aircraft approximate within the airspace of one en route air traffic control center.

Table 9    -	ATN End-to-End Transit Delay with VDL Mode 2 Subnetwork in En Route airspace with 600 Aircraft on a VDL Channel Dedicated to ATS Safety Communication

| Network Element(s) | Preferred Configuration - Transit Delay @ 95% probability | Retrofit Configuration - Transit Delay @ 95% probability |
| --- | --- | --- |
| VDL Mode 2 Subnetwork | 5.0 seconds | 5.0 seconds |
| Contribution of Other Network Elements (from Table 1) | 0.915 seconds | 3.127 seconds |
| # TOTAL ATN Transit Delay | # 5.915 seconds | # 8.127 seconds |

With the preferred avionics configuration an ATN end-to-end transit delay of 5.915 seconds is anticipated when accounting for all of the network elements shown in Figure 1.    This corresponds to ATSC traffic class C .    However with the retrofit avionics configuration the ATN end-to-end delay increases to 6.127 seconds which corresponds to a traffic class of D.

4.	Other Considerations

The above reported transit delay values exclude the input and output processing times. The ICAO ATN Panel used the assumption that these functions would contribute 10% to the overall end user-to-user delivery times.    This may be the case where modern end systems (i.e., host) processors are used and/or for the lower traffic classes where the 10% value represents a significant time (e.g., 5 seconds for traffic class G).    However, existing end system processors, either used for the ground ATC automation or for the aircraft's flight management system may substantially exceed this 10% allocation for the more demanding traffic classes.    Recent estimates by the FAA for the end system processing time required for CPDLC with the FAA's current en route ATC automation system indicate that approximate 4.6 seconds would be required.    Likewise, less capable flight management computers may require perhaps 2 seconds for application processing.    If we assume a combination of input processing by the originator of the application message and the output processing by the recipient of the message will contribute an additional 6 seconds worst case, then this may become a limiting factor for supporting ATS data link services.      However, as the ATC automation systems are upgraded with more modern processors, as is planned in the U.S. before the ATN services would become operational, the application processing times are expected to decrease to values more consistent with those assumed by the ICAO ATNP.    Even with modern automation systems, non-optimal software designs can results in considerable delays in displaying data link message.    Estimates provided by the German DFS indicate that their new ATC automation system may require 2.5 to 3.0 seconds from input of a data link message until the information is displayed to the controller.

5.	Conclusions/Observations

5.1	Properly configured VDL Mode 2 and Mode S subnetworks are probably both technically viable alternatives, in terms of providing adequately short transit delays to support both en route an terminal ATS safety messages.    However, there are other factors that may impact the overall user-to-user message delivery times to the point they will not be acceptable in a given operational environment. The first specific example is the case where aircraft are using the connection oriented Williamsburg protocol and low speed ARINC 429 for the airborne subnetwork to interconnect the VDL avionics to the ATN router and the ATN router to the FMC.    The 2+ seconds of additional transit delay introduced by this configuration, as compared the preferred avionics configuration may preclude it from supporting terminal ATS safety services (i.e., CPDLC).

A second example that represents an even greater variable is the actual end system processing time as this may vary widely depending the capabilities of the ATC automation system and the flight management computer.    If we use a worst case time of 6 seconds for the total application processing (i.e., airborne plus gound) and apply it to the en route case where VDL Mode 2 is used as the mobile subnetwork, the total user-to-user times would become approximately 12 seconds with the preferred avionics configuration and 14 with the retrofit avionics configuration.    This pushes the total user-to-user delivery times to the very threshold of operational acceptability for an en route environment.    For the terminal case where the VDL Mode 2 transit delays are two seconds less, such poor application processing performance would drive the overall user-to-user delivery times to unacceptably long values.

Therefore the conclusion one can draw is that in order to support the most demanding ATS data link applications (i.e., CPDLC in the terminal ATC environment), the airborne and ground application processors must have adequate performance and the preferred avionics configuration would include use of the connectionless Williamsburg protocol and high speed ARINC 429, or an alternative configuration with similar or better performance.

5.2	ATNP WG1 should note the ongoing work of RTCA/EUROCAE on the subject of defining RCP and consider using the forthcoming material from these activities as the basis of defining the required performance of the ATN.

Appendix A

Aircraft Subnetwork Transit Delay using Williamsburg Protocol

| Transit Delays with Existing Williamsburg Connection Oriented Protocol |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Low Speed ARINC 429W Best Case |  |  |  |  | Low Speed Williamsburg |  |  | min (ms) | max (ms) | typ (ms) |  |
|  | data bytes | # words | time (ms) |  | T1 CTS Send Time |  |  | 2 | 100 | 20 |  |
| A. | 0 | 0 | 6 |  | T8 ACK Time |  |  | 2 | 200 | 50 |  |
| B. | 0 | 0 | 6 |  | T13 SOT after CTS |  |  | 2 | 100 | 20 |  |
| C. | 0 | 0 | 1209.32 |  | Word Gap (bits) |  |  | [4] | [64] | [8] |  |
|  |  |  |  |  | Word + WG (bits) |  |  | [36] | [96] | [40] |  |
| Low Speed ARINC 429W Typical Case |  |  |  |  | bit rate (bits/sec) |  |  | 12500 | 12500 | 12500 |  |
|  | data bytes | # words | time (ms) |  | Time (ms) per word/wg |  |  | 2.88 | 7.68 | 3.2 |  |
| A. | 256 | 105 | 426 |  |  |  |  |  |  |  |  |
| B. | 512 | 207 | 752.4 |  |  |  |  |  |  |  |  |
| C. | 1024 | 255 + 159 | 1509.8 |  |  |  |  |  |  |  |  |
| Low Speed ARINC 429W Worst Case (protocol errors excluded) |  |  |  |  |  |  |  |  |  |  |  |
|  | data bytes | # words | time (ms) |  |  |  |  |  |  |  |  |
| A. | 256 | 105 | 1206.4 |  |  |  |  |  |  |  |  |
| B. | 512 | 207 | 1989.76 |  |  |  |  |  |  |  |  |
| C. | 1024 | 255 + 159 | 3984.52 |  |  |  |  |  |  |  |  |
| High Speed ARINC 429W Best Case |  |  |  |  |  | High Speed Williamsburg |  |  | min (ms) | max(ms) | typ (ms) |
|  | data bytes | # words | time (ms) |  |  | T1 CTS Send Time |  |  | 0 | 100 | 20 |
| A. | 256 | 105 | 37.8 |  |  | T8 ACK Time |  |  | 0 | 200 | 50 |
| B. | 512 | 207 | 74.52 |  |  | T13 SOT after CTS |  |  | 0 | 100 | 20 |
| C. | 1024 | 255 + 159 | 154.04 |  |  | Word Gap (bits) |  |  | [4] | [64] | [8] |
|  |  |  |  |  |  | Word + WG (bits) |  |  | [36] | [96] | [40] |
| High Speed ARINC 429W Typical Case |  |  |  |  |  | bit rate (bits/sec) |  |  | 100000 | 100000 | 100000 |
|  | data bytes | # words | time (ms) |  |  | Time (ms) per word/wg |  |  | 0.36 | 0.96 | 0.4 |
| A. | 256 | 105 | 132 |  |  |  |  |  |  |  |  |
| B. | 512 | 207 | 172.8 |  |  |  |  |  |  |  |  |
| C. | 1024 | 255 + 159 | 350.6 |  |  |  |  |  |  |  |  |
| High Speed ARINC 429W Worst Case (protocol errors excluded) |  |  |  |  |  |  |  |  |  |  |  |
|  | data bytes | # words | time (ms) |  |  |  |  |  |  |  |  |
| A. | 256 | 105 | 500.8 |  |  |  |  |  |  |  |  |
| B. | 512 | 207 | 598.72 |  |  |  |  |  |  |  |  |
| C. | 1024 | 255 + 159 | 1202.44 |  |  |  |  |  |  |  |  |

| Transit Delays with future Williamsburg Connectionless Protocol |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Connectionless Williamsburg (Version 3), High Speed - Best Case |  |  |  |  |  |  |  |  |  |  |  |
|  | data bytes | # words | time (ms) |  |  |  |  |  |  |  |  |
| A. | 256 | 105 | 37.8 |  |  |  |  |  |  |  |  |
| B. | 512 | 207 | 74.52 |  |  |  |  |  |  |  |  |
| C. | 1024 | 255 + 159 | 154.04 |  |  |  |  |  |  |  |  |
| Connectionless Williamsburg (Version 3), High Speed - Typical Case |  |  |  |  |  |  |  |  |  |  |  |
|  | data bytes | # words | time (ms) |  |  |  |  |  |  |  |  |
| A. | 256 | 105 | 42 |  |  |  |  |  |  |  |  |
| B. | 512 | 207 | 82.8 |  |  |  |  |  |  |  |  |
| C. | 1024 | 255 + 159 | 175.6 |  |  |  |  |  |  |  |  |
| Connectionless Williamsburg (Version 3), High Speed - Worst Case; excluding protocol errors |  |  |  |  |  |  |  |  |  |  |  |
|  | data bytes | # words | time (ms) |  |  |  |  |  |  |  |  |
| A. | 256 | 105 | 100.8 |  |  |  |  |  |  |  |  |
| B. | 512 | 207 | 198.72 |  |  |  |  |  |  |  |  |
| C. | 1024 | 255 + 159 | 417.44 |  |  |  |  |  |  |  |  |

APPENDIX B

Draft Material from RTCA/EUROCAE on

Required Communications Performance (RCP)

RTCA SC189/Eurocae WG-53

Safety and Interoperability Requirements for

Air Traffic Services (SIR ATS)

- Sub-Group 3 Draft Position Paper Number 3

1.	Introduction	28

1.1.	Purpose	28

1.2.	Introduction to RCP Types	28

2.	Overview of Methodology	28

3.	Performance Templates	31

3.1.	Environment Characteristics	31

3.2.	COMMUNICATION PERFORMANCE	32

4.	RCP Type Selection	34

4.1.	Description of RCP Types	34

4.2.	RCP Parameters	34

Message Delay	34

Message Expiration Time	34

Availability	35

Integrity	35

4.3.	RCP Selection Methodology	35

# Introduction

## Purpose

This paper is intended to provide guidelines for the development of RCP parameter specifications for specific data link communications services or applications.    This methodology begins with the completion of a communications service template.    The template identifies a set of services with their corresponding transactions and messages.    The sequence and timing of the messages is specified in associated message sequence charts as are other characteristics of the service.    The completed template will permit the selection of an appropriate required communication performance (RCP) type from the set of RCP types.

## Introduction to RCP Types

Each RCP-type is identified with a number which represents the end-to-end delivery time in seconds.    This characterization is analogous to the definition of Required Navigation Performance (RNP) which is characterized by a number which represents navigation accuracy in nautical miles.    Each RCP-type carries with it a set of performance parameters.

# Overview of Methodology

The goal of this part is to describe the method which should be followed to formalize credible RCP-types, in order to classify any system, and qualification means for these systems.

We shall describe a step-by-step method, in which we will explain, for each step

- what are the objectives of the considered step,

- which inputs are required,

- who should be involved in the making of the outputs

- how can these inputs be handled to reach our objectives

- which output document is expected.

Step 1 : Performance template

The goal of this step is to provide Regional Groups (PIRGs) with enough pieces of information in order to allow them to fill in the needed performance template, as described in chapter 3.

This output template should contain the required end-to-end performance levels, regarding the considered kind of transaction, with different classes of messages. They will rely, on one hand, on the parameters defined in Position Paper 7 and, on the second hand, on the tables and explanations given in Position Paper 11.

PIRGs and communication service providers will define precisely the transactions for which they are interested in and then make some measurements of performance parameters for these, independently of one another. Levels obtained by the PIRGs should be consistent with the expectations of the operational users and the measurements of the communication service providers.

The output document should be made of the tables given in Position Paper 11, completed with values and explanations.

| Objectives | Give defined performance parameters a coherent value |
| --- | --- |
| Input | Position Paper 7, Position Paper 11 |
| Involved | Regional Groups, ATS providers, communication providers |
| Output | Performance template |

Step 2 : System performance requirements

The goal of this step is to define some performance requirements for the End-to-End system.

The templates filled by the PIRGs express needs for End-to-End transactions. From these needs, SG3 shall propose a method to define and allocate performance criteria for any End-to-End message concerned by these transactions. The table below defines the correspondence between inputs from the performance template and outputs for the system performance requirements.

| Communication performance templates (Needs expressed by PIRGs on transactions) | System performance requirements (Needs expressed by SG3 on messages) |
| --- | --- |
| Required End-to-End transaction delay | Required End-to-End message delay |
| Transaction expiration time | Message expiration time |
| Transaction required time accuracy | Message required time accuracy |
| Area availability | Area availability |
| Link availability | Link availability |
| Integrity | Integrity |

Required End-to-End message delay

This delay can be correlated to the message delays requested in the performance template

Message expiration time

Transaction delay is made of a series of technical transmission delay and human response delay.

We should first define human response delays for each message and then define a theoretical message expiration time as follows :

(Transaction expiration time-å(human response delays)) / (number of messages in the transaction)

For some kind of messages, transmission delay may be longer than the theoretical message expiration time. Thus, a practical message expiration time should be defined as three times the required End-to-End message delay

For a general purpose, we may define message expiration time as the maximum value of theoretical and practical message expiration time.

Message required time accuracy

This accuracy can be correlated to the message accuracy requested in the performance template.

Availability

The unavailability of a message leads to the unavailability of a transaction. So, the availability of a message should be greater than the availability of a transaction, for the same service availability. Message availability could be the Nth-root of the transaction availability, where N is the number of messages in the transaction.

The message availability requested to make a difference between the area availability and the link availability are still to be defined.

Integrity

Message integrity should be the same as the requested transaction integrity in which the message is included.

If a message is part of different transactions with different integrity requirements, the message should have the highest integrity.

| Objectives | Define performance requirements for messages |
| --- | --- |
| Input | Performance template |
| Involved | Sub-Group 3 of EUROCAE WG53/RTCA 189 |
| Output | System performance requirements |

Step 3 : RCP-types

The goal of this step is to provide a table with RCP-types, as described in chapter 4. This table is due to help classification of any system involved in air-ground datalink communications.

This output table will be handled by SG-3, according to the elements that will have been collected in the former step. From these, SG-3 shall define performance levels that will apply to different message classes.

RCP-types table shall include the most significant parameters, chosen by SG3, for each messages class (Personal note: Too many parameters would lead to a non-usable table). These choices and levels should be consistent with the expectations of the operational users and the measurements of service providers (ATS and Telcos).

The output document should be designed as a table in which, for each given RCP-type, a set of performance parameters will be associated. Any system should reach all the performance parameters described in a RCP-type column, in order to be qualified for that RCP-type. These may help to specify some requirements on the end-to-end system.

| Objectives | Define RCP-types |
| --- | --- |
| Input | System performance requirements |
| Involved | Sub-Group 3 of EUROCAE WG53/RTCA 189 |
| Output | RCP table and requirements on the end-to-end system |

# Performance Templates

This chapter provides the data link performance characteristics expected by users. These characteristics, defined by the ICAO Planning and Implementation Regional Groups (‘PIRG), are used by the chapter Methodology as inputs to develop Required Communication Performance (‘RCP) for the end to end data link path for a given communication environment supporting the operational concept.

## Environment Characteristics

The communication performances depend on the data link service's operational environment. Indeed, the traffic density and the volume of exchanged messages that the communication architecture must be able to support have an impact on the message delivery time. As well, the number and type of sub-networks available and the coverage have a direct relationship with the availability of each end to end communication link.

| Characteristics | Unit | Value |
| --- | --- | --- |
| Required Operating Time | Hours/month | xxxx |
| Average Flight Time in the Control Area | Hours | xx |
| Expected Density of Traffic | Aircraft/hours | xxx |
| Peak Number of Expected Exchanged Message | Messages/hours | xxxxx |

Table 3.1-1 : Environment Characteristics Table

The above "Environment Characteristics Table" are used as an input to compute the "Performance Requirement Table" detailed in the methodology chapter.

## COMMUNICATION PERFORMANCE

The data link applications such as for FANS 1/A CPDLC and ADS could be described as Operational Services.    An Operational Service is a set of ATM related transactions. These Transaction sets for communication, navigation or surveillance use data link communications, and are made up of a sequence of messages.

For each services, transaction and messages, communication performance are by PIRGs, in terms of integrity, allowed unavailability, messages delay and condition on failed transaction.

These different parameters presented in the following table are used as an input to compute the "Performance Requirement Table" detailed in the methodology chapter.

|  |  | Time Accuracy |  | Allowed Unavailability |  |  |  | Transaction Expiration Time |  | Messages Performance Requirement |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Service Type | Service Integrity | Time Stamping | Message Recording | Control Area | Aircraft Link | Transaction name & MSC | Allowed Human Factor Time | Allowed Maximum Time | Allowed Failed Transaction note : (2) | Messages Class note : (1) | 95% Required Delay |
|  |  |  |  |  |  | Transaction 1 | xx sec | xxx sec | 10-x | Messages A | xx sec |
| Service xxxx | 10-x | x.x sec | x.x sec | 10-x Hours | 10-x Hours |  |  |  |  | Messages A | xx sec |
|  |  |  |  |  |  | Transaction 2 | xx sec | xxx sec | 10-x | Messages C | xx sec |
| Service yyyy | 10-x | x.x sec | x.x sec | 10-x Hours | 10-x Hours | Transaction 3 | xx sec | xxx sec | 10-x | Messages A | xx sec |
| Service zzzzz | 10-x | x.x sec | x.x sec | 10-x Hours | 10-x Hours | Transaction 4 | xx sec | xxx sec | 10-x | Messages B | xx sec |

Table 3.2-1 : Communication Performance Table

Note 1 : The messages which compose a transaction shall be IAW the SARPS and DO recommended classification.

Note 2 : The Allowed volume of failed transaction may be dictated by the severity classification of the failure when the percentage is exceeded and may follow the Severity and Probability of Effects table below :

# RCP Type Selection

## Description of RCP Types

As discussed in Section 1.2, each RCP type is identified with a number which represents the communications system end-to-end delay in seconds.    This number is the 95 percentile message delay.    That is, for a given RCP type, specified by a number of seconds, 95 percent of the messages must have an end-to-end delay of equal or less than the RCP number in seconds.    This approach recognizes the stochastic nature of message delay and is equivalent to specifying the required navigation performance (RNP) as a number representing the 95 percentile navigation accuracy that must be achieved in a given air space and phase of flight.    As an example, for RCP type 10, 95 percent of the messages are required to have end-to-end delays of up to 10 seconds.    That leaves 5 percent of the messages that are allowed to have delays exceeding 10 seconds.

## RCP Parameters

Because RCP embodies performance requirements other than end-to-end message delay, additional performance parameters are associated with each RCP type.    Each RCP type will have specified availability and integrity parameters, and in some cases a total transaction time, that must be met in addition to the 95 percent end-to-end message delay time designated by the RCP type number.    Table 4-1, below, gives an example of notional RCP parameters associated with some hypothetical RCP types ranging from 0.1 to 60.    Each type has a specified Area Availability, Link Availability, Integrity and, where applicable, a Total transaction time.    The values given are notional but were chosen to represent an assumed hierarchy of requirements that associates higher availability and integrity values with the shorter, and presumably more critical, required message delays.    The total transaction time values are separated to indicate that they are not fixed for each RCP type but should be assigned, when applicable, based on the number of messages comprising the particular transaction to which it applies.

### Message Delay

Table 4-1:    RCP Types – Message Delays

| Message Delay | 0.7 | 1.5 | 2.5 | 5 | 8 | 15 | 20 | 30 | 55 | 110 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### Message Expiration Time

Table 4-2:    RCP Types – Message Expiration Time

| Message Expiration time |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### Availability

#### Area Availability

Table 4-3:    RCP Types – Area Availability

| Area Availability | .999 | .9999 | .99996 |  |  |
| --- | --- | --- | --- | --- | --- |

#### Single Aircraft Availability

Table 4-4:    RCP Types – Single Aircraft Availability

| Single Aircraft Availability | .99 | .999 | .9999 |  |  |
| --- | --- | --- | --- | --- | --- |

### Integrity

Table 4-5:    RCP Types – Integrity

| Integrity | 10-6 | 10-7 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

## RCP Selection Methodology

As briefly discussed in SG3_PP11, the assignment of an RCP type is to be made for each message class, and for each phase of flight (aircraft separation) for which data link service is provided in a given service area.    That is, an RCP value and associated parameters from a defined set, as represented in Table 4-1, should be assigned to each message class to be used for conducting air traffic control or flight information services provided in a particular air space.    A particular combination of aircraft equipment and    terminal or ATC center would then be certified to meet various RCP levels for specified sets of communications environmental conditions (traffic loading,    network configuration, etc.).    When any of the RCP parameters are exceeded by some defined amount, e.g., the 95% message delay reaches 150%    of the specified value, the environment must be adjusted, if possible, so that the required RCP level required for the more critical services can be maintained..    This might be done by restricting less critical message traffic. Otherwise, the actual RCP level for that control area must be down-graded and the consequences of that handled procedurally by increasing aircraft separation or some other action that maintains critical ATC services.    This is analogous to an ILS installation being downgraded from CAT III to CAT II when a level of redundancy is lost and the higher continuity required for CAT III can no longer be met.