ATNP WG-1/7 WP 7-6

15 October 1996

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL

WORKING GROUP 3/SUB-GROUP 2

AIR-GROUND APPLICATIONS

Alexandria, Virginia

7-18 October 1996

Agenda Item 3:  Review And Approval of Input to ATNP/2

Proposal To Complete The Performance Requirements of Sub-Volume 1

(Presented by M.J.A. Asbury)

(Prepared by G. Anderson)

The Automatic Dependent Surveillance Panel Fourth Meeting (ADSP/4) held in Montreal from 9-20 September 1996 approved the forwarding of the ICAO Manual of ATS Data Link Applications for formal ICAO coordination.  This working paper highlights a difference which exists between the work of the ADSP and ATNP in the performance requirements area.

1.	INTRODUCTION

1.1	The ICAO Manual of ATS Data Link Applications was recommended for formal processing under the auspices of the ICAO Secretariat as a result of the ADSP/4 meeting held in September.

1.2	This action results in the adoption of a stable reference from which the ATNP/WG-1 can take guidance.

2.	DISCUSSION

2.1	At the ATNP meeting held in Brisbane, Australia WG-3 and WG-2 had agreed to accept the Performance Requirements output provided by the ADSP WGW held in Dakar, Senegal during March 1996.

2.2	This information was provided to the ATNP meeting held in Brussels.

2.3	The output of the Brussels meeting resulted in Table 3-1 for “Transit Delays For ATSC Classes” in Sub-Volume 1 (SV-1) with values A-H, based on the A-J input received from the ADSP WGW.

2.4	The understanding is that the SV-1 values are consistent with “expected” networks available in the “CNS/ATM-1” timeframe.  This could be construed to mean that the ATNP has defined what the lifetime of the “CNS/ATM-1” package is.  For the Internet or Upper Layers, this may be a true statement, however no comparable statement can be made for SV-1 or the Air-Ground Application SARP’s.

2.5	The values for ATSC Classes A & B are shown as Reserved.  Given the discussion in 2.4, the question could be raised as to what these values are reserved for?

Presumably, the answer is, they are reserved for future growth or new networks on the horizon.

2.6	As the ADSP provides the operational requirements for performance, the ATNP approach could be viewed as not meeting specified requirements for transit delay.

2.7	To complicate the issue, organizations who currently have on-going activities for implementation of ATS Data Link Applications, which use the SARP’s as a requirement of their contract, are having problems in that the implementor cannot select an ATSC Class which meets the stated requirement for a given environment (eg; Approach Control Service).

2.8	Another complication is the RTCA activities to update the MOPS for CPDLC, ADS, and CMA are using the ATSC Class to set end system timers to reflect the operational need to know that a message which arrives beyond a specified time (derived from the ATSC Class established in the initial connection) are old, and that the message may need to be validated with the originator.

2.9	In addition to the above, the performance requirements defined by the ADSP for Availability, Integrity*, Reliability, and Continuity are not addressed in the SARP’s at all.  Where are the technical requirements associated with these parameters found?

*  Integrity figure of 10-8  is found in SV-1 at paragraph 3.28.  Is this meant to be 	an inclusive number for all applications.  It is more stringent than the ADSP 	requirements.

2.10	If the ATNP has a need to identify a level of validation for these requirements, it would seem appropriate that simulation or analysis and inspection could be the level achieved for the ATSC Classes that can not be implemented due to the lack of suitable networks.

3.	RECOMMENDATION

3.1	It is recommended that WG-1 review the tables from the Manual of ATS Data Link Applications provided below and consider whether the approach taken by the ATNP meets the operational requirements.

3.2	Assuming a negative reply, WG-1 is invited to:

a)  adopt the performance requirements as specifed by the ADSP, or

b)  fill in the A & B ATSC Classes with the next two levels specified by ADSP 	values, and

c)  consider the need for the parameters cited in 2.9.

3.3	WG-1 is invited to coordinate the result(s) with WG’s 2 and 3, as appropriate.

Tables From the Manual of ATS Data Link Applications

3A.2	Performance Requirements

3A.2.1	The figures in Tables 3A-1 and 3A-2 reflect the various levels of performance that may be selected for the purpose of providing Data Link services.  Depending on the level of service to be provided, a given State can determine what the performance needs are in a given domain by factors such as the separation minima being applied, traffic density, or traffic flow.

| Performance Levels | Mean End-to-End Transfer Delay (Seconds) | 95% End-to-End Transfer Delay (Seconds) | 99.996% End-to-End Transfer Delay (Seconds) |
| --- | --- | --- | --- |
| A | 0.5 | 0.7 | 1 |
| B | 1 | 1.5 | 2.5 |
| C | 2 | 2.5 | 3.5 |
| D | 3 | 5 | 8 |
| E | 5 | 8 | 12.5 |
| F | 10 | 15 | 22 |
| G | 12 | 20 | 31.5 |
| H | 15 | 30 | 51 |
| I | 30 | 55 | 90 |
| J | 60 | 110 | 180 |

Table 3A-1:  Transfer Delay Performance Requirements

| Application | Availability | Integrity | Reliability | Continuity |
| --- | --- | --- | --- | --- |
| DLIC | 99.9% | 10-6 | 99.9% | 99.9% |
| ADS | 99.996% | 10-7 | 99.996% | 99.996% |
| CPDLC | 99.99% | 10-7 | 99.99% | 99.99% |
| FIS | 99.9% | 10-6 | 99.9% | 99.9% |
| AIDC | 99.996% | 10-7 | 99.9% | 99.9% |
| ADS-B | 99.996% | 10-7 | 99.996% | 99.996% |

Table 3A-2:  Application Specific Performance Requirements

3A.2.2	Except in catastrophic situations, no single end-to-end outage should exceed 30 seconds (end-to-End availability may be achieved through provision of alternate communications routings where feasible).