ATNP WG-1  WP/2-30

AIR/GROUND APPLICATION REQUIREMENTS ON COMMUNICATIONS SERVICES

1	Introduction

1.1	This Flimsy represents a consensus agreement among adhoc participants (list enclosed) concerning ATNP air/ground application requirements on a communication service.

1.2	This Flimsy only applies to CNS/ATM-1 Air/Ground Application SARPS.

2	Requirements/Issues

2.1	General Design Requirements

2.1.1	Transit Delay shall be specified for each application in the SARPS.

2.1.2	Residual Error Rate shall be specified for each application in the SARPS.  All ATNP applications will have the same value.  Other applications (e.g. AOC) may have different values.

2.1.3	Service Loss Reporting shall be specified for each application in the SARPS.  All ATNP applications will have the same value.

2.1.4	Availability shall be specified for each application in the SARPS.

2.1.5	Service Restoration Time shall be specified for each application in the SARPS.

2.2	Message Sequencing

2.2.1	Sequential delivery is required (e.g., where succeeding message delivery is dependent upon the successful delivery of preceding messages).

2.3	Communication Service Termination

2.3.1	The communications service shall provide an orderly termination of service upon indication by the application (e.g. if messages  have been passed to the communications service and then a termination of service is requested, the preceding messages are to be delivered as per normal operations before the service is terminated).  It is noted that the ATN Upper Layer Architecture will provide this service.

2.3.2	Upon failure of orderly termination an indication shall be provided to the application.  It is noted that the ATN Upper Layer Architecture will provide this service.

2.4	Priority

2.4.1	Applications shall specify priority in accordance with ICAO ANNEX 10 (i.e., ITU) regulations.

2.4.2	There shall be a one-to-one relationship between application specified priority and any communication service priorities (e.g. transport layer, network layer, etc.).

2.4.3	Note that for the CNS/ATM-1 package, application specified priority will not necessarily result in a transport layer service, but will provide a queue handling service for the network layer.

2.5	Routing Policy

2.5.1	It is not a valid assumption that lowest practical cost is always required.

2.5.2	The application shall be able to set routing policy based upon best time path, lowest cost path, or no preference.

2.5.3	There is a potential requirement for application specific routing.

2.6	Message Duplication

2.6.1	A message delivered to the communications service shall not be delivered more than once to its peer entity.

3	Action Items

3.1	Provide general design requirement values.  (WG3, Jun 95)

3.2	Develop draft application specific routing proposal.  (Mr. Hennig, 22 Mar 95)

3.3	Provide a range for likely lifetime values.  (WG2, Jun 95)

3.4	Functional Interfaces

3.4.1	Provide draft functional interface description for ATN transport layer and below.  (WG2, Jun 95)

3.4.2	Provide draft functional interface description for ATN Upper Layers.  (WG3, SG3, Jun 95)

4	Participants

Asbury, Mike

Brangier, Francis

Colliver, Forrest

Crenais, Jean-Michel

Crocker, Ken

Day, John

Grout, Jean-Francois

Hennig, Paul

Link, Wes

Murphy, Mike

Overgaauw, Bo

Pearce, Stephen

Snively, Austin

Valentine, Ian