Summary This paper proposes to use safety analyses methods to address the implications of certification during development of Standards and Recommended Practices (SARPs) for the communication, navigation, and surveillance/air traffic management (CNS/ATM) packages. The safety analyses methods would provide substantiation of decisions that directly affect the design of the aeronautical telecommunication network (ATN), the distributed data applications, and the overall system architecture. These decisions would take the form of high-level safety requirements or safety objectives. That part of the safety analyses, which is accomplished during SARPs development, would support the validation of SARPs and the intent is that each state gives credit to safety analyses activities performed at the SARPs level to minimize the safety analyses activities performed for each specific implementation. This paper presents the underlying philosophy for addressing the safety aspects of systems and equipment installed on aircraft and proposes a framework for applying that philosophy to the SARPs activities. These safety aspects apply directly to the airworthiness approval and operational authorizations for aircraft and the commissioning of air traffic services within each state.

1.0	Introduction

This paper proposes to use safety analyses methods to address the implications of certification during development of Standards and Recommended Practices (SARPs) for the communication, navigation, and surveillance/air traffic management (CNS/ATM) packages.  The safety analyses methods would provide substantiation of decisions that directly affect the design of the aeronautical telecommunication network (ATN), the distributed data applications, and the overall system architecture.  These decisions would take the form of high-level safety requirements or safety objectives.  That part of the safety analyses, which is accomplished during SARPs development, supports the validation of SARPs and the intent is that each state gives credit to safety analyses activities performed at the SARPs level to minimize the safety analyses activities performed for each specific implementation.  This paper presents the underlying philosophy for addressing the safety aspects of systems and equipment installed on aircraft and proposes a framework for applying that philosophy to the SARPs activities.  These safety aspects apply directly to the airworthiness approval and operational authorizations for aircraft and the commissioning of air traffic services within each state.

The methods proposed in this paper substantiate decisions made during SARPs development in the context of assessing hazards associated with the communications infrastructure and the distributed data applications for the ATN.  Particular design features, architecture, and development assurance methods can be used to adequately preclude failure modes from contributing to hazards.  The safety analysis would provide substantiation, a means to identify specific safety requirements contained in the SARPs, and ensure adequate treatment of those requirements during implementation (i.e., that there is an acceptable level of assurance that implementations satisfy safety requirements).  Additionally, the safety analysis would ensure that the SARPs provide a viable solution to implementing the ATN and application processes from a certification perspective as systems containing parts that are highly integrated, complex, and non-deterministic may be impractical to certify if their characteristics are not addressed in the overall system architecture.

1.1	Scope

This paper discusses the following topics and proposes a means for their consideration during SARPs development:

a.  Definition of functionality.  Methods for determining minimum performance for safety critical systems.

b.  Definition of integrity and availability.  Methods for considering hazards and failure conditions of functions in safety critical systems.  Determines minimum integrity and availability.

c.  System architecture, design features, and development assurance.  Methods for treatment of complexity and non-determinism in safety critical systems.

d.  “End-to-end” safety and interoperability assessment.  Methods for treatment of aircraft/ground interaction of safety critical systems.

To clarify the intent of the safety analysis, as applicable to SARPs development, examples are included as they relate to the definition of the CNS/ATM-1 package.

1.2	U.S. Activity

The FAA is developing guidelines for conducting safety analyses on ground/space system implementations for systems it owns and operates or services it acquires.

This activity is based on the methods currently employed within the aircraft certification community to evaluate systems and ensure adequate protection against the effects of failure conditions leading to known hazards to the aircraft.  Although the FAA has certified aircraft with data communication capabilities that are currently being used for certain air traffic services (ATS), the Boeing 747-400 Flight Management Computer System (FMCS) Future Air Navigation System One (FANS-1) package, provided the main thrust behind the FAA’s development of notice 8110.50, Guidelines for Airworthiness Approval of Airborne Data Link Systems and Applications, dated April 20, 1994.  The notice provides a means to account for failure conditions, which exist outside the aircraft and which could contribute to the cause of known hazards; however, its scope is limited to the airworthiness approval of the aircraft and only provides a means to support an “end-to-end” safety assessment, not to perform it.  Figure 1-1, taken from notice 8110.50, outlines the scope of the airworthiness approval process and its relationship to other FAA processes associated with the ground/space implementations that interoperate with the aircraft.

Currently, each controlling authority uses different standards and means to implement different parts of the system.  From a total systems perspective, there appears to be a mis-match in the use of these standards.  Table 1 exemplifies the different and incompatible standards that are used within the U.S. for ground-based systems and aircraft systems.  The FAA plans to develop an FAA-STD-xx, which would provide guidelines for performing safety analysis on ground/space implementations and which will be used to negotiate contractual agreements with contractors that are developing parts of an integrated system or providing services for the FAA.  The FAA-STD-xx is intended to ensure compatiility and consistency among each controlling authority.

To support this activity, the FAA is conducting a safety analysis for the advanced oceanic automation system (AOAS).  The safety analysis will initially apply to the ground-based oceanic ATS system with AOAS Build 2 capabilities.  Conducting this safety analysis early in the development process should allow for the allocation of requirements to the AOAS Build 2.  To provide adequate substantiation of safety, assumptions will be made pertaining to parts of the “end-to-end” system that are not part of the AOAS Build 2 design.  It is imperative that the controlling authority validate these assumptions as they apply to their respective air, space, and ground-based systems and ensure that system implementations satisfy the assumptions using system development assurance methods.  It is envisaged that these assumptions will be validated using a consensus process among controlling authorities as part of an “end-to-end” safety and interoperability assessment.

The ATN is intended to provide a seamless worldwide data communication system to implement the FANS CNS concepts.  As the ATN evolves to its full capabilities, it is imperative that an acceptable level of safety and interoperability be maintained.  Therefore, acceptable levels of safety and interoperability must be defined and a means provided to ensure that those levels are maintained.  The specification of such acceptable levels of safety and interoperability in the SARPs provides an elegant solution that ensures consistency in the application of safety analysis throughout the implementation of the ATN.

2.0	Safety and performance considerations for the ATN and CNS/ATM packages

Figure 1-1:  Overview of the airworthiness approval process

Table 1:  Standards and Means Used in the U.S. for Aircraft Systems and Ground-based Systems

| Ground-based System | Aircraft |
| --- | --- |
| Federal Acquisition Regulations (FAR) | Title 14 Code of Federal Regulations (CFR) |
| FAA Standards, Directives, etc. | Advisory Circulars (public comment) |
| FAA makes decisions regarding ground system design for FAA-owned systems. Non-federal systems FAA approved under the provisions of 14 CFR, part 171, are typically done by similarity of FAA owned system. | FAA has statutory authority to regulate aircraft design, but does not make design decisions. |
| FAR do not regulate design decisions for ground system. Design decisions are not regulated. | 14 CFR regulates design decisions. |
| FAA may use service providers to support ground-system functionality. Service level certification is typically done. | Service providers are not part of aircraft certification. |

The ICAO and related ISO standards provide a certain degree of assurance that systems will interoperate with each other.  Today, those standards that apply to the future air navigation system (FANS) communication, navigation, and surveillance (CNS) functions, such as the ATN standards, are not mature enough to provide this assurance.  However, the aviation community is seeking a means to allow evolutionary development of such standards, and use the capabilities provided by implementations of interim standards.  To use these capabilities, it is imperative to assess safety along the migration path, by providing traceability of CNS functionality, which is defined in terms of performance, integrity, and availability, to the intended uses.

The ground-based components and the aircraft components may contain potential error sources that could contribute to a hazard.  Safety analyses substantiate that operational procedures and systems adequately mitigate the hazards.  Currently, the scope of safety analyses are limited to the aircraft or ground-based system domain.  Assumptions must be made about the unknown part with no effective means to validate those assumptions. Ideally, the goal of the activities performed at the standards level should be to maximize the substantiation of high-level safety requirements that are applicable to each implementation.  The safety analysis needs to be defined formally and executed systematically through the development of standards by ICAO and through the implementation of those standards by each state. Achieving this goal would minimize efforts in performing a safety analysis for each implementation of the standard.  This goal can be achieved particularly through the validation of the requirements themselves.

Figure 2-1 portrays the one-to-many relationship between standards development and implementations of those standards and suggests that the safety analysis plays a role in each domain.  Recognize that it would not be possible to provide the complete safety analysis within standards development because of implementation specific dependencies.  These dependencies are addressed by using safety analyses methods to assess system architecture and design features unique to each implementation and employing development assurance methods throughout system development.  Also, it is important to note that within each domain, there is a different controlling authority responsible for performing the safety analysis and specifying safety requirements resulting from the analysis.  As a result, it is necessary to develop a minimal set of conventions and terminology for conducting the safety analyses.  This aspect of the analysis will be discussed further in Section 3.4.

Figure 2-1:  Relationship of safety analysis, standards development, and implementation of standards

2.1	Validation and verification of requirements

It is important to note the distinction between the validation and the verification of the requirements.  There are similarities in the methods or tools used to validate a requirement or verify that a system satisfies that requirement, but the objectives are quite different.  Validation ensures a complete and correct set of requirements.  Verification ensures that the implementations satisfy those requirements.  Given that ICAO SARPs include safety requirements for the ATN, it would seem appropriate to use safety analysis methods to validate those safety requirements to ensure that they are correct and complete.  As each implementation of the requirement will likely be different, the validation of derived requirements specific to each implementation would be necessary, but the validation effort completed for the high level safety requirements defined in the SARPs could be applied to each implementation.  Derived requirements are those that do not map directly to a higher level requirement, but may be safety-related (e.g., a partitioning requirement).  Therefore to minimize the level of effort to validate requirements during implementation, it would be necessary to minimize the creation of derived requirements.  This could be achieved through increased specificity of the SARPs.  For other reasons, it may be advantageous to specify standards in general terms, but from a safety perspective, it complicates the safety analysis by requiring more of the analysis to be applied at the implementation level.  This paper is not necessarily suggesting that SARPs provide specific details of implementations, but it does suggest balancing the specification of requirements with the extent to which one would have to demonstrate the safety of each specific implementation.

3.0	Safety analysis

This section of the paper provides a framework for discussing the safety analysis based on concepts applied within the aircraft certification community.  It suggests how these concepts could be applied to other parts of a system, particularly in cases where the system consists of aircraft and ground-based components, and performs complementary functions.  Emphasis is placed on the role that the safety analysis plays during SARPs development.  The framework could be further developed or modified and applied to the ATN and associated distributed data applications.

For U.S. type certificated transport category aircraft, the regulatory basis for requiring a safety analysis is Title 14 Code of Federal Regulations (CFR), part 25, § 25.1309.  There are specific safety regulations for individual functions that use probabilistic terms.  However, within the U.S., § 25.1309 is a generic rule and applies to aircraft functions for which there is no specific requirement (e.g., controller-pilot data communications).  Other parts of 14 CFR regulate other types of aircraft:  part 23 for normal category; part 27 for normal rotorcraft, part 29 for transport rotorcraft, and part 33 for engines.  The regulatory basis for ensuring interoperability is § 25.1301, performance of intended function.  Attachment 1 contains 14 CFR, §§ 25.1301 and 25.1309.  Within Europe and Canada, similar requirements are contained in Joint Aviation Requirements (JAR), Part 25, and Canadian Airworthiness Manual (CAM), Chapter 523, respectively.

Today, a safety analysis is required for aircraft systems to examine aircraft level functions, identify potential hazards, and classify related failure conditions according to the severity of its effects on safety considering the operational environment.  The operational environment includes the air/ground subnetwork, the ground data communications, the ground portion of the distributed data applications, and operational aspects, such as operational and maintenance procedures.  The safety analysis determines the safety requirements for the aircraft data communication system and the distributed data applications in the context of the safety requirements that define the operational environment.  Therefore, the safety analysis needs to make assumptions about the operational environment to substantiate the classification of failure conditions and to substantiate that the failure conditions are adequately precluded.  For the aircraft systems, the safety analysis interacts with the system development processes to validate the safety requirements allocated to aircraft systems and to ensure that the aircraft implementation satisfies its requirements.

It would be advantageous to employ safety analysis methods to assess hazards of the FANS CNS functions, particularly the communications and surveillance functions, because of the nature of distributed data applications, which consist of aircraft, space, and ground-based components, performing complementary functions.  In these cases, there exists potential error sources in systems located in all the components, error sources that can contribute to hazards associated with the aircraft.  Using safety analysis methods similar to those methods used for aircraft certification would provide a consistent approach to conducting safety analyses and provide a means to assess “end-to-end” safety and interoperability throughout the implementation of the ATN worldwide.

For SARPs development, the safety analysis should substantiate the minimum performance, integrity, and availability required for an operational use.  Minimum performance requirements for a particular function are substantiated by analysis of the operational concept and the role that function plays in the overall concept.  Minimum integrity and minimum availability requirements are substantiated by analysis of the hazards associated with malfunction (e.g., misleading information) or loss of the function, respectively.

The following summarizes this section of the paper:

| Section 3.1 | Presents a means to assess the minimum performance requirements for the ADS and controller pilot communications. |
| --- | --- |
| Section 3.2 | Presents a means to assess the minimum integrity and availability requirements. |
| Section 3.3 | Discusses the use of architecture and design features to address safety aspects. |
| Section 3.4 | Provides a framework for integrating all the pieces of the safety analysis using an “end-to-end” safety and interoperability assessment. |

3.1	Definition of functionality and minimum performance requirements

To support the safety analysis, it is necessary to specify the functionality of the ATN in terms of performance, integrity, and availability requirements, which are based on an operational context.  This can be accomplished by providing a mapping of logical functionality of the ATN to operational uses expected from the implementation of that functionality.  Operational uses of the ATN, however, are part of a complete suite of functionality provided by the FANS CNS model.  Therefore, it is necessary to consider the ATN as part of a FANS CNS model.  This will provide viable options for substantiation of the safety of the ATN; however, it will be necessary to validate any assumptions made about functions that are beyond the scope of the ATN (e.g., navigation functions).  Figure 3-1 suggests a top level model depicting the relationship between communication, navigation, and surveillance functions associated with tactical control of aircraft.  The model is intended to provide a high level framework to further discuss safety analyses concepts associated with defining the ATN functionality.  The safety analysis is integral with the activities associated with defining the function; therefore, it may be necessary to revise the functional definition to accommodate the analysis.

To exemplify the methods of the safety analysis used to substantiate the minimum performance requirements, this paper presents a safety argument for specifying the end-to-end response times for automatic dependent surveillance (ADS) reports and for the corrective action following a discrepancy detected by the surveillance functions.  The analysis requires knowledge about the operational uses of the function.  The analysis assumes an oceanic environment with separation standards of 30 nautical miles laterally and longitudinally.  Note for simplicity of the analysis, vertical separation is not considered. End-to-end response time of the data communications between the controller and the flight crew can be determined based on the time required for an aircraft to depart the allocated airspace in the event an undetected error occurs in the navigation function.  Figure 3-2 provides an illustration of an operational scenario for determining minimum end-to-end response time for controller-pilot communication.

Figure 3-1:  Relationships of the elements of the FANS CNS Model associated with tactical control of the aircraft

Figure 3-2:  Operational scenario for determining minimum end-to-end response time for controller-pilot communication

Certain assumptions must be made to support the analysis, such as the angle at which the aircraft departs its intended flight path, the maximum speed of the aircraft, the point of detection relative to the RNP boundary, and response times of the controller and flight crew.  Table 3-2 presents examples of an analyis that assumes a 30 nautical mile route width, an angle at which aircraft departs its intended flight path of 15 degrees, and indicated airspeed of 600 knots.  The analysis also assumes a required navigation performance (RNP) of 4 nautical miles, which defines the position accuracy requirements for the navigation function to be 4 nautical miles at 95% on a normal distribution curve.  The point of detection for the surveillance functions is determined to occur at 1.2*RNP.  This point of detection was arbitrarily chosen; however, it should be a function of RNP, not interfere with aircraft monitors that are capable of detecting exceedance of RNP, and be at a point in the route to allow sufficient time to contain the aircraft within the allocated airspace defined by the containment boundary and buffer zone.  Based on the time (190 sec) from the point of detection to the point at which the aircraft enters the buffer zone, minimum performance times can be allocated to various parts of the system, including reaction times for the controller and flight crew.

Note that for higher RNP values, such as those that may be applied to inertial navigation systems, the distribution curve is different.  Drift rates over time must be a factor in the analysis.  Also, the track deviation angle is purely an assumption and a more systematic approach could be taken to establish its validity, such as conducting an analysis of error sources that could cause the angle at which the aircraft would depart its intended flight path.  The buffer zone was introduced to account for uncertainties in the engineering analysis and primarily represents a predetermined safety margin.  The minimum response times, shown in Table 3-2, for surveillance and controller-pilot communications are each assumed to be one half of the total time remaining after allocating times to controller-pilot reaction times.

Because the point of detection and the width of the buffer zone are a function of RNP, the minimum communication performance requirements are dependent on minimum navigation performance requirements for a given operational context.  Given the scenario described in Figure 3-2, if, hypothetically, the minimum navigation performance requirement was RNP 6, then 111 seconds would provide the basis for allocating response times.  If the minimum navigation performance requirement was RNP 1, then 308 seconds would provide the basis.  A proper balance between the minimum communication performance requirement and minimum navigation performance requirement could be determined by balancing the capabilities of the technologies providing the functions, the costs associated with implementing the requirement, and other factors.

Table 3-2:  Examples of analysis for determining minimum end-to-end response time for controller-pilot communication

| Assumptions | Value |  |  | Notes |
| --- | --- | --- | --- | --- |
| Route width | 30 nm | 30 nm | 30 nm |  |
| RNP | 1 nm | 4 nm | 6 nm | Based on a family of RNP values: 20, 12.6, 4, 1, (RGCSP) |
| Angle at which A/C departs intended flight path | 15 deg | 15 deg | 15 deg |  |
| Indicated airspeed (IAS) | 600 kts | 600 kts | 600 kts | Speed of aircraft |
| Calculations |  |  |  |  |
| Distance from route centerline to containment boundary | 15 nm | 15 nm | 15 nm | ½ route width |
| Point of detection from route centerline | 1.2 nm | 4.8 nm | 7.2 nm | 1.2*RNP to avoid interference with aircraft monitoring of RNP. |
| Width of buffer zone | .5 nm | 2 nm | 3 nm | ½*RNP |
| Distance from point of detection to buffer zone | 13.3 nm | 8.2 nm | 4.8 nm | Controller intervention should not be before aircraft monitor indicates exceedance of RNP to flight crew. |
| Along track distance from point of detection to buffer zone | 51 nm | 32 nm | 18 nm |  |
| Time to exceed buffer zone from point of detection | 308 sec | 190 sec | 111 sec | This value is allocated to parts of the system |
| Time to detect discrepancy and provide indication to controller | 124 sec | 65 sec | 26 sec | Minimum “end-to-end” response time for ADS, Conflict Probe, and any other automation used to detect and intervene. Assumed to be ½ of the total time remaining after allocating times to controller-pilot reaction times. |
| Time for controller to decide course of action | 30 sec | 30 sec | 30 sec | Time for a controller to determine course of action. Validate using a sample of air traffic controllers. |
| Time to transmit course of action and provide indication to flight crew | 124 sec | 65 sec | 26 sec | Minimum “end-to-end” response time for controller-pilot communications. Assumed to be ½ of the total time remaining after allocating times to controller-pilot reaction times. |
| Time for flight crew to respond to controller commands | 30 sec | 30 sec | 30 sec | Time for flight crew to act on controller’s command. Validate using sample of flight crews. |

The analysis assesses required performance times from a safety perspective.  The surveillance and communication functionality are provided to monitor and protect against the aircraft entering the buffer zone.  There may be other factors that may take precedence over the results from the safety analysis.  Human factors, such as the cognitive and psychological characteristics of the controller and flight crew, may play a significant role in determining minimum performance response times.  For RNP 4, even though the safety analysis indicates a minimum end-to-end response time of 65 seconds for ADS, etc., this may be unacceptable from a psychological perspective, since current radar updates occur at approximately 4-second intervals.  The safety analysis is not intended to provide a means in itself.  Engineering and operational judgment may be necessary to specify more stringent requirements; however, the safety analysis may serve to provide confidence in the validity of these judgments.

3.2	Definition of minimum integrity and availability requirements

The minimum integrity and availability requirements can be derived from an analysis of hazards associated with malfunction (e.g., misleading information) and loss of function.  Failures and design errors in the system contribute to hazards of different severity, for example, a clearance message that the flight crew never received may not be as severe as a corrupted clearance message that misleads the flight crew.  The aircraft certification community classifies failure conditions according to the severity of its effects on the aircraft.  This section presents this classification scheme and suggests modifications to the definitions, to provide for consideration of failure conditions of air traffic services.

Currently, failure conditions are classified according to the severity of the effect the failure condition has on the aircraft.  Title 14 CFR identifies 3 categories; two categories are quoted from 14 CFR, part 25, § 25.1309(b):

(b) The airplane systems and associated components, considered separately and in relation to other systems, must be designed so that-

(1) The occurrence of any failure condition which would prevent the continued safe flight and landing of the airplane is extremely improbable, and

(2) The occurrence of any other failure conditions which would reduce the capability of the airplane or the ability of the crew to cope with adverse operating conditions is improbable.

The third category is implied - the occurrence of any failure condition that has minor effects is probable.

The FAA, Aircraft Certification Service, has published an advisory circular (AC) 25.1309-1A, which provides guidance for how to interpret Title 14 CFR, part 25, § 25.1309.  The European Joint Aviation Authorities (JAA) have published advisory material joint (AMJ) 25.1309, which is nearly equivalent and is used throughout Europe.  AC 25.1309-1A maps the three categories described in the rules into 4 categories: catastrophic, severe-major (or the JAA use the term “hazardous”), major, and minor.  The four categories evolved because the range of probability of failure conditions classified as “improbable” was too broad  (e.g., in numeric terms 10-5 > improbable > 10-9 per flight hour).  This AC together with the rule itself are currently under review by the Systems Design and Analysis Harmonization Working Group (SDAHWG) chartered under the FAA-JAA Harmonization Work Program and the Aviation Regulatory Advisory Committee (ARAC).  Based on current directions of the SDAHWG and other industry committees, the following categories are suggested:

| Figure 3-3 provides the relationship between the failure condition classification and the minimum requirement for its probability of occurrence and development assurance level for systems that implement the function. The minimum integrity requirement is determined based on the classification of failure conditions that can cause malfunction. For data communications, FAA notice 8110.50 has identified the following failure conditions that can cause malfunction: “ATS messages received out of sequence, errors in the ATS message address, and errors in ATS messages.” The notice classifies these failure condition effects on the aircraft as “major.” This translates to a minimum integrity requirement of 10-5 per flight hour probability. The requirement would apply to parts of the system, whose anomalous behavior would contribute to the cause of these failure conditions. The minimum availability requirement is determined based on the classification of the failure condition, “loss of function.” For ATS that use data communications, the FAA notice classifies this failure condition as “minor,” provided “the operational authorizations continue to require alternative communication systems that meet current operating rules.” This translates to a minimum availability requirement of 10-2 per flight hour probability. Catastrophic | Failure conditions which prevent continued safe flight and landing of one or more aircraft |
| --- | --- |
| Hazardous | Failure conditions which would reduce the capability of the aircraft, air traffic services, or the ability of the flight crew or controller to cope with adverse operating conditions to the extent there would be: |
|  | · A large reduction in safety margins, functional capabilities, or airspace separation |
|  | · Physical distress or higher workload such that the flight crew or controller cannot be relied upon to perform their tasks accurately or completely |
|  | · Serious or fatal injury to a relatively small number of aircraft occupants |
| Major | Failure conditions which would reduce the capability of the aircraft, air traffic services, or the ability of the flight crew or controller to cope with adverse operating conditions to the extent that there would be, for example, a significant reduction in safety margins or functional capabilities, a significant increase in flight crew or controller workload or in conditions impairing flight crew or controller efficiency, or some discomfort to aircraft occupants, possibly including injuries. |
| Minor | Failure conditions which would not significantly reduce safety, and which involve flight crew or controller actions that are well within their capabilities. Minor failure conditions may include, for example, a slight reduction in flight crew or controller workload, such as routine flight plan changes, or some inconvenience to aircraft occupants. |
| No effect | Failure conditions which do not affect the operational capability of the aircraft, air traffic services, and do not increase flight crew or controller workload. |

Because of the subjective nature of the definitions, to aid in the classification of particular failure conditions, Table 3-3 provides a decision matrix based on the attributes of a failure condition.  These attributes are:

a.  Consequences that the failure condition has on the aircraft or the air traffic service

b.  Indication of the failure condition to the flight crew or the air traffic controller

c.  Intervention capabilities

d.  Reconfiguration capabilities

Based on these attributes, one could provide a more precise definition of the failure condition classification, particularly between the major and hazardous category.  The matrix could also be used during system development to reduce the classification of a failure condition by specifying requirements that would change its characteristics.

Figure 3-3:  Relationship between failure condition classification and the minimum requirement for its probability of occurrence and the development assurance level for systems that implement the function.

| Effect on aircraft or air traffic services | Normal | Nuisance | Operating limitations; emergency procedures | Significant reduction in safety margins; difficult for flight crew or air traffic controller to cope with adverse conditions; passenger injuries | Large reduction in safety margins; flight crew or air traffic controller extended because of workload or environmental conditions; serious or fatal injury to a small number of occupants | Multiple deaths, usually with loss of aircraft |
| --- | --- | --- | --- | --- | --- | --- |
| Classification of Failure Conditions | <---No effect---> | <-----------------Minor-------------> |  | <--Major--> | <Hazardous> | <Catastrophic> |
| U.S. probabilistic terms | <---------------------------Probable-----------------------> |  |  | <--------------Improbable------------> |  | Extremely <-----------------> Improbable |
| European probabilistic terms | <--------------Frequent--------------> |  | Reasonably <----------------> Probable | <----Remote---> | Extremely <-----------------> Remote | Extremely <-----------------> Improbable |
| Integrity requirement | <100 | <10-2 | <10-3 | <10-5 | <10-7 | <10-9 |
| Development assurance level | E | D |  | C | B | A |
| Note: The value signifies probability of undetected error per flight hour. The minimum integrity requirement is associated with the effect of failure conditions that would cause malfunction. The minimum availability requirement is associated with the effect of loss of function. |  |  |  |  |  |  |

3.3	System architecture and design features

As mentioned earlier, the safety analysis is integral to system development.  Functions are defined and associated hazards and failure conditions are identified and classified.  The functions are allocated within a system architecture based on the severity of the hazards associated with the function.  The system architecture and certain design features can be used to minimize the complexity of parts of the system performing safety critical functions.  The classification scheme provides a means to identify which parts of the system are critical.  Based on the safety analysis, it may be more cost effective to use one system architecture instead of another.  Where the system architecture and/or design features do not provide adequate protection against the anomalous behavior of certain parts of the system, then assurance must be derived through rigorous development methods to ensure adequate protection.  Also, for those parts of the system that are considered complex and non deterministic, development assurance methods may not be practical, particularly in cases where the anomalous behavior of those parts could contribute to the more severe failure conditions.

An example of the use of system architecture and design features to address safety aspects is the requirement for an integrity algorithm as part of the communication infrastructure for the ATN:  the cyclic redundancy check (CRC) implemented at the link layer and the Fletcher’s checksum implemented at the transport layer.  This feature of the ATN is based on optimizing the performance of the network.  From a communication performance perspective, it makes sense to employ an integrity algorithm that is most appropriate for the types of errors one is looking for, and implement the integrity algorithm as close to the error source as possible.  From a safety perspective, one is trying to minimize the complexity and non-determinism of the safety-critical parts of the system, and the system development assurance methods.  Therefore, it makes sense to implement an integrity algorithm as close to the user of the data as possible.  Otherwise, the integrity algorithm specified by the ATN standards will not detect anomalous behavior of the distributed data applications and the human computer interface and, therefore, must be addressed by each specific implementation.

Table 3-3:  Failure condition classification decision matrix

| Attributes of failure condition | Scenarios |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | S1 | S2 | S3 | ... | Sn |
| Aircraft/Air Traffic Services Consequences |  |  |  |  |  |
| Safe flight and landing jeopardized | X |  |  |  |  |
| Operationally, significant functional capability lost, or increased risk of major equipment damage |  |  |  |  |  |
| Operational efficiency reduced or system redundancy compromised |  | X |  |  |  |
| No operational impact, but dispatch requirements impact |  |  | X |  |  |
| Failure Condition Indication |  |  |  |  |  |
| Masked or misleading condition not readily determinable, or determinable only through subtle indications of performance changes, or evident only to persons having unique knowledge. | X |  |  |  |  |
| Implicit condition clearly determinable from indications used in normal operation or from readily sensible changes in performance |  |  |  |  |  |
| Explicitly unique indication provided specifically identifying the condition or intervention requirement |  | X | X |  |  |
| Pilot/Controller Intervention Alternatives |  |  |  |  |  |
| None, no intervention alternatives provided or possible | X |  |  |  |  |
| Excessive, continuous and unusual skilled task or decision making without direct procedural or training support |  |  |  |  |  |
| Demanding, continuous exercise or routine skilled task or extended decision making with procedural and training support |  |  |  |  |  |
| Routine, brief skilled task or simple decision making with procedural or training support |  |  |  |  |  |
| Dispensability decision only, no workload impact |  | X | X |  |  |
| Reconfiguration Capability |  |  |  |  |  |
| No reconfiguration possible | X |  |  |  |  |
| Manual reconfiguration possible |  | X |  |  |  |
| Automatic reconfiguration active |  |  | X |  |  |
| Decision alternatives |  |  |  |  |  |
| Catastrophic | X |  |  |  |  |
| Hazardous |  |  |  |  |  |
| Major |  |  |  |  |  |
| Minor |  | X |  |  |  |
| No Effect |  |  | X |  |  |

Guidelines for software development assurance methods, according to software level, are contained in RTCA DO-178B/EUROCAE ED-12B, Software Considerations in Airborne Systems and Equipment.  Software level is defined based upon the contribution the anomalous behavior the software has to the potential failure condition.  As such, the level of effort required to show that software complies with certification requirements varies with the failure condition category.  There is no requirement to reduce the amount of software whose anomalous behavior contributes to a failure condition; however, placing emphasis on the use of system architecture and design features to mitigate the hazard makes sense, particularly when considering distributed data applications.  For example, the use of the Fletcher’s checksum in the transport layer alleviates the concern for undetected corruption of data messages caused by software used in the ground networks.  This eliminates the need for certification requirements on softwre in ground networks, which are considered complex, both from a technical and institutional standpoint.

The system architecture and design features of the ATN may be directly applicable to substantiating the safety aspects of the system.  It may also be desirable to identify, in the SARPs for the ATN, system architecture and design features related to safety to minimize certification requirements for development assurance methods.

3.4	“End-to-end” safety and interoperability assessment

Figure 2-1 suggests that safety analyses methods, currently being applied to systems installed on aircraft at the time of implementation may be applied during ICAO standards development.  This part of the safety analysis would validate safety requirements identified in the ICAO SARPs and provide a consistent approach to the safety analyses performed later during implementation of the standards.  In Section 3.1, the paper discusses the need to define functions in terms of minimum performance, integrity, and availability required for a specific operational use.  Because of the inherent nature of distributed data applications and the ATN and the relationship of communication performance requirements to the navigation performance requirements, the definition of functionality and the safety analysis itself logically (e.g., C,N,S) and physically (e.g., aircraft, space, ground-based) transcend any one piece of the system.  Section 3.0 discusses the safety analysis as integral to the system development process, yet there is a different controlling authority responsible for implementation of each part of the system and for each safety analysis associated with that part of the system.  As a result, there is a need for a consistent approach to the safety analyses.  The use of a consistent approach would provide a means for each piece of the safety analysis to support an “end-to-end” safety and interoperability assessment.

Figure 3-4 shows the relationship of the “end-to-end” safety and interoperability assessment to the safety analyses, systems development process, and approval process for each part of the system.  The assessment would be commence at the start of a change development activity that would require a revision to assumptions made during the original safety analyses about other parts of the system or the operational context.  A change to operational context would include, for example, a change in separation criteria, but the system would not necessarily change.  The “end-to-end” safety and interoperability assessment would provide a means to ensure continued operational safety.

The “end-to-end” safety and interoperability assessment should be performed by the controlling authorities responsible for each part of a specific system.  The ICAO SARPs could provide recommended practices for conducting the “end-to-end” safety and interoperability assessment.  This “end-to-end” assessment would be considered part of implementation of the standards and would ensure that the operational requirements specified for the system are complete and correct (i.e., validate requirements).  The operational requirements include the safety requirements, which are derived from safety analyses and interoperability requirements, which are derived from international standards (e.g., ICAO, ISO).  More specifically, the “end-to-end” safety and interoperability assessment would:

a.  Rely on substantiation provided by each of the controlling authorities for its piece of the system, but ensure that the assumptions in each safety analysis made about other parts of the system were, in fact, valid.

Figure 3-4:  Relationship of safety and interoperability assessment to safety analyses, systems development, and approval for each part of the system

b.  Provide a two-way conduit across the domains to resolve inconsistencies.

c.  Maintain distinctions between domains to ensure flexibility during the implementation of the standards.

d.  Accommodate change activity, inherent in an evolutionary development, rather than prohibit changes.  For example, the findings of this assessment may provide substantiation for changes to the subsequent CNS/ATM packages.

4.0	Recommendations

This paper presents the methods of safety analysis currently used within the aircraft certification community and proposes modifications to the definitions to allow the use of these methods to substantiate the minimum performance, integrity, and availability requirements for CNS functions.  The working groups of the ATNP should consider a joint effort with the certification community and implementers to develop these methods further and apply these methods to the development and validation of the ICAO SARPs for the CNS/ATM-1, CNS/ATM-2, and CNS/ATM-3 packages.  This effort reduces the certification effort and provides a systematic approach to ensure safety during the evolutionary development of the ATN in operational service.  The following summarizes specific recommendations discussed in this paper:

a.	Assess the safety of the functionality of the CNS/ATM packages from a total system perspective and develop guidelines and recommended practices for conducting safety analyses to ensure a consistent approach among the different controlling authorities involved in the deployment of a complete system.  These guidelines should be described in the ICAO SARPs and include tangible and recognizable products as outputs of the safety analysis.

b.	Specify, in the ICAO SARPs for CNS/ATM packages, safety requirements in terms of minimum performance, integrity, and availability requirements for each function of the CNS/ATM-1 package.  The minimum requirements should be derived from the results of the safety analysis to validate the CNS/ATM functions in the context of a FANS CNS model.  Table 3-4 proposes a structure for specifying these minimum requirements.  The contents of the structure are shown for clarity only and may not necessarily reflect expected operational uses.  The structure, however, is intended to show the mapping of operational uses to CNS/ATM functionality and the transition from one CNS/ATM package to another.

c.	Continue the U.S. efforts to conduct a safety analysis for the Advanced Oceanic Automation System (AOAS) and encourage other states to engage in similar activities to ensure that operational systems satisfy safety requirements identified by appropriate safety analyses.  Identify the need for an “end-to-end” safety and interoperability assessment performed by controlling authorities responsible for implementing various parts of a system that will satisfy the standards and requirements of the CNS/ATM-1 package.  This assessment should validate the assumptions made during the safety analysis, address specific implementation issues that pertain to the end-to-end safety and interoperability, and identify specific issues with the CNS/ATM-1 standards for consideration in future CNS/ATM packages.

| Low Volume (Current) | 100 nm/15 min/2000 ft PANSRAC DOC4444 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| High Density (Current) | 60 nm/100 min/2000 ft MNPS/NAT |  |  |  |  |  |  |  |
| High Density CENPAC/NOPAC (Current) | 50 nm/10 min/2000 ft Composite (Pacific) |  |  |  |  |  |  |  |
| Lateral: Flex Tracks High Density Longitudinal High Volume (Current) | 60 nm/50 nm/2000 ft Australian RNAV Tasman Sea |  |  |  |  |  |  |  |
| Airspace Access Lateral Flex Tracks DARPS Random Route Vertical 2000 ft Climb | 30 nm/30 nm/1000 ft | CNS/ATM-1 CPC ADS FIS (ATIS) inter-center coord message handling ATN Subnetwork |  |  |  |  |  |  |
| Airspace Access Lateral Flex Tracks DARPS Random Route Vertical 2000 ft Climb | 15 nm rad/1000 ft reduced vertical | CNS/ATM-2 |  |  |  |  |  |  |
| Airspace Access Lateral DARPS Random Route Dynamic Route Vertical Cruise Climb | 10 nm rad/1000 ft | CNS/ATM-3 |  |  |  |  |  |  |
| Airspace Access Lateral Random Route Dynamic Route Vertical Cruise Climb | 5 nm rad/1000 ft | CNS/ATM-4? |  |  |  |  |  |  |

Title 14, Code of Federal Regulations (CFR), part 25

25.1301 Function and installation.

Each item of installed equipment must-

(a) Be of a kind and design appropriate to its intended function;

(b) Be labeled as to its identification, function, or operating limitations, or any applicable combination of these factors;

(c) Be installed according to limitations specified for that equipment; and

(d) Function properly when installed.

Section 25.1309 Equipment, systems, and installations.

(a) The equipment, systems, and installations whose functioning is required by this subchapter, must be designed to ensure that they perform their intended functions under any foreseeable operating condition.

(b) The airplane systems and associated components, considered separately and in relation to other systems, must be designed so that-

(1) The occurrence of any failure condition which would prevent the continued safe flight and landing of the airplane is extremely improbable, and

(2) The occurrence of any other failure conditions which would reduce the capability of the airplane or the ability of the crew to cope with adverse operating conditions is improbable.

(c) Warning information must be provided to alert the crew to unsafe system operating conditions, and to enable them to take appropriate corrective action. Systems, controls, and associated monitoring and warning means must be designed to minimize crew errors which could create additional hazards.

(d) Compliance with the requirements of paragraph (b) of this section must be shown by analysis, and where necessary, by appropriate ground, flight, or simulator tests. The analysis must consider-

(1) Possible modes of failure, including malfunctions and damage from external sources.

(2) The probability of multiple failures and undetected failures.

(3) The resulting effects on the airplane and occupants, considering the stage of flight and operating conditions, and

(4) The crew warning cues, corrective action required, and the capability of detecting faults.

(e) Each installation whose functioning is required by this subchapter, and that requires a power supply, is an ‘’essential load’’ on the power supply. The power sources and the system must be able to supply the following power loads in probable operating combinations and for probable durations:

(1) Loads connected to the system with the system functioning normally.

(2) Essential loads, after failure of any one prime mover, power converter, or energy storage device.

(3) Essential loads after failure of-

(i) Any one engine on two-engine airplanes; and

(ii) Any two engines on three-or-more-engine airplanes.

(4) Essential loads for which an alternate source of power is required by this chapter, after any failure or malfunction in any one power supply system, distribution system, or other utilization system.

(f) In determining compliance with paragraphs (e) (2) and (3) of this section, the power loads may be assumed to be reduced under a monitoring procedure consistent with safety in the kinds of operation authorized. Loads not required in controlled flight need not be considered for the two-engine-inoperative condition on airplanes with three or more engines.

(g) In showing compliance with paragraphs (a) and (b) of this section with regard to the electrical system and equipment design and installation, critical environmental conditions must be considered. For electrical generation, distribution, and utilization equipment required by or used in complying with this chapter, except equipment covered by Technical Standard Orders containing environmental test procedures, the ability to provide continuous, safe service under foreseeable environmental conditions may be shown by environmental tests, design analysis, or reference to previous comparable service experience on other aircraft.