ATNP/WG1

IP/1

9 to 13 October 1995

AERONAUTICAL TELECOMMUNICATION NETWORK PANEL

WORKING GROUP 3

U.S. Federal Aviation Administration Support for OSI, ATN and IPS Protocols

Presented by Leon Sayadian (FAA)

Prepared by the Center for Advanced Aviation System Development (CAASD)

SUMMARY This paper describes the current U.S. Federal Aviation Administration (FAA) plans to support OSI and ATN protocols for wide area applications and IPS protocols for local area applications. The FAA is presently investigating the expansion of support for IPS in internal local area networks. However, this is contigent upon IPS studies and the approval of the applicable standards.

1.0 BACKGROUND

Based on the current and anticipated FAA architecture, it is expected that both Transmission Control Protocol /Internet Protocol (TCP/IP) and International Standards Organization (OSI) networks will continue to exist in the future.  Based on events that have occurred in 1994, the FAA has decided to re-examine the use of IPS data communications protocols in internal networks. This does not mean that the FAA will not continue to support ATN systems. On the contrary, the FAA is commited to communicate with external ATN equipped aircraft and external ground systems using OSI ATN protocols.

2.0 DISCUSSION

Federal Aviation Administration (FAA) policy for network data communication protocols was originally established in 1987 and was driven by the policies documented in the U.S. Government Open System Interconnection Profile (GOSIP). U.S. GOSIP policy stated that government data communication systems would utilize a specific set of International Organization for Standardization (ISO) protocols, that complied with the Open Systems Interconnection (OSI) Reference Model for data communication.  This policy was adopted with the intention of taking advantage of a competitive marketplace for commercial products that adhered to the ISO standards.  However, while a limited number of OSI products were available, they failed to gain a significant portion of the commercial market.  The U.S. commercial marketplace was dominated by products implementing the Internet Protocol Suite (IPS).

In 1994, in response to a growing recognition of the realities of the commercial trends in data communications, the U.S. Federal Internetworking Requirements Panel (FIRP) was convened to re-examine U.S. policies for data communication systems.   As stated in the FIRP Report, the FIRP was chartered to study issues and recommend actions, which the Federal Government can take to address the short and long term issues of internetworking and convergence of networking protocols - particularly the Internet Protocol Suite and the Open Systems Interconnection (OSI) protocol suite, and proprietary protocols.  The draft FIRP recommendations were completed in January 1994 and a 30 day public comment on the document was provided.  The final report was released on 31 May 1994.  The FIRP concluded that there is not a single technology or protocol solution to satisfy the Federal internetworking requirements, and recommended that the current GOSIP policy be replaced with a new Federal Information Processing Standards Publications (FIPS) that includes appropriate standards drawn from both the OSI and IPS protocol suites.  The recommendations of the FIRP were accepted, and on 16 September 1994 the U.S. National Institute of Standards and Technology (NIST) drafted the Profiles for Open Systems Internetworking Technologies (POSIT) Federal Information Processing Standards Publications (FIPS PUB 146-2).  FIPS 146-2 modified FIPS 146-1 by removing the requirement that Federal agencies specify GOSIP protocols when they acquire networking products and services and communications systems and services.  On November 1994, GOSIP was formally superseded by POSIT.

In July 1994, the Internet Engineering Task Force (IETF) also completed an examination of a set of network layer protocols to provide the basis of the Internet Protocol version 6 (IPv6).  The IETF rejected the use of  ISO 8473, and begun specification development based on a set of agreed assumptions.  The IETF has completed the IPv6 specification, and has provided the following enhancements over IP version 4:

=	IPv6 expands the address space to 16 octets, and optionally allows expansion to 20 octets

=	IPv6 contains inherent support for mobility and resource reservation

=	IPv6 contains inherent support for security mechanisms.

Since these events occurred, the internet and ISO standards communities have begun working together towards covergence of the internet and ISO standards.  In particular, the IETF and ISO have started working together towards harmonization of the OSI and TCP/IP networking layers. At the July 1995 IETF Internet and OSI Harmonization Birds of a Feather (BOF) meeting, ISO/IEC JTC1/SC6 goals were presented: convergence of OSI and TCP/IP networking layers, IPv6 and CLNP coexistence, and the adoption of IETF Request for Comment (RFC) 1006 (This RFC proposes that the ISO/IEC 8073 Transport Protocol Class 0 be supported on top of TCP/IP).  As an outcome from this meeting, SC6 was requested to apply its attention to the use of OSI Transport over both the traditional IP and the new IPv6.  IETF members also invited to help in solving some problems which exist when operating the OSI convergence standard RFC1006 over TCP/IP.

In addition, the Internet Society has established a liaison relationship with International Telecommunications Union (ITU)-T.  In July 1995, the ITU-T Executive Council agreed to accept the Internet Society as a Recognized Private Operating Agency (RPOA) and, likewise, the ITU-T will become members of Internet Society for a trial period of one year, during which the fees on both sides will be waived.  The objective is to parallel the liaison which is already in progress between ISO/IEC and the Internet Society and to continue progress on the convergence of future Internet and OSI protocols as started by SC6.

Given the direction that the commercial network data communication market and the results of the FIRP, the FAA is examining the use of IPS for internal data communications to benefit from the large commercial marketplace that currently exists for IPS products.  In parallel, the FAA will adhere to International Civil Aviation Organization (ICAO) requirements and implement ATN t