ATNP WG1 / WP3-18

AERONAUTICAL TELECOMMUNICATION NETWORK PANEL (ATNP)

Working Group 1 - Systems Planning and Concepts

Third Meeting

BANFF,  9 - 13 October 1995

Agenda Item :

USE OF OPTIONAL DATA FIELD (ODF) FOR ATN TRANSITION PURPOSES

(Presented by )

1.	Introduction

According to ATNP WG1 meeting  held in France from 21 to 24  March  1995, the meeting recognized that the use of ODF procedures will be beneficial for ATM systems during AFTN/ATN transition period.  To achieve this purposes, ODF procedures should be developed and the amendment to Annex 10, Volume II should be proposed to be made accordingly.

2.	Background

2.1	At present, there are five options of the ODF procedures that have been developed.  Option 1 of the ODF has been approved by the ASPP and presented during the first meeting of the ATNP.

2.2	Option 2 to 5 were used for conveying message/data unit identification,   message/data unit reference information,   message/

data unit time stamp information and message/data unit Cyclic Redundancy Check (CRC) information, respectively.  These options have been considered by the AFTN experts in the ASIA/PAC Regions and have been included in the ASIA/PAC Draft AIDC ICD to support ATN transition in the region.

3.	Discussion

3.1	The use of ODF procedures for conveying the needed information such as   message/data unit identification ,   message/data  unit

A-1

reference information, time stamp information and CRC information will be useful for data exchange between the ATM Systems during AFTN/ATN transition.

4.	Recommendation

The Meeting is invited to consider the attached working paper to be submitted to the second ATNP Meeting.

END

ATNP/2 - WP/

/    /96

AERONAUTICAL TELECOMMUNICATION NETWORK PANEL (ATNP)

Second Meeting

Montreal,  1996

Agenda Item :  To be supplied by secretariat

USE OF OPTIONAL DATA FIELD (ODF) FOR ATN TRANSITION PURPOSES

(Presented by the Rapporteur)

1	Proposal

1.1	It is proposed that the Panel recommend the amendment to Annex 10, Volume II, as included in the appendix of this working paper.

2	Purpose

2.1	The purpose of the proposed recommendation in the paper is to show various options on uses of ODF for conveying the needed information between the ATM Systems.

3	History

3.1	The need for using OPTIONAL DATA FIELD (ODF) to convey the information for data exchange between the ATM systems was recognized during the ATNP/1 Meeting. It was recognized that ODF procedures should be recommended for AFTN message format and be included in ANNEX 10 Volume II in order to support AFTN/ATN communication during transition period.

4	Discussion

4.1	For ATN transition purposes, the members of the former ASPP have developed the procedures for using in OPTIONAL DATA FIELD (ODF). Each of these procedures are also identified and adopted by the ASIA/PAC regional AFTN experts. Appendix A contains the background information and recommendation developed by the ATN Working Group.

ATNP/2 - WP/

APPENDIX A

USE OF OPTIONAL DATA FIELD (ODF) FOR ATN TRANSITION PURPOSES

1	Introduction

1.1	Transition planning activities in various ICAO Panels and regional working groups have highlighted specific technical needs to support the AFTN/ATN transition. The requirements for message/data unit identification, and message/data unit reference information, have been identified during these planning processes, as well as the need for providing Time Stamp, and Cyclic Redundancy Check (CRC) information. The optional data field (ODF), which currently exists in Annex 10, Vol II, provides a flexible way to convey this needed information to ATM Systems on an end-to-end basis.

2	Background

2.1	A number of communication related transition issues have been identified and are concerned with the provision of a common application interface to the various communication services found in the AFS, and the eventual transition to the ATN. The ASPP identified various "Options" to make use of the current ODF. Option 1 of the ODF has been allocated for additional addressing use as a result of the work completed by the ASPP and presented during the first meeting of the ATNP.  Option 2 and 3 were identified and defined for computer applications to convey message/data unit identification and message/data unit reference information.  Option 4 has been identified to express a 12 digit Time Stamp, and Option 5 to convey a four digit hexadecimal CRC number.  These Options have been considered by the AFTN experts in the ASIA/PAC region and have been included in the ASIA/PAC Draft AIDC ICD to support ATN transition in the region.

3.	Discussion

3.1	During the AFTN/ATN transition period, message/data unit identification, message/data unit reference information, Time Stamp and Cyclic Redundancy Check (CRC) information are necessary for ATM systems.

3.2	The ODF procedures developed by the ASPP, and identified and adopted by the ASIA/PAC regional AFTN  experts will meet the users' requirement for data exchange between the ATM Systems.

A-1

4.	Recommendation

4.1	The Panel is invited to recommend the following amendment to Annex 10, Vol II, by adding new procedures following paragraphs   4.18.1.3.7.  These additions provide optional features to support the conveyance of the needed information between ATM Systems.

x.x.x.x.7.3 Recommendation. - When unique message/data unit identification is needed between application processes or terminals, it should be conveyed in the ODF, using the following specific format:

a) the characters 2. to indicate the parameter code for the message/ data  unit identification function;

b) the message/data unit identification value, not to exceed six (6) numeric characters; and

c) the character - (hyphen) to terminate the message/data unit identification parameter field.

x.x.x.x.7.4  Recommendation. - When unique message/data unit reference information is needed between application processes or terminals, it should be conveyed in the ODF, using the following specific format:

a) the character 3. to indicate the parameter code for the message/data unit reference information function;

b) the message/data unit reference value, not to exceed four alphabetic and six numeric characters; and

c) the character - (hyphen) to terminate the message/data unit reference information parameter field.

Note: - The message/unit reference value would typically use the first four AFTN address characters of the message source, followed by six characters identifying the previously sent message/unit identification value being referenced.

A-2

x.x.x.x.7.5  Recommendation. - When message/data unit time stamp information is required to convey a more accurate value than the AFTN date-time group between application processes or terminals, it should be conveyed in the optional data field (ODF) using the following format:

a)   the characters 4. to indicate the parameter code for time-stamp;

b) the message/data unit time stamp value encoded as YYMMDDHHMMSS; and

c)   the character - (hyphen) to terminate the time stamp field.

Note:- This procedure is a way for the receiving process to capture local system clock time and calculate the process to process transition delay time of the complete communications service.

x.x.x.x.7.6  Recommendation. - When message/data unit cyclic redundancy check (CRC) information is required to improve data integrity between application processes, or terminals, it should be conveyed in the optional data field (ODF) using the following format:

a) the character 5. to indicate the parameter code for CRC;

b) the CRC-16 value encoded as four IA-5 characters, or the CRC-32 value encoded as eight IA-5 characters; and

c)  the character - (hyphen) to terminate the CRC field.

x.x.x.x.x.6.1  The bit combinations generated by the user of the CRC parameter shall preclude IA-5 transmission control character bit patterns by setting the four high order bits of each byte to an all ones (1) value. On reception, bit eight, the high order bit, shall be ignored. The 16-bit generator polynomial shall be x16 + x12 + x5 + 1, and the 32-bit generator polynomial shall be x32 + x26 + x23 + x22 + x16  x12 + x11 + x10 + x8 + x7 + x5 + x4 + x2 + x + 1, in accordance with ISO 3309.

Note:- Additional procedures for the use of CRC must be agreed between the two communicating users prior to operational exchanges.

A-3

x.x.x.x.x.6.2  When computing the CRC specified in x.x.x.x.6.1 the occurrence of the characters CR and LF shall be omitted in the calculation of the CRC. This procedure ensures that as the message flows over the AFTN the occurrence of insertion of CR and LF characters will not effect the accuracy of the calculation when the receiving application computes a new CRC for comparison with the one received as part of the AFTN message.

- END -