ATNP-WG/1-WP 3-15

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL

(ATNP)

WORKING GROUP 1

Banff, 9-12 October

Agenda item 5: Status of specific WG1 work items/deliverables

WG1-18 CIDIN AS A SUBNETWORK FOR ATN

(Presented by Theo H.M. Hagenberg)

Summary

This Working Paper provides an analysis on the use of CIDIN as a ATN subnetwork.

Ref.

Analysis of CIDIN protocols and their relationships to the use of  X.25 as a subnetwork of the ATN; Information paper; presented by the USA at the limited middle east (MID) air navigation meeting (Cairo, August 27 to Sept. 7, 1995)

1. Introduction

1.1 Background

Working Group 1 was at its Toulouse meeting considering a proposal for eliminating the use of the CIDIN as an ATN subnetwork from the draft ATN manual.

It was concluded that further work had to be done on the use of CIDIN as an ATN subnetwork before a recommendation on either maintaining or eliminating CIDIN as an ATN subnetwork can be taken.

During SICASP/4 the subject of integration of ground subnetworks within the ATN has been addressed (see Annex 1). On the basis of the material then developed, it appeared that it might be feasible that CIDIN can become a (ground) subnetwork of the ATN. Three  alternatives have been considered  for integration of CIDIN in ATN. A clear preference for one solution has been expressed then in par. 6.3.3.1 of the SICASP/4 report for reasons of simplicity  and efficiency. It was recognized, however, that this approach would require a revision of the existing ICAO SARPs for CIDIN and that the issue calls for a thorough study which was considered beyond the scope of the SICAS Panel.

For that reason the meeting recommended that its considerations be brought to the attention of the Aeronautical Fixed Service (AFS) Systems Planning for Data Interchange Panel (ASPP) and developed therefore recommendation 6/2.

The initial action taken by the ANC at the 16th meeting or its 122nd session on 12 dec 1989 has been presented in supplement no.2 of SICASP/4 as follows:

Noted the recommendation and requested the Secretary to bring it to the Commission's attention during review of the report of the second meeting of the ASPP/2.

A follow-up action on recommendation 6/2 from SICASP/4 has been taken by ASPP/3. This panel, under its agenda item 3, has carefully analyzed this problem and the possible

alternatives.

ASPP/3 (Jan. 1992) has proposed a solution. Following is a reference to the report of ASPP/3:

=============== begin of extract from ASPP/3 report =========================

3.7.1 After the Analysis was completed, it was concluded that:

a)   the ATN internetworking architecture should be adopted as the

objective for a common internetworking structure for air traffic

management applications;

b)   SARPs should be developed for a gateway which supports

operations of AFTN applications over the ATN internet via

ISO potocols (including transport);

c)   SARPs should be developed for a subnetwork dependent convergence

protocol which supports operation of the ATN internet protocols

over the CIDIN layer 4 protocol;

d)   SARPs should reference the need for a subnetwork dependent convergence

protocol which supports operation of the ATN internet protocols

over a fully conforming ISO-8208 protocol;

=============== end of extract from ASPP/3 report ============================

The Appendices to the report on agenda item 3 contain, inter alia, draft SARPs material

for the convergence provisions and requirements for the priority mapping.

In summary, it can be stated that the ASPP/3 did well undertake the follow-up action on SICASP/2's recommendation, and has clearly demonstrated the relevance of using CIDIN as an ATN subnetwork (completely in line with this, convergence provisions for CIDIN have been

included in the ATN manual).

2. Overview of CIDIN and ATN architectures

2.1 CIDIN Architecture.

The CIDIN architecture as presented in Figure 1, is an ICAO standard. The following  extract from Annex 10, Vol. I, is relevant in this context:

a.	4.12.5.1.4.1  Each transport header and the associated segment shall be preceded by a CIDIN packet header.  No further segmentation of the CIDIN message shall be used between transport protocol level and the CIDIN packet protocol level.  Both headers, therefore, shall be used in combination.  Together they shall be referred to as the Communications Control Field (CCF).  Together with the message segment they form CIDIN packets that shall be transmitted from entry centre to exit centre(s), when necessary through one or more relay centres, as an entity.

b.	4.12.1.4.2  CIDIN packets of one CIDIN message shall be relayed independently via predetermined routes through the network thus allowing alternative routing on a CIDIN packet basis as necessary.

c.	4.12.1.4.3  The CIDIN packet header shall contain information to enable relay centres to handle CIDIN packets in the order of priority, to transmit CIDIN packets on the proper outgoing circuit(s) and to duplicate or multiplicate CIDIN packets when required for multiple dissemination purposes.  The information shall be sufficient to apply address stripping on the exit addresses as well as on the addressee indicators of messages in AFTN format.

d.	The CIDIN protocol architecture as shown in Figure 1, serves to illustrate some of the unique characteristics of the associated procedures.  The level 3a procedures are based on the  X.25 protocol standard (1980 version) produced by the ITU.  The Permanent Virtual Circuit (PVC) transit centre function may be fulfilled by a public or private packet node. The functions of a CIDIN Relay Centre are unique to CIDIN procedures and can not be fulfilled by a public or private packet node.  CIDIN packets are restricted to a maximum size of 256 octets, and the maximum amount of data that can be exchanged as one unit over a CIDIN connection is 2119 packets.  This restriction is not foreseen as a problem for the exchange of ATM related data between AFTN switching centres, or MET data banks. In ISO terms, the CIDIN procedures provide a connectionless transport service, with acknowledgement, between the two communicating CIDIN centers.

2.2	ATN Architecture.

Fig. 1  CIDIN PROTOCOL ARCHITECTURE

The ATN protocol architecture depicted in Figure 2 is based on current versions of ISO and ITU standards and exactly follows the OSI model for an open operating environment.  There is no restriction to the maximum amount of data that may be exchanged as a unit across the ATN, when connection oriented upper layer protocols are used.  This function is strictly a matter of the application and its associated application layer service elements.  When using

connectionless upper layer protocols the maximum amount of data that can be exchanged as a unit across the ATN is 2512 octets.  Figure 2  also presents that connectivity is implemented by network access, or on a point-to-point link.

Fig. 2  ATN Protocol Architecture

a.	When using ATN protocols the interconnection between end systems, acting as DTEs at the subnetwork level, can be via a point-to-point circuit, or via a subnetwork.  If the two end systems are located in two different routing domains or routing areas, the data flow must also pass between them via one or more ATN routers.  All systems attached to a subnetwork must employ the same protocol standards and implementation profiles in order to exchange data.

b.	In the ATN, several different types of subnetworks are allowed. However, when CIDIN is used as ATN subnetwork then the CIDIN procedures are restricted to the provision of a permanent logical circuit (PVC) between the two communicating systems.  ATN protocols support switched virtual circuits (SVCs), but CIDIN does not.  Also, the CIDIN multiple dissemination function is not allowed where CIDIN procedures are used in an ATN subnetwork.

3.0	Analysis of the CIDIN archtecture

The strategy for using  the CIDIN  as an ATN subnetwork as proposed by ASPP/3 will avoid the use of gateways or protocol converters in the ATN internet.  Figure 4 depicts the required protocol architecture for employing CIDIN protocols as a subnetwork.

a.	In Figure 3 the situation regarding the use of point-to-point or subnetwork access, using a packet network has been presented.  If the two end systems are in different routing areas one or more ATN routers would be in the path between the two communicating end systems.  If one of the end systems wished to communicate with another system located in another routing domain, then the ATN router would also be part of the path to reach the remote system.

b.	Draft SARPs for using CIDIN as an ATN subnetwork are as yet not validated.  Validation should be preferrebly be completed by mid 1996 in order to be included in the recommended ATN SARPs.  The completion of  ATN SARPs is expected at the second meeting of the ATN Panel, which is now scheduled for late 1996.

Fig. 3  ATN Protocol Architecture with CIDIN Protocols as a Subnetwork

3.1	Relation of CIDIN Protocols to the OSI Model.

The unique CIDIN transport and packet header must be used together to derive addressing for the relay and end-to-end transfers, while in ISO protocols addressing for relaying and transport are totally separate.  Other differences between  the CIDIN layer 3b and 4 protocols are noted in Table 1.

Table 1  CIDIN/ATN Functional Comparison

| CIDIN Function | ISO 3B | ISO 4 | Remarks |
| --- | --- | --- | --- |
| CIDIN Packet Protocol - Message Priority - Exit Address(es) - Destination Address(es) - Address Stripping Missing ISO/ATN Functions - Source Addressing - Segmentation - Protocol Identification - Protocol Version - Congestion Notification - Traffic Type Security - Interdomain Addressing - Dynamic Routing Protocols - Dynamic Forwarding Data Base - Switched Virtual Circuits | E E V V R R R R R R R R R R |  | Layer 3b ISO 8473, et el For Wide Area Networks |
| CIDIN Transport Protocol - Message Identification Number - CIDIN Packet Sequence Number - Final CIDIN Packet Indicator - Message Code and Format - Network Management Field - Message Type Indicator - Conversation Protect Indicator - Network Acknowledgement Indic. - Entry Address - End of Header Missing ISO/ATN Functions - Flow Control - Destination Address - Data Integrity - Protocol Identification - Protocol Version - Quality of Service - Packet Size Negotiation - Transport Connections |  | E E E V V V V E E E R R O R R O R R | ISO 8073 Layer 3 Function Required by ATN Required by ATN |

E = Equivalent Function in ISO, O = Option in ISO, V = Violation of ISO, R= Required by the ATN Manual

4. The operational use of CIDIN through the world

CIDIN is currently in operation in the following European Centres:

Amsterdam, Athens, Brussels, Frankfurt, Copenhagen, Lisbon, Madrid, Nicosia, Moscow, Tel Aviv, Vienna, and finally in the Eurocontrol IFPS/CFMUs in Haren and in Bretigny.

In addition, the centres Karlsruhe and Maastricht are connected to CIDIN.

Rome, Prague, London, Montreal and Reykjavik are expected to be put

into service this year. London is expected to be operational this fall, which would immediately allow connection to Montreal and Reykjavik, which are already available since one year.

Consequently, CIDIN must be regarded as an existing, operationally used communication

network for ATS purposes in a large number of countries.

Significant investments have been made in order to have CIDIN operational in these centres.

Although it is recognized that CIDIN used as a ATN subnetwork has certain disadvantages,

it seems reasonably to enable CIDIN to be used as an ATN subnetwork if:

-	the cost benefit ratio is reasonable: to amortize CIDIN and replace it by a X 25 subnetwork may have a low cost benefit ratio.

-	CIDIN can accomodate the ATN data message for the initial ATN transition period.

5. CONCLUSION

ATNP should seriously  consider to retain SARPs which allow CIDIN to be used  in an future ATN environment.

Investments already made in existing CIDIN communication systems are extensive and must be allowed to be amortized in a reasonable time period.

CIDIN may technically and operationally be used as a ATN subnetwork, although with some technical and operational drawbacks. Therefore a clear migration path must be identified

from the current use of CIDIN to CIDIN in an ATN envirionment and subsequently to replace CIDIN by an X 25 type subnetwork.

ANNEX 1

Extract from the report of the 4th meeting of the SICAS Panel

6.3	 Integration of ground subnetworks within the ATN

6.3.1	The meeting's attention was drawn to the common ICAO data interchange network (CIDIN) specifications defined in Annex 10, Volume !, Part I, chapter 4, for aeronautical ground-ground data interchange.

6.3.1	The panel noted that CIDIN employs bit-oriented data transfer techniques; thus, it is possible to use CIDIN to carry ATN internetwork packets between ATN routers. It was also noted that Annex 10 specifies four protocols levels for transfer of data between switching centres. These protocols are:

a)	the data link protocol level (CIDIN Level 2)

b)	the X.25 packet protocol level (CIDIN level 3a); and

c)	the transport protocol level (CIDIN Level 4).

6.3.2.1	The meeting noted that the above CIDIN levels implement layers 2,3 and 4 of the OSI model; however, the corresponding CIDIN protocols do not conform to ISO protocol standards. Moreover according to Annex 10, CIDIN access is assumed to occur only at the CIDIN level 4.

6.3.2.2	It was also noted that the architecture specifies the use of ISO protocol standards within layers 3 and 4 of the OSI  model. As such, interconnections of subnetworks occur at level 3. For this reason, special considerations are required for the use of CIDIN within the ATN .

6.3.3	The meeting felt that the definition of the requirements for the integration of appropriate ground subnetworks within the ATN is an important task. Three approaches for integration of CIDIN within the ATN were discussed as follows:

6.3.3.1	The ATN internetwork protocol packets may be carried via the CIDIN Level 3a packet protocol. This approach would be the simplest, since an ISO standard exist for convergence of the ATN internetwork protocol with X.25 packet level protocol. However this approach would require that a Level 3a interface be provided within the CIDIN protocol architecture. It should also be noted that while the ATN architecture allows operation over the X.25 PVCs, optimal use of CIDIN Level 3a access would also require SVC support.

6.3.3.2.	The ATN internetwork protocol packets may be carried via the CIDIN Level 4 transport protocol. This approach would allow the use of the CIDIN protocol architecture as currently specified in Annex 10, however, the interface within the CIDIN transport protocol would be more complex than with the X.25 packet protocol. Also, the additional complexity offers no additional benefits since the CIDIN transport protocol would then be used only in a network layer role, thus creating protocol overhead.

6.3.3.3	A transport layer gateway may be defined for interworking between the ISO transport protocol and the CIDIN Level 4 transport protocol. This approach would require a mapping of transport layer services between the two protocol standards; however, further analysis is required to assess the feasibility of this mapping function. Assuming that this mapping is possible, this approach has at least one important disadvantage. The use of transport  layer gateways interrupts the end-to-end relationship between host computers serving as end-systems, and does not conform to the OSI model. This implies that in order to assure reliable end-to-end data transfer as defined in the ATN architecture, additional protocol features (i.e. message buffering and acknowledgement at each transport gateway) must be provided.

6.3.4	Although any of the approaches discussed above could be adopted, the meeting was of the opinion that the approach referred in paragraph 6.3.3.1 above was preferable for integration of CIDIN within the ATN architecture for reasons of simplicity and efficiency. It was recognized, however, that this approach would require a revision of the existing ICAO SARPs of CIDIN and that the issue calls for a thorough study which was beyond the purview of the SICAS Panel. For this reason, the meeting recommended that its considerations be brought to the attention of the Aeronautical Fixed Service (AFS) Systems Planning for Data interchange Panel (ASPP) and developed the following recommendation:

Recommendation 6/2 Integration of the CIDIN within the ATN

That ICAO review and amend as necessary the work programme of the ASP Panel to include the development of SARPs for the interface between aeronautical ground-ground subnetworks with a view to ensuring the compatibility with the aeronautical telecommunication  Network (ATN).