Annex A to WG1/WP 3-12

AERONAUTICAL TELECOMMUNICATION NETWORK PANEL

Working Group 1

ATN Naming, Addressing and Registration Concept

Deliverable WG 1-16

DRAFT

SUMMARY

This document is a draft version of the ATN naming, addressing and registration concept. It outlines the addressing, naming and registration provisions related to the ATN information objects within the scope of the CNS/ATM-1 Package.

DOCUMENT CONTROL LOG

| Section(s) | Date | Amendment Number | Reason for change and/or Change Note Number. |
| --- | --- | --- | --- |
| All | 13/2/95 | 0_A | Initial draft |
| All | 3/3/95 | 0_B | Additional content / Technical updates |
| All | 6/10/95 | 1.0 | Additional content / Technical updates |

TABLE OF CONTENTS

1 Introduction

1.1 Scope and Purpose

1.2 References

1.3 Glossary

2 Problem Statement

3 Naming, Addressing And Registration Overview

4 CNS/ATM-1 Package Assumptions

4.1 Overview of Applications and Upper Layer Architecture

4.2 Internetwork Topology

4.3 Subnetworks Used

4.4 Elements of ATN Naming and Addressing

4.4.1 Subnetwork Elements

4.4.2 Internetwork Elements

4.4.3 Applications

4.4.4 Aircraft

4.4.5 ATC Centres

5 Naming, Addressing and Registration Requirements

5.1 Subnetwork Addressing

5.1.1 X.25 Subnetworks

5.1.2 VDL Subnetworks

5.1.3 AMSS Subnetworks

5.1.4 Mode S Subnetworks

5.1.5 Minimising The Impact of Topology Changes and Outages

5.2 Internetwork Addressing

5.2.1 Internetwork Addressing

5.2.1.1 The NSAP Address

5.2.1.2 The Network Entity Title

5.2.2 Internetwork Address Registration

5.2.3 Relationship To ATN Routing

5.2.4 Minimising The Impact of Topology Changes and Outages

5.3 Transport and Upper Layer Addressing

5.4 Application Naming

5.4.1 Naming Scheme

5.4.2 Ground-Based ATM Applications

5.4.3 Airborne ATM Applications

5.4.4 Other Ground-Based Applications

5.4.5 Other Airborne Applications

5.4.6 Name and Address Relationship in CNS/ATM-1 Package

5.4.7 Application Name Registration

6 The Use of Context Management as a Basic Directory Service

# Introduction

## Scope and Purpose

This paper is a draft version of the ATN naming, addressing and registration concept. It identifies certain information objects relevant to the CNS/ATM-1 Package as well as some of the longer-term issues required for successful world-wide development and operational use of ATN systems and services expected as follow-on functionalities, perhaps in the yet-to-be defined Package 2 timeframe.  This material may serve as a baseline for the refinement and further specification of the ATN naming, addressing and registration strategy for CNS/ATM-1 Package.

## References

1.	Draft Manual of the Aeronautical Telecommunication Network (ATN), ICAO, November 19, 1993

2.	VDL Manual, ICAO

3.	AMSS SARPs, ICAO

4.	Automatic Dependent Surveillance (ADS) and Air Traffic Services (ATS) Data Link Applications guidance Material, ADS Panel, ICAO, May 1994

5.	DO-223 - Minimum Operational Performance Standards for Aircraft Context Management (CM) Equipment, RTCA SC-169, July 13, 1994

6.	DO-212 - Minimum Operational Performance Standards for Automatic Dependent Surveillance, RTCA SC-169

7.	DO-219 - Minimum Operational Performance Standards for Two-Way Data Link, RTCA SC-169

8.	Mode S SARPs, ICAO

9.	Proposed Draft SARPs for CNS/ATM-1 FIS Application, France, January 30, 1995

10.	AFTN Specification, TBD.

11.	CNS/ATM-1 Package SARPs and Guidance Material, Part V - Internet Communication Service, version 3.0, 07-August-1995

X.25	Recommendation X.25 - Interface Between DTE and DCE for Terminals Operating In the Packet Mode And Connected to Public Data Network by Dedicated Circuit, CCITT, 1988

X.121	Recommendation X.121 - International Numbering Plan for Public Data Networks, CCITT, 1992

7498-3	ISO/IEC 7498-1 - Basic Reference Model, Par 3: Naming and Addressing, ISO, 1989

9834-1	ISO/IEC 9834-1 - Procedures for the Operation of OSI Registration Authorities: General Procedures, ISO, 1993

9834-6	ISO/IEC 9834-6 - Procedures for the Operation of OSI Registration Authorities: Application Processes and Application Entities, ISO, 1993

9594	ISO/IEC 9594 - The Directory - first edition 1990

10730	ISO/IEC 10730 - Tutorial on Naming and Addressing, first edition, 1994

## Glossary

ADM		Administration Identifier

AOR-E	Atlantic Ocean Region-East

AOR-W	Atlantic Ocean Region-West

ADS		Automatic Dependent Surveillance

AFI 		Authority Format Identifier

AMSS	Aeronautical Mobile Satellite Service

ARS 		Administrative Region Selector

ATCC		Air Traffic Control Centre

ATN		Aeronautical Telecommunication Network

ATNP		ATN Panel

ASO		Application Service Object

BCD		Binary Coded Decimal

CLNP		Connectionless Network Protocol

CM		Context Manegement

CPC		Controller-Pilot Communication

DIT		Directory Information Tree

DNIC		Data Network Identification Code

DTE		Data Terminal Equipment

FIS		Flight Information Services

IDI 		Initial Domain Identifier

IOR		Indian Ocean Region

LOC		Location Identifier

MTN		Mega Transport Network

NSAP		Network Service Access Point

OOOI		Out-Off-On-In application

OSIE		OSI Environment

POR		Pacific Ocean Region

PSDN		Packet Switched Data Network

RDI		Routing Domain Identifier

RDF 		Routing Domain Format

SARPS	Standards and Recommended Practices

SEL		Selector

SNPA		Subnetwork Point of Attachment

SYS		System Identifier

TSAP		Transport Service Access Point

TSEL		Transport Selector

VER 		Version Identifier

VDL		VHF Data Link

# Problem Statement

The ATN naming, addressing and registration issues is potentially vast.  The principal question is the meaning of naming and addressing elements:  how do ATN systems and applications use this information and how may it be administered on a global basis?  Although a well-defined internetwork addressing (NSAP) plan already exists ([1]), there are many related issues which require resolution before the ATN can be successfully deployed.  In the short term, a naming, addressing and registration plan must be defined for the ATN CNS/ATM-1 Package deployment.  In the long term, a broader approach must be taken to cover the more general ATN applications and architecture.  Issues requiring short term resolution include

1.	Mobile subnetwork addressing and registration and how this relates to the ATN internetwork addressing and routing;

2.	Use of the internetwork addressing plan and internetwork address registration procedures;

3.	CNS/ATM-1 Package application naming, addressing and registration and how these elements are used in relation to directory services;

4.	Assumed upper layer architecture and ASO naming;

5.	Migration requirements for Package 2 and beyond.

# Naming, Addressing And Registration Overview

The concepts of name and address are defined in The OSI Basic Reference Model, Part 3 (ISO 7498-3).  In brief, a name is used to identify an object (e.g. a system, a protocol, an application, etc.) while an address is used to locate an object.  Names typically have meaning to upper layer elements, applications, and are thus generally expressed in a mnemonic format.  Correspondingly, the significance of addresses typically increases as we descend in the OSI stack and are generally expressed in a coded or numeric format for machine processing.  In either case, in order to ensure unambiguous assignments, names and addresses must be registered by registration authorities within the context or environment in which they are to be used.

In order to manage the inherent complexity of large networks, names and addresses should be assigned according to hierarchical name/address trees.  The structure of these trees should, in principal, represent the structure of the network operating environment.  For example, as the ATN is an international network involving primarily two principal interest groups (ICAO and IATA), the top levels of the ATN internetwork (NSAP) address tree has been chosen as depicted in Figure 1.  The address structure, and thus the responsibility for registration, has effectively been divided between ICAO and IATA.  Each organisation is responsible for address allocation and registration within their sub-trees.  The responsibility for NSAP address field allocation would then likely be further delegated to the countries and airlines within ICAO and IATA.  The tree structure closely follows the air transport community’s administrative topology.  Correspondingly, other name/address trees should follow a similar structure as detailed in paragraph 5.

Naming, addressing and registration concepts are closely related to directory services.  A directory may be used to store name and address information and can provide lookup functions to determine the address or characteristics of a given application or service. Directory information may be stored local to a given application process if the addresses of all other applications and services with which it will communicate are known ahead of time.  However, in an operating environment like the one for which ATN is intended, applications cannot always know in advance the address of all other relevant applications.  Thus, a simple directory service, the context management function, is required per [4] and [5].

# CNS/ATM-1 Package Assumptions

## Overview of Applications and Upper Layer Architecture

The following applications are expected to be part of the initial application set for the CNS/ATM-1 Package.  These functionalities of these applications are used to determine the associated naming, addressing and registration requirements.

1.	Automatic Dependent Surveillance (ADS).  ADS will be the primary application used to support air traffic management services.  The operational requirements for ADS are described in [4].

2.	Controller-Pilot Communications (CPC).  CPC will also be an application used to support air traffic management.  The operational requirements for CPC are described in [4] and [6].

3.	Inter-Centre Co-ordination (ICC).  This application is required for exchange of information between ATS units such as aircraft tracking information, coordination of hand-offs and transfer of control. The operational requirements for ICC are implicitly described in [4].

4.	Context Management (CM).  CM is a basic directory service used to support ATM-based applications.  The operational requirements for CM are implicitly described in [4] and [7].

5.	Flight Information Services (FIS).  Flight information services may also be included in CNS/ATM-1 Package. The operational requirements for FIS are currently being defined in [9].

6.	Message Handling Services (MHS).  MHS services may also be included in CNS/ATM-1 Package.  Although it is expected that MHS will be used to support the AFTN,  specific operational requirements for MHS in CNS/ATM-1 Package are currently not defined.

The ADS, CPC, CMA, AIDC and FIS applications are taken into consideration in the development of the CNS/ATM-1 Package naming, addressing and registration concept.  Naming, addressing and registration requirements should be derived from the ATN MHS when this application and the corresponding operating concept is defined.

CNS/ATM-1 Package is also expected to support airline-specific messaging and distributed applications.  The operational requirements of these applications are not globally-defined and are outside the scope of this activity.  It is not expected that the introduction of airline-specific applications will adversely affect the ICAO naming, addressing and registration concept.  However, airline-specific considerations should be considered as a follow-on activity.

Until early 1995, the upper layer architecture for all CNS/ATM-1 Package applications considered in this document was expected to be minimal.  Specifically, the principal standards from RTCA defining the ADS, CPC, and CM applications ([5], [6] and [7]) indicated that the applications would run directly over the connection oriented transport service without any support from the session, presentation or application services.  However, recent developments in the ATNP/WG3/SG3 (ATN Upper Layer Architecture Subgroup) have led to the conclusion that common ATN application services should be based on the extended application layer structure including the use of an extended ACSE.  As ATNP/WG3/SG3 is still working on the details of the definition, this paper will only introduce some of the aspects corresponding to upper layer addressing.

## Internetwork Topology

The ATN internetwork topology is expected to be a loosely connected internetwork of routing domains providing the connectivity required to support ATM and airline-specific services.  Specifically, we can foresee ATC centres being connected together via local means within an ICAO Member State and via international WANs and the ATN backbone between ICAO Member States.  Communications between aircraft and ATC centres will be ensured by the ATC’s Mode S subnetwork as well as air/ground service providers AMSS and VDL subnetworks.  Both airlines and ATC authorities may rely on traditional air/ground service providers to provide managed ATN services including air/ground as well as ground/ground ATN communication services.

The ATN internetwork will be comprised of both transit routing domains and end routing domains.  Transit routing domains are typically responsible for switching ATN traffic between themselves and to and from end routing domains.  For example, SITA ATN routing domains, as part of the ATN backbone, would be able to switch traffic for the entire ATN community.  ATN end users are typically only responsible for end routing domains, which do not switch traffic for other ATN routing domains.  For example, each ATN aircraft may have exactly one ATN end routing domain on board.

## Subnetworks Used

In order to interconnect ATN routing domains, the respective interdomain routers must be connected via a subnetwork.  The principal ATN subnetworks to be used during the CNS/ATM-1 Package timeframe will be X.25 PSDNs for ground-based communications as well as the VDL, AMSS and Mode S air/ground subnetworks.

## Elements of ATN Naming and Addressing

This section outlines the basic elements within the ATN environment requiring names and addresses.

### Subnetwork Elements

1.	X.25 Network Addresses.  A X.25 network address identifies the SNPA of a DTE/router on a given X.25 network.  It has meaning only within the context of the X.25 subnetwork on which it is used subject to the provisions in [X.25].

2.	AMSS Network Addresses.  An AMSS network address identifies the SNPA of a DTE/router on a given AMSS subnetwork.  An AMSS DTE may be either be located on an aircraft or at a fixed site.  The AMSS network address has meaning only within the context of the AMSS subnetwork in which it is used subject to the provisions in [3].

3.	VDL Network Addresses.  A VDL network address identifies the SNPA of a DTE/router on a given VDL subnetwork.  An VDL DTE may be either be located on an aircraft or at a fixed site.  The VDL network address has meaning only within the context of the VDL subnetwork in which it is used subject to the provisions in [2].

4.	Mode S Network Addresses.  A Mode S network address identifies the SNPA of a DTE/router on a given Mode S subnetwork.  A Mode S DTE may be either be located on an aircraft or at a fixed site.  The Mode S network address has meaning only within the context of the Mode S network in which it is used subject to the provisions in [8].

### Internetwork Elements

1.	Network Entity Title (NET).  An ATN system’s NET is a 20-octet string used to uniquely identify and locate the internetwork protocol entity of the system, and thus, the system itself.  Thanks to the hierarchical ATN internetwork addressing plan defined in [1], a system’s NET can be used to locate it anywhere within the ATN topology.  The syntax of an ATN NET is equivalent to that of an ATN NSAP address and should, in fact, only differ from the systems’ NSAPs in the last octet as described in A7.5.9.3 of [2]. A given NET must identify exactly one ATN system.

2.	NSAP Address.  An ATN NSAP address is a 20-octet string used to uniquely identify and locate a given NSAP within the context of the ATN.

3.	Administrative Domain Identifier.  Each ATN administrative domain can be identified by a unique domain address prefix comprising the AFI, IDI, VER and ADM fields of an ATN NSAP address.  An administrative domain identifier is the prefix of all ATN NSAP addresses and NETs within the same ATN administrative domain.

4.	Routing Domain Identifier.  Each ATN routing domain can be identified by a unique, fixed 11-octet ATN routing domain identifier comprising of the AFI, IDI, VER, ADM, RDF and ARS fields of the ATN NSAP.  A routing domain identifier is the prefix of all ATN NSAP addresses and NETs within the same ATN routing domain.

### Applications

1.	Transport Address.  As defined in [1], a transport address, or more specifically a TSAP address, uniquely identifies a given transport service user within the context of the ATN.  A TSAP address is comprised of a one- or two-octet TSAP selector appended to the host system’s NSAP address.

2.	Application Address.  The address of an OSI application is typically its presentation addresses.  As CNS/ATM-1 Package applications may be defined to be users of the extended application layer structure, ATN application addressing should follow the OSI standards currently under development.

3.	Application Name.  OSI applications are generally given a globally unique application process title (AP-title) and one or more globally unique application entity titles (AET).  These names are used to identify the application within the ATN operating context.  They may be used as inputs to a directory service to determine the presentation address of a given application.  A means of naming CNS/ATM-1 Package applications is introduced in [5] and further explained in paragraph 5.3.

### Aircraft

1.	ICAO 24-bit Address.  Each aircraft equipped with ATN-compatible data link equipment can be identified by a unique 24-bit identifier assigned by the registration administration under the authority of ICAO.  This identifier will stay unmodified except in cases of re-certification, e.g. due to owner’s change. This address is used in many address components including the NSAP addresses as well as the VDL, AMSS and Mode S SNPAs. In some cases, the ICAO 24-bit address may be derived from the registration mark, while in others there is no mapping with the registration mark.

2.	Registration Mark.  Each aircraft can be identified by a uniqueregistration mark being constant except in cases of re-certification.  The registration mark of the aircraft may be used in certain messages and may also be included in the aircraft’s flight plan.

3.	Flight Number.  During any given flight or flight segment, every ATC-controlled aircraft is assigned a flight number comprised of the operating company’s IATA 3-character identifier and a unique number of up to four digits.  The flight number is included with an aircraft’s flight plan.  ATC authorities currently use an aircraft’s flight number to identify aircraft within their control.

4.	Aircraft Routing Domain Identifier.  Each aircraft can be identified by unique, unvarying 11-octet ATN routing domain address assigned as a function of the airline operator (identified in the NSAP ADM field) and the aircraft’s ICAO 24-bit address (identified in the NSAP ARS field).  The aircraft domain address is the prefix of all ATN NSAP addresses on board the aircraft.

### ATC Centres

1.	Air traffic control centres (ATCCs) are responsible for ATM within an ATC control region.  There is exactly one ATCC system responsible within each control region.  A control region is an area which operates its own flight plan data processing systems. There may be one or more control regions per flight information region (FIR).  Currently, each ATCC is assigned an address per [10].  Within the ATN, each ATCC and, hence a control region, should be identified by an RDI based on the current ATCC administrative topology.

# Naming, Addressing and Registration Requirements

## Subnetwork Addressing

Subnetwork naming and addressing is important to consider in the ATN context as ATN systems must use subnetwork points of attachment (SNPAs) to address other ATN systems.  In general, SNPAs are treated as addresses and have no relationship to an object name.  Additionally, registration of SNPAs is generally a consideration local to a subnetwork, but the SNPAs assigned to specific systems or services should be made available to all interested parties attached to a given PSDN.  This section presents addressing issues related to subnetworks likely to be used for interconnecting various ATN organisations and routing domains.

### X.25 Subnetworks

Addressing of systems (typically referred to as DTEs on X.25 networks) attached to an X.25 network is treated in [X.25] and [X.121].  Generally an X.25 network has a well-established local addressing plan according to the needs identified by the network operator.   DTE addresses on a given network typically contain up to 10 BCD digits.  For calls placed to destinations outside of the network, a 4-digit BCD DNIC defined by [X.121] or other international agreements must be prepended to the DTE network address.  DTE network addresses do not need to be registered within the ATN-wide context as their use does not have ATN-global impact. Rather, organisations will exchange DTE network addresses as appropriate according to a prior agreement or service contract.

An ATN system connected to an X.25 network must have preconfigured knowledge of the DTE network addresses of the systems with which it will communicate using the X.25 network.  Typically, this information should be loaded in the CLNP routing table by systems management functions prior to putting the system into service.

### VDL Subnetworks

The addressing plan for the VDL subnetwork is described in [2].  The scope and impact of the VDL addressing plan is limited to the VDL subnetwork. Systems not directly attached to the VDL subnetwork are not affected by the VDL addressing plan.

The two principal types of ATN systems which use the VDL subnetwork are airborne routers and air/ground routers.  The VDL address of an airborne router is comprised of an 8 digit BCD-encoded representation of its ICAO 24-bit address plus an optional one or two digit subaddress.  The VDL address of an air/ground router may simply be its DTE network address as assigned by the network operator to which it is attached.  For example, air/ground routers attached to the SITA VDL network will be assigned DTE network addresses according to the SITA X.25 numbering plan.  A system not directly attached to a VDL subnetwork may be able to call a router attached to the VDL subnetwork by prepending the DNIC of the destination network to the VDL address of the called system.

As all calls placed over the VDL subnetwork must be originated by airborne routers, the addresses of appropriate air/ground routers must be known before a call is placed.  An airborne router must have the ability to determine which air/ground routers can be called via a given ground station.  Ground stations providing the VDL service will broadcast information about reachable air/ground routers and will allow airborne routers to call a default router without having to know its DTE network address. The specific operation of these VDL functions are addressed in [2].

### AMSS Subnetworks

The addressing plan for AMSS is treated in [3].  The scope and impact of the AMSS addressing plan is limited to the AMSS subnetwork. Systems not directly attached to the AMSS network are not affected by the AMSS addressing plan.

The two principal types of ATN systems which use the AMSS subnetwork are airborne routers and air/ground routers.  The AMSS address of an airborne router is formatted using BCD-encoded digits as follows:

<AMSS airborne address> :: <DNIC> ‘5’ <AES> > <D>

<DNIC> :: 	‘1111' (AOR-E satellite) or ‘1112’ (POR satellite) or ‘1113’ (IOR satellite) or ‘1114’ (AOR-W satellite)

<AES> ::	8-digit BCD-encoded 24-bit address of aircraft

<D> ::	Optional subaddress digit

The digit ‘5’ following the DNIC is a discriminator indicating that the address refers to an airborne system.  A example AMSS address of an airborne router when located over the Atlantic Ocean may be 1111.5.46721005.

The AMSS address of an air/ground router is formatted using BCD-encoded digits as follows:

<AMSS ground address> :: ‘26’ <DNIC> <NTN>

<DNIC> :: 	4-digit DNIC of the ground network as registered in [X.121] or by international convention (e.g. SITA’s DNIC  is ‘1116’).

<NTN> ::	Up to 9-digit network terminating number (DTE network address) of the air/ground router on the provider’s network identified by the ASNID.

The digits ‘26’ comprise a prefix indicating that the address is used to access an internetwork router within the AMSS addressing plan.  As an example, a  SITA air/ground router AMSS address may be 26.1116.2331123.

As part of the routing initiation process over an AMSS subnetwork, airborne routers are required to place the initial to the air/ground router.  Therefore, the addresses of appropriate air/ground routers must be known before a call is placed.  An airborne router must have the ability to determine which air/ground routers can be called via a given ground station.  This information can be pre-configured into the airborne router by systems management.  Therefore, limited address registration may be required within the scope of the AMSS.

### Mode S Subnetworks

Mode S subnetworks are effectively seen as X.25 networks from the users’ (routers’) perspective.  Airborne Mode S systems will be addressed by deriving a DTE network address from the aircraft’s 24-bit ICAO address.

### Minimising The Impact of Topology Changes and Outages

From time to time, a subnetwork topology may change (including intended reconfigurations e.g. due to ATC center re-organization) or a system outage may occur which effectively changes the subnetwork address of a given system.  Because ATN subnetworks are expected to be used to support air traffic management applications, operational availability must be very high.

Subnetwork topology changes or subnetwork address changes should be made tranparent to ATN users. This is possible, if only subnetwork addresses, but not NSAP addresses or NETs are changed. Only the local intradomain routing tables must be updated, and subnetwork topology changes or subnetwork address changes are not propagated to other routing areas or routing domains.

## Internetwork Addressing

### Internetwork Addressing

The ATN internetwork addressing plan is defined in section A.7 of [1].  This hierarchical addressing plan reflects the expected topology of the ATN internetwork based on the administrative topology derived from how the network may be used by the ATN community.  During the early use of the ATN, this internetwork addressing plan will be subject to validation as the community evaluates the appropriateness of the selected plan.  However, strict adherence to this hierarchical addressing plan is required for proper operation of the ATN.

#### The NSAP Address

| Field Name | Size (Octets) | Value / Range (Hex) | Meaning | Registration Authority | CNS/ATM-1 Package Status |
| --- | --- | --- | --- | --- | --- |
| AFI | 1 | 47 | Basic NSAP Format | ISO | Fixed |
| IDI | 2 | 00 27 | ATN NSAP | ISO | Fixed |
| VER | 1 | 01 41 81 c1 | fixed AINSC Address mobile AINSC address fixed ATSC Address mobile ATSC address | ICAO | Fixed Fixed Fixed Fixed |
| ADM | 3 | 00 00 00 - ff ff ff | ISO Country Code IATA Code | ICAO IATA | Variable Variable |
| RDF | 1 | 00 | reserved | ICAO | Fixed |
| ARS | 3 | 00 00 00 - ff ff ff | Admin. Region / ATSC Admin. Region / AINSC 24-bit Aircraft ID | Per ADM Field Per ADM Field ICAO | Variable Variable Fixed |
| LOC | 2 | 00 00 - ff ff | ATN Routing Area | Per ADM Field | Variable |
| SYS | 6 | 00 00 00 00 00 00 - ff ff ff ff ff ff | Local System Identifier | Per ADM Field | Variable |
| SEL | 1 | 00 01-fd fe ff | Network Entity of a router suporting IDRP spare codes Network Entity of an airborne router not suporting IDRP spare code | ICAO | Fixed Variable Fixed Fixed |

Table 1.  CNS/ATM-1 Package ATN NSAP Address Definition

The use of the ATN internetwork addressing plan for CNS/ATM-1 Package is outlined in Table 1.  This table describes the complete ATN NSAP address format from which are derived NETs, RDIs and NSAP address prefixes.  Of the nine address components, five already have fixed values per ICAO definitions.  The values of the remaining four fields, marked as Variable in Table 1, may be allocated as follows:

ADM	The value of the ADM, combined with the values of the preceding fields, determines the system’s ATN administrative domain.  Each ICAO Member State, airline or other organisation participating in the ATN should be responsible for exactly one ATN administrative domain.

If the VER field indicates an ATSC address format, ICAO is the registration authority responsible for the ADM field and the corresponding ATN administrative domain.  In this case, the ADM field should contain the ISO country code of the ICAO Member State responsible for the system addressed by the corresponding NSAP.

If the VER field indicates an AINSC address format, IATA is the registration authority responsible for the ADM field and the ATN administrative domain.  In this case, IATA will assign ADM fields according to the procedures outlined in paragraph 5.2.3.

ARS	The value of the ARS, when combined with the values of the preceding fields, determines the system’s ATN routing domain.  Each ICAO Member State, airline or other organisation identified by the value in the ADM field will be responsible for establishing one or more ATN routing domains according to their local addressing requirements.

If the VER field indicates a mobile routing domain, the ARS shall be the hexadecimal-encoded value of the aircraft’s ICAO 24-bit identifier.  In this case, the ATN systems onboard an aircraft comprise one single routing domain.

If the VER field indicates a fixed routing domain and an ATSC address format, the ARS may contain either the ICAO location identifier or other meaningful indication (e.g. the flight information region indicator) of the routing domain.

If the VER field indicates a fixed routing domain and an AINSC address format, the ARS may contain the IATA location identifier or other meaningful indication of the routing domain.

LOC	The value of the LOC field is under the responsibility of the organisation identified in the ADM field and should be assigned a meaningful value within the context of the administrative region identified in the ARS field.  This field may be used to subdivide large routing domains into routing areas according to local needs.

SYS	The value of the SYS field is under the responsibility of the organisation identified in the ADM field and should be assigned a meaningful value within the context of the administrative region and location identified in the ARS and LOC fields.  This field may be used to identify a specific ATN system within a given routing domain.

| AFI | IDI | VER | ADM | RDF | ARS | LOC | SYSSEL |
| --- | --- | --- | --- | --- | --- | --- | --- |

Figure 2.  ATN NSAP Format

#### The Network Entity Title

A NET is in fact the name (title) of the network entity in any given ATN system.  However, as an ATN system may have exactly one network entity, the NET in fact ”names” the entire system.  Further, as ATN NETs are derived from the ATN addressing plan, the NET may also be used as an ATN address to locate a given system, or as a source from which to derive address prefixes of all systems within a given routing domain (e.g. during routing initiation).

For internetwork addressing consistency within the ATN, the NET and all NSAPs in use on a given system should only differ by the value of the SEL field per Table 1.

### Internetwork Address Registration

To ensure internetwork name and address unambiguity within the ATN, certain fields of the NSAP address format, and therefore also the NET, RDI and administrative domain identifier formats must be registered according to a well-defined process.  Specifically, values for the ADM field should be registered as follows:

- The ADM field should be assigned a value representing the IATA or ICAO organisation which is responsible for the corresponding name or address as indicated by the value of the VER field.  This will effectively define the ATN administrative domain identifier. The ADM should generally be an IATA 3-letter code for airlines (e.g. the ”airline designator”) and other stakeholders or a 3-letter country code for ICAO member states.  IATA has already proposed a registration procedure through their Geneva offices based on the use of current alphanumeric ”airline designators” with extensions for ”other stakeholders” compatible with the IATA Passenger Services.  ICAO has yet to define a registration process, but should follow the IATA example by assigning common country name abbreviations or those defined by ISO.

- One (or more) registration document(s) should be established, where values for the ADM field of the ATN NSAP address will be registered. This could be similar to the ISO 6523 standard document, where International Code Designators (ICD) are listed for addressing authorities registered with ISO (e.g. the value "0027" for ICAO can be found there). The registration document for ADM values may for instance contain tables with the following layout:

| ICAO member / ATSC Administration | ISO Country Code (3 letter code) | ADM value (hexadecimal) |
| --- | --- | --- |
| ... |  |  |
| Finland | FIN | 46 49 4E |
| France | FRA | 46 52 41 |
| Germany | DEU | 44 45 55 |
| ... |  |  |

| IATA member / AISC Administration | IATA Airline Code (3 letter code) | ADM value (hexadecimal) |
| --- | --- | --- |
| ... |  |  |
| Air France | AFR | 41 46 52 |
| British Airways | BAE | 42 41 45 |
| Lufthansa | DLH | 44 4C 48 |
| South African Airlines | SAL | 53 41 4C |
| United Airlines | UAL | 55 41 4C |
| ... |  |  |

Table 2. Example ADM value Assignments for ATSC and AISC Administrations

ARS values can be assigned, administered and registered by the authority designated in the ADM field. A guideline for registration procedures and an outline for registration documents could be established by ICAO and/or IATA.

Note: Administrations may derive the ARS values from the IA-5 encoding of the IATA or ICAO location identifier [11].

- A common ICAO/IATA database maintaining the above information, plus the ARS values of all ATN-equipped aircraft, should be kept and published through the appropriate ICAO and IATA channels.  This database should be arranged to identify all possible ATN routing domains and the organisations responsible for each one.  This database may be the basis of generalised directory services for post-CNS/ATM-1 Package ATN deployments.

- Values for the LOC and SYS fields may be registered within a routing domain or administrative domain, but, in general, there is little need for ATN-wide coordination of these values.  However, in order to support ATN directory services in the long term, the NSAP and presentation addresses of ATN applications should be maintained in a possibly-distributed ATN directory application.

### Relationship To ATN Routing

In order to support the ATN routing concept, the ATN NSAP addressing plan must be respected, especially with regards to the registration and use of ADM and ARS values.  As the NSAP addressing plan is based on the expected ATN topology, it has been defined hierarchically allowing routing information to be managed efficiently.  Specifically, address prefixes comprising of simply an administrative domain identifier or routing domain address may be used to describe routes to all systems within a given domain.  For example, the IDRP protocol may be used to indicate that a given router has routes to all systems in a given administration or airline rather than identifying every system individually.  As an extension of this idea, there may some merit in considering minor modifications to the addressing plan allowing air/ground routers to efficiently  advertise routes to ”all mobiles” as a first step in broadcasting reachability information.

During the CNS/ATM-1 Package timeframe, IDRP may not be supported over the mobile subnetworks.  This will have no impact in the addressing plan as the routing initiation process will ensure that both the air/ground and airborne routers will have enough information about each other and their corresponding communications capabilities.  The air/ground router’s IDRP entity can forward the routing domain address of the aircraft (derived from the airborne router’s NET) to other domains requiring this information.  On the other hand, the airborne router must have some pre-configured knowledge of the reachability of destination systems via a given air/ground router as there may not be any means of dynamically adverting route information to all aircraft.

### Minimising The Impact of Topology Changes and Outages

From time to time, the ATN internetwork topology may change or a system outage may occur which effectively changes the internetwork topology.  Because the ATN is expected to be used for air traffic management applications, operational availability must be very high. Consequently, the effects of ATN internetwork topology changes should be transparent to ATN users or must be minimised in order to avoid losing vital communications paths.

There are two aspects to be considered: Planned or unforeseen changes:

Planned Changes

If a system is moved to another routing area or another routing domain or if routing areas or routing domains are reconfigured, new NSAP addresses or NETs must be assigned. Any system known to need to communicate with the system whose address will change must be informed well in advance. A period of time in which a system maintains two or more NSAP addresses for the same NSAP may also ease the transition.  If this is the case, IDRP-equipped routers may broadcast the reachability of a given system by both its old and new address for a limited time, thus minimising the overall impact of the change.  Additionally, use of directory services may be used to effectively map a desired application name to the appropriate NSAP.

Reconfiguration of the network topology inside a routing area or inside a routing domain, which is not partitioned into routing areas, should be made transparent to ATN users, i.e. the NSAP address should not be changed.

Unforeseen Changes

Unforeseen changes may occur due to failure of a given end system or router. If an end system fails, a standby or backup system must be available to avoid loss of communication. If a router fails, either a standby or backup router or a redundant path may guarantee operational availability of the ATN. In the latter case, the problem is covered by routing procedures (re-routing of packets); NSAP addresses or NETs are not changed.

If a stand-by end system or router replaces the failed system, the replacement system should effectively adopt the NSAP address or NET of the failed system and the local routing tables should be updated to indicate the local topology change.  Thus the availability of ATN services can be ensured and made transparent to ATN users; the effects of the failure are not be propagated outside of the local domain.

## Transport and Upper Layer Addressing

As indicated previously, CNS/ATM-1 Package applications will likely be supported over an upper layer stack offering certain common application services to ATN applications.  As a consequence, the address of a CNS/ATM-1 Package application will be its presentation address, which is based on an NSAP address plus transport and additional ”selectors”.  There may be one or more applications on a given end system each one having an unique presentation address.

As the upper layer architecture is still evolving, it is difficult to define the exact application address format.  However, if there is a requirement to use a single transport connection for multiple applications whenever possible, an ATN presentation address must consist the host system’s NSAP, TSEL and at least one additional selector to identify a specific application using a given TSAP.  ICAO should consider and develop the application addressing requirements and associated solution as the upper layer architecture is being developed.

## Application Naming

During the CNS/ATM-1 Package timeframe certain ATM applications and other services, implemented as ATN applications, may need to be named in such a way to avoid the pre-assignment of application addresses.  Application names are generally referred to as AP-titles and must be globally unique within the ATN context. Following ISO naming principals [7498-3], we can ensure that each AP-title is unambiguous within the ATN (and consequently the OSIE) by assigning it according to a well-defined AP-title naming tree under the ultimate authority of ICAO. The CNS/ATM-1 Package application naming tree should reflect the administrative topology of the network. Thus, we can envision four principal types of applications in the CNS/ATM-1 Package timeframe:

1.	Ground based ATM applications under the responsibility of an ICAO Member State;

2.	Airborne ATM applications under the responsibility of an airline, but deployed in order to obtain improved ATM services from ICAO Member States;

3.	Ground based airline community applications under the responsibility of an IATA member;

4.	Airborne airline community applications under the responsibility of an IATA member.

Each of the four application types could be found in an AP-title subtree under the authority of one or more registration subauthorites.  Note that as the ATN is already registered by ICAO under the ICD 00027, ICAO is the ultimate ATN registration authority and has the responsibility to assign subauthorities as needed.

### Naming Scheme

An overall naming scheme needs to be established to guarantee consistency of all names used in the ATN. An ATN naming schema shall consider all names used in the ATN that have to be standardised.

Once established, such a naming scheme can be used as a directory scheme, when directory services shall be provided to ATN users. A directory scheme is a set of rules and constraints concerning the Directory Information Tree (DIT) structure and its contents. These rules ensure that the contents are valid entries and contain valid information. They define, what the user can do with entries and their contents.

Note: Even if no OSI Directory Service is intended to be used within the CNS/ATM-1 Package timeframe, it is highly recommended to use the principles described in the OSI Directory standard.

The following is a first approach to design a naming tree for ground based and airborne ATN applications.

All ATN names, which have to be standardised shall have the Distinguished Name form. A Distinguished Name is an ordered sequence of name fields referred to as Relative Distinguished Names (RDNs). A RDN consists of two elements: an attribute type and an attribute value. The attribute type is an Object Identifier characterising the name field and the attribute value is an IA5 string for the name itself.

An ATN DIT may contain the following object classes:

Organisation (O): This object class is used at the first level to identify either ICAO or IATA as an international organisation, which is direct subordinate under the root.

Administration (ADM): This object class may be used to identify either ATSC administrations (national CAA, Eurocontrol etc.) or AINSC administrations (airlines).

Country (C): The country object class may be used instead of the administration object class to identify national CAAs. It shall contain the two letter ISO country code.

Flight Number (FN): This object class shall be used for all airborne applications or other names to identify objects on board of aircraft.

ATC Centre (ATCC): This object class shall contain the ATCC name for all ATSC applications or other names to identify objects located in an ATCC.

Location (L): This object class shall identify the location of a ground located AINSC application.

Sector (SE): It may be useful to distinguish between different control sectors in an ATCC. This object class may be used to identify a control sector.

Application Process (AP) and Application Entity (AE): These object classes shall be used to identify the type of the ground or airborne ATSC or AINSC application. The AE object class is always subordinate of the AP object class, while the AP object class may be subordinate to Sector, ATC Centre, Location or Flight Number.

Figure 3 shows a proposed DIT structure for ATN application processes and application entities.

Note: Besides APs and AEs other object classes may be added to the DIT in the future, if other name types need to be standardised, e.g. end system or server names, user or user group names etc.

Figure 3.  Proposed DIT Structure

### Ground-Based ATM Applications

The following are examples for ground based ATM application names:

Note: AE types have not been identified yet. Therefore, only AP names are given in the examples.

1. Example for an Automatic Dependent Surveillance (ADS) application running in sector "SR 1" in the ATCC Munich:

O = 	ICAO

ADM =	DFS

ATCC =	Munich

SE =	SR 1

AP =	ADS

2. Same example as above, but using the ISO country code instead of an administration identifier:

O = 	ICAO

C =	DE

ATCC =	Munich

SE =	SR 1

AP =	ADS

### Airborne ATM Applications

Example for a Controller Pilot Communications (CPC) operated in a Lufthansa aircraft on flight LH4347:

O = 	IATA

FN =	LH4347

AP =	CPC

### Other Ground-Based Applications

Example for a Flight Information Service (FIS) application operated by Lufthansa operational centre in Munich:

O = 	IATA

ADM =	Lufthansa

L =	Munich

AP =	FIS

### Other Airborne Applications

Example for a Flight Information Service (FIS) application operated in a Lufthansa aircraft on flight LH4347:

O = 	IATA

FN =	LH4347

AP =	FIS

### Name and Address Relationship in CNS/ATM-1 Package

To date very little effort has been focused on the relationship between ATN application names and corresponding addresses outside of the scope of ADS, CPC and the supporting CM function.  Based on the expected CNS/ATM-1 Package upper layer architecture and the expected limited deployment of ATN systems during the CNS/ATM-1 Package timeframe, the mapping between application names and addresses could be resolved a priori thus eliminating the need to impose application names in the short term.  This assumption is not unreasonable, especially for applications such as AIDC which will likely have few address and name changes.  However, unless the structure of application naming tree and the corresponding registration authorities are defined during CNS/ATM-1 Package, the scalability of the ATN may be limited by requiring that all ATN applications individually maintain addressing information about all other applications with which they may need to communicate.  ICAO should pursue the definition of a naming tree allowing the development of ATN directory services to support the potential growth of ATN.

For distributed air/ground applications such as ADS and CPC, a basic name-to-address resolver, CM, has already been defined.  CM is a critical function allowing systems onboard an aircraft and within a given ATC facility (or FIR) to identify their service capabilities (applications) and their corresponding addresses to each other.  It should be noted however that, as demonstrated in paragraph 6, CM provides limited capabilities in a limited context.

### Application Name Registration

ATN AP-titles should be registered based on the registration procedures [9834-1] and [9834-6]. Specifically, ICAO should operate as the International Registration Authority and delegate the name registration responsibilities to both itself (for ATM services and applications) and IATA (for non ATM services and applications).  The name registration responsibilities may be further delegated according to the administrative taxonomy of the ICAO and IATA organisations. For example a given ICAO Member State could take responsibility for registering the nation’s ATM services, while a given IATA member could be responsible for the registration of its applications.  It is recommended, however, that certain ATM services such as ADS, CPC and CM be assigned a common ”application identification” within the context of a given ATC facility or FIR to minimise any potential confusion at the international level.

Both the object identifier form and directory name form described in [9834-1] may be used, especially when considering the potential use of OSI directory services for the ATN.  As both forms relate to the same naming tree, either nomenclature could be used to identify a given application.

The object identifier form should be based on ICAO ICD (00027) and the ATN arc (0).  As ICAO is registered under ISO, all ATN applications (in fact, all ATN objects) begin with the prefix {1 3 27 0}.  It is currently understood that a hierarchy for the ATN object identifier ”name space” has not yet been established.  Unless an ATN object naming trees is defined, the ATN community will be forced to continue with virtually a ”flat” object (and application) ”name space”.  It is recommended that an object naming tree be defined by ICAO.

# The Use of Context Management as a Basic Directory Service

This section introduces the use of the ATN naming and addressing concept for ATM applications distributed between airborne and ground-based systems.  Further and more detailed information about the context management application can be found in [5].

Within the CNS/ATM-1 Package timeframe, the CM application will provide a basic directory service which provides the following capabilities:

1.	Ground-based ATM applications may determine the ATN application capabilities of a given aircraft and determine the addresses of relevant applications.

2.	Airborne ATM applications may determine the ATN application capabilities within a given FIR and determine the addresses of relevant applications.

3.	The ground-based CM function may advertise the update of relevant ground based ATM application addresses to the airborne CM application.

4.	The ground-based CM function may direct its airborne peer CM to contact another ground-based CM application, perhaps in another FIR.

Once an aircraft has ”logged on” (see [5]) using CM the onboard systems are able to determine the application addresses of ATM services available from the local control region.  Correspondingly, the local control region’s ATCC will know the aircraft’s ATM application capabilities and their respective ATN application addresses at completion of logon.  The logon process can be described as follows:

1.	Assuming one does not already exist, the aircraft’s CMA establishes a transport connection with the ATCC CMA in the control region currently responsible for the aircraft.  The aircraft is required to know which control region it is in and the corresponding address of the CMA application.

Note:  In the absence of generalised directory services, it may be worthwhile to establish default CMA application addresses within each ATN equipped control region.  This would simply mean defining, at the ICAO level, fixed LOC, SYS, NSEL, TSEL and additional upper layer selector values for the ground CMA applications.  The RDI of a given control region can be derived from the name of the control region, presumably already known by the aircraft, and expressed in the ARS field.  The airborne CMA would then append the predefined values of the above-listed fields to the RDI identifier to produce the required CMA address.

2.	The aircraft CMA sends a CMLogonRequest message indicating the aircraft flight identification (its flight number), and an associative list of application names and addresses.  The application names are in fact currently defined to simply be 3-character Application IDs (from paragraph 5.3.2.2) It is assumed that the ATCC can derive the full application name from the associated addressing information and flight identification.  At this point, the ATCC will be able to associate the aircraft, identified in the flight plan by its flight identification, to the supported ATM applications and their corresponding addresses.

3.	If the ground CMA is able to support the logon request, it sends a CMLogonResponse message indicating that the aircraft logon request has been accepted along with an associative list of applications and their associated application addresses similar to the aircraft’s list. At this point, the aircraft will be able to associate the ATM applications supported by the ATCC with their corresponding applications addresses.

Following the logon procedure, peer application names and addresses can be distributed within the aircraft and ATCC domains by local means.  At this point, ATM applications may now communicate between the aircraft and ATCC.  If, for some reason, the ground application addresses change, the ATCC may send an updated associative name/address list to the aircraft.  Additionally, to support transfer of control to the aircraft’s next control region, the ATCC may send a ContactRequest message to the airborne CMA indicating the next control centre’s CMA application address.