ATNP WG1/IP 3-3

2 October 1995 Revison

Aeronautical Telecommunication Network Panel

Working Group 2

Priority Definitions within Annex 10 and the Relationship to the ATN SARPs

Prepared by:  Ron Jones

This working paper is an update to a previous version that was reviewed at the Rome WG2 meeting in July 1995.  The pervious version was accepted by WG2 and distributed as an attachment to Flimsy 3 from the Rome WG2 meeting.  The minor revisions reflected in this paper are based on additional inputs received subsequent to the Rome WG2 meeting.  This working paper presents the results of a review of Amendment 70 to ICAO Annex 10, as related to Chapter 4, as well as the existing Annex 10 Chapter 5, ITU Radio Regulations - Part 51, and the ATN Manual second edition definitions of message categories and communication priorities.  Only the use of priorities associated with the physical through network layers are addressed by this working paper.

References:  	Flimsy 3 from WG2 meeting in Rome, July 1995

ICAO Annex 10, Chapter 4 and Chapter 5

Proposed Amendment 70 to ICAO Annex 10

ATN Manual, second edition

1.0  Background:

The categories associated with the priority levels in the ATN Manual, second edition, were modified at SICASP/5 in an attempt to better align with what we believed would eventually be included in Amendment 70 to Annex 10 for the definition of message priorities.  I have looked over the proposed revisions to Annex 10 (Amendment 70) and can report that the proposed changes are related to the Aeronautical Fixed Service standards in Chapter 4 of Annex 10.  The definitions in Amendment 70 do not track one-for-one with what is in the ATN Manual or what is defined for the Aeronautical Mobile Service in Chapter 5 of Annex 10.  Currently Annex 10, when taken along with ITU Radio Regulations Part 51, include three relevant sets of definitions related to message priority.  First is for r.f. spectrum utilization for Aeronautical Mobile Service (ITU Part 51 - referenced in Annex 10, Chapter 5), the second is the material included directly in Annex 10, Chapter 5 for the Aeronautical Mobile Services and the third is for the Aeronautical Fixed Services (ICAO Annex 10, Chapter 4). Note that Annex 10 is already not totally self-consistent where the definitions of message categories and priorities are concerned.  This is clearly an area that needs to be reviewed by ATNP since the ATN will encompass both the aeronautical fixed and aeronautical mobile services

2.0  Discussion.

The following three tables describe the current definition of message categories and associated priorities from the ICAO Annex 10 and the ITU Radio Regulations.

__________________________________________________________________________

Table 1 - Definitions from ITU Article 51 from 1990 edition of the ‘Radio Regulations’

(Note that Category 1 is the highest priority)

1.  Distress calls, distress messages and distress traffic

2.  Communications proceeded by the urgency signal

3.  Communications relating to radio direction finding

4.  Flight safety messages

5.  Meteorological messages

6.  Flight regularity messages

7.  Messages relating to the application of the United Nations Charter

8.  Government messages for which priority has been expressly requested

9.  Service communications relating to the working of the telecommunication

service or to communications previously exchanged

10. Other aeronautical communications

Note the term communications is defined in Part 51 to include:  Radiotelegrams, radiortelephone and radiotelex calls.

________________________________________________________________________

Table 2 - Definitions from ICAO Annex 10 (Chapter 5 -Aeronautical Mobile Service)

From Para. 5.1.8, ‘Categories of Messages’.  Note that no categories, or priorities,

are defined lower than ‘flight regularity messages’ for the Aeronautical Mobile Service.

(Note category ‘a’ is the highest priority)

a)  Distress calls, distress messages and distress traffic

b)  Urgency messages, including messages proceeded by the medical transports

signal

c)  Communications relating to direction finding

d)  Flight safety messages

e)  Meteorological messages

f)  Flight regularity messages

Note that para. 5.1.8 includes a note that indicates NOTAMS may qualify for any of the categories or priorities c) to f) inclusive depending on the contents of the NOTAM.

Note that para. 5.1 references the ITU Radio Regulations.

________________________________________________________________________

Table 3 - Definitions from ICAO Annex 10 (Chapter 4 - Aeronautical Fixed Services)

From Para. 4.4.1 ‘Categories of Messages’ (over AFTN).  The current Annex 10, para. 4.4.1 with the proposed changes from Amendment 70 are shown below as strikeout and underline for text that Amendment 70 proposes to delete (strikeout) or proposes to add (underline).

a)  distress calls and distress traffic

b)  urgency messages

c)  flight safety messages

d)  meteorological messages

e)  Flight regularity messages

f)  aeronautical administrative messages

g)  NOTAM - Class I distribution

h)  reservation message

i)  general aircraft operating agency messages

j)  service messages

f)  aeronautical information services (AIS) messages

g)  aeronautical administrative messages

h)  service messages

Unlike the ITU Radio Regulation or the ICAO Annex 10 Chapter 5 standards for Aeronautical Mobile Service, the above message categories are mapped into just 3 transmission priority levels (as per the current Annex 10, Chapter 4 and remains unchanged with Amendment 70), although the AFTN header will use 5 different priority indicators to denote application priority.  Transmission priority 1 (highest priority) includes message category a) only.  Transmission priority 2 includes message categories b) and c).  Transmission priority 3 includes message categories d), e), f) and g).  Message category h) ‘service messages’ “shall take the same priority indicator as the category of the message being requested except where higher priority is warranted for flight safety.”

Note that even the lowest category of message, for which a specific transmission priority level is specified (i.e., Aeronautical Administrative messages), appears to include messages associated with safety and regularity of flight.

The following table (Table 4) provides a summary of the message categories and associated priorities as defined by ITU, ICAO Annex 10 (Chapter 4, with Amendment 70 and Chapter 5), and the ATN Manual Second Edition.  Table 1 also shows a recommended mapping between the message categories defined by these documents.

Table 4 - Mapping of Message Categories and Associated Priority Levels

| ITU Radio Regulations, Part 51 [Priority] | ICAO Annex 10, Chapter 5, Aeronautical Mobile Service [Priority] | ICAO Annex 10, Chapter 4, Aeronautical Fixed Service [Priority] | ICAO ATN Manual, Second Edition [CLNP Priority] |
| --- | --- | --- | --- |
| [1] Distress calls, distress messages and distress traffic | [a] Distress calls, distress messages and distress traffic | [1] Distress calls | [14] Network/Systems Management |
| as above | as above | as above | [13] Distress Communications |
| [2] Communications proceeded by the urgency signal | [b] Urgency messages, including messages proceeded by the medical transports signal | [2] Urgency messages | [12] Urgent Communications |
| [3] Communications relating to radio direction finding | [c] Communications relating to direction finding | Not Applicable | [11] Communications relating to radio direction finding |
| [4] Flight safety messages | [d] Flight safety messages | [2] Flight safety messages | [10] Flight safety messages |
| [5] Meteorological messages | [e] Meteorological messages | [3] Meteorological messages | [9] Meteorological communications |
| [6] Flight regularity messages | [f] Flight regularity messages | [3] Flight regularity messages | [8] Flight regularity communications |
| as above | as above | [3] Aeronautical information services | [7] Aeronautical information service messages |
| as above | as above | Not Applicable | [6] Network/Systems administration |
| Not Applicable | Not Applicable | [3] Aeronautical administrative messages | [5] Aeronautical administrative messages |
| Not Applicable | Not Applicable | [*] Service message *priority as appropriate | [4] Unassigned |
| [7] Messages relating to the application of the United Nations Charter | Not Applicable | Not Applicable | [3] Urgent priority administrative and U.N. Charter communications |
| [8] Government messages for which priority has been expressly requested | Not Applicable | Not Applicable | [2] High priority administrative and state/government communications |
| [9] Service communications relating to the working of the telecommunication service or to communications previously exchanged | Not Applicable | Not Applicable | Not Applicable |
| [10] Other aeronautical communications | Not Applicable | Not Applicable | [1] Normal priority administrative |
| as above | Not Applicable | Not Applicable | [0] Low priority administrative |

3.  Conclusions:

3.1  ICAO Annex 10 only defines message categories for Aeronautical Fixed or Mobile Services that support safety and regularity of flight.  ITU Radio Regulations, Part 51, define lower priorities of aeronautical communications in addition to those supporting safety and regularity of flight.

3.2  ICAO has not defined the message categories and associated priorities such that they map one-for-one with the ITU message categories.  For example, in the case of the aeronautical fixed services multiple categories of messages are mapped into the single overall ITU category of “flight regularity messages.”

3.3  Since the ATN will support communications traffic that falls into both mobile and fixed services and a single message may traverse both mobile and fixed service subnetworks, it is necessary for the ATNP to define a rational mapping between the ATN message categories (with associated communications priorities) and the message categories and communications priorities defined for aeronautical mobile and for aeronautical fixed services.  Mapping should also be provided to the ITU Radio Regulations, Part 51 categories, as this is a controlling standard for the use of radio spectrum (i.e., the physical layer of the ATN mobile subnetworks).

3.4  The ATN Manual, second edition, Table A5-1 indicated that message categories with CLNP priority level 8 or greater correspond to communications associated with safety and regularity of flight.  Likewise Table 10-1 indicated that CLNP priority levels less than 8 will not be mapped to a Mode S subnetwork priority level, because these categories were not considered to be associated with safety or regularity of flight communications.

3.5  After a review of the Annex 10, ITU Radio Regulations Part 51 and the ATN Manual definitions of message categories and associated priorities, it appears that ATN CLNP priority levels of 6 or greater should be associated with safety and regularity of flight.  Specifically CLNP priority levels 6, 7 and 8 are all associated with the single ITU category for regularity of flight communications.

4.0 Proposal

It is proposed that ATNP WG2 take the following actions:

4.1  Review the above information and consider the use of the message categories and associated priorities, as currently defined in the draft ATN SARPs, with mapping to the ITU categories and the Annex 10, Chapter 4  and Chapter 5 message categories and priorities.  Agree that the ATN need not provide the same number of priorities as used today on AFTN nor as defined by ITU Radio Regulations.  Agree that the mapping, as proposed in Table 4, with the modifications proposed in Table 5 below, would be provided as at least guidance material on how to map the message categories between ATN and the aeronautical fixed and aeronautical mobile services as defined in Annex 10.

4.2  The ATN SARPs should be modified to reflect that message categories for CLNP priorities 6 or greater are associated with communications supporting safety and regularity of flight.

4.3  Since it is unlikely that “communications relating to radio direction finding” will ever have a significant role for communications via the ATN, it is proposed that this ITU level be mapped to CLNP priority 11 which would be defined for ‘high priority safety communications’.  It is also proposed that CLNP priority 10 be defined for ‘normal priority safety communications’ as shown in Table 5 below

4.4  It does not appear that these proposed changes would have any impact on the VDL (Mode 1 or Mode 2) SARPs but the AMSS SARPs may need minor editorial changes to reflect the revised definitions associated with the CLNP priorities 10 and 11.  It does not appear that this proposal would necessitate any technical changes in the AMSS SARPs since the mapping between CLNP and AMSS subnetwork priorities is not directly impacted. The AMCP is now considering the need for the VDL Mode 3 (TDMA) subnetwork to support the use of priority which could be impacted by the proposed changes to CLNP priority definition.  A flimsy should be prepared informing the AMCP of the revisions to the CLNP priority definition.

4.5  The ATN Guidance Material (derived from the old Table 10-1 of ATN Manual) should show mapping of CLNP priorities 6, 7, 8 and 9 to the low priority Mode S subnetwork connection and CLNP priorities of 10 and greater to the high priority Mode S subnetwork connection.  A flimsy should be prepared and forwarded to the SICAS Panel informing them of the decisions related to use of priority on the ATN and requesting their concurrence related to the proposed mapping by the SNDCF of the CLNP priorities to the high and low priority Mode S subnetwork connections.

4.6  A flimsy should be prepared and forwarded to ATNP WG3 informing them of the decisions related to use of priority on the ATN.

Table 5 - Proposed Mapping of Message Categories and Associated Priority Levels

| ITU Radio Regulations, Part 51 [Priority] | ICAO Annex 10, Chapter 5, Aeronautical Mobile Service [Priority] | ICAO Annex 10, Chapter 4, Aeronautical Fixed Service [Priority] | ICAO ATN SARPs [CLNP Priority] |
| --- | --- | --- | --- |
| [1] Distress calls, distress messages and distress traffic | [a] Distress calls, distress messages and distress traffic | [1] Distress calls | [14] Network/Systems Management |
| as above | as above | as above | [13] Distress Communications |
| [2] Communications proceeded by the urgency signal | [b] Urgency messages, including messages proceeded by the medical transports signal | [2] Urgency messages | [12] Urgent Communications |
| [3] Communications relating to radio direction finding | [c] Communications relating to direction finding | Not Applicable | [11] High piority fight safety messages |
| [4] Flight safety messages | [d] Flight safety messages | [2] Flight safety messages | [10] Normal priority flight safety messages |
| [5] Meteorological messages | [e] Meteorological messages | [3] Meteorological messages | [9] Meteorological communications |
| [6] Flight regularity messages | [f] Flight regularity messages | [3] Flight regularity messages | [8] Flight regularity communications |
| as above | as above | [3] Aeronautical information services | [7] Aeronautical information service messages |
| as above | as above | Not Applicable | [6] Network/Systems administration |
| Not Applicable | Not Applicable | [3] Aeronautical administrative messages | [5] Aeronautical administrative messages |
| Not Applicable | Not Applicable | [*] Service message *priority as appropriate | [4] Unassigned |
| [7] Messages relating to the application of the United Nations Charter | Not Applicable | Not Applicable | [3] Urgent priority administrative and U.N. Charter communications |
| [8] Government messages for which priority has been expressly requested | Not Applicable | Not Applicable | [2] High priority administrative and state/government communications |
| [9] Service communications relating to the working of the telecommunication service or to communications previously exchanged | Not Applicable | Not Applicable | Not Applicable |
| [10] Other aeronautical communications | Not Applicable | Not Applicable | [1] Normal priority administrative |
| as above | Not Applicable | Not Applicable | [0] Low priority administrative |