CNS/ATM-1 Package

Sub-Volume 1

Introduction and 
System-Level Requirements

Version 0.6

7-June-1996

## Sub-Volume 1:  Introduction and System-Level Requirements

## Introduction

The International Civil Aviation Organization (ICAO) is developing Standards and Recommended Practices (SARPs) to advance the Communication, Navigation, Surveillance/Air Traffic Management (CNS/ATM) concepts.  These advanced CNS/ATM concepts include the use of distributed data applications and supporting data communication services to:

- Deliver air traffic services (ATS) to aircraft;

- Exchange air traffic management (ATM) information between fixed-based ATS facilities on the ground; and

- Control the movements of aircraft and vehicles operating on airport surfaces.

The Aeronautical Telecommunication Network (ATN) provides communication services and applications to support CNS/ATM concepts.

### Scope.

Figure 1.1-1 shows an overview of ATN.

Figure 1.1-1:  Overview of ATN.

These SARPs define:

- Context Management (CM), (Data link initiation capability)

- Controller/Pilot Data Communications (Controller Pilot Data Link Communications (CPDLC), includes (Pre)Departure Clearance)

- Automatic Dependent Surveillance (ADS)

- Automatic Terminal Information Services (ATIS) as an Flight Information Services (FIS) application

- Inter-Centre Co-ordination (ICC) as an Air Traffic Services (ATS) Interfacility Data Communications (AIDC) application

- ATS Message Handling Services (AMHS)

- Upper Layers Communication Services

- Internet Communication Services

### Overview

Figure 1.1-2 shows an overview of these SARPs.

Figure 1.1-2: ATN SARPs overview.

## System Level Requirements

Note.—  The system level requirements are high-level technical requirements that have been derived from operational needs, technological constraints, safety objectives and regulatory constraints (administrative and institutional).  These system-level requirements are the basis for the lower level technical and derived requirements.  Figure 1.2-1 is a conceptual overview of the relationship between high-level requirements, low-level requirements and external constraints

Figure 1.2-1:  System Level Requirement Concept Diagram

### ATN System Level Requirements

#### The ATN shall use the International Organization for Standardization (ISO) Open Systems Interconnection (OSI) standards.

#### The ATN shall provide a means to facilitate migration to future versions.

#### The ATN shall enable the transition of existing AFTN users and systems into the ATN architecture.

#### The ATN shall enable only the authorized ATC authority to provide ATC instructions to aircraft operating in its airspace.

#### The ATN shall employ policy based routing.

#### The ATN shall enable data communications to be carried only over authorized paths for the type of traffic specified by the user.

#### The ATN shall employ ATSC traffic classes in accordance with the criteria in Table 1.2-1.

Table 1.2-1:  ATSC Traffic Classes

| One way End-to-End Transit Delay at 95% probability (seconds) | ATSC Traffic Class |
| --- | --- |
| Reserved | A |
| Reserved | B |
| 7.2 | C |
| 13.5 | D |
| 18 | E |
| 27 | F |
| 50 | G |
| 100 | H |
| No value specified | No preference |

#### The ATN shall enable communication priorities in accordance with Table 1.2-2.

Table 1.2-2.  Relationship of ATN applications to communication priorities

| Message Categories | ATN Application |  | Corresponding Protocol Priority |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Transport Layer Priority |  | Internet Layer Priority |
|  |  |  |  | Transport Connection Priority | TSDU Priority | CLNP Priority |
| Network/Systems Management |  |  |  | 0 | 0 | 14 |
| Distress Communications |  |  |  | 1 | 1 | 13 |
| Urgent Communications |  |  |  | 2 | 2 | 12 |
| High Priority Flight Safety Messages |  |  |  | 3 | 3 | 11 |
| Normal Priority Flight Safety Messages |  | CPDLC ADS AIDC |  | 4 | 4 | 10 |
| Meteorological Communications |  |  |  | 5 | 5 | 9 |
| Flight Regularity Communications |  | CM |  | 6 | 6 | 8 |
| Aeronautical Information Service Messages |  | FIS |  | 7 | 7 | 7 |
| Network/Systems Administration |  |  |  | 8 | 8 | 6 |
| Aeronautical Administrative Messages |  |  |  | 9 | 9 | 5 |
| <unassigned> |  |  |  | 10 | 10 | 4 |
| Urgent Priority Administrative and U.N. Charter Communications |  |  |  | 11 | 11 | 3 |
| High Priority Administrative and State/Government Communications |  |  |  | 12 | 12 | 2 |
| Normal Priority Administrative |  |  |  | 13 | 13 | 1 |
| Low Priority Administrative |  |  |  | 14 | 14 | 0 |
| Note:— Priorities above double line are for communications related to safety and regularity of flight. |  |  |  |  |  |  |

#### The ATN shall enable peer to peer application exchange of information when an authorized path exists.

#### The ATN shall notify the appropriate application processes when no authorized path exists.

#### The ATN shall provide a means to unambiguously address all ATN End and Intermediate systems.

#### The ATN shall enable the recipient of a message to positively identify the originator of that message within a dialogue.

#### The ATN addressing plan shall permit States and organisations to assign addresses within their own administrative domains.

#### The ATN shall support fixed and mobile systems.

#### The ATN shall accommodate ICAO standardized mobile subnetworks.

#### The ATN shall enable an aircraft Intermediate System to be connected to a ground Intermediate System via concurrent mobile subnetworks.

#### The ATN shall enable an aircraft Intermediate System to be connected to multiple ground Intermediate Systems.

#### The ATN shall enable exchange of application address information.

#### The ATN shall be capable of establishing, maintaining, releasing, forwarding and aborting peer to peer application associations for Automatic Dependent Surveillance (ADS).

#### The ATN shall be capable of establishing, maintaining, releasing, forwarding and aborting peer to peer application associations for Controller Pilot Data Link Communications (CPDLC).

#### The ATN shall be capable of establishing, maintaining, releasing, forwarding and aborting peer to peer application associations for Context Management (CM).

#### The ATN shall be capable of establishing, maintaining, releasing and aborting peer to peer application associations for Flight Information Services/Automatic Terminal Information System (FIS/ATIS).

#### The ATN shall be capable of establishing, maintaining, releasing and aborting peer to peer application associations for ATS Message Handling Services over the ATN (AMHS).

#### The ATN shall be capable of establishing, maintaining, releasing and aborting peer to peer application associations for Inter-Centre Co-ordination/ATS Interfacility Data Communications Application (ICC/AIDC).

#### The ATN shall reference time based on Co-ordinated Universal Time (UTC).

## References

| ICAO ADSP/4 | Report of the ICAO ADSP, “Automatic Dependent Surveillance (ADS) and Air Traffic Services (ATS) Data Link Applications Guidance Material,” dated tbd 1996. ADS, CMA, CPDLC |
| --- | --- |
| NOCAR | ICAO NATSPG IMG/x, Appendix y, “nth Edition of the North Atlantic Oceanic Concept and Requirements Document (NOCAR),” tbd |
| ICAO Doc. 9623 | FANS(II)/4, Special Committee for the Monitoring and Co-ordination of Development and Transition Planning for the Future Air Navigation System (FANS Phase II), Fourth Meeting, Montreal, Sept 15 - Oct. 1, 1993. |
| ICAO Doc 7910 | Location Indicators. |
| ICAO Doc 8585 | Designators for Aircraft Operating Agencies, Aeronautical Authorities and Services. |
| ISO 6523: 1994 | Data interchange – Structures for the identification of organizations (Registration of International Code Designators). ULA |
| ISO/IEC 7498-1:1994 ITU-T Rec. X.200 (1994) | Information processing systems – Open Systems Interconnection – Basic Reference Model. Internet, ULA |
| ISO 7498-2:1989 | Information processing systems – Open Systems Interconnection – Basic Reference Model – Part 2: Security Architecture Internet |
| ISO 7498-3:1989 ITU-T Rec. X.650 (1992) | Information processing systems – Open Systems Interconnection – Basic Reference Model – Part 3: Naming and Addressing Internet, ULA |
| ISO/IEC 7498-4:1989 | Information processing systems – Open Systems Interconnection – Basic Reference Model – Part 4: Management framework Internet |
| ISO/IEC 8072:1994 ITU-T Rec. X.214 (1993) | Information technology – Open Systems Interconnection – Transport service definition (second edition) Internet ULA |
| ISO/IEC 8073:1992 | Information technology – Telecommunications and information exchange between systems – Open Systems Interconnection – Protocol for providing the connection-mode transport service Internet |
| ISO/IEC 8073:1992/PDAM5 | Information technology – Telecommunications and information exchange between systems – Open Systems Interconnection – Protocol for providing the connection-mode transport service - Amendment 5: Provision of Non-blocking Expedited Service Internet |
| ISO/IEC 8208:1995 | Information technology – Data communications – X.25 Packet Layer Protocol for Data Terminal Equipment (Revision of ISO 8208:1990) Internet |
| ISO/IEC 8326:1994 ITU-T Rec. X.215 (1994) ¦ | Information technology – Open Systems Interconnection – Basic Session Service Definition (draft second edition) Internet, ULA |
| ISO/IEC 8326:1995/DAM 1 | Information Technology – Open Systems Interconnection – Basic Session Service Definition - Amendment 1: efficiency enhancements ULA |
| ISO/IEC 8327-1:1994 ITU-T Rec. X.225 (1994) ¦ | Information Technology – Open Systems Interconnection – Basic Connection Oriented Session Protocol: Part 1 – Protocol Specification (draft second edition) Internet, ULA |
| ISO/IEC 8327-1:1995/DAM 1 | Information Technology – Open Systems Interconnection – Basic Connection Oriented Session Protocol: Part 1 – Protocol Specification - Amendment 1: efficiency enhancements ULA |
| ISO/IEC 8327-2:1994 | Information technology – Open Systems Interconnection – Basic connection oriented session protocol specification – Part 2: Protocol Implementation Conformance Statement (PICS) Proforma Internet |
| ISO/IEC 8348:1993 | Information technology – Open Systems Interconnection – Network Service Definition Internet |
| ISO/IEC 8473-1:1994 | Information technology – Protocol for providing the connectionless-mode network service: Protocol specification Internet |
| ISO/IEC 8473-2:1994 | Information technology – Protocol for providing the connectionless-mode network service – Part 2: Provision of the underlying service by an ISO/IEC 8802 subnetwork Internet |
| ISO/IEC 8473-3:1995 | Information technology – Protocol for providing the connectionless-mode network service – Part 3: Provision of the underlying service by an X.25 subnetwork Internet |
| ISO/IEC 8473-4:1995 | Information technology – Protocol for providing the connectionless-mode network service – Part 4: Provision of the underlying service by a subnetwork that provides the OSI data link service Internet |
| ISO TR 8509:1987 | Information processing systems – Open Systems Interconnection – OSI Service Conventions. Internet, ULA |
| ISO 8571-1:1988 | Information processing systems – Open Systems Interconnection – File Transfer, Access and Management – Part 1: General introduction Internet |
| ISO 8571-2:1988 | Information processing systems – Open Systems Interconnection – File Transfer, Access and Management – Part 2: Virtual Filestore Definition Internet |
| ISO/IEC 8602:1995 | Information technology – Protocol for providing the OSI connectionless-mode transport service Internet |
| ISO 8648:1988 | Information processing systems – Open Systems Interconnection – Internal organization of the Network Layer Internet |
| ISO/IEC 8649: 1994 ITU-T Rec. X.217 (1992) | Information processing systems– Open Systems Interconnection – Service definition for the Association Control Service Element (draft second edition) Internet, ULA |
| ISO/IEC 8649:1995/PDAM 1 | Information technology– Open Systems Interconnection – Service definition for the Association Control Service Element – Amendment 1: Fast associate mechanism. ULA |
| ISO/IEC 8650-1: 1996 ITU-T Rec. X.227 (1994) | Information processing systems– Open Systems Interconnection – Protocol specification for the Association Control Service Element (draft second edition) ULA |
| ISO/IEC 8650-1:1995 /DAM 2 | Information processing systems – Open Systems Interconnection – Protocol specification for the Association Control Service Element – Amendment 2: Fast associate mechanism. ULA |
| ISO/IEC 8650-1:1995 /DAM 1 | Information processing systems – Open Systems Interconnection – Protocol specification for the Association Control Service Element – Amendment 1: Incorporation of extensibility markers. ULA |
| ISO/IEC 8650-2:1995 | Information technology – Open Systems Interconnection – Protocol specification for the Association Control Service Element – Part 2: Protocol Implementation Conformance Statement (PICS) proforma Internet |
| ISO/IEC 8822:1994 ITU-T Rec. X.216 (1994) ¦ | Information technology – Open Systems Interconnection – Presentation service definition (Second edition) Internet, ULA |
| ISO/IEC 8822:1994 /DAM 1 | Information technology – Open Systems Interconnection – Presentation service definition – Amendment 1: efficiency enhancements. ULA |
| ISO/IEC 8823-1:1994 ITU-T Rec. X.226 (1994) ¦ | Information technology – Open Systems Interconnection – Basic connection-oriented presentation protocol – Part 1: Protocol specification (second edition) Internet, ULA |
| ISO/IEC 8823-1:1994/DAM 1 | Information technology – Open Systems Interconnection – Basic connection-oriented presentation protocol – Part 1: Protocol specification – Amendment 1: Efficiency enhancements ULA |
| ISO/IEC 8823-2:1995 | Information technology – Open Systems Interconnection – Basic connection-oriented presentation protocol – Part 2: Protocol Implementation Conformance Statement (PICS) proforma Internet |
| ISO/IEC 8824-1:1994 ITU-T Rec. X.682 | Information Technology - OSI Abstract Syntax Notation One (ASN.1). – Specification of basic notation. ADS, CMA, CPDLC, FIS |
| ISO/IEC 8824-1 /Amd.1 (1995) | Information Technology – Open Systems Interconnection – Abstract Syntax Notation One (ASN.1) - Specification of Basic Notation - Amendment 1: Rules for Extensibility. ULA |
| ISO/IEC 8824-2:1995 | Information technology - Open Systems Interconnection -Abstract Syntax Notation One (ASN.1) - Part 2: Information object specification Internet |
| ISO/IEC 8824-3:1995 | Information technology - Open Systems Interconnection - Abstract Syntax Notation One (ASN. 1) - Part 3: Constraint specification Internet |
| ISO/IEC 8824-4:1995 | Information technology - Open Systems Interconnection -Abstract Syntax Notation One (ASN. 1) - Part 4: Parameterization of ASN.1 Specifications Internet |
| ISO/IEC 8825-1:1995 ITU-T Rec. X.691 (1993) | Information technology – ASN.1 encoding rules – Part 1: Specification of Basic Encoding Rules (BER), Canonical Encoding Rules (CER) and Distinguished Encoding Rules (DER) . Internet, ULA |
| ISO/IEC 8825-2 ITU-T Rec. X.691 | Information technology – Open Systems Interconnection – Specification of Basic Encoding Rules for Abstract Syntax Notation One (ASN.1) – Part 2: Packed encoding rules ADS, CMA, CPDLC, FIS |
|  | Part |
| ISO DIS 8825-3 | Information Technology - Open Systems Interconnection -Specification of ASN.1 Encoding Rules - Part Distinguished and Canonical Encoding Rules Internet |
| ISO/IEC 8878:1992 | Information technology – Telecommunications and information exchange between systems – Use of X.25 to provide the OSI Connection-mode Network Service Internet |
| ISO/IEC 9072-1:1989 | Information Technology - Text Communication - Remote Operations - Part 1: Model Notation and Service Definition Internet |
| ISO/IEC 9072-2:1989 | Information Technology - Text Communication - Remote Operations - Part 2: Protocol Specification Internet |
| ISO CD 9072-4 | Information Technology - Text Communication - Remote operations service element – Protocol implementation conformance statement (PICS) proforma Internet |
| ISO/IEC 9542:1988 | Information processing systems – Telecommunications and information exchange between systems – End system to Intermediate system routeing exchange protocol for use in conjunction with the Protocol for providing the connectionless-mode network service (ISO 8473) Internet |
| ISO/IEC 9542:1988/DAM1 | Information processing systems – Telecommunications and information exchange between systems – End system to Intermediate system routeing exchange protocol for use in conjunction with the Protocol for providing the connectionless-mode network service ( ISO 8473) – Amendment 1: Dynamic Discovery of OSI NSAP Addresses by End Systems Internet |
| ISO/IEC 9545:1994 | Information technology – Open Systems Interconnection – Application Layer structure (second edition) Internet, ULA |
| ISO 9548:1993 | Information Technology - Open Systems Interconnection - Connectionless Session Protocol Specification Internet |
| ISO/IEC 9548-2:1995 | Information technology – Open Systems Interconnection – Connectionless session protocol specification – Part 2: Protocol implementation conformance statement (PICS) proforma Internet |
| ISO/IEC TR 9575:1995 | Information technology – Telecommunications and information exchange between systems – OSI Routeing Framework Internet |
| ISO/IEC 9576-1:1995 | Information technology – Open Systems Interconnection – Connectionless presentation protocol specification Internet |
| ISO/IEC 9576-2:1995 | Information technology – Open Systems Interconnection – Connectionless presentation protocol specification – Part 2: Protocol Implementation Conformance Statement (PICS) proforma Internet |
| ISO/IEC TR 9577:1993 | Information technology – Telecommunications and information exchange between systems – Protocol identification in the network layer Internet |
| ISO/IEC 9595:1991 | Information technology – Open Systems Interconnection – Common management information service definition Internet |
| ISO 9596-1:1991 | Information technology – Open Systems Interconnection – Common management information protocol – Part 1: Specification Internet |
| ISO 9596-2:1993 | Information technology – Open Systems Interconnection – Common management information protocol – Part 2: Protocol Implementation Conformance Statement (PICS) proforma Internet |
| ISO/IEC 9646-1:1994 | Information technology – Open Systems Interconnection – Conformance testing methodology and framework – Part 1: General concepts Internet |
|  | Part |
|  | Part |
| ISO/IEC 9646-2:1994 | Information technology – Open Systems Interconnection – Conformance testing methodology and framework – Part 2: Abstract Test Suite specification Internet |
|  | Part |
|  | Part |
| ISO/IEC 9646-4:1994 | Information technology – Open Systems Interconnection – Conformance testing methodology and framework – Part 4: Test realization Internet |
|  | Part |
|  | Part |
| ISO/IEC 9646-5:1994 | Information technology – Open Systems Interconnection – Conformance testing methodology and framework – Part 5: Requirements on test laboratories and clients for the conformance assessment process Internet |
|  | Part |
|  | Part |
| ISO/IEC 9646-7:1995 | Information technology – Open Systems Interconnection – Conformance testing methodology and framework – Part 7: Implementation Conformance Statements Internet |
| ISO/IEC 9834-1:1993 | Information technology – Open Systems Interconnection – Procedures for the operation of OSI Registration Authorities – Part 1: General procedures Internet |
| ISO/IEC 9834-2:1993 | Information Technology - Open Systems Interconnection - Procedures for specific OSI Registration Authorities - Part 2: Registration Procedures for OSI Document Types Internet |
| ISO/IEC 9834-6:1993 | Information technology – Open Systems Interconnection – Procedures for the operation of OSI Registration Authorities – Part 6: Application processes and application entities Internet |
| ISO/IEC TR 10000-1:1995 | Information technology – Framework and taxonomy of International Standardized Profiles – Part 1: General principles and documentation framework Internet |
| ISO/IEC TR 10000-2:1995 | Information technology – Framework and taxonomy of International Standardized Profiles – Part 2: Principles and Taxonomy for OSI profiles Internet |
| ISO/IEC 10021-1: 1990 | Information Technology – Text Communication – Message-Oriented Text Interchange System (MOTIS) - Part 1: System and Service Overview MHS |
| ISO/IEC 10021-1/Amd.2:1994 | MHS |
| ISO/IEC 10021-2: 1990 | Information Technology – Text Communication – Message-Oriented Text Interchange System (MOTIS) - Part 2: Overall Architecture MHS |
| ISO/IEC 10021-2/Amd.1:1993 | MHS |
| ISO/IEC 10021-2/Amd.2:1994 | MHS |
| ISO/IEC 10021-3: 1990 | Information Technology – Text Communication – Message-Oriented Text Interchange System (MOTIS) - Part 3: Abstract Service Definition Conventions MHS |
| ISO/IEC 10021-4: 1990 | Information Technology – Text Communication – Message-Oriented Text Interchange System (MOTIS) - Part 4: Message Transfer System: Abstract Service Definition and Procedures MHS |
| ISO/IEC 10021-4/Amd.1:1994 | MHS |
| ISO/IEC 10021-5: 1990 | Information Technology – Text Communication – Message-Oriented Text Interchange System (MOTIS) - Part 5: Message Store: Abstract Service Definition MHS |
| ISO/IEC 10021-5/Amendment 1 | Message Store Extensions and Message Store Logs MHS |
| ISO/IEC 10021-6: 1990 | Information Technology – Text Communication – Message-Oriented Text Interchange System (MOTIS) - Part 6: Protocol Specifications MHS |
| ISO/IEC 10021-7: 1990 | Information Technology – Text Communication – Message-Oriented Text Interchange System (MOTIS) - Part 7: Interpersonal Messaging System MHS |
| ISO/IEC 10025-1:1995 | Information technology – Telecommunications and information exchange between systems – Transport conformance testing for connection mode transport protocol operating over connection mode network service – Part 1: General principles Internet |
| ISO/IEC DIS 10025-2 | Information technology – Telecommunications and information exchange between systems – Transport conformance testing for connection mode transport protocol operating over connection mode network service – Part 2: Test suite structure and test purposes Internet |
| ISO/IEC 10025-3:1993 | Information technology – Telecommunications and information exchange between systems – Transport conformance testing for connection-mode transport protocol operating over connection-mode network service – Part 3: Transport test management protocol specification Internet |
| ISO/IEC 10028:1993 | Information technology – Telecommunications and information exchange between systems – Definition of the relaying functions of a Network layer intermediate system Internet |
| ISO/IEC 10035-1:1995 | Information Technology - Open Systems Interconnection - Connectionless ACSE Protocol to Provide the Connectionless-Mode ACSE Service Internet |
| ISO/IEC 10035-2:1995 | Information technology – Open Systems Interconnection – Connectionless ACSE protocol to provide the Connectionless-Mode ACSE service– Part 2: Protocol implementation conformance statement (PICS) proforma Internet |
| ISO/IEC 10040:1992 | Information technology – Open Systems Interconnection – Systems management overview Internet |
| ISO/IEC 10040:1992 /DAM1 | Information technology - Open Systems Interconnection - Systems management overview - DAM 1 : Management Knowledge Management Architecture Internet |
| ISO/IEC 10164-1:1993 | Information technology – Open Systems Interconnection – Systems Management – Part 1: Object Management Function Internet |
| ISO/IEC 10164-2:1993 | Information technology – Open Systems Interconnection – Systems Management – Part 2: State Management Function Internet |
| ISO/IEC 10164-3:1993 | Information technology – Open Systems Interconnection – Systems Management – Part 3: Attributes for representing relationships Internet |
| ISO/IEC 10164-4:1992 | Information technology – Open Systems Interconnection – Systems management – Part 4: Alarm reporting function Internet |
| ISO/IEC 10164-5:1993 | Information technology – Open Systems Interconnection – Systems management – Part 5: Event Report Management Function Internet |
| ISO/IEC 10164-6:1993 | Information technology – Open Systems Interconnection – Systems Management – Part 6: Log control function Internet |
| ISO/IEC 10164-7:1992 | Information technology – Open Systems Interconnection – Systems Management – Part 7: Security alarm reporting function Internet |
| ISO/IEC 10164-8:1993 | Information technology – Open Systems Interconnection – Systems Management – Part 8: Security audit trail function Internet |
| ISO/IEC 10164-9:1995 | Information technology – Open Systems Interconnection – Systems management – Part 9: Objects and attributes for access control Internet |
| ISO/IEC 10164-10:1995 | Information technology – Open Systems Interconnection – Systems Management – Part 10: Usage Metering Function Internet |
| ISO/IEC 10164-11:1994 | Information technology – Open Systems Interconnection – Systems Management – Part 11: Metric objects and attributes Internet |
| ISO/IEC 10164-12:1994 | Information technology – Open Systems Interconnection – Systems Management – Part 12: Test Management Function Internet |
| ISO/IEC 10164-13:1995 | Information technology – Open Systems Interconnection – Systems Management – Part 13: Summarization Function Internet |
| ISO/IEC DIS 10164-14 | Information technology – Open Systems Interconnection – Systems management – Part 14: Confidence and diagnostic test categories Internet |
| ISO/IES 10164-15:1995 | Information technology – Open Systems Interconnection – Systems Management – Part 15: Scheduling Function Internet |
| ISO/IEC DIS 10164-16 | Information technology – Open Systems Interconnection – Systems Management – Part 16: Management Knowledge management function Internet |
| ISO/IEC DIS 10164-17 | Information technology – Open Systems Interconnection – Systems Management – Part 17: Change over function Internet |
| ISO/IEC DIS 10164-18 | Information technology – Open Systems Interconnection – Systems management – Part 18: Software management function Internet |
| ISO CD 10164-gr | Information Technology - Open Systems Interconnection - Systems Management - Part sm: General Relationship Model Internet |
| ISO DIS 10164-19 | Information Technology - Open Systems Interconnection - Systems Management - Part md: Management Domain Management Function Internet |
| ISO CD 10164-rt | Information Technology - Open Systems Interconnection - Systems Management - Part rt: Response Time Monitoring Function Internet |
| ISO CD 10164-ti | Information Technology - Open Systems Interconnection - Systems Management - Part ti: Time Management Function Internet |
| ISO/IEC 10164-X | Information Technology - Open Systems Interconnection - Systems Management - Enhanced Control Function Internet |
| ISO/IEC 10164-X | Information Technology - Open Systems Interconnection - Systems Management - Additional Selection Functionality for Discriminator Constructs Internet |
| ISO/IEC 10165-1:1992 | Information technology – Open Systems Interconnection – Management Information Services – Structure of management information – Part 1: Management Information Model Internet |
| ISO/IEC 10165-2:1992 | Information technology – Open Systems Interconnection – Structure of management information – Part 2: Definition of management information Internet |
| ISO/IEC 10165-4:1992 | Information technology – Open Systems Interconnection – – Structure of management information – Part 4: Guidelines for the definition of managed objects Internet |
| ISO/IEC 10165-5:1994 | Information technology – Open Systems Interconnection – Structure of management information – Part 5: Generic management information Internet |
| ISO/IEC 10165-6:1994 | Information technology – Open Systems Interconnection – Structure of management information – Part 6: Requirements and guidelines for implementation conformance statement proformas associated with OSI management Internet |
| ISO/IEC 10169-1:1991 | Information technology – Open Systems Interconnection – Conformance test suite for the ACSE protocol – Part 1: Test suite structure and test purposes Internet |
| ISO/IEC DIS 10181-1 | Information technology - Open Systems Interconnection -Security Frameworks Part 1: Overview Information technology – Open Systems Interconnection – Security Frameworks for Open Systems – Part 1: Overview Internet |
| ISO/IEC DIS 10181-2 | Information technology – Open Systems Interconnection – Security Frameworks for Open Systems – Part 2: Authentication Framework Internet |
| ISO/IEC DIS 10181-3 | Information technology – Open Systems Interconnection – Security frameworks in open systems – Part 3: Access control Internet |
| ISO/IEC DIS 10181-4 | Information technology – Open Systems Interconnection – Security frameworks in Open Systems – Part 4: Non-repudiation Internet |
| ISO/IEC DIS 10181-5 | Information technology – Open Systems Interconnection – Security frameworks in open systems – Part 5: Confidentiality Internet |
| ISO/IEC DIS 10181-6 | Information technology – Open Systems Interconnection – Security frameworks in open systems – Part 6: Integrity Internet |
| ISO/IEC DIS 10181-7 | Information technology – Open Systems Interconnection – Security Frameworks for Open Systems – Part 7: Security Audit Framework Internet |
| ISO/IEC 10589:1992 | Information technology – Telecommunications and information exchange between systems – Intermediate system to intermediate system intra-domain-routeing routine information exchange protocol for use in conjunction with the protocol for providing the connectionless-mode Network Service ( ISO 8473) Internet |
| ISO ISP 10607-1:1995 | Information technology – International Standardized Profiles AFTnn – File Transfer, Access and Management – Part 1: Specification of ACSE, Presentation and Session protocols for the use by FTAM Internet |
| ISO ISP 10607-2:1995 | Information technology – International Standardized Profiles AFTnn – File Transfer, Access and Management – Part 2: Definition of document types, constraint sets and syntaxes Internet |
|  | Part |
| ISO ISP 10607-3:1995 | Information technology – International Standardized Profiles AFTnn – File Transfer, Access and Management – Part 3: AFT11 – Simple File Transfer Service (unstructured) Internet |
| ISO ISP 10607-4:1995 | Information technology – International Standardized Profiles AFTnn – File Transfer, Access and Management – Part 4: AFT12 – Positional File Transfer Service (flat) Internet |
| ISO ISP 10607-5:1995 | Information technology – International Standardized Profiles AFTnn – File Transfer, Access and Management – Part 5: AFT22 – Positional File Access Service (flat) Internet |
| ISO ISP 10607-6:1995 | Information technology – International Standardized Profiles AFTnn – File Transfer, Access and Management – Part 6: AFT3 – File Management Service Internet |
| ISO DISP 10608-1 | Information technology – International Standardized Profile TAnnnn – Connection-mode Transport Service over Connectionless-mode Network Service – Part 1: General overview and subnetwork-independent requirements Internet |
| ISO/IEC ISP 10611-1 : 1994 | Information technology – International Standardized Profiles AMH1n – Message Handling Systems – Common Messaging – Part 1: MHS Service Support MHS |
| ISO/IEC ISP 10611-2 : 1994 | Information technology – International Standardized Profiles AMH1n – Message Handling Systems – Common Messaging – Part 2: Specification of ROSE, RTSE, ACSE, Presentation and Session Protocols for use by MHS MHS |
| ISO/IEC ISP 10611-3 : 1994 | Information technology – International Standardized Profiles AMH1n – Message Handling Systems – Common Messaging – Part 3: AMH11-Message Transfer (P1) MHS |
| ISO/IEC ISP 10611-4 : 1994 | Information technology – International Standardized Profiles AMH1n – Message Handling Systems – Common Messaging – Part 4: AMH12-MTS Access (P3) MHS |
| ISO/IEC ISP 10611-5 : 1994 | Information technology – International Standardized Profiles AMH1n – Message Handling Systems – Common Messaging – Part 5: AMH13-MS Access (P7) MHS |
| ISO TR 10730:1993 | Information technology - Open systems Interconnection - Tutorial on naming and addressing Internet |
| ISO/IEC 10731:1994 ITU-T Rec. X.210 (1993) | Information technology– Open Systems Interconnection –Conventions for the definition of OSI services Internet, ULA |
| ISO/IEC 10733:1993 | Information technology – Telecommunications and information exchange between systems – Elements of management information relating to OSI Network Layer standards Internet |
| ISO/IEC 10733:1993/PDAM1 | Information technology – Telecommunications and information exchange between systems – Elements of management information relating to OSI Network Layer standards Amendment 1: Managed Object Conformance Statement Proforma Internet |
| ISO/IEC 10736:1995 | Information technology – Telecommunications and information exchange between systems – Transport layer security protocol Internet |
| ISO/IEC 10737:1994 | Information technology – Telecommunications and information exchange between systems – Elements of Management Information Related to OSI Transport Layer Standards Internet |
| ISO/IEC 10737:1994/PDAM2 | Information technology – Telecommunications and information exchange between systems – Elements of Management Information Related to OSI Transport Layer Standards – Amendment 2: Managed Object Conformance Statement Proforma Internet |
| ISO/IEC 10745:1995 | Information technology – Open Systems Interconnection – Upper layers security model Internet |
| ISO/IEC 10747:1994 | Information technology – Telecommunications and information exchange between systems – Protocol for exchange of inter-domain routeing information among intermediate systems to support forwarding of ISO 8473 PDUs Internet |
| ISO/IEC ISP 11183-1:1992 | Information technology – International Standardized Profiles AOM1n OSI Management – Management Communications – Part 1: Specification of ACSE, presentation and session protocols for the use by ROSE and CMISE Internet |
| ISO/IEC ISP 11183-2:1992 | Information technology – International Standardized Profiles AOM1n OSI Management – Management Communications – Part 2: CMISE/ROSE for AOM12 – Enhanced Management Communications Internet |
| ISO/IEC ISP 11183-3:1992 | Information technology – International Standardized Profiles AOM1n OSI Management – Management Communications – Part 3: CMISE/ROSE for AOM11 – Basic Management Communications Internet |
| ISO/IEC 11570:1992 | Information technology – Telecommunications and information exchange between systems – Open Systems Interconnection – Transport protocol identification mechanism Internet |
| ISO/IEC 11577:1995 | Information technology – Open Systems Interconnection – Network layer security protocol Internet |
| ISO/IEC DIS 11586-1 | Information technology – Open Systems Interconnection – Generic Upper Layers Security – Part 1: Overview, Models and Notation Internet |
| ISO/IEC DIS 11586-2 | Information technology – Open Systems Interconnection – Generic Upper Layers Security – Part 2: Security Exchange Service Element (SESE) Service Specification Internet |
| ISO/IEC DIS 11586-3 | Information technology – Open Systems Interconnection – Generic Upper Layers Security – Part 3: Security Exchange Service Element (SESE) Protocol Specification Internet |
| ISO/IEC DIS 11586-4 | Information technology – Open Systems Interconnection – Generic Upper Layers Security – Part 4: Protecting Transfer Syntax Specification Internet |
| ISO/IEC ISP 12059-0:1995 | Information technology – International Standardized Profiles – OSI Management – Common information for management functions – Part 0: Common definitions for management function profiles Internet |
| ISO/IEC ISP 12059-1:1995 | Information technology – International Standardized Profiles – OSI Management – Common information for management functions – Part 1: Object management Internet |
| ISO/IEC ISP 12059-2:1995 | Information technology – International Standardized Profiles – OSI Management – Common information for management functions – Part 2: State management Internet |
| ISO/IEC ISP 12059-3:1995 | Information technology – International Standardized Profiles – OSO Management – Common information for management functions – Part 3: Attributes for representing relationships Internet |
| ISO/IEC ISP 12059-4:1995 | Information technology – International Standardized Profiles – OSI Management – Common information for management functions – Part 4: Alarm reporting Internet |
| ISO/IEC ISP 12059-5:1995 | Information technology – International Standardized Profiles – OSI Management – Common information for management functions – Part 5: Event report management Internet |
| ISO/IEC ISP 12059-6:1995 | Information technology – International Standardized Profiles – OSI Management – Common information for management functions – Part 6: Log control Internet |
| ISO/IEC ISP 12060-1:1995 | Information technology – International Standardized Profiles – OSI Management – Management functions – Part 1: AOM211 – General management capability Internet |
| ISO/IEC ISP 12060-2:1995 | Information technology – International Standardized Profiles – OSI Management – Management functions – Part 2: AOM212 – Alarm reporting and state management capabilities Internet |
| ISO/IEC ISP 12060-3:1995 | Information technology – International Standardized Profiles – OSI Management – Management functions – Part 3: AOM213 – Alarm reporting capabilities Internet |
| ISO/IEC ISP 12060-4:1995 | Information technology – International Standardized Profiles – OSI Management – Management functions – Part 4: AOM221 – General event report management Internet |
| ISO/IEC ISP 12060-5:1995 | Information technology – International Standardized Profiles – OSI Management – Management functions – Part 5: AOM231 – General log control Internet |
| ISO/IEC ISP 12062-1 : 1995 | Information technology – International Standardized Profiles AMH2n – Message Handling Systems – Interpersonal Messaging – Part 1: IPM MHS Service Support MHS |
| ISO/IEC ISP 12062-2 : 1995 | Information technology – International Standardized Profiles AMH2n – Message Handling Systems – Interpersonal Messaging – Part 2: AMH21 – IPM Content MHS |
| ISO/IEC ISP 12062-3 : 1995 | Information technology – International Standardized Profiles AMH2n – Message Handling Systems – Interpersonal Messaging – Part 3: AMH22 – IPM Requirements for Message Transfer (P1) MHS |
| ISO/IEC ISP 12062-4 : 1995 | Information technology – International Standardized Profiles AMH2n – Message Handling Systems – Interpersonal Messaging – Part 4: AMH23 – IPM Requirements for MTS Access (P3) MHS |
| ISO/IEC ISP 12062-5 : 1995 | Information technology – International Standardized Profiles AMH2n – Message Handling Systems – Interpersonal Messaging – Part 5: AMH24 – IPM Requirements for Enhanced MS Access (P7) MHS |
| ITU-T Rec. X.215 Addendum 1 (1995) | Information processing systems – Open Systems Interconnection – Service Definition for Session Layer Efficiency Enhancements ULA |
| ITU-T Rec. X.216 Addendum 1 (1995) | Information processing systems – Open Systems Interconnection –Service Definition for Presentation Layer Efficiency Enhancements ULA |
| ITU-T Rec. X.225 Addendum 1 (1995) | Information processing systems – Open Systems Interconnection –Protocol Specification for Session Layer Efficiency Enhancements ULA |
| ITU-T Rec. X.226 Addendum 1 (1995) | Information processing systems – Open Systems Interconnection – Protocol Specification for Presentation Layer Efficiency Enhancements ULA |

## Glossary

| Accounting Management | Accounting management enables charges to be established for the use of resources, and for costs to be identified for the use of those resources. Accounting management includes functions to inform users of costs incurred or resources consumed, enable accounting limits to be set and tariff schedules to be associated with the use of resources, and enable costs to be combined where multiple resources are invoked to achieve a given communications objective. Internet |
| --- | --- |
| ACSE. | The Association Control Service Element is the common mechanism in the ALS for establishing and releasing ASO-associations ULA |
| Active User | A user that is currently in an applications dialogue, such as for CM or CPDLC. ADS, CM, CPDLC, FIS |
| Actual TSAP | The actual TSAP is composed of the IDP and the long TSAP. CM |
| Address Domain | An Address Domain is a set of address formats and values administered by a single address authority. Under the ISO plan, any address authority may define subdomains within its own domain, and delegate authority within those subdomains. Internet |
| Addressing (logical) | Logical addressing means that the address defined in the addressing plan and used to locate the addressed object is a virtual address which is a substitute of the actual (physical) address of an object. Address mapping functions have to fulfil this substitution, carefully maintaining unambiguity of identification of objects. Lexicon |
| Addressing (physical) | Physical addressing means that the address defined in the addressing plan and used to locate the addressed object is the physical, i.e. hardwired, hard-coded, or configured address of the object. An example of a physical address is the ICAO 24-bit Aircraft Address used for the SSR Mode S Transponder. Lexicon |
| Addressing Authority | An Addressing Authority defines formats and/or values of NSAP addresses within its jurisdiction. Internet |
| Administrative Domain | A collection of end systems, intermediate systems, and subnetworks operated by a single organisation or administrative authority. An administrative domain may be internally divided into one or more routing domains. Internet |
| ADS abstract service interface | The abstract interface between the ADS-air-ASE and the ADS-air-user or between the ADS-ground-ASE and the ADS-ground-user ADS |
| ADS emergency report | An ADS report provided as part of an emergency contract ADS |
| ADS report | A report provided by the ADS-air-user and sent to the ADS-ground-user concerning conditions on the aircraft, notably its location and FOM ADS |
| ADS service primitive | See Service Primitive. ADS |
| ADS service provider | See Service Provider. ADS |
| ADS-AE Abstract Service Interface | The abstract interface between the ADS-users and the ADS-service-provider. ADS |
| ADS-air-ASE | That abstract part of the aircraft system that performs the communications related functions of ADS ADS |
| ADS-air-user | That abstract part of the aircraft system that performs the non communications related functions of ADS ADS |
| ADS-ASE Abstract Service Interface | The abstract interface through which the ADS-ASE services are accessed. Note 1.— In version 1 of the ADS application, this interface coincides with the ADS-AE abstract service interface. ADS |
| ADS-CF | That abstract part of the AE that performs the mapping between the ADS-ASE service primitives and other elements within the ADS application ADS |
| ADS-ground-ASE | That abstract part of the ground system that performs the communications related functions of ADS ADS |
| ADS-ground-user | That abstract part of the ground system that performs the non communications related functions of ADS ADS |
| AE Qualifier | That part of the AE title that unambiguously identifies the particular application entity. Note – some subvolumes use “uniquely” ADS, CM, CPDLC, FIS, ULA |
| AE Title | An unambiguous name for an application entity.ADS, CM, CPDLC, FIS, ULA |
| Aeronautical Administrative Communications (AAC) | Communications used by aeronautical operating agencies related to the business aspects of operating their flights and transport services. These communications are used for a variety of purposes, such as flight and ground transportation bookings, deployment of crew and aircraft,, or any other logistic purposes that maintains or enhances the efficiency of overall flight operation. Internet |
| Aeronautical Administrative Messages | Messages regarding the operation or maintenance of facilities provided for the safety or regularity of aircraft operation, messages concerning the functioning of the aeronautical telecommunication services, and messages exchanged between government civil aviation authorities relating to aeronautical services. Internet |
| Aeronautical Industry Service Communication (AINSC) | AINSC comprises aeronautical industry communications between airlines, aeronautical industry service providers, general aviation operators, and any other industry stakeholders. This term is used for purposes of address administration. Internet |
| Aeronautical Information Service Messages. | Messages concerning NOTAMS and messages concerning SNOWTAMS. Internet |
| Aeronautical Mobile Satellite Service (AMSS) | AMSS provides packet-mode data and circuit-mode data and voice service to aircraft and ground users provided by a satellite subnetwork which comprises satellites, Aircraft Earth Stations (AESs), Ground Earth Stations (GESs), and associated ground facilities such as a network coordination center. Internet |
| Aeronautical Operational Control (AOC) | Communications required for the exercise of authority over the initiation, continuation, diversion, or termination of a flight in the interest of the safety of the aircraft and the regularity and efficiency of flight. Internet |
| Aeronautical Passenger Communications (APC) | Communications relating to the non-safety voice and data services to passengers and crew members for personal communications. Internet |
| Aeronautical stakeholder | Definition tbd |
| Aeronautical Telecommunication Network (ATN) | The Aeronautical Telecommunication Network is an internetwork architecture which allows ground, air-to-ground, and avionics data subnetworks to interoperate by adopting common interface services and protocols based on the International Organization for Standardization (ISO) Open Systems Interconnection (OSI) reference model. Internet |
| AINSC Administrative Domain | An AINSC Administrative Domain is an ATN Administrative Domain owned and/or administered by an aeronautical industry service organisation. Internet |
| AINSC RDC | The ATN AINSC RDC consists of all AINSC RDs in the ATN. Internet |
| AINSC Routing Areas | An AINSC Routing Area is a routing subdomain comprising one or more ISs, and optionally, one or more ESs owned and/or administered by an aeronautical industry service organisation. For example, an AINSC Routing Area may correspond to a physical location such as an airline’s systems located at an airport. Internet |
| AINSC Routing Domains | An AINSC Routing Domain comprises ESs and ISs that are part of an AINSC Administrative Domain. Internet |
| Air Traffic Control (ATC) | ATC is a service operated by an appropriate authority to promote the safe, orderly, and expeditious flow of air traffic. Internet |
| Air Traffic Management (ATM) | ATM consists of a ground and air part, both needed to ensure the safe and efficient movement of aircraft during all phases of operation. Internet |
| Air Traffic Services (ATS) | A generic term meaning variously, flight information service, alerting service, air traffic advisory service, air traffic control service (area control service, approach control service, or aerodrome control service).Internet |
| Air Traffic Services Communications (ATSC) | Communications related to air traffic services including air traffic control, aeronautical and meteorological information, position reporting, and services related to safety and regularity of flight. This communication must involve one or more air traffic service administrations. This term is used for purposes of address administration. Internet |
| Air Traffic Services Unit | A generic term meaning variously, air traffic control unit, flight information centre or air traffic services reporting office. |
| Airframe-Id | The ICAO 24 bit aircraft identifier. ADS, ULA |
| ALS | The Application Layer Structure (ALS) refers to the internal architecture of the OSI Application Layer as described in ISO/IEC 9545, Edition 2. ULA |
| APDU | An Application Protocol Data Unit (APDU) is an (N)-PDU where N refers to the Application Layer. An APDU is the basic unit of information exchanged between the airborne application and the ground application. ADS, CM, CPDLC, FIS, ULA |
| Application | Software providing services to its users, in the guise of a consistent set of functionality; example given, the ATC related functions implemented in the server(s) and/or controller work position host computers.(from EATCHIP Glossary of Terms / COPS/CWP Report) Lexicon |
| Application Control Service Element (ACSE) | The association control service element (ACSE) establishes, maintains and releases associations between application entities. Internet |
| Application Entity (AE) | Part of an application process that is concerned with communications within the OSI environment. The aspects of an application process that need to be taken into account for the purposes of OSI are represented by one or more AEs. . ADS, CM, CPDLC, FIS, Internet, ULA |
| Application identifier | An abstract identifier which distinguishes one application from another. ADS |
| Application Layer | The layer of the OSI reference model that controls application user access to the communication system. Internet |
| Application process (AP) | A set of resources, including processing resources, within a real open system which may be used to perform a particular information processing activity. Internet |
| Application service | The abstract interface between the (N)-service and the (N)-service user, where N refers to the Application layer; thus it is the boundary between the ATN-App-AE and the Application-user. ULA |
| Application Service Element (ASE) | A set of application functions which provide a capability for the interworking of application-entity-invocations for a specific purpose; ASEs are a component of application service objects. An ASE can be considered to be a protocol module that is combined with others to form a complete protocol. ULA |
| Application Service Object (ASO) | An active element within (or equivalent to the whole of) the application-entity embodying a set of capabilities defined for the Application Layer that corresponds to a specific ASO-type (without any extra capabilities being used). An ASO is a combination of ASEs and ASOs that perform a specific function. An ASO that provides the functions of the establishment and data transfer phases is considered a complete protocol. ULA |
| Application-user | That abstract part of the aircraft or ground system that performs the non-communications related functions of the Application ULA |
| ATM/ATS Applications | These are applications supporting ATM or other ATS functions and do not necessarily correspond to ATN applications. The term is usually used to distinguish between ATM functions and other non-ATM functions using the same communication service. Lexicon |
| ATN App | A generic name for an ATN application. ULA |
| ATN Applications | Refers to applications that support ATM or aeronautical industry functions and that are designed to operate across an OSI communications system. ATN applications are always distributed applications, i.e. peer processes are hosted by different end systems which are interconnected. Lexicon |
| ATN End-System | An ATN end system which can send and receive ATSC Traffic over the ATN Internet. |
| ATN Environment | The term ATN environment relates to functional and operational aspects around the ATN as a complete end-to-end communication system. Lexicon |
| ATN host computer | An ATN host computer is a civil aeronautical computer system which contains one or more end user applications and that communicates using the ATN internet. In OSI terms, it denotes an End System. An ATN Host Computer may also implement the upper layers necessary to support the Systems Management Agent and Systems Management Manager and upper layer protocols as specified for the supported end-user applications. Internet |
| ATN Intermediate System | An ATN intermediate system which can route ATSC Traffic. |
| ATN Internet (ATNI) | An implementation of the ISO OSI network layer services and protocols for support of interprocess data communication between aeronautical host computers. It is defined to be the collection of the connected internetwork routers and subnetworks that conform to ATN internetwork requirements. Internet |
| ATN Island Backbone RDC | An ATN backbone is an RDC comprising a subset of Transit Routing Domains within an ATN Island which provide general connectivity. Internet |
| ATN Island Bridge | A bridge between two ATN Islands is a communications link between backbones over a suitable subnetwork. Internet |
| ATN Island RDC | An ATN Island is an RDC comprising CAA-operated ATN RDs within a geographical region, and may include associated ATN service providers, or an RDC comprising Aeronautical Industry members which are users of communications services of a single Aeronautical Industry Service Provider, or more than one such provider providing services in combination with each other. Internet |
| ATN Network Operating Concept | An ATN Network Operating Concept will address the administrative, operational, institutional, and policy issues and additional (non-SARPs relevant) technical aspects to enable the efficient and correct operation of the ATN. Lexicon |
| ATN Presentation Address | In the ATN, presentation addresses must, as a minimum, include an NSAP Address and a TSAP Selector and may include a PSAP Selector and SSAP Selector based on the addressing structure adopted within the ES and whether the application requires the OSI session or presentation protocol. Internet |
| ATN Profile Requirement List (APRL) | APRLs contained in the Draft ATN SARPs identify, in a tabular form, requirements together with the options and parameters for protocols used in the ATN. The supplier of an ATN protocol implementation claiming to conform to the ATN SARPs must indicate conformance to those requirements by preparing a Protocol Implementation Conformance Statement (PICS) based on the set of APRLs presented in the SARPs. Lexicon |
| ATN Router | The communication element that manages the relaying and routing of data while in transit from an originating ATN host computer to a destination ATN host computer. In ISO terms, an ATN router comprises an OSI intermediate system and an end system supporting a systems management agent. Internet |
| ATN Routing Domain Confederation | The ATN RDC is the set of interconnected RDs that together form the ATN. Internet |
| ATN Services | The ATN services are provided to ATN users that require ground-ground or air-ground data communication. The ATN internet service is provided at the transport layer (service access point). The ATN accommodates different grades of services which can be expressed by Quality of Service parameters. Lexicon |
| ATN System Applications | System Applications support the operation of the ATN communication services and are either not directly or not at all used by ATN users but rather by the service providers or operators. Typical examples of ATN system applications are the ATN directory service, ATN context management or ATN systems management. Lexicon |
| ATN Systems Management | The ATN Systems Management provides mechanisms for monitoring, control and co-ordination of resources necessary to provide ATN services. ATN Systems Management is based on OSI System Management principles and may be distributed, centralised, or local. Lexicon |
| ATN-App ASE | That abstract part of the ATN end system that performs the communications related functions of the ATN application. Examples of the ATN-App ASE: the ADS-Air ASE and the CM-Air ASE. ULA |
| ATS Message | A unit of user-data, coded in binary form, which is conveyed from an originator of the data to one or more recipients of the data. It is possible to associate a unique message identifier and a priority with each ATS message. MHS |
| ATS Message Handling Service | Procedures used to exchange ATS Messages over the ATN such that the conveyance of an ATS Message is in general not correlated with the conveyance of another ATS Message by the service provider. Two ATS Message Handling Services are defined in this Part of Sub-Volume III. They are the ATS Message Service and the ATN Pass-Through Service. MHS |
| ATS Organization | An ICAO State or organisation which administers one or more ATS end and/or intermediate system. |
| ATSC Administrative Domain | An ATSC Administrative Domain is an ATN Administrative Domain owned and/or administered by an air traffic services organisation. Internet |
| ATSC RDC | The ATN ATSC RDC consists of all ATSC RDs in the ATN. Internet |
| ATSC Routing Areas | An AINSC Routing Area is a routing subdomain comprising one or more ISs, and optionally, one or more ESs owned and/or administered by an ATS organisation. For example, an ATSC Routing Area may correspond to a physical location such as an airport. Internet |
| ATSC Routing Domain | An ATSC Routing Domain comprises ESs and ISs that are part of an ATSC Administrative Domain. Internet |
| Authentication information | Information used to authenticate the identity of an application or user. ADS, ULA |
| Automatic Dependent Surveillance (ADS) | A technique for aircraft surveillance that is based upon the reporting of aircraft-derived position via a communication system. Internet |
| Boundary Intermediate System (BIS) | An intermediate system that is able to relay data between two separate routing or administrative domains. Internet |
| Broadcast Subnetwork | Broadcast subnetworks (e.g. LANs) are often used to connect ISs and ESs within a small geographical area with media offering relatively high data throughput with relatively low delays. Internet |
| Control Function (CF) | That abstract part of the AE that performs the mapping between the ATN-App ASE service primitives, the ACSE service primitives, and other elements within the Application Entity. ULA |
| CM AE abstract service interface | The abstract interface between the CM-users and the CM-service provider. CM |
| CM service primitive | See Service Primitive. CM |
| CM service provider | See Service Provider. CM |
| CM-air-ASE | An abstract part of the aircraft system that performs the communication related functions of CM. CM |
| CM-air-user | The abstract part of the aircraft system that performs the non communication related functions of CM. CM |
| CM-ASE abstract service interface | The abstract interface through which the CM-ASE service are accessed. Note— In version 1 of the CM application, this interface coincides with the CM-AE abstract service interface. CM |
| CM-CF | That abstract part of the application entity that performs the mapping between the CM-ASE service primitives and other elements within the CM application. CM |
| CM-ground-ASE | An abstract part of the ground system that performs the communication related functions of CM. CM |
| CM-ground-user | The abstract part of the ground system that performs the non communication related functions of CM. CM |
| Configuration Management | Configuration management identifies, exercises control over, collects data from and provides data to open systems for the purpose of preparing for, initialising, starting, providing for the continuous operation of, and terminating interconnection services. Internet |
| Congestion | In the ATN Internet sense, congestion describes the state where the network is overloaded. Typical effects of congestion are extended transit delays, drastically reduced throughput, and the loss of data packets. Lexicon |
| Congestion Avoidance | Techniques which continuously control the data flow into the network in order to prevent the network from getting overloaded. These encompass both open-loop techniques which ensure that a traffic contract specified by the source is respected, and closed-loop techniques which monitor signals generated by the network and adapt the traffic generated by the sources accordingly. Lexicon |
| Congestion Management | This term refers to a set of rules and techniques that prevent congestion , e.g. by monitoring actual network load. Co-operative interaction of all end systems is required in order to prevent individual end-systems taking up the throughput saved by well-behaving systems. Lexicon |
| Congestion Recovery / Congestion Control | This term refers to a mechanism that reacts to congestion after it has occurred in order to remove the overload condition. Congestion Recovery can be initiated only after congestion has been experienced, and is not able to safely prevent congestion in the network. Lexicon |
| Connection mode Service (CO) | The communication service technique that transfers data between peer layers using a prior connection to logically associate the sequence of protocol data units (PDUs). Internet |
| Connectionless mode Service (CL) | The communication service technique transfers data between peer layers without prior coordination. All protocol data units (PDUs) are transferred with no explicit association between them. Internet |
| Context Management | In the ATN environment, the term “Context Management (CM)” refers to an ATN application. This application implements an ATN logon service allowing initial aircraft introduction into the ATN. The logon service also allows indication of all other data link applications on the aircraft. CM also includes functionality to forward addresses between ATC centres. Thus, CM is a logon and simple directory service. Note: “Context Management” is a recognised OSI presentation layer term. The OSI use and the ATN use have nothing in common. Lexicon |
| contract | An agreement between the ADS-ground-user and the ADS-air-user that the latter will provide reports to the former under the conditions specified in the contract. ADS |
| CPDLC AE abstract service interface | The abstract interface between the CPDLC-users and the CPDLC-service provider. CPDLC |
| CPDLC ASE abstract service interface | The abstract interface through which the CM-ASE service are accessed. Note: — In version 1 of the CPDLC application, this interface coincide with the CPDLC-AE abstract service interface. CPDLC |
| CPDLC service primitive | See Service Provider. CPDLC |
| CPDLC service provider | See Service Provider. CPDLC |
| CPDLC-air-ASE | An abstract part of the aircraft system that performs the communication related functions of CPDLC. CPDLC |
| CPDLC-air-user | The abstract part of the aircraft system that performs the non-communication related functions of CPDLC. CPDLC |
| CPDLC-CF | That abstract part of the application entity that performs the mapping between the CPDLC-ASE service primitives and other elements within the CPDLC application. CPDLC |
| CPDLC-ground-ASE | An abstract part of the ground system that performs the communication related functions of CPDLC. CPDLC |
| CPDLC-ground-user | The abstract part of the ground system that performs the non-communication related functions of CPDLC. CPDLC |
| Current Data Authority | The ground system that is technically permitted to conduct a CPDLC dialogue with an aircraft. CPDLC |
| Data Communications Equipment (DCE) | An interface between data terminal equipment and the transmission mechanism. Internet |
| Data Link Applications | This term refers to applications using either a specific data link (air/ground subnetwork) or air-ground communications in general. It should be regarded as an outdated term and be replaced by the term ATN Air/Ground Applications. Lexicon |
| Data Link Layer | The layer of the OSI reference model that manages the operations of the physical layer and may utilise special error detection or retransmission techniques to achieve acceptable error rates. Internet |
| Data Terminal Equipment (DTE) | A digital data transmitter/receiver device that includes terminals and computers. Internet |
| Datagram service | A service providing the transmission and reception of packets of data as discrete messages. Internet |
| Demand Contract | A “contract” between a requestor and a provider of information service, such as ADS or FIS, to provide a single report to the requestor (vs. Continual reports to one request). |
| Dialogue | A co-operative relationship between elements which enables communication and joint operation. ULA |
| Dialogue service | A term for the lower service boundary of an ATN-App ASE; the service allows two ATN-App ASEs to communicate, such as a CM-ground-ASE to communicate with a CM-air-ASE. ULA |
| Directory | The ATN Directory is a facility that supports on request the retrieval of ATN address information or the resolution of application names. Lexicon |
| Directory Service | The ATN Directory Service provides the ATN user with the addressing information that is associated with the application process title or application entity title used as input to the directory. The addressing information provided by the directory service includes the network address as well as further technical addresses on the layers above, as required or applicable. Furthermore, the ATN Directory Service resolves generic application process titles or application entity titles, i.e. names which may be incomplete or contain “don’t care” elements, into the corresponding (list of) non-generic application process titles or application entity titles. Lexicon |
| Distinguishing Path Attribute (DPA) | In ISO/IEC 10747 a DPA is used to discriminate among multiple routes to a destination, based on differences in the quality of service between the routes (for example, expense, transit delay or residual error probability.) Internet |
| Domain | A set of end systems and intermediate systems that operate according to the same routing procedures and that is wholly contained within a single Administrative domain. Internet |
| Domain Specific Part (DSP) | An Addressing Authority is responsible for its own Addressing Subdomain, and NSAP Addresses within that addressing domain are distinguished, where necessary, by the value of the DSP. Internet |
| Downstream Data Authority | The ground system that is technically permitted to conduct a DSC dialogue with an aircraft. CPDLC |
| emergency contract | A contract to provide ADS reports at regular intervals during an emergency situation ADS |
| emergency contract | A contract to provide ADS reports at regular intervals during an emergency situation CM |
| emergency mode | A mode of operation of the aircraft when a Active User: a user that is currently involved in a CM dialogue. ADS |
| End Routing Domain (ERD) | A RD that only routes PDUs from/to its own RD. Internet |
| End System (ES) | A system that contains the seven OSI layers and contains one or more end user application processes. Internet |
| end user | The human who is using the user interface to the system CM |
| Engineering Trials | In contrast to operational trials, engineering trials may be based on pre-operational, prototype, or experimental equipment. Aim is to demonstrate the technical feasibility and correctness of applied techniques, concepts, and specifications. Lexicon |
| Entity | An active element in any layer which can either be a software entity (such as a process) or a hardware entity (such as an intelligent I/O chip). Internet |
| Ethernet | Ethernet is based on a local area network standard ISO 8802-3 Carrier Sense Multiple Access with Collision Detection (CSMA/CD) Access Method, and Physical Layer Specifications using broadcast technology which may connect as an ATN subnetwork. Internet |
| event contract | A contract to provide ADS reports when certain events occur. CM |
| Expected Quality of Service (QoS) | Expected QoS is based on a combination of a priori knowledge and analysis of performance information received from the operation of routing protocols. Internet |
| Expected Transit Delay | Expected Transit Delay is defined as the time elapsed between the invocation of CLNS by the source ATN NS user and the arrival of an NSDU at the destination ATN NS user, based on an NPDU size of 512 octets. Transit Delay values are typically expressed in increments of 500 milliseconds. Internet |
| Expense | Expense is the cost to perform some task. In the context of internetworking, expense is defined in terms of the incremental expense incurred for transfer of a single NSDU of 512 octets in size. Internet |
| extended projected profile | A projected profile extended up to a number of way points. CM |
| Fast Byte | definition tbd. Lexicon |
| Fault Management | Fault management encompasses fault detection, isolation, and the correction of abnormal operation, and includes functions to maintain and examine error logs, accept and act upon error detection notifications, trace and identify faults, carry out sequences of diagnostic tests, and correct faults. Internet |
| FIS Abstract Service Interface | The abstract interface between the FIS-air-AE and the FIS-air-user or between the FIS-ground-AE and the FIS-ground-user. This interface identifies both the FIS AE abstract service interface and the FIS-ASE abstract service interface. FIS |
| FIS service-primitive | A function of an FIS AE that is not broken down further into subfunctions, and is presented as part of the abstract service interface (i.e. request, indication, response, or confirmation). FIS |
| FIS service-provider | The FIS service-provider is composed of the ground and airborne FIS AEs, all underlying data communication protocol entities and the physical media. As a consequence, it encompasses everything between the FIS-AE service interfaces of the end-users of the FIS application. FIS |
| FIS-AE Abstract Service Interface | The abstract interface between the FIS-users and the FIS-service-provider. FIS |
| FIS-air-ASE | The abstract part of the aircraft system that performs the communications related functions of FIS. FIS |
| FIS-air-user | The abstract part of the aircraft system that performs the non communication related functions of FIS. FIS |
| FIS-ASE | The FIS-air-ASE and the FIS-ground-ASE. FIS |
| FIS-ASE Abstract Service Interface | The abstract interface through which the FIS-ASE services are accessed. Note 1.— In version 1 of the FIS Application, this interface coincides with the FIS-AE abstract service interface FIS |
| FIS-CF | The abstract part of the AE that performs the mapping between the FIS-ASE service primitives and others elements within the FIS Application. FIS |
| FIS-ground-ASE | The abstract part of the ground system that performs the communications related functions of FIS. FIS |
| FIS-ground-user | The abstract part of the ground system that performs the non communication related functions of FIS. FIS |
| FIS-user | The FIS-air-user or the FIS-ground-user. FIS |
| Fixed ATN RDC | The Fixed ATN RDC consists of all ground-based RDs that form the ATN. Internet |
| flight id | An identifier, to an ICAO approved format, for a particular flight. CM, ULA |
| Flight plan |  |
| Flow control | A function that controls the flow of data to perform buffer management within a layer or between adjacent layers. Internet |
| FOM | The figure of merit is an indication of the level of accuracy of positional information given in an ADS report. CM |
| forward contract | A contract to provide a ground ADS system with ADS reports. CM |
| Forwarding Information Base (FIB) | The information base that is maintained by each ATN router and contains the set of forwarding paths reflecting the various policy and QoS rankings available to reach each known destination. Internet |
| Four-D profile |  |
| Function | A function is a coherent set of activities which fulfils, by itself or together with other functionality, a concept. Examples of functions: conflict free planning; electronic representation of the flight. Lexicon |
| Functional Requirements | Operational requirements that determine what function a system should perform. They can usually be expressed by a verb applying to a type of data, e.g. display aircraft position. Lexicon |
| Gateway | A system used to interconnect dissimilar networks. A gateway may contain all seven layers of the OSI reference model. Internet |
| General Communications | A category of communications which includes APC, public correspondence, and other non-operational and non-administrative communications. Internet |
| General Topology Subnetwork | General topology subnetworks (e.g. WANs) are often used to connect geographically dispersed ISs and ESs. Internet |
| Global Network Addressing | An internetwork addressing plan covering worldwide aeronautical operations which enables all participating subnetworks to function in a single integrated global network. Internet |
| Global Network Addressing Domain | An addressing domain consisting of all the NSAP addresses in the OSI environment. Internet |
| Indicated QoS | Indicated QoS is determined by the QoS parameters passed in protocol control information, and may reflect varying accuracy with respect to actual characteristics. Internet |
| Initial Domain Part (IDP) | The IDP identifies the Addressing Authority responsible for an Addressing Subdomain that assigned the NSAP Address, and that specified the abstract syntax and structure of the remainder of the NSAP Address. Internet |
| Institutional Issues | Issues related to ownership, control, and responsibility for correct implementation and operation of systems that involve more than one state or organisation. Lexicon |
| Integrated Services Digital Network (ISDN) | A public telecommunications network that supports the transmission of digitised voice and data traffic on the same transmission links. Internet |
| Intermediate System (IS) | A system comprising the lower three layers of the OSI reference model and performing relaying and routing functions. Internet |
| Internetwork | A set of interconnected, logically independent heterogeneous subnetworks. The constituent subnetworks are usually administrated separately and may employ different transmission media. Internet |
| Internetwork Protocol | A protocol that performs the basic end-to-end mechanism for the transfer of data packets between network entities. In the ATN Internet, the ISO 8473 internetwork protocol is used. Internet |
| Interoperable | Describes the ability of the ATN to provide, as a minimum, a transparent data transfer service between end systems even though the ATN comprises various ground, air-to-ground, and avionics subnetworks. The ability to interoperate between end systems can be extended to include commonality of upper layer protocols. Internet |
| Intra-domain routing information exchange protocol | In the ATN, the ISO 10589 IS-IS intra-domain routing information exchange protocol may be used to exchange connectivity and topology information between ATN routers within a routing domain. Internet |
| Level 1 subdomain | A routing subdomain of end systems and intermediate systems that maintains detailed routing information about its own internal composition and routing information which allows it to reach other routing areas. A level 1 subdomain area is also denoted a routing area. Internet |
| Level 2 subdomain | The subset of all level 2 intermediate systems within a routing domain. Internet |
| Local Area Network (LAN) | A network connecting various data communication devices in a localised geographical area such as a single aircraft, office building, or a group of buildings. Internet |
| Long TSAP | The long TSAP is composed of the RDP and the short TSAP. CM |
| Lower layers | A term pertaining to the physical, data link, network and transport layers of the OSI reference model. Internet |
| Managed Object | Managed objects refer to those data processing and data communication resources that may be managed through the use of the OSI Management protocol. Internet |
| Management Administrative Domain | A management administrative domain is a management domain where the managed objects in the domain are all under the responsibility of one and only one administrative authority. Internet |
| Management Agent | The Management Agent performs management operations on managed objects within its local environment as a consequence of management operations communicated from a manager. An Agent may also forward notifications emitted by managed objects to a manager. Internet |
| Management Domain | Management domains are resources that for systems management purposes are represented by managed objects. A management domain possesses at least the following quantities: a name that uniquely identifies that management domain, identification of a collection of managed objects that are members of the domain, and identification of any inter-domain relationships between this domain and other domains. Internet |
| Management Information Base (MIB) | The management information base is a conceptual composite of management information within an open system. Internet |
| Management Information System (MIS)-User | An MIS-User is a management application. For the purposes of network management, an MIS-User is allowed to take on one of two possible roles — either an agent role or a manager role. Internet |
| Manager | A manager is the term given to a system that requests or otherwise receives information about managed objects. Internet |
| Mean Transit Delay | The average time it takes to transfer a standard packet size from source to destination. Internet |
| Message | Basic unit of user information exchanged between an airborne application and its ground counterpart, or between two ground applications. Messages are passed in one or more data blocks from one end user to another through different subnetworks. |
| Message Element | A component of a message used to define the context of the information exchanged. CPDLC |
| Message Element Identifier | The ASN.1 tag of the ATCUplinkMsgElementID or the ATCDownlinkMsgElementId. CPDLC |
| Message Header (air/ground) | The control information used to maintain synchronisation between the aircraft and the ground ATC system. CPDLC |
| Message Header (ground/ground) | Control information used to maintain synchronisation between the two ground ATC systems. CPDLC |
| Message Identification Number | A unique number assigned to each air/ground message. This number is used to differentiate messages and is conveyed in an air/ground message header. CPDLC |
| Message Reference Number | Used to uniquely associate a response with a previously received message. The Message Identification Number of a previously received message becomes the Message reference number of the response message. The Message Reference number is conveyed in the message header. CPDLC |
| Mobile Routing Domains | A mobile routing domain is formed from ATSC and AINSC systems onboard an aircraft (or any other mobile platform), within the aircraft operator’s Administrative Domain. A mobile RD is characterised as an End Routing Domain (ERD). Internet |
| Mobile Subnetwork | A subnetwork connecting a mobile system with another system not resident in the same mobile platform. These subnetworks tend to use free-radiating media (e.g. VHF/UHF radio, D-band satellite or D-band secondary surveillance radar) rather than “contained” media (e.g. wire or coaxial cable); thus they exhibit broadcast capabilities in the truest sense. Internet |
| Mode Select (Mode S) | An enhanced mode of secondary surveillance radar (SSR) which permits the selective interrogation of Mode S transponders, the two-way exchange of digital data between Mode S interrogators and transponders, and also the interrogation of Mode A or Mode C transponders. Internet |
| Multi-homed End Routing Domain | An ERD that is in communication with more than one RD. Internet |
| navigational intent | The intended path of the aircraft for a period of time in the future. CM |
| Network Addressing Domain | A subset of the global addressing domain consisting of all the NSAP addresses allocated by one or more addressing authorities. Internet |
| Network Entity | A functional portion of an internetwork router or host computer that is responsible for the operation of internetwork data transfer, routing information exchange, and network layer management protocols. Internet |
| Network Entity Title (NET) | The global address of a network entity. Internet |
| Network Layer | The ISO network layer is responsible for providing a uniform service interface for the transfer of data among end systems and intermediate systems (ISs) utilising the ISO protocol architecture. Internet |
| Network Management | Network Management is the set of functions related to the management of various OSI resources and their status across the Network Layer of the OSI architecture. Internet |
| Network Service Access Point (NSAP) | Point within the ISO protocol architecture at which global end users may be uniquely addressed on an end-to-end basis. Internet |
| Network Service Access Point (NSAP) Address | The NSAP Address is a hierarchically organised global address, supporting international, geographical, and telephony-oriented formats by way of an address format identifier located within the protocol header. Although the top level of the NSAP address hierarchy is internationally administered by ISO, subordinate address domains are administered by appropriate local organisations. Internet |
| Network Topology Map | Network topology maps provide an overall view of the global network connectivity, and are used in path computations by the operative routing algorithm. Internet |
| Next Data Authority | The ground system so designated by the Current Data Authority. CPDLC |
| Non-ATS End System | An ATN end system which can send and receive non-ATSC (e.g. AOC) Traffic over the ATN Internet. |
| Non-ATS Intermediate System | An ATN intermediate system which can route non-ATSC (e.g. AOC) Traffic. |
| NSAP address prefix | NSAP Address prefixes are used to identify groups of systems that reside in a given routing domain or confederation. An NSAP prefix may have a length that is either smaller than, or the same size as, the base NSAP address. Internet |
| Open Systems Interconnection (OSI) Protocol Architecture | A set of protocols used to implement the OSI reference model. Internet |
| Open Systems Interconnection (OSI) reference model | A model providing a standard approach to network design introducing modularity by dividing the complex set of functions into seven more manageable, self-contained, functional layers. By convention these are usually depicted as a vertical stack. Internet |
| Operating Concept | The Operating Concept describes the technical functionality of a system and its inherent capabilities regarded from the system operator’s point of view. This includes the interaction between user and system, the services provided by the system as well as the internal operation of the system. Lexicon |
| Operational Concept (1) | The Operational Concept describes from the user’s point of view the operational requirements, constraints, and prerequisites within which a technical system is supposed to work as well as the inherent capabilities of the system. Furthermore it describes the interaction between the user and the system as well as the services the user may expect from the system. Lexicon |
| Operational Concept (2) | Broad outline of an operational structure able to meet a given set of high level user requirements. It comprises a consistent airspace organisation, general operational procedures, and associated operational requirements for system support. Lexicon |
| Operational Requirements | This term refers to a set of requirements that define the operational needs and constraints within which a technical system has to operate in order to fulfil its anticipated role. The ATN operational requirements relate to ATN communications as seen from the user point of view. Operational requirements are composed of functional and non-functional requirements. Lexicon |
| Operational Trials | Operational trials are based on operational environment. This includes operational systems and operational equipment, e.g. routinely scheduled flights in an operational ATS environment. Aim is to demonstrate the operational acceptance and correctness of applied mechanisms, applications, and concepts. Lexicon |
| Packet | The basic unit of data transfer among communications devices within the network layer. Internet |
| Performance Management: | Performance management enables the behaviour of resources and the effectiveness of communication activities to be evaluated. Performance management includes functions to gather statistical information, maintain and examine logs of system state histories, determine system performance under natural and artificial conditions, and alter system modes of operation for the purpose of conducting performance management activities. Internet |
| Performance Requirements | Performance Requirements are requirements with respect to the performance of a system (e.g. reliability, availability, response time, processing delay, etc.) and are derived from Operational Requirements. In general, they describe the minimum performance figures that a system must provide in order to fulfil the operationally required functions. Lexicon |
| periodic contract | A contract to provide ADS reports at regular intervals. CM |
| Physical Layer | The layer of the OSI reference model that controls access to the transmission medium which forms the basis for the communication system. Internet |
| Policy Information Base (PIB) | The PIB is contained with a BIS, and consists of a set of policy statements specified by the Systems Manager which together describe the applicable Routing Policy. Internet |
| Presentation Layer | The layer of the OSI reference model that controls the coding, format, and appearance of the data transferred to and from the application layer. Internet |
| Presentation Service Selector (PSAP Selector) | The element of the presentation address that identifies the user of the presentation protocol entity. Internet |
| Priority | Priority is defined as the relative importance of a particular PDU relative to other PDUs in transit, and is used to allocate resources which become scarce during the transfer process. Internet |
| Profile | A profile defines implementation conformance constraints on a set of reference specifications. Internet |
| projected profile | An indication of where and when the aircraft anticipates it will be at the following two way points. CM |
| Protocol | A set of rules and formats (semantic and syntactic) which determines the communication behaviour between peer entities in the performance of functions at that layer. Internet |
| Protocol Control Information (PCI) | Information included in a layer header which contains service primitives specific to that layer. Internet |
| Protocol Data Unit (PDU) | A unit of data specified in an (N)-protocol and consisting of (N)-protocol-control-information and possibly (N)-user-data, where N indicates the layer. ULA |
| Presentation Data Value (PDV) | the unit of information specified in an abstract syntax, which is transferred by the OSI presentation-service (ISO/IEC 8822). ULA |
| Quality of Service (QoS) | Information relating to data transfer characteristics (for example, requested throughput and priority) used by a router to perform relaying and routing operations across the subnetworks which make up a network. Internet |
| Relaying | The process of transferring packets across subnetworks including any necessary packet conversion. Internet |
| Requested QoS | Requested QoS reflects the service characteristics desired by the service user. Internet |
| Reserved Value | Reserved values constitute legal values for the respective fields but have not yet been assigned specific meanings by ICAO. These values should be processed normally in order to allow future assignment. Meanings may be assigned in the future and are not available for local use. The allocation of these values requires no change in the version identifier. Internet |
| Residual Error Probability | Residual Error Probability indicates the likelihood that an PDU will be lost, duplicated, or corrupted. This probability is defined as the ratio of lost, duplicated, or corrupted NSDUs to the total number of NSDUs transmitted by an ATN NS provider, normalised for an NSDU size of 512 octets. Internet |
| Residual error rate (RER) | The ratio of messages misdelivered, non-delivered, or delivered with an error undetected by the system, to the total number of messages delivered to the system during a measurement period (adapted from ISO/IEC 8072). Note that for ATN, ICAO is considering not counting non-delivered messages in the total, as this may not be appropriate for mobile end systems, which can lose messages when leaving ATN coverage. ULA |
| Route | A route consists of the set of addresses that identifies the destinations reachable over the router, and information about the route’s path including the QoS and security available over the route. Internet |
| Router | A router is the communication element that manages the relaying and routing of data while in transit from an originating end system to a destination end system. An ATN router comprises an OSI intermediate system and end system supporting a systems management agent. Internet |
| Routing | A function within a layer that uses the address to which an entity is attached in order to define a path by which that entity can be reached. Internet |
| Routing Area (RA) | A routing area is a routing subdomain comprising one or more ISs, and optionally one or more ESs. Internet |
| Routing Domain | A set of end systems and intermediate systems that operate the same routing protocols and procedures and that are wholly contained within a single administrative domain. A routing domain may be divided into multiple routing subdomains. Internet |
| Routing Domain Confederation (RDC) | A Routing Domain Confederation (RDC) is a set of Routing Domains and/or RDCs that have agreed to join together and form a Routing Domain Confederation. The formation of a RDC is done by private arrangement between its members without any need for global coordination. Internet |
| Routing Domain Identifier (RDI) | An RDI is a generic NET as described in ISO 7498, and is assigned statically in accordance with ISO 8348. An RDI is not an address, and cannot be used as a valid destination of an ISO 8473 PDU. However, RDIs are, like ordinary NETs, assigned from the same Addressing Domain as NSAP Addresses. Internet |
| Routing Information Base(RIB) | A data base that is maintained by each router and comprises the information regarding the connectivity and topology of the ESs and ISs within a particular Routing Domain and path information pertinent to paths interconnecting Routing Domains. It is maintained by way of the information received by a routing information exchange protocol. Each Routing Information Exchange Protocol has its own RIB specification. Internet |
| Routing information exchange protocol | The protocol used to exchange subnetwork connectivity information between end systems and intermediate systems and between intermediate systems and intermediate systems. Internet |
| Routing Policy | Routing policy is a set of rules that control the selection of routes and the distribution of routing information by ATN Boundary Intermediate Systems (BISs). These rules are based on policy criteria rather than on performance metrics such as hop count, capacity, transit delay, cost, etc. which are usually applied for routing. There are two groups of routing policy in the ATN: (1) general routing policy specified in the Draft ATN Internet SARPs in order to ensure necessary connectivity in the ATN at a reasonable routing information update rate and (2) user specified routing policy, i.e. individual policy rules which may be additionally implemented in ATN BISs by administrations and organisations to meet their specific operational and policy needs. Lexicon |
| Routing Policy | The set of rules in a BIS that determines the advertisement and use of routes is known as a Routing Policy. Each organisational user of the ATN must determine and apply their own Routing Policy. Internet |
| Safety Case | A safety case is an analysis presenting an overall justification for the declaration that a particular systems satisfies its safety requirements. Internet |
| Security Label | A Security Label may indicate requirements for protection of a PDU and provide information used by network layer access control functions. Internet |
| Security Management | The purpose of security management is to support the application of security policies by means of functions which include the creation, deletion and control of security services and mechanisms, the distribution of security-relevant information, and the reporting of security-related events. Internet |
| Service Data Unit | A unit of data transferred between adjacent layer entities, which is encapsulated within a PDU for transfer to a peer layer. Internet |
| Service primitive | A function of an ASE that is not broken down further into subfunctions, and is presented as part of the abstract service interface (i.e. request, indication, response, or confirmation). ULA |
| Service Provider | An application service provider is composed of the ground and airborne AEs for the application, all underlying data communication protocol entities and the physical media. As a consequence, it encompasses everything between the Application-AE service interfaces of the peer end-users of the application. |
| Session layer | The layer of the OSI reference model that establishes the rules of dialogue between two end-user entities. Internet |
| Session Service Selector (SSAP Selector) | The element of the session address that identifies the user of the session protocol entity. Internet |
| Short TSAP | The short TSAP is composed of the ARS, the LOC, the SYS, and the SEL. CM |
| Single Homed ERD | An ERD that is in communication with one other RD only. Internet |
| Spare Value | A Spare value is a value for which no meaning is currently defined. These values are available by the administering authority for local use, and may be assigned in the future. Internet |
| Stack (or protocol stack) | A set of co-operating OSI protocols selected from different layers of the basic reference model. Hence, “upper layer stack” refers to session, presentation, and application protocols, while “lower layer stack” refers to physical, data link, network, and transport protocols. Internet |
| Subnetwork | An actual implementation of a data network that employs a homogeneous protocol and addressing plan, and is under control of a single authority. Internet |
| Subnetwork Access Facility (SNAcF) | The subset of the OSI network layer that provides the interface with the data link layer and is specific to a particular subnetwork. Internet |
| Subnetwork Access Protocol (SNAcP) | The actual protocol used to receive services form a particular subnetwork. For example, the subnetwork access protocol to many public data networks is X.25. Internet |
| Subnetwork Dependent Convergence Function (SNDCF) | The set of rules and procedures needed to convert the data transfer needs of the subnetwork independent convergence protocol to the actual services provided by a subnetwork. Internet |
| Subnetwork Domain | The set of end systems and intermediate systems connected to the same physical network. Internet |
| Subnetwork Independent Convergence Function (SNICF) | The subnetwork independent convergence protocol is the common protocol for all ATN host computers and routers that is used for the transfer of data. In the ATN internet, the SNICF is the connectionless network protocol defined by ISO 8473. Internet |
| Subnetwork Point of Attachment (SNPA) | The SNPA is the point at which a real end system, interworking unit, or real subnetwork is attached to a real subnetwork, and is a conceptual point within an end or intermediate system at which the subnetwork service is offered. Internet |
| Subnetwork Point of Attachment (SNPA) Address. | The SNPA address provides information used in the context of a particular real subnetwork to identify a SNPA. An SNPA address is a subnetwork address such as X.25 DTE Addresses, Ethernet MAC Addresses, etc. Internet |
| Subnetwork sublayer | A component of the OSI reference model that provides the protocol mechanism for data transfer between peer entities within the same subnetwork. This sublayer is an implementation of the OSI subnetwork access facility (SNAcF). Internet |
| Systems Management | The set of functions related to the management of various OSI resources and their status across all layers of the OSI architecture. Internet |
| Systems Management Application Entity (SMAE) | An application entity for the purpose of systems management communications. Internet |
| Systems Management Function | Systems Management Functions encompass the monitoring, controlling, operating, supervising, co-ordination, and administration of a communications network. Internet |
| Traffic Type | The data conveyed by the ATN is divided into four traffic types: ATN Operational Communications representing safety and regularity of flight communications, ATN Administrative Communications representing non-safety and regularity of flight communications sent by aircraft operating agencies and ATS administrations, General Communications representing APC, public correspondence, and other non-operational and non administrative communications, and ATN Systems Management Communications representing systems management information that is critical for support of network operations. The differentiation of traffic types is required because different data traffic may have different access to subnetworks. The traffic type is conveyed in the ATN Security Label of ISO 8473 (CLNP) and ISO 10747 (IDRP) PDUs. It is used to qualify (CLNP) data packets and (inter-domain) routes according to the class of traffic that they carry. Based on this qualification, access of subnetworks is controlled at the ATN Internet level. Lexicon |
| Transit Routing Domain (TRD) | A Transit Routing Domain is a domain whose policies permit its BISs to provide relaying for PDUS whose source is located in either the local routing domain or in a different routing domain. Internet |
| Transport layer | The layer of the OSI reference model that assures reliable end-to-end transfer between transport service users. Internet |
| Transport service (TS) user | The entity that uses transport layer services. Internet |
| Transport Service Access Point (TSAP) | The Transport Service Access Point is the logical access point to the transport layer. Internet |
| Transport Service Access Point (TSAP) address | The complete communications address which unambiguously defines a transport service user. The TSAP address comprises the NSAP address and a TSAP-selector. Internet |
| Transport Selector (TSEL) | The element of the address that identifies the user of the transport protocol entity. Internet |
| Update Contract | A contract to provide a piece of FIS information and any update of this information. FIS |
| Upper layers | A term pertaining to the session, presentation, and application layers of the OSI reference model. Internet |
| User Requirements | User requirements describe what users expect to obtain from the system (not how the system should do it). User requirements are usually expressed on a high level and do not care about technical details. Technically, the direct user of the ATN is an application within an end system supporting Air Traffic Management or Aeronautical Industry functions. The Air Traffic Controller, other ground staff, or the Pilot are the human beings using directly or indirectly the ATN. The user may also be seen more on the abstract level as an organisation, e.g. airline or air navigation service provider. Lexicon |
| Validation | In the ICAO context, validation is a process that ensures that systems meet user requirements to an agreed level of confidence and can be produced from written SARPs and Guidance material. Therefore, one has to distinguish between performance based and functional validation. Single subsystems of the ATN, like routers, may be validated on a functional basis. In contrast, validation of the ATN’s suitability with respect to network performance etc. requires definition of performance requirements. Both, performance based and functional validation is necessary for the ATN. Lexicon |
| Very High Frequency (VHF) | VHF is a frequency band from 30 to 300 megahertz. Internet |
| VHF Data Link (VDL) | VDL provides packet data communications to aircraft and ground users provided by a VDL subnetwork which comprises airborne VHF data radios (VDRs), VHF ground stations, and connectivity to routers on the aircraft and the ground. Internet |
| Virtual circuit priority | The priority associated with a connection (virtual circuit) which is established between two systems prior to the transmission of data. Internet |
| Wide Area Network (WAN) | Wide Area Networks are used to interconnect geographically dispersed routers and host computers. These subnetworks may be internally complex packet switching entities of their own, or they may be as simple as point-to-point dedicated lines. Internet |
| X.25 Packet Switched Data Network (PSDN) | An X.25 PSDN is a communications network that provides a network access service in compliance with CCITT Recommendation X.25. Internet |

## Acronyms

| A/G | Air-ground MHS |
| --- | --- |
| AAC | Aeronautical Administrative Communications Internet |
| AARE | ACSE Associate Response APDU ULA |
| AARQ | ACSE Associate Request APDU ULA |
| ABRT | ACSE Abort APDU ULA |
| AC | Accept SPDU ULA |
| ACSE | Association Control Service Element ULA |
| ACT | Activity Management ULA |
| ACA | Address compression algorithm Internet |
| ACAS | Airborne Collision Avoidance System ADS |
| AD | Administrative domain Internet |
| AD | Addendum (of an ISO/IEC standard) Internet |
| ADJBISMO | Adjacent BIS MO Internet |
| ADJRIBMO | Adjacent RIB MO Internet |
| ADM | Administrative identifier Internet |
| ADMD | Administration management domain MHS |
| ADMF | ADM Flag Internet |
| ADS | Automatic Dependent Surveillance ADS, CPDLC, FIS |
| ADS-ATC | ADS-based Air Traffic Control system ADS |
| AE | Application Entity ADS, CPDLC, FIS, Internet |
| AES | Aircraft earth station Internet |
| AF-Address | AFTN-form address MHS |
| AFI | Authority and format identifier Internet |
| AFS | Aeronautical fixed service Internet |
| AFTN | Aeronautical fixed telecommunication network MHS |
| A-FU | ACSE Authentication Functional Unit ULA |
| AIDC | ATS Interfacility Data Communications |
| AINSC | Aeronautical Industry Service Communication Internet, MHS |
| AK | Data acknowledgement Internet |
| ALS | Application Layer Structure ULA |
| AMHS | ATS message handling system MHS |
| AMSS | Aeronautical mobile satellite service Internet |
| ANC | Air Navigation Commission Internet |
| AOC | Aeronautical Operational Communications Internet |
| AOC | Aeronautical Operational Control ULA |
| AOM | Systems Management Upper Layer profile Internet |
| AP | Application process Internet, ULA |
| APC | Aeronautical Passenger Communications Internet |
| APDU | Application Protocol Data Unit ADS, CM, CPDLC, FIS, ULA |
| API | Application Program Interface MHS |
| App | Application ULA |
| APRL | ATN profile requirements list Internet |
| ARPA | Advanced Research Projects Agency Internet |
| ARPANET | ARPA network Internet |
| ARS | Administrative Region Selector CM, CPDLC, Internet |
| ARSD | ARS Default [Flag] Internet |
| ASE | Application Service Element ADS, CM, CPDLC, FIS, Internet, ULA |
| ASI | Abstract Service Interface FIS |
| ASN.1 | Abstract Syntax Notation One Lexicon, ADS, CM, CPDLC, FIS, Internet, MHS, ULA |
| ASO | Application Service Object ULA |
| ATC | Air Traffic Control ADS, CM, CPDLC, FIS, Internet, MHS |
| ATFM | Air traffic flow management Internet |
| ATIS | Automatic Terminal Information Service FIS, ULA |
| ATM | Air Traffic Management CM, CPDLC, FIS, Internet |
| ATN | Aeronautical Telecommunication Network ADS, CM, CPDLC, FIS, Internet, MHS, ULA |
| ATNI | ATN internet Internet |
| ATNPA | ATN protocol architecture Internet |
| ATNSM | ATN systems management Internet |
| ATS | Air Traffic Services ADS, CM, CPDLC, FIS, Internet, MHS |
| ATSC | Air Traffic Services Communications CPDLC, Internet, MHS, ULA |
| ATSU | Air Traffic Services Unit FIS |
| AU | Access unit MHS |
| BCD | Binary Coded Decimal Internet |
| BER | Basic Encoding Rules (of ASN.1) Internet, ULA |
| BIS | Boundary intermediate system Internet |
| BISPDU | BIS PDU Internet |
| BPS | Bits per second Internet |
| BSI | British Standards Institute Internet |
| C | Counter Internet |
| CAA | Civil aviation administration Internet |
| CAN | Cancellation Internet |
| CC | Connection confirm Internet |
| CCITT | International Telegraph and Telephone Consultative Committee Internet,MHS |
| CDSE | Confirmed Data Service Element ULA |
| CDT | Credit Internet |
| CE | Congestion experienced flag Internet |
| CF | Control Function ADS, CM, CPDLC, FIS, ULA |
| CIDIN | Common ICAO data interchange network Internet |
| CL | Connectionless mode Internet |
| CLNP | CL network protocol Internet |
| CLNPMMO | CL network protocol machine MO Internet |
| CLNS | CL network service Internet |
| CLTP | CL transport protocol Internet |
| CLTPMMO | CL transport protocol machine MO Internet |
| CLTS | CL transport service Internet |
| CM | Context Management Lexicon, ADS, CM, CPDLC, FIS, ULA |
| CMIP | Common management information protocol Internet |
| CMIS | Common management information service Internet |
| CMISE | CMIS element Internet |
| CN | Connect SPDU ULA |
| CNS | Communications, Navigation, and Surveillance CM, CPDLC, FIS, Internet |
| CNS/ATM | Communications Navigation Surveillance / Air Traffic Management ADS, CM, CPDLC, ULA |
| CO | Connection mode Internet |
| COMSEC | Communications security Internet |
| COTP | CO transport protocol Internet |
| COTP | Connection-oriented transport protocol MHS |
| COTPMMO | COTP protocol machine MO Internet |
| COTS | CO transport service Internet |
| CP | Connect PPDU ULA |
| CPA | Connect Accept PPDU ULA |
| CPC | Controller-Pilot Communications Lexicon, CM, CPDLC |
| CPDLC | Controller-Pilot Data Link Communications Lexicon, CM, CPDLC, ULA |
| CPR | Connect Reject PPDU ULA |
| CR | Connection request Internet |
| CR | Context Restoration Presentation FU ULA |
| CVER | Compressed VER Internet |
| DC | Demand Contract FIS |
| DC | Disconnect confirm Internet |
| DCC | Data country code Internet |
| DCE | Data circuit terminating equipment Internet |
| DCPC | Direct Controller-Pilot Communications (ATNP preferred term = “CPC”) Lexicon |
| DFDAU | Digital flight data acquisition unit Internet |
| DL | Distribution List MHS |
| DN | Disconnect SPDU ULA |
| DOD | Department of Defense Internet |
| DPA | Distinguishing path attribute Internet |
| DR | Disconnect request Internet |
| DS | Dialogue Service CPDLC, FIS, ULA |
| DSP | Data link service provider Intro |
| DSP | Domain specific part Internet |
| DST-REF | Destination reference Internet |
| DT | Data Internet |
| DT | Data Transfer SPDU ULA |
| DTE | Data terminal equipment Internet |
| E/C | Error probability over cost flag Internet |
| E/R | Error report requested Internet |
| E/T | Error probability over transit delay flag Internet |
| EA | ED acknowledge Internet |
| ED | Expedited data Internet |
| EGP | Exterior gateway protocol Internet |
| EoS | Element of Service MHS |
| EOT | End of TSDU Internet |
| ER | Error [TPDU] Internet |
| ER | Error report [NPDU] Internet |
| ER | Error report requested flag Internet |
| ERD | End routing domain Internet |
| ERP | Echo Response [NPDU] Internet |
| ERQ | Echo Request [NPDU] Internet |
| ES | End System Internet |
| ESCT | ES configuration timer Internet |
| ESH | ES hello Internet |
| EX | Expedited Data SPDU ULA |
| EXCEP | Exceptions Session FU ULA |
| EXP | LOCREF extension flag Internet |
| F/M | Fixed/Mobile Internet |
| FANS | Future Air Navigation Systems Internet |
| FD | Functional Description ADS, CM, CPDLC, FIS |
| FD | Full Duplex Session FU ULA |
| FDPS | Flight Data Processing System ADS, CM, CPDLC |
| FG | Functional Group MHS |
| FIB | Forwarding information base Internet |
| FIBMO | FIB MO Internet |
| FIFO | First in first out Internet |
| FIS | Flight Information Services Lexicon, CM, CPDLC, FIS, ULA |
| FMS | Flight management system Internet |
| FN | Finish SPDU ULA |
| FOM | Figure Of Merit ADS |
| FP | Full/Prefix Internet |
| FSM | Finite state machine Internet |
| FTAM | File transfer, access and management Internet |
| FU | Functional Unit ULA |
| G | Gauge Internet |
| GA | General Aviation Internet |
| GDMO | Guideline for definition of MOs Internet |
| GES | Ground earth station Internet |
| GT | Give Tokens SPDU ULA |
| HD | Half Duplex Session FU ULA |
| HF | High Frequency ULA |
| HI | High Interface FIS |
| IA5 | International Alphabet Number 5 CM, CPDLC, FIS, Internet, MHS |
| IATA | International Air Transport Association Internet |
| ICAO | International Civil Aviation Organization ADS, CM, CPDLC, FIS, Internet, ULA |
| ICC | Inter-Centre Communications Lexicon |
| ICD | International code designator Internet, ULA |
| ICS | Implementation conformance statement Internet |
| ID | Identifier CM, Internet, ULA |
| IDI | Initial domain identifier Internet |
| IDP | Initial Domain Part CM, CPDLC, Internet |
| IDRP | Interdomain routing protocol Internet |
| IDRPCFGMO | IDRP configuration MO Internet |
| IEC | International Electrotechnical Commission CM, CPDLC, FIS, Internet, MHS, ULA |
| IIH | IS-IS hello Internet |
| IMF | International Monetary Fund Internet |
| IOC | Internet operations centre Internet |
| IP | Internetwork protocol Internet, MHS |
| IPI | Initial protocol identifier Internet |
| IPM | Interpersonal message MHS |
| IPMS | Interpersonal Messaging System Lexicon, MHS |
| IPN | Interpersonal notification MHS |
| IPRL | ISP Protocol RL Internet |
| IS | International Standard CM, CPDLC, FIS |
| IS | Intermediate system Internet |
| ISDN | Integrated Services Digital Network Internet |
| IS-SME | IS SME Internet |
| ISH | IS hello Internet |
| ISN | Initial sequence number Internet |
| ISO | International Organization for Standardization CM, CPDLC, FIS, Internet, MHS, ULA |
| ISOPA | ISO protocol architecture Internet |
| ISORM | ISO reference model Internet |
| ISP | International standardized profile Internet, MHS |
| ISPICS | ISP Implementation Conformance Statement MHS |
| ITA-2 | International Telegraph Alphabet No. 2 MHS |
| ITU | International Telecommunication Union CPDLC, FIS, Internet, MHS, ULA |
| ITU-T | ITU Telecommunication Standardization Sector Internet |
| IUT | Implementation under test Internet |
| IVMO | Initial value MO Internet |
| K | Kilo Internet |
| L1R | Level 1 Router Internet |
| L2R | Level 2 Router Internet |
| LAN | Local area network Internet |
| LI | Low Interface FIS |
| LI | Length indicator Internet |
| LINKMO | Linkage MO Internet |
| LOC | Location Identifier CM, CPDLC, Internet |
| LOCD | LOC Default [Flag] Internet |
| LOCREF | Local reference Internet |
| LOCRIBMO | Local RIB MO Internet |
| LSP | Link state PDU Internet |
| M | More [bit] (X.25) Internet |
| MA | Major Synchronisation Session FU ULA |
| MAC | Medium Access Control Internet |
| MAD | Management administrative domain Internet |
| MD | Management Domain Internet, MHS |
| MET | Meteorological Internet |
| MF-Address | MHS-form address MHS |
| MHS | Message Handling Services Lexicon |
| MHS | Message handling system MHS |
| MIB | Management information base Internet |
| MIDS | Management information definition statement Internet |
| MIS | Management information service Internet |
| MO | Managed object Internet |
| MOA | MO attribute Internet |
| MOCS | MO conformance statement Internet |
| MOD | Modulus Internet |
| Mode S | Mode Select Internet |
| MORTS | MO requirement template specification Internet |
| MOTIS | Message-oriented text interchange system MHS |
| MS | More segments flag Internet |
| MS | Message store MHS |
| MTA | Message transfer agent MHS |
| MTS | Message Transfer System Lexicon, MHS |
| MTSE | Message transfer service element MHS |
| N/A | Not applicable Internet |
| NATO | North Atlantic Treaty Organization Internet |
| NE | Network entity Internet |
| NEMO | NE MO Internet |
| NET | NE title Internet |
| NL | Network layer Internet |
| NLE | NL entity Internet |
| NLM | NL management Internet |
| NLPI | NL protocol information Internet |
| NLRI | NL reachability information Internet |
| NLSP | NL security protocol Internet |
| NM | Network management Internet |
| NOR | No orderly release Session FU ULA |
| NOTAM | Notice to Airmen FIS, Internet |
| NPAI | Network protocol address information Internet |
| NPDU | Network protocol data unit Internet |
| NR | Negotiated Release Session FU ULA |
| NRN | Non-Receipt Notification MHS |
| NS | Network service Internet |
| NSAP | Network Service Access Point Internet, MHS |
| NSAPMO | NSAP MO Internet |
| NSDU | NS data unit Internet |
| NSMO | Network subsystem MO Internet |
| O/R | Originator/recipient MHS |
| OA | Overflow Accept SPDU ULA |
| OCA | Object class attributes Internet |
| OCN | Object class notifications Internet |
| OCNB | Object class name bindings Internet |
| OHI | Optional Heading Information MHS |
| OID | Object Identifier ULA |
| OOC | Operations on object classes Internet |
| OR | Operational Requirement ADS, CM, CPDLC, Internet |
| OSI | Open Systems Interconnection FIS, Internet, MHS, ULA |
| OSIE | OSI environment Internet |
| OSIM | OSI management Internet |
| OSISME | OSI SM environment Internet |
| P | Priority Internet |
| PC | Personal Computer Internet |
| PCI | Protocol control information Internet, ULA |
| PDAI | Predetermined address indicator MHS |
| PDAM | Proposed Draft Addendum Internet |
| PDN | Public data network Internet |
| PDU | Protocol Data Unit ADS, CM, CPDLC, Internet, ULA |
| PDV | Presentation Data Value ULA |
| PER | Packed Encoding Rules (of ASN.1) ADS, CM, CPDLC, FIS, Internet, ULA |
| PIB | Policy information base Internet |
| PIBMO | PIB MO Internet |
| PICS | Protocol implementation conformance statement Internet, ULA |
| PIREP | Pilot Report FIS |
| PPDU | Presentation PDU ULA |
| PR | Prepare SPDU ULA |
| PRL | Profile Requirements List MHS, ULA |
| PRMD | Private management domain MHS |
| PSAP | Presentation service access point Internet |
| PSDN | Packet switched data network Internet |
| PT | Please Tokens SPDU ULA |
| PTT | Post, telephone, and telegraph Internet |
| Q | QOS Maintenance Internet |
| QoS | Quality of Service ADS, CM, CPDLC, FIS, Internet, ULA |
| R&R | Requirements and Recommendations Internet |
| R/W | Read/write Internet |
| RA | Routing area Internet |
| RCP | Required Communication Performance ADS, CM, CPDLC, FIS |
| RD | Routing domain Internet |
| RD PDU | Redirect PDU Internet |
| RDC | Routing domain confederation Internet |
| RDF | Routing domain format Internet |
| RDFD | RDF Domain [Flag] Internet |
| RDI | Routing domain identifier Internet |
| RDP | Router Domain Part CM, CPDLC |
| REL | Release ULA |
| RER | Residual Error Rate ADS, CM, CPDLC, FIS, Internet, ULA |
| RESYNC | Resynchronisation Session FU ULA |
| RF | Radio frequency Internet |
| RF | Refuse SPDU ULA |
| RIB | Routing information base Internet |
| RJ | Reject Internet |
| RL | Requirements list Internet |
| RLRE | ACSE Release Response APDU ULA |
| RLRQ | ACSE Release Request APDU ULA |
| RLS | Release ULA |
| RN | Receipt Notification MHS |
| ROA | Request of Acknowledgement Internet |
| ROSE | Remote operation service element Internet |
| RP | Routing information exchange protocol Internet |
| RPF | Reference publication format Internet |
| RPOA | Recognised private operating agency Internet |
| RTE | Receiving TE Internet |
| RTSE | Reliable transfer service element Internet, MHS |
| S | W (creation), R (lifetime) Internet |
| S/T | Segmentation over transit delay flag Internet |
| SAC | Short Accept SPDU ULA |
| SARPs | Standards and Recommended Practices ADS, CM, CPDLC, FIS, Internet, ULA |
| SCN | Short Connect SPDU ULA |
| SDU | Service data unit Internet |
| SEL | (Transport) Selector CM, CPDLC, Internet |
| SHORT-CP | Short Connect PPDU ULA |
| SHORT-CPA | Short Connect Accept PPDU ULA |
| SHORT-CPR | Short Connect Reject PPDU ULA |
| SICASP | SSR Improvements and Collision Avoidance Systems Panel Internet |
| SM | Systems management Internet |
| SMA | SM application Internet |
| SMAE | SM AE Internet |
| SME | SM entity Internet |
| SMF | SM function Internet |
| SMFA | SM functional area Internet |
| SN | Subnetwork Internet |
| SNAcF | SN access function Internet |
| SNAcP | SN access protocol Internet |
| SNCR | SN connection reference Internet |
| SNDCF | SN dependent convergence function Internet |
| SNDCP | SN dependent convergence protocol Internet |
| SNICF | SN independent convergence function Internet |
| SNICP | SN independent convergence protocol Internet |
| SNSDU | SN SDU Internet |
| SN-SME | SN SME Internet |
| SNL | SN layer Internet |
| SNOWTAM | Snow NOTAM Internet |
| SNP | Sequence number PDU Internet |
| SNPA | SN point of attachment Internet |
| SNQOS | SN QoS Internet |
| SNS | SN service Internet |
| SP | SN Processor Internet |
| SP | Segmentation permitted flag Internet |
| SPDU | Session PDU MHS, ULA |
| SPI | Subsequent protocol identifier Internet |
| SPM | Session Protocol Machine ULA |
| SRC-REF | Source reference Internet |
| SRF | Short Refuse SPDU ULA |
| SS | Symmetric Synchronise Session FU ULA |
| SS | Session Service ULA |
| SSR | Secondary surveillance radar Internet |
| ST/SYS | Storage and transfer system MHS |
| STE | Sending TE Internet |
| SY | Minor Synchronise Session FU ULA |
| SYS | System Identifier CM, CPDLC |
| SYS4 | SYS 4th Octet [Flag] Internet |
| SYS5 | SYS 5th Octet [Flag] Internet |
| SYS6 | SYS 6th Octet [Flag] Internet |
| T | Tidemark Internet |
| T/C | Transit delay over cost flag Internet |
| T/SYS | Transfer system MHS |
| TC | Transport connection Internet |
| TCIVMO | TC IVMO Internet |
| TCMO | TC MO Internet |
| TCP | Transmission control protocol Internet |
| TCQIVMO | TC QoS IVMO Internet |
| TD | Typed Data Session FU ULA |
| TE | Transport entity Internet |
| TEMO | TE MO Internet |
| TI | Transmission identification MHS |
| TLE | Transport layer entity Internet |
| TP4 | Transport protocol class 4 Internet |
| TPDU | Transport protocol data unit Internet |
| TPDU-NR | TPDU send sequence number Internet |
| TR | Technical report Internet |
| TRD | Transit routing domain Internet |
| TS | Transport service Internet |
| TSAP | Transport Service Access Point CM, CPDLC, Internet, MHS, ULA |
| TSAPMO | TSAP MO Internet |
| TSDU | TS data unit Internet |
| TSMO | Transport subsystem MO Internet |
| TSN | Tag set name Internet |
| TWDL | Two-Way Data Link (ATNP preferred term = “CPC”) Lexicon |
| TWS | Terminal Weather Service FIS |
| U | User option Internet |
| UA | User agent MHS |
| UC | Update Contract FIS |
| UD | Unit data Internet |
| UHF | Ultra high frequency Internet |
| UTC | Co-ordinated Universal Time ADS, CM, CPDLC |
| VC | Virtual circuit Internet |
| VDL | VHF data link Internet |
| VER | Version Internet |
| VHF | Very high frequency Internet, ULA |
| WAN | Wide area network Internet |
| WR | Receive window value Internet |
| WS | Send window value Internet |
| WX | Weather Internet |
| X | Hexadecimal Internet |
| YR-EDTU-NR | Expected ED TPDU sequence number in EA (‘your ED TPDU number') Internet |
| YR-TU-NR | Expected TPDU sequence number (‘your TPDU number') Internet |