ATNP/WG1/5

WP/5-22
	14 June 1996

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL

Working Group 1

Munich, Germany

17. - 20. 6. 1996

ATN Naming, Addressing and Registration Concept

Deliverable WG 1-16

Presented By K. Platz

Editor: K.-P. Graf

Contributions from M. Adnams, R. Hoffmann, G. Oliveau, S. van Trees

SUMMARY

This document is an updated version of the ATN naming, addressing and registration concept. It outlines the addressing, naming and registration provisions related to the ATN information objects within the scope of the CNS/ATM-1 Package.

DOCUMENT CONTROL LOG

| Section(s) | Date | Version Number | Reason for Change |
| --- | --- | --- | --- |
| All | 13/2/95 | 0.A | Initial draft |
| All | 3/3/95 | 0.B | Additional content / Technical updates |
| All | 6/10/95 | 1.0 | Additional content / Technical updates |
| All | 26/1/96 | 1.1 | Review by ATNP WG1/3 |
| All | 14/6/96 | 1.2 | Review by ATNP WG1/4 / Additional content |

TABLE OF CONTENTS

µ1	Introduction	1

1.1	Scope and Purpose	1

1.2	Overview	1

1.3	References	1

1.4	Glossary	3

2	Naming and Addressing Principles And Concepts	6

2.1	Names and Addresses	6

2.2	Naming and Addressing Domains	6

2.3	Naming and Addressing Authorities	8

3	ATN Elements Subject to Naming and Addressing	9

3.1	Subnetwork Elements	10

3.2	Internetwork Elements	11

3.3	Upper Layer and Application Elements	11

3.4	ATN Application Users	12

3.5	Aircraft	12

3.6	ATC Centres	13

4	ATN Address and Name Definitions	13

4.1	Subnetwork Addressing	13

4.1.1	X.25 Subnetworks	13

4.1.2	VDL Subnetworks	13

4.1.3	AMSS Subnetworks	14

4.1.4	Mode S Subnetworks	15

4.1.5	Minimising The Impact of Subnetwork Topology Changes and Outages	15

4.2	Internetwork Addressing	15

4.2.1	The ATN NSAP Address	16

4.2.2	The Network Entity Title	19

4.2.3	Relationship To ATN Routing	19

4.2.4	Minimising The Impact of Topology Changes and Outages	19

4.3	Transport, Upper Layer and Application Addressing	20

4.4	Application Naming	21

4.4.1	Object Identifier Form of Application Names	22

4.4.2	Directory Name Form of Application Names	23

4.4.2.1	Ground-Based ATM Applications	25

4.4.2.2	Aircraft ATM Applications	26

4.4.2.3	Other Ground-Based Applications	26

4.5	Name and Address Relationship in Package 1	26

4.6	ATN Name and Address Cross-Reference	26

5	ATN Name and Address Registration	28

5.1	Registration Authority	28

5.2	General Procedures for Registration of ATN Objects	28

5.3	Internetwork Address Administration and Registration	29

5.3.1	Administration	29

5.3.2	Registration	29

5.4	Transport Address Administration and Registration	30

5.4.1	Administration	31

5.4.2	Registration	31

5.5	Application Name Administration and Registration	31

5.5.1	AP Registration	31

5.5.2	AE Registration	32

6	The Use of Context Management as Basic Directory
	Service	32

6.1	Resolvement of Address/Naming Information Provided By CMA	33

# 1	Introduction

## 1.1	Scope and Purpose

This paper intends to define a framework for naming, addressing and registration of ATN information objects relevant to the CNS/ATM-1 Package (Package 1). It further identifies some of the longer term naming, addressing and registration issues required for successful world-wide development and operational use of ATN systems and services expected as follow-on functionalities, perhaps in the yet-to-be defined Package 2 timeframe.

This material may serve as

r	a baseline for the refinement and further specification of the ATN naming, addressing and registration provisions for Package 1 and beyond;

r	guidance material on the naming, addressing and registration procedures of information objects in the ATN;

r	an initial register of addresses and names assigned to ATN objects for Package 1.

## 1.2	Overview

There is a requirement in the ATN environment for

r	the unambiguous identification of certain objects;

r	the assignment of names and addresses to these objects to achieve this identification;

r	the recording of definitions of the instances to which those names and addresses are assigned; and

r	the propagation of registered names and addresses both on a global level and on a local level.

The principal question concerning naming and addressing is the meaning of naming and addressing elements:  how do ATN systems and applications use this information ? The principal question concerning registration is the distribution of the authority and responsibility for the assignment of names and addresses:  how may this information be administered and propagated on a global basis?

Although a well-defined ATN internet addressing plan exists [11] and the syntax and values for ATN upper layer addresses and ATN application names have been defined [10], there are some remaining naming, addressing and registration issues which require resolution before the ATN can be successfully deployed. These issues are mainly of an administrative nature and are addressed in this document.

In the short term, following the principle of delegated responsibility for certain name and address portions as outlined in the ATN SARPs, naming/addressing authorities and registration authorities must be set up, and name and address register must be established for the Package 1 deployment. In the long term, i.e. beyond Package 1, a broader approach must be taken to cover the enhanced ATN internet and upper layer architecture, and the more advanced ATN applications.

## 1.3	References

[1]	Aeronautical Telecommunication Network (ATN) Manual, Version 2, ICAO, November 19, 1993

[2]	VHF Digital Link (VDL) Draft SARPs, Version 2.53, ICAO, November 1, 1995

[3]	Aeronautical Mobile-Satellite Service (AMSS) Draft SARPs, ICAO, April 22, 1994

[4]	Automatic Dependent Surveillance (ADS) and Air Traffic Services (ATS) Data Link Applications Guidance Material, Version 5, ICAO, May 1994

[5]	DO-223 - Minimum Operational Performance Standards for Aircraft Context Management (CM) Equipment, RTCA SC-169, July 13, 1994

[6]	DO-212 - Minimum Operational Performance Standards for Automatic Dependent Surveillance, RTCA SC-169

[7]	DO-219 - Minimum Operational Performance Standards for Two-Way Data Link, RTCA SC-169

[8]	Secondary Surveillance Radar (SSR) Mode S Subnetwork SARPs, Annex 10, Aeronautical Telecommunications, Volume I, Part 1, Chapter 4, Amendment 69, ICAO

[9]	Aeronautical Fixed Telecommunication Network (AFTN) SARPs, Annex 10, Aeronautical Telecommunications, Volume II, Amendment 69, ICAO

[10]	ATN Draft SARPs, Subvolume 4 - ATN Upper Layers, Version 3.0, April 19, 1996

[11]	ATN Draft SARPs, Subvolume 5 - Internet Communications Service, Version 5.1, June 7, 1996

[12]	ATN Draft SARPs, Subvolume 2 - Air/Ground Applications, Part 1 - Context Management Application, Version 3.0, April 22, 1996

[13]	CNS/ATM-1 Package Registration Authority, ATNP/JWG/WP3-5, 1995

[14]	ATN Draft SARPs, Subvolume 3, Part 1 - ATS Message Handling Services over the ATN, Version 1.0, April 24, 1996

[15]	ATN Draft SARPs, Subvolume 3, Part 2 - ATS Interfacility Data Communications, Version 1.1, April 1996

X.25	Recommendation X.25 - Interface Between DTE and DCE for Terminals Operating In the Packet Mode And Connected to Public Data Network by Dedicated Circuit, CCITT, 1988

X.121	Recommendation X.121 - International Numbering Plan for Public Data Networks, CCITT, 1992

3166	ISO 3166 - Codes for the representation of names of countries, ISO, 1988

6523	ISO 6523 - Structures for the identification of organizations, ISO, 1984

7498-3	ISO/IEC 7498-3 - Basic Reference Model, Part 3: Naming and Addressing, ISO, 1989

8824	ISO/IEC 8824 - Specification of Abstract Syntax Notation One (ASN.1), ISO, 1990

8824-1	ISO/IEC 8824-1/Amendment 1 - Abstract Syntax Notation One (ASN.1), Specification of Basic Notation, Amendment 1: Rules for extensibility, ISO, 1995

9834-1	ISO/IEC 9834-1 - Procedures for the Operation of OSI Registration Authorities: General Procedures, ISO, 1993

9834-6	ISO/IEC 9834-6 - Procedures for the Operation of OSI Registration Authorities: Application Processes and Application Entities, ISO, 1993

9594-2	ISO/IEC 9594 - The Directory, Part 2: Models, ISO, 1990

9594-6	ISO/IEC 9594 - The Directory, Part 6: Selected Attribute Types, ISO, 1990

10730	ISO/IEC 10730 - Tutorial on Naming and Addressing, First edition, 1994

11188-1	ISO/IEC 11188-1 - Common Upper Layer Requirements - Part 1: Basic Connection Oriented Requirements, ISO, 1995

## 1.4	Glossary

ACSE		Application Control Service Element

ADI		Administrative Domain Identifier

ADM		Administration Identifier

ADS		Automatic Dependent Surveillance

AE		Application Entity

AES		Aircraft Earth Station

AFI 		Authority Format Identifier

AFTN		Aeronautical Fixed Telecommunication Network

AI		Aircraft Identifier

AIDC		ATS Interfacility Data Communications

AINSC	Aeronautical Industry Services Communications

AMHS	ATS Message Handling System

AMSS		Aeronautical Mobile Satellite Service

AO		Aircraft Operator

AOR-E	Atlantic Ocean Region East

AOR-W	Atlantic Ocean Region West

AP		Application Process

ARS		Administrative Region Selector

ASE		Application Service Entity

ATC		Air Traffic Control

ATCC		Air Traffic Control Centre

ATM		Air Traffic Management

ATN		Aeronautical Telecommunication Network

ATNP		ATN Panel

ATS		Air Traffic Services

ATSC		Air Traffic Services Communications

ASNID

ASO		Application Service Object

BCD		Binary Coded Decimal

CCITT	International Telegraph and Telephone Consultative Committee

CLNP		Connectionless-mode Network Protocol

CM		Context Management

CMA		Context Management Application

CNS		Communication, Navigation, Surveillance

CPC		Controller Pilot Communications

CPDLC	Controller Pilot Data Link Communications

DIT		Directory Information Tree

DNIC		Data Network Identification Code

DTE		Data Terminal Equipment

FDDI		Fiber Distributed Data Interface

FIR		Flight Information Region

FIS		Flight Information Services

GDLP		Ground Data Link Processor

GWA		AFTN/ATN Gateway Type A

GWB		AFTN/ATN Gateway Type B

HF		High Frequency

IA5		International Alphabet No. 5

IATA		International Air Transport Association

ICAO		International Civil Aviation Organisation

ICC		Inter-Centre Communications

ICD		International Code Designator

ID		Identifier

IDI		Initial Domain Identifier

IDP		Initial Domain Part

IDRP		Inter-Domain Routing Protocol

IEC		International Electrotechnical Commission

IOR		Indian Ocean Region

ISO		International Organisation for Standardisation

ITU-T		International Telecommunication Union - Telecommunication Section

LAN		Local Area Network

LOC		Location Identifier

MHS		Message Handling Services

NE		Network Entity

NET		Network Entity Title

NSAP		Network Service Access Point

N-SEL	Network Selector

OID		Object Identifier

OSI		Open Systems Interconnection

OSIE		OSI Environment

O/R		Originator/Recepient

NTN

PCI		Protocol Control Information

POR		Pacific Ocean Region

PSAP		Presentation Service Access Point

PSDN		Packet Switched Data Network

RD		Routing Domain

RDF		Routing Domain Format

RDI		Routing Domain Identifier

RDN		Relative Distinguished Name

RTCA

SARPs	Standards and Recommended Practices

SEL		Selector

SNPA		Subnetwork Point of Attachment

SSAP		Session Service Access Point

S-SEL	Session Selector

SYS		System Identifier

TSAP		Transport Service Access Point

TSEL		Transport Selector

ULA		Upper Layers Architecture

VER		Version Identifier

VDL		VHF Data Link

WG		Working Group

# 2	Naming and Addressing Principles And Concepts

The ATN naming and addressing scheme is based on the OSI Reference Model (ISO 7498-3) which supports the principles of unique and unambiguous identification of information objects and global address standardisation which are essential features for an international, mixed-user communications system as the ATN.

## 2.1	Names and Addresses

The OSI Basic Reference Model, Part 3 (ISO 7498-3) distinguishes the concepts of name and address.

In brief, a name is a linguistic construct which is expressed in some language and is used to identify an object (e.g. a system, a protocol entity, an application, etc.) while an address is used to locate an object. A name stays with an object as long as it exists, while the address of the object may change during the lifetime of the object, e.g. if the object is moved from one system to another, or from one (routing) domain to another.

Names typically have meaning to upper layer elements, applications and people and are thus generally expressed in a mnemonic format. Correspondingly, the significance of addresses typically increases when descending in the OSI stack and are generally expressed in a coded or numeric format. Typical examples of addresses are the NSAP address of a network service user and the address prefixes used by routing algorithms. In either case, in order to ensure unambiguous assignments, names and addresses must be registered by registration authorities within the context or environment in which they are to be used.

In order to manage the inherent complexity of large networks, names and addresses should be assigned according to hierarchical name/address trees.  The structure of these trees should, in principal, represent the structure of the network operating environment.  For example, as the ATN is an international network involving primarily two principal interest groups (ICAO and IATA), both the ATN internetwork (NSAP) address tree and the ATN application (AP and AE) name tree split the overall naming and addressing space between these groups at a high level of the tree (see Figures 4.2.1-2 and 4.4-1). At the same time, the responsibility for registration of addresses and names has effectively been divided between ICAO and IATA on a high level of the address stucture, with the provision of further delegation to subordinate authorities, if appropriate. Each authority is responsible for address allocation and registration within their sub-spaces and may further delegate responsibility to subordinate organisations or bodies. For example, the responsibility for NSAP address field allocation may be further delegated to the States for the ICAO naming/addressing space and to airlines for the IATA naming/addressing space.

Naming, addressing and registration concepts are closely related to directory services.  A directory may be used to store name and address information and can provide lookup functions to retrieve address information or other information on the associated object (e.g. a given application or service element). Directory information may be stored local to a system if the information is small and static, i.e. the relevant address and naming information does not change with time.  However, in an operating environment like the one for which ATN is intended, applications cannot always know in advance the current addresses and names of all other communication partners.  A simple directory service for use between aircraft and ground entities, the context management application, has been specified in [12].

## 2.2	Naming and Addressing Domains

Unambiguity of ATN names and ATN addresses is achieved through the use of naming/addressing domains with firmly allocated naming/addressing authorities.

A naming/addressing domain is the set of names/addresses that are assignable to objects of a particular type. Independent naming/addressing domains exist for objects of different types (e.g. an application process, a network entity or a subnetwork point of attachment). Each naming/addressing domain is administered by a naming/addressing authority (see section 2.3).

According to ISO 7498-3, naming/addressing domains may be hierarchically decomposed into subsets which are known as naming/addressing sub-domains. Each subset (sub-domain) is under the control of an individual naming/addressing authority and does not intersect with other subsets (sub-domains) administered by different naming/addressing authorities. The top of this hierarchical structure is the global OSI domain.

The ATN naming/addressing domains, i.e. the sets of all possible names/addresses of objects within the ATN, are subdomains of the global OSI naming/addressing domain. Several such ATN naming/addressing domains exist, as there are different types of ATN objects which haved to be named or assigned addresses respectively, as outlined in section 3. An ATN naming domain exists, for example, for ATN Application Processes and an ATN addressing domain exists, for example, for ATN network addresses.

The naming/addressing authority for the ATN naming/addressing domains is ICAO which controls and manages these domains through the ATN SARPs.

In order to facilitate the address assignment and registration in the ATN, which is expected to comprise several thousands of objects, the ATN SARPs further decompose the ATN naming/addressing domains into a set of hierarchical sub-domains. This principle of hierachically structured ATN sub-domains within the global OSI naming/addressing domain is illustrated in Figure 2.2-1 for the example of the ATN NSAP addressing domain.

Figure 2.2-1:	Hierarchical Decomposition of Addressing Domains Illustrated for ATN Network Addresses

Figure 2.2-1 shows how the global OSI network addressing domain (which is itself a sub-domain of the global OSI addressing domain) is partitioned into several sub-domains, one of which is the ATN NSAP address sub-domain. This sub-domain is itself decomposed into a number of subordinate addressing sub-domains in a recursive fashion. Each such sub-domain is associated with an NSAP addressing authority which is responsible for this sub-domain, and may further delegate authority for those sub-domains into which it has partioned its own addressing subdomain. This principle allows to construct ATN addresses as a sequence of individual address fields (see Figure 2.2-1 and section 4.2.1), with each field corresponding to an addressing sub-domain. As these sub-domains are individually administered, the address field formats and values can be assigned without the need to co-ordinate between addressing authorities.

## 2.3	Naming and Addressing Authorities

A naming/addressing authoritiy defines the rules, including syntax (i.e. sizes and formats) and semantics (i.e. contents and interpretation), for specifying names/addresses within its naming/addressing domain and for the creation of further sub-domains. Furthermore, it allocates names/addresses within its domain according to specified rules, but does not perform the binding of the allocated names/addresses to the associated objects. This latter task is within the responsibility of the registration authority (see section 5).

A naming/addressing authority may administer and allocate names/addresses itself, or, if it has partioned its naming/addressing domain into naming/addressing sub-domains, may delegate the responsibility for naming/addressing within each such sub-domain to a sub-domain naming/addressing authority.

The overall naming and addressing authority for the ATN naming/addressing domains are the ATN SARPs. Besides partioning the ATN naming/addressing domains into appropriate sub-domains and specifying the syntax, semantics and encoding for these sub-domains, the ATN SARPs also directly allocate and register names/addresses within these sudomains, where appropriate or required (see for example section 4.2.1). Furthermore, provisions have been made within the ATN SARPs which delegate full or partial responsibility for certain sub-domains (i.e. certain address fields) to organisations other than ICAO, such as IATA, regional ATS organisations and national ATS administrations. In order to comply with the ATN SARPs, these organisations have to implement the procedures for a naming/addressing authority and, where required, for a registration authority (see section 5.2).

Within a hierarchy of naming/addressing domains, the operation of each naming/addressing authority is independent of that of the other naming/addressing authority at the same level, subject only to any common rules established by the procedures of the parent authorities.

# 3	Package 1 Assumptions

## 3.1	Applications

The following ATS applications will be part of the initial application set for the CNS/ATM-1 Package:

r	Automatic Dependent Surveillance (ADS)

r	Controller Pilot Data Link Communications (CPDLC)

r	Flight Information Services (FIS)

r	Context Management (CM)

r	ATS Inter-Centre Communications (ICC)

r	Message Handling Services (MHS).

Package 1 is also expected to support airline-specific messaging and distributed applications.  The operational requirements of these applications are not globally-defined and are outside the scope of the activity of ICAO.  It is not expected that the introduction of airline-specific applications will adversely affect the ATN naming, addressing and registration concept.

## 3.2	Upper Layer Architecture

Until early 1995, the upper layer architecture for all Package 1 applications considered in this document was expected to be minimal.  Specifically, the principal standards from RTCA defining the ADS, CPDLC, and CM applications ([5], [6] and [7]) indicated that the applications would run directly over the connection oriented transport service without any support from the session, presentation or application services.  However, recent developments in the ATNP/WG3/SG3 (ATN Upper Layer Architecture Subgroup) have converged in an ATN upper layer architecture (ULA) for air/ground applications [10] which is based on an extended Association Control Service Element (ACSE edition 2) and the presentation and session efficiency enhancements specified by the ITU-T.  As ATNP/WG3/SG3 is still working on the final details of the ULA, the aspects corresponding to upper layer addressing and naming presented in this paper are still in a state of flux.

## 3.2	Internetwork Topology

The ATN internetwork topology is expected to be a loosely connected internetwork of routing domains providing the connectivity required to support ATM and airline-specific services.  Specifically, we can foresee ATC centres being connected together via local means within an ICAO Member State and via international WANs and the ATN backbone between ICAO Member States.  Communications between aircraft and ATC centres will be provided by the ATS’s Mode S subnetwork as well as AMSS and VDL subnetworks which may be owned and operated by both air/ground communication service providers or ATS authorities.

The ATN internetwork will be comprised of both transit routing domains and end routing domains.  Transit routing domains are typically responsible for switching ATN traffic between themselves and to and from end routing domains.  ATN end users are typically only responsible for end routing domains, which do not switch traffic for other ATN routing domains.  For example, each ATN aircraft will have at least one ATN end routing domain on board.

## 3.3	Subnetworks Used

In order to interconnect ATN routing domains, the respective interdomain routers must be connected via a subnetwork.  The principal ATN subnetworks to be used during the Package 1 timeframe will be X.25 PSDNs for ground-based communications as well as the VDL, AMSS and Mode S subnetworks for air/ground communications.

# 3	ATN Elements Subject to Naming and Addressing

The ATN elements which require the assignment of unambiguous names are:

| ATN Element | Generic Name |
| --- | --- |
| Network Layer Entities | Network Entity Title (NET)1 |
| Routing Domains | Routing Domain Identifier (RDI) |
| Routing Domain Confederations | Routing Domain Identifier (RDI) |
| Administrative Domains | Administrative Domain Identifier (ADI) |
| Application Processes | Application Process Title (AP-Title) |
| Application Entities | Application Entity Title (AE-Title) |
| Application Context | Application Context Name |
| Presentation Context | Presentation Context Identifier |
| AMHS User | AMHS Originator/Receipient Name (O/R Name) |
| Security Type Objects | Security Registration Identifier |
| Managed Objects | Managed Object Name |

Table 3-1:	ATN Elements Requiring Unambiguous Names

The ATN elements which require the assignment of unambiguous addresses are:

| ATN Element | Generic Address |
| --- | --- |
| Subnetwork Access Point | Subnetwork Point of Attachment (SNPA) Address |
| Network Layer Entities | Network Entity Title (NET) |
| Network Service Users | Network Service Access Point (NSAP) Address |
| Transport Service Users | Transport Service Access Point (TSAP) Address |
| Session Service Users | Session Service Access Point (SSAP) Address |
| ATN Applications | Presentation Service Access Point (PSAP) Address |

Table 3-2:	ATN Elements Requiring Unambiguous Addresses

Whereas the SNPA addresses have to be unambiguous within the context of a given subnetwork, all other names and addreses listed in the Tables 3-1 and 3-2 have to be unambiguous throughout the whole ATN. As the ATN internetwork addressing plan [11] defines NSAP addresses which are globally (i.e. within the OSI environment) unique, all addresses, except the SNPA addresses, listed in Table 3-2 are even globally unambiguous.

Figure 3-1 illustrates the location of those ATN elements, which require unambiguous names or addresses and which relate to an ATN system, within a conceptual representation of an ATN End System. Those elements of the above tables not displayed in this figure either relate to an entity which is beyond the scope of an ATN System (such as Administrative Domains and Routing Domains), or which may be found in any layer of an ATN End System (such as Managed Objects or Security Type Objects) and have been omitted in the Figure 3-1 for presentation reasons.

Figure 3-1: Adresses and Names of ATN Objects Contained in an ATN End System

As can be seen from Figure 3-1, ATN addresses and names/addresses relate to objects contained in different layers of an ATN system's communication stack and even identify users of this stack. There may be more than one object of a given type, e.g. application process or transport entity, within an ATN System. In this case, the name/address of each such object has the same syntax, semantics and encoding, but differs in the assigned value.

## 3.1	Subnetwork Elements

In order to provide ATN end-to-end data transfer ATN Systems will be interconnected via real subnetworks.  The most likely ATN subnetworks to be used during the Package 1 timeframe will be X.25 PSDNs for ground-based communications as well as the VDL, AMSS and Mode S subnetworks for air/ground communications. This section briefly outlines the addressing elements within these subnetworks. It should be noted that the subnetworks listed in this section are only a subset of the network technologies which can be used to interconnect ATN Systems.

1.	X.25 Network Addresses.  An X.25 network address identifies the subnetwork point of attachment (SNPA) of a DTE/router on a given X.25 network.  It has meaning only within the context of the X.25 network on which it is used subject to the provisions in [X.25].

2.	AMSS Network Addresses.  An AMSS network address identifies the SNPA of a DTE/router on a given AMSS subnetwork.  An AMSS DTE may either be located on an aircraft or at a fixed site.  The AMSS network address has meaning only within the context of the AMSS network in which it is used subject to the provisions in [3].

3.	VDL Network Addresses.  A VDL network address identifies the SNPA of a DTE/router on a given VDL subnetwork.  An VDL DTE may either be located on an aircraft or at a fixed site.  The VDL network address has meaning only within the context of the VDL network in which it is used subject to the provisions in [2].

4.	Mode S Network Addresses.  A Mode S network address identifies the SNPA of a DTE/router on a given Mode S subnetwork.  A Mode S DTE may either be located on an aircraft or at a fixed site.  The Mode S network address has meaning only within the context of the Mode S network in which it is used subject to the provisions in [8].

## 3.2	Internetwork Elements

1.	Network Entity Title (NET).  An ATN NET is a 20-octet string used to uniquely identify and locate a network layer entity of an ATN system (router or end system), and thus, in networking terms, is used to identify the system itself.  Thanks to the global nature of the ATN internetwork addressing plan defined in [11], a system’s NET can be used to locate it anywhere within the ATN.  The syntax of an ATN NET is equivalent to that of an ATN NSAP address. It differs from the NSAP addresses assigned to the same system only in the last octet, i.e. the network selector (N-SEL) field value (see section 4.2.1).

2.	NSAP Address.  An ATN NSAP address is a 20-octet string used to uniquely identify and locate a given NSAP (i.e. a network service user) within the context of the ATN.

3.	Administrative Domain Identifier.  Each ATN administrative domain can be identified by a unique domain address comprising of the AFI, IDI, VER and ADM fields of an ATN NSAP address (see section 4.2.1).  An administrative domain identifier is the prefix of all ATN NSAP addresses and NETs within the same ATN administrative domain.

4.	Routing Domain Identifier.  Each ATN routing domain can be identified by a unique, unvarying 11-octet ATN routing domain identifier comprising of the AFI, IDI, VER, ADM, RDF and ARS fields of an ATN NSAP address.  A routing domain identifier is the prefix of all ATN NSAP addresses and NETs within the same ATN routing domain.

## 3.3	Upper Layer and Application Elements

1.	Transport Address.  An ATN transport address, or in OSI terms an ATN TSAP address, uniquely identifies and locates a given transport service user within the context of the ATN.  A TSAP address is comprised of a one- or two-octet TSAP selector (T-SEL) appended to the host system’s NSAP address. Consequently, the T-SEL identifies and locates a given transport service user within the context of the end system's NSAP address, i.e. within a given end system. Therefore T-SEL values have only a local scope and do not need global registration.

2.	Session Address.  An ATN session address, or in OSI terms an ATN SSAP address, uniquely identifies and locates a given session service user within the context of the ATN.  A SSAP address is comprised of a SSAP selector (S-SEL) appended to the host system’s TSAP address. Consequently, the S-SEL identifies and locates a given session service user within the context of the end system's TSAP address, i.e. within a given end system. Therefore S-SEL values have only a local scope and do not need global registration.

3.	Application Address.  An ATN application address, or in OSI terms an ATN PSAP address, uniquely identifies and locates a given application within the context of the ATN.  A PSAP address is comprised of a PSAP selector (P-SEL) appended to the host system’s SSAP address. Consequently, the P-SEL identifies and locates a given session service user within the context of the end system's SSAP address, i.e. within a given end system. In Package 1 the application address of an ATN datalink application is a PSAP address with absent session and presentation selectors (S-SEL and P-SEL). Thus it has the form of a TSAP address (see section 4.3).

4.	Application Name.  An ATN application name, or in OSI terms an ATN application entity title (AE-title), uniquely identifies a given application within the context of the ATN. An AE-title consists of an application process title (AP-title) and an AE-qualifier. The AP-title is the name of the application process which contains the ATN application, and the AE-qualifier uniquely identifies this ATN application within the context of the application process. In Package 1, the AP-title of an ATN datalink application which is hosted by a ground system includes the ICAO Facility Designator, whereas the AP-title of an ATN datalink application which is hosted by an aircraft system includes the ICAO 24-bit aircraft address.

AP-Titles and AE-Qualifiers may be assigned either an attribute-based name form or an object identifier name form (see section 4.4). When an AP-Title is allocated an attribute-based name form, all of the associated AE-Qualifiers must also be assigned an attribute-based name form. Correspondingly, when an AP-Title is allocated an object identifier name form, all of the associated AE-Qualifiers must also be assigned an object identifier name form.

The application name may be used as input to a directory service to determine the address, i.e. the PSAP address, of a given application. A simple form of such a directory service is provided by the ATN Context Management Application (CMA) [12].

5.	Application Process Title.  Application Processes are each named in terms of a unique ATN application process title (AP-title) which uniquely identifies the application process throughout the OSI environment. Application processes are those elements in a given ATN end system which perform the information processing for particular user applications. In doing so, application processes make use of one or more application entities which are included in the application process.

## 3.4	ATN Application Users

1.	AMHS User Name.  An AMHS user name or in OSI-terms an AMHS O/R name is used to locate an AMHS user in the AMHS address space. An AMHS O/R name is composed of an AMHS management domain identifier and a set of attributes, which depend on the addressing scheme implemented by the AMHS management domain, and which uniquely identify the AMHS user within this management domain.  Furthermore, an AMHS O/R name may optionally include the directory name of the AMHS user [14].

## 3.5	Aircraft

1.	ICAO 24-bit Aircraft Address.  Each aircraft can be identified by a unique, unvarying 24-bit identifier assigned by the aircraft manufacturer under the authority of ICAO.  This ICAO 24-bit aircraft address is used as component (i.e. field value) in several of the ATN addresses and names, including the AE-title, the NSAP address as well as the VDL, AMSS and Mode S SNPAs.

2.	Registration Mark.  Each aircraft can be identified by a unique, unvarying registration mark.  In some cases, the registration mark may be derived from the ICAO 24-bit aircraft address, while in others there is no one-to-one mapping with the ICAO 24-bit aircraft address.  Currently, the ATN adressing and naming provisions do not make use of the registration mark in their address and name definitions.

3.	Flight Number.  During any given flight or flight segment, commercial aircraft are generally assigned a flight number comprised of the operating company’s IATA 3-character identifier and a unique number of up to four digits.  The flight number is included with an aircraft’s flight plan.  ATC authorities currently use an aircraft’s flight number to identify aircraft within their control.  Currently, the ATN adressing and naming provisions do not make use of the flight number in their address and name definitions.

## 3.6	ATC Centres

1.	Currently, each Air traffic Control Centre (ATCC) is assigned an address per [9].  There is no requirement from the ATN to address a particular ATCC, but rather to address ATN systems and applications within an ATCC. Consequently, there are no ATN addresses or names defined for ATCCs. However, the ICAO Facility Designator is part of the AE-title of ground-based ATN datalink applications in order to identify an ATN application within the context of a given ATC responsibility.

# 4	Atn Address and Name Definitions

## 4.1	Subnetwork Addressing

Although subnetwork addresses are no ATN addresses in the strict sense , as their scope is limited to a given subnetwork and is not unique and unambiguous throughout the ATN, subnetwork addressing is important to consider in the ATN context as ATN systems must identify the appropriate subnetwork point of attachment to communicate with other ATN systems.  In general, only addresses are assigned to SNPAs and no names.

Registration of SNPAs is generally a consideration local to a subnetwork, but the SNPA addresses assigned to specific systems or services should be made available to all interested parties attached to a given subnetwork.

This section presents addressing issues related to subnetworks likely to be used for interconnecting various ATN organisations and routing domains in the timeframe of Package 1. Other subnetwork types, such as HF and Gatelink to interconnect ATN airborne routers and ATN air/ground Routers, or LAN and FDDI to interconnect ATN airborne systems or ATN ground systems, may be deployed as ATN subnetworks in the longer term.

### 4.1.1	X.25 Subnetworks

Addressing of systems (typically referred to as DTEs on X.25 networks) attached to an X.25 network is treated in [X.25] and [X.121].  Generally an X.25 network has a well-established local addressing plan according to the needs identified by the network operator.  DTE addresses on a given network typically contain up to 10 BCD digits.  For calls placed to destinations outside of the network, a 4-digit BCD DNIC defined by [X.121] or other international agreements must be prepended to the DTE network address.

DTE network addresses do not need to be registered within the ATN-wide context as their use does not have ATN-global impact. Rather, organisations will exchange DTE network addresses as appropriate according to a prior agreement or service contract.

An ATN system connected to an X.25 network must have preconfigured knowledge of the DTE network addresses of the systems with which it will communicate using the X.25 network.  Typically, this information should be loaded in the CLNP routing table by systems management functions prior to putting the system into service.

### 4.1.2	VDL Subnetworks

The addressing plan for the VDL subnetwork is described in [2].  The scope and impact of the VDL addressing plan is limited to the VDL subnetwork. Systems not directly attached to the VDL subnetwork are not affected by the VDL addressing plan.

The two principal types of ATN systems which use the VDL subnetwork are aircraft routers and air/ground routers.  The VDL address of an aircraft router is comprised of an 8 digit BCD-encoded representation of its ICAO 24-bit aircraft address plus an optional one or two digit subaddress.  The VDL address of an air/ground router may simply be its DTE network address as assigned by the network operator to which it is attached.

A system not directly attached to a VDL subnetwork may be able to call a router attached to the VDL subnetwork by prepending the DNIC of the destination network to the VDL address of the called system.

As all calls placed over the VDL subnetwork must be originated by aircraft routers, the addresses of appropriate air/ground routers must be known before a call is placed.  An aircraft router must have the ability to determine which air/ground routers can be called via a given ground station.  Ground stations providing the VDL service will broadcast information about reachable air/ground routers and will allow aircraft routers to call a default router without having to know its DTE network address. The specific operation of these VDL functions are specified in [2].

### 4.1.3	AMSS Subnetworks

The addressing plan for AMSS is treated in [3].  The scope and impact of the AMSS addressing plan is limited to the AMSS subnetwork. Systems not directly attached to the AMSS network are not affected by the AMSS addressing plan.

The two principal types of ATN systems which use the AMSS subnetwork are aircraft routers and air/ground routers.  The AMSS address of an aircraft router is generated using BCD-encoded digits as follows:

<AMSS aircraft address> :: <DNIC> ‘5’ <AES> > <D>

<DNIC> :: 	‘1111' (for AOR-E satellite) or ‘1112’ (for POR satellite) or ‘1113’  (for IOR satellite) or ‘1114’ (for AOR-W satellite)

<AES> ::	8-digit BCD-encoded ICAO 24-bit aircraft address

<D> ::	Optional subaddress digit

The digit ‘5’ following the DNIC is a discriminator indicating that the address refers to an aircraft system.  An example AMSS address of an aircraft router in the satellite coverage area of the Atlantic Ocean Region East (AOR-E) satellite may be 1111.5.46721005.

The AMSS address of an air/ground router is formatted using BCD-encoded digits as follows:

<AMSS ground address> :: ‘26’ <DNIC> <NTN>

<DNIC> :: 	4-digit DNIC of the ground network as registered in [X.121] or by international convention

<NTN> ::	Up to 9-digit network terminating number (DTE network address) of the air/ground router on the provider’s network identified by the ASNID.

The digits ‘26’ comprise a prefix indicating that the address is used to access an internetwork router within the AMSS addressing plan.  As an example, a SITA air/ground router's AMSS address may be 26.1116.2331123.

As part of the routing initiation process over an AMSS subnetwork, aircraft routers are required to place the initial call to the air/ground router.  Therefore, the addresses of appropriate air/ground routers must be known before a call is placed.  An aircraft router must have the ability to determine which air/ground routers can be called via a given ground station.  As the number of air/ground routers connected to the AMSS subnetwork is supposed to be small, this information can be pre-configured into the aircraft router by systems management.  Therefore, limited address registration may be required within the scope of the AMSS.

### 4.1.4	Mode S Subnetworks

The addressing plan for the Mode S Subnetwork is defined in [8].  The scope and impact of the Mode S subnetwork addressing plan is limited to the Mode S subnetwork. Systems not directly attached to the Mode S subnetwork are not affected by the Mode S addressing plan.

The two principal types of ATN systems which use the Mode S subnetwork are aircraft routers and air/ground routers.  The Mode S address of an aircraft router is generated from the ICAO 24-bit aircraft address using BCD-encoded digits as follows:

<Mode S aircraft address> :: 		<ICAO 24-bit aircraft address> ‘00’

<ICAO 24-bit aircraft address> :: 	Octal representation of the ICAO 24-bit aircraft address coded in BCD.

The total lenght of the Mode S DTE address of an aircraft router is 10 digits. The subaddress formed by the last two digits of the aircraft Mode S address identifies specific DTEs onboard an aircraft. The subaddress '00' is firmly assigned to the DTE of the aircraft ATN router per [11]. An example Mode S DTE address of an aircraft router may be 46721005.00

The Mode S DTE address of an air/ground router is a 3-digit BCD-encoded value in the range of 0 through 255. Assignment of ground Mode S DTE addresses is a local issue. However, all DTEs connected to Mode S ground data link processors (GDLPs) having overlapping coverage must have unique addresses. This requires some form of bi- or multilateral administrative coordination between adjacent ICAO Member States when assigning ground Mode S subnetwork addresses.

### 4.1.5	Minimising The Impact of Subnetwork Topology Changes and Outages

From time to time, a subnetwork topology may change or a system outage on subnetwork level may occur which effectively may change the subnetwork address of a given system.  Because ATN subnetworks are used to support air traffic management applications, operational availability must be very high.

Subnetwork topology changes or subnetwork address changes should be transparent to ATN users. This is possible, if a change in the subnetwork address does not affect NSAP addresses or NETs. As illustrated in Table 1, the NSAP addressing plan allows in the SYS field the assignment of a local identifier for a given endsystem or router. Transparancy of ATN addresses from subnetwork topology changes is ensured if a SYS field value is assigned which is independent from the endsystem's or router's subnetwork (DTE) address(es). In this case only the local intra-domain routing tables must be updated when the subnetwork topology or subnetwork addresses changes. These changes are limited to the local routing area and must not be propagated to other routing areas or routing domains.

## 4.2	Internetwork Addressing

The ATN internetwork addressing plan (i.e. ATN NSAP addressing plan) is defined in [11].  This hierarchical addressing plan reflects the expected (network and routing) topology of the ATN and accommodates the requirements of ATSC and AINSC service users and providers derived from how the network may be used by the ATN community.  Strict adherence to this hierarchical addressing plan is required for proper and efficient operation of the ATN internetwork.

### 4.2.1	The ATN NSAP Address

The use of the ATN internetwork addressing plan for Package 1 is outlined in Table 4.2.1-1.  This table describes the complete ATN NSAP address format, semantics and field contents from which NETs, RDIs and NSAP addresses and NSAP address prefixes are derived. Furthermore, Table 4.2.1-1 shows the addressing and registration authorities for each of the address fields as specified in [11].

| Field Name | Size (Octets) | Value / Range (Hex) | Semantic | Contents | Package 1 Status | Addressing Authority | Registration Authority |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AFI | 1 | 47 | Basic NSAP Format | 47 | Fixed | ISO | ISO/IEC 8473 |
| IDI | 2 | 00 27 | ATN NSAP | 00 27 | Fixed | ISO | British Standards Institute (BSI) |
| VER | 1 | 01 41 81 c1 | Fixed AINSC Address Mobile AINSC Address Fixed ATSC Address Mobile ATSC Address | 01 41 81 c1 | Fixed Fixed Fixed Fixed | ATN SARPs Subvolume 5 | ATN SARPs Subvolume 5 |
| ADM | 3 | 00 00 00 - ff ff ff | ATN Network Address (Sub-)Domain Authority | ISO Country Code or IATA Code | Fixed Variable | ATN SARPs Subvolume 5 | ISO 3166 IATA Draft Doc ? |
| RDF | 1 | 00 | Unassigned | 00 | Fixed | ATN SARPs Subvolume 5 | ATN SARPs Subvolume 5 |
| ARS | 3 | 00 00 00 - ff ff ff | Routing Domain | For ground RDs: Not defined For mobile RDs: ICAO 24-bit Aircraft Address | Variable Fixed | ATN SARPs Subvolume 5 | ICAO/IATA ICAO |
| LOC | 2 | 00 00 - ff ff | Routing Area | Not defined | Variable | ATN SARPs Subvolume 5 | ATN Network Addressing (Sub-) Domain Authority defined in ADM field |
| SYS | 6 | 00 00 00 00 00 00 - ff ff ff ff ff ff | ATN System | Not defined | Variable | ATN SARPs Subvolume 5 | ATN Network Addressing (Sub-) Domain Authority defined in ADM field |
| N-SEL | 1 | 00 - ff | Network Entity or Network Service User | fe (Network entity implementing optional non-use of IDRP) 00 (all other NEs) Remaining values not defined | Fixed Fixed Variable | ATN SARPs Subvolume 5 | ATN SARPs Subvolume 5/ ATN Network Addressing (Sub-) Domain Authority defined in ADM field |

Table 4.2.1-1:  Package 1 ATN NSAP Address Definition

Of the nine address fields of an ATN NSAP address, four already have fixed values allocated by the ATN SARPs Subvolume 5. The values of the remaining five fields, marked as variable in Table 4.2.1-1, may be allocated as follows:

| AFI | IDI | VER | ADM | RDF | ARS | LOC | SYS | N-SEL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Figure 4.2.1-1:  ATN NSAP Format

ADM	The value of the ADM field, combined with the values of the preceeding fields, determines an ATN NSAP address sub-domain.  Each ICAO Member State, airline or other organisation participating in the ATN should be responsible for exactly one such addressing sub-domain and perform the address allocation and establishment of subordinate registration authorities, if required, within this responsibility.

In the case of an ATSC addressing domain (i.e. VER field value is either 81 or c1) the ADM field contains the ISO country code, as defined in ISO 3166, of the ICAO Member State responsible for the relevant addressing sub-domains which are identified by the address prefixes 47 0027 81 <3-digit ISO Country Code> and 47 0027 c1 <3-digit ISO Country Code>.

If the case of an AINSC addressing domain (i.e. VER field value is either 01 or 41), IATA is the registration authority responsible for the associated addressing subdomains which are identified by the address prefixes 47 0027 01 and 47 0027 41. In this case, IATA will assign ADM field values according to the procedures outlined in section 5.2.

ARS	The value of the ARS field, when combined with the values of the preceding fields, determines the system’s ATN routing domain.  Each ICAO Member State, airline or other organisation identified by the value in the ADM field will be responsible for establishing one or more ATN routing domains according to their local addressing and routing requirements.

If the VER field indicates a mobile routing domain, the ARS field value shall be the hexadecimal-encoded value of the aircraft’s ICAO 24-bit aircraft address.  In this case, the ATN systems onboard an aircraft comprise one single routing domain.

If the VER field indicates a fixed routing domain and an ATSC address format, the ARS field may contain either the ICAO location identifier or other meaningful indication (e.g. the flight information region indicator) of the routing domain.

If the VER field indicates a fixed routing domain and an AINSC address format, the ARS field may contain the IATA location identifier or other meaningful indication of the routing domain.

LOC	The value of the LOC field is under the responsibility of the organisation identified in the ADM field and should be assigned a meaningful value within the context of the administrative region identified in the ARS field.  This field may be used to subdivide large routing domains into routing areas according to local needs.

SYS	The value of the SYS field is under the responsibility of the organisation identified in the ADM field and should be assigned a meaningful value within the context of the administrative region and location identified in the ARS and LOC fields.  This field may be used to identify a specific ATN system within a given routing domain.

N-SEL	The value of the N-SEL field is currently under the responsibility of the organisation identified in the ADM field.  The N-SEL field is used to identify either a network layer entity (such as the IDRP protocol machine) or a network service user within the context of a given ATN system.

### 4.2.2	The Network Entity Title

A NET is the name of a network layer entity in any given ATN system. If an ATN system has exactly one network layer entity, the NET in fact ”names” the entire system.  Furthermore, as ATN NETs are derived from the ATN addressing plan, the NET may also be used as an ATN address to locate a given system, or as a source from which to derive address prefixes of all systems within a given routing domain (e.g. during routing initiation).

For internetwork addressing consistency within the ATN, the NET and all NSAPs in use on a given system should only differ by the value of the N-SEL field per Table 4.2.1-1.

### 4.2.3	Relationship To ATN Routing

In order to support the ATN routing concept, the ATN NSAP addressing plan must be respected, especially with regards to the registration and use of ADM and ARS values.  As the NSAP addressing plan is based on the expected ATN topology, it has been defined hierarchically allowing routing information to be managed efficiently.  Specifically, address prefixes comprising of simply an administrative domain identifier or routing domain address may be used to describe routes to all systems within a given domain.  For example, the IDRP protocol may be used to indicate that a given router has routes to all systems in a given administration or airline rather than identifying every system individually.

During the Package 1 timeframe, IDRP may not be supported over the mobile subnetworks by all aircraft routers.  The N-SEL field of the ATN NSAP addressing plan is currently used to indicate the IDRP capability of the aircraft router.  In the case of a non-IDRP equipped aircraft the routing initiation process will ensure that both the air/ground and aircraft routers will have enough information about each other and their corresponding communications capabilities.  The air/ground router can derive the routing domain address of the aircraft from the received NET of the aircraft router and forward it  to other domains requiring this information.  On the other hand, the aircraft router must have some pre-configured knowledge of the reachability of destination systems via a given air/ground router as there may not be any means of dynamically adverting route information to all aircraft.

### 4.2.4	Minimising The Impact of Topology Changes and Outages

From time to time, the ATN internetwork topology may change or a system outage may occur which effectively changes the internetwork topology.  Because the ATN is expected to be used for air traffic management applications, operational availability must be very high. Consequently, the effects of ATN internetwork topology changes should be transparent to ATN users or must be minimised in order to avoid losing vital communications paths.

There are two aspects to be considered: Planned or unforeseen changes.

Planned Changes

If a system is moved to another routing area or another routing domain or if routing areas or routing domains are reconfigured, new NSAP addresses or NETs must be assigned. The operator of any system known to need to communicate with the system whose address will change must be informed well in advance. A period of time in which a system maintains two or more NSAP addresses for the same NSAP may also ease the transition.  If this is the case, IDRP-equipped routers may broadcast the reachability of a given system by both its old and new address for a limited time, thus minimising the overall impact of the change.

Reconfiguration of the network topology inside a routing area or inside a routing domain, which is not partitioned into routing areas, should be made transparent to ATN users, i.e. the NSAP address should not be changed.

Unforeseen Changes

Unforeseen changes may occur due to failure of a given end system or router. If an end system fails, a standby or backup system must be available to avoid loss of communication. If a router fails, either a standby or backup router or a redundant path may guarantee operational availability of the ATN. In the latter case, the problem is covered by routing procedures which re-route packets through alternative paths.

If a stand-by end system or router replaces the failed system, the replacement system should effectively adopt the NSAP address or NET of the failed system and the local routing tables should be updated to indicate the local topology change.  Thus the availability of ATN services can be ensured and made transparent to ATN users; the effects of the failure are not be propagated outside of the local domain.

## 4.3	Transport, Upper Layer and Application Addressing

According to ISO 7498-3, a TSAP, SSAP and PSAP address is composed of a globally unique NSAP address plus a local transport selector (T-SEL), session selector (S-SEL) and presentation selector respectively, as illustrated in Figure 4.3-1. This means that the ATN NSAP, TSAP and SSAP address of a given system can be derived from the address of an application, i.e. PSAP address, hosted by this system by cutting off the relevant selectors. In the same way the TSAP, SSAP and application addresses within a system can be constructed from the NSAP address using a priori knowledge of the local selector values.

Figure 4.3-1:   Relationship of Network Addresses and Upper Layer/Application Addresses within a given ATN End System

For Package 1 the ATN address formats shown in Figure 4.3-1 apply to AMHS Servers, AMHS User Agents and AFTN-AHMS Gateways as specified in [14], whereas for Package 1 datalink applications, AIDC applications and the AFTN/ATN Type A Gateway the session selector (S-SEL) and presentation selector (P-SEL) will be null. This means that the address of these latter group of ATN applications has exactly the same form as the TSAP address. As a consequence, the TSAP address is sufficient to unambiguously identify and locate ATN datalink applications, AIDC applications and AFTN/ATN Type A Gateways in the frame of the CNS/ATM-1 package.

The format of the application address, i.e. PSAP address, of an ATN datalink application, an AIDC application and an AFTN/ATN Type A Gateway is illustrated in Figure 4.3-2.

Figure 4.3-2:  ATN Application Address Format for Package-1 Datalink Applications, AIDC Applications and AFTN/ATN Type A Gateways

If there is a requirement to use a single transport connection for multiple applications whenever possible, an ATN presentation address must consist the host system’s NSAP, TSEL and at least one additional selector to identify a specific application using a given TSAP.

## 4.4	Application Naming

Application names are generally referred to as AP-titles and must be globally unique within the ATN context. The abstract syntax of AP-titles (and also of AE-qualifiers and AE-titles) is defined in [8650]. It defines two syntactic forms of each type of name: an object identifier (OID) form and a directory name form.

Following ISO naming principals [7498-3] consistency of all application names used in the ATN can be ensured by assigning them according to a well-defined ATN application naming tree under the ultimate authority of ISO. Figure 4.4-1 illustrates the ATN application naming tree for Package 1 applications in the object identifier form as defined in [10].

Figure 4.4-1:  OID Form of the ATN Application Naming Tree

### 4.4.1	Object Identifier Form of Application Names

The object identifier form of an AP-title and AE-title respectively is the sequence of all object identifier components in the order of the assignment by the individual registration authorities. An object identifier component is an integer value, together with an (optional) component name.

ISO/IEC 8824 specifies the top of this hierarchical name space. It has the following three entries, i.e. object identifier components, in its register:

| Component Name | Integer Value |
| --- | --- |
| ccitt | 0 |
| iso | 1 |
| joint-iso-ccitt | 2 |

The iso name space has been selected as the root of the ATN application naming tree. This name space is further subdivided by ISO/IEC 8824 into:

| Component Name | Integer Value |
| --- | --- |
| standard | 0 |
| registration-authority | 1 |
| member-body | 2 |
| identified-organisation | 3 |

ICAO has requested the allocation of an International Code Designator (ICD) and has obtained the ICD 0027 according to the dictates of ISO 6523. This corresponds to the object identifier (OID) "icao (27)", where "icao" represents the component name and "27" the integer value. This assignment allows ICAO to establish its own name space within the name space associated with the object identifier "identified-organisation (3)" and to assign subauthorities as needed.

Subvolume 4 of the ATN SARPs is currently the registration authority for this name space. It has 4 entries, i.e. object identifier components, in its register at the highest level:

| Component Name | Integer Value |
| --- | --- |
| atn | 0 |
| atn-end-system-air | 1 |
| atn-end-system-ground | 2 |
| atn-ac | 3 |

Under the "atn-end-system-air (1)" name space, Subvolume 4 of the ATN SARPs specifies an object identifier component whose values correpond to the IACO 24-bit aircraft address. Likewise, under the "atn-end-system-ground (2)" name space Subvolume 4 specifies an arc whose values correspond to the ICAO Facility Designator.

According to Subvolume 4, the ATN AP-titles are allocated underneath either of these object identifiers. For the Package 1, the object identifier component "ops (0)" has been assigned by Subvolume 4.

Examples

Referring to the ATN naming tree illustrated in Figure 4.4-1, the object identifier form of an AP-title of an application process hosted by a mobile ATN endsystem is:

{1 3 27 1 n 0} where n is the integer value of the ICAO 24-bit aircraft address of the aircraft carrying the ATN endsystem which hosts the AP.

The object identifier form of an AP-title of an application process hosted by a ground ATN endsystem is:

{1 3 27 2 n 0} where n is the integer value corresponding to the ICAO Facility Designator of the location of the ATN endsystem which hosts the AP.

Subvolume 4 assigns object identifiers to ATN application entities (AEs) along the type of the ATN application service entity (ASE). Eight entries are defined in the register for ATN AE-qualifiers. These are:

| ASE Type | ATN AE-Qualifier Name | ATN AE-Qualifier Integer Value |
| --- | --- | --- |
| Automatic Dependen Surveillance | ADS | 0 |
| Context Management Application | CMA | 1 |
| Controller Pilot Data Link Communication | CPC | 2 |
| Automatic Terminal Information Services | ATI | 3 |
| Type A Gateway | GWA | 4 |
| Systems Management Application | SMA | 5 |
| ATS Inter-Facility Data Communications | IDC | 6 |
| ATS Message Server | AMS | 7 |
| AFTN-AMHS Gateway | GWB | 8 |
| ATS Message User Agent | AUA | 9 |

Table 4.4-1:  ATN Package 1 AE-Qualifier Names and Values

The object identifier form of an AE-title is constructed by appending the object identifier form of the AE-qualifier (in its integer value form) to the sequence of object identifier components that comprise the object identifier of the associated AP-title.

In the ATN application naming tree of Figure 4.4-1 the object identifier values for the ATN AE-qualifiers are illustrated beneath the object identifier for the ATN AP-titles. Consequently, the complete sequence of object identifier components from the root of the ATN application naming tree to the last level of the hierarchy provides the object identifier form of an ATN AE-title.

Example

The object identifier form of the AE-title of the CPDLC AE contained in an AP hosted by a mobile ATN endsystem is:

{1 3 27 1 n 0 2} where n is the integer value of the ICAO 24-bit aircraft address of the aircraft carrying the ATN endsystem which hosts the given AP and the last digit, i.e. 2, is the AE-qualifier.

### 4.4.2	Directory Name Form of Application Names

Whereas the OID form of AP-titles, AE-titles and AE-qualifiers has some benefits in terms of efficiency when passing naming information in the protocol control infomation (PCI) field of ATN application protocols, the directory name form provides a much more user-friendly representation of application names.

Once established, such a naming scheme can be used as a directory scheme, when directory services shall be provided to ATN users. A directory scheme is a set of rules and constraints concerning the Directory Information Tree (DIT) structure and its contents. These rules ensure that the contents are valid entries and contain valid information. They define, what the user can do with entries and their contents.

Note: Even if no OSI Directory Service is intended to be used within the Package 1 timeframe, it is highly recommended to use the principles described in the OSI Directory standard.

The directory name form of an AP-title is the sequence of all relative distinguished names (RDNs) in the order of the assignment by the individual registration authorities. This ordered sequence of RDNs is referred to as Distinguished Name. A RDN consists of two elements: an attribute type and an attribute value. The attribute type is an Object Identifier characterising the name field and the attribute value is an IA5 string for the name itself.

The directory name form of an AE-qualifier is a relative distinguished name, which is unambiguous within the scope of the associated application process.

The directory name form of an AE-title is constructed by appending the relative distinguished name form of the AE-qualifier to the directory name of the associated AP-title.

Currently no AP-titles and AE-qualifers in the form of directory names or relative distinguished names are assigned to ATN APs and AEs by the ATN SARPs.

Figure 4.4.2-1 shows a proposed ATN Application Naming Tree structure for ATN application processes and application entities following the DIT approach.

§

Figure 4.4.2-1: DIT Form of Proposed ATN Application Naming Tree

The ATN DIT of Figure 4.4.2-1 contains the following object classes:

Organisation (O): This object class is used at the first level to identify either ICAO or IATA as the international organisation, which is direct subordinate under the root.

Administration (ADM): This object class may be used to identify either ATSC administrations (e.g. national CAA, Eurocontrol etc.) or AINSC organisations (e.g. airlines).

Country (C): The country object class may be used instead of the administration object class to identify national CAAs. It shall contain the three letter ISO country code.

Aircraft Operator (AO): This object class shall be used to identify the aircraft operator in the case of IATA aircraft.

Aircraft Identifier (AI): This object class shall be used for all aircraft applications or other names to identify objects on board of aircraft.

ATC Centre (ATCC): This object class shall contain the ATCC name for all ATSC applications or other names to identify objects located in an ATCC.

Location (L): This object class shall identify the location of a ground located AINSC application.

Sector (SE): It may be useful to distinguish between different control sectors in an ATCC. This object class may be used to identify a control sector.

Application Process (AP) and Application Entity (AE): These object classes shall be used to identify the type of the ground or aircraft ATSC or AINSC application. The AE object class is always subordinate of the AP object class, while the AP object class may be subordinate to Sector, ATC Centre, Location or Aircraft Identifier.

Note: Besides APs and AEs other object classes may be added to the proposed DIT of Figure 4.4.2-1 in the future, if other name types need to be standardised, e.g. end system or server names, user or user group names etc.

#### 4.4.2.1	Ground-Based ATM Applications

The following are examples for ground based ATM application names:

Note: AE types have not been identified yet. Therefore, only AP names are listed in the following examples.

1. Example for an Automatic Dependent Surveillance (ADS) application running in sector "SR 1" in the ATCC Munich:

O = 	ICAO

ADM =	DFS

ATCC =	Munich

SE =	SR 1

AP =	ADS

2. Same example as above, but using the ISO country code instead of an administration identifier:

O = 	ICAO

C =	DE

ATCC =	Munich

SE =	SR 1

AP =	ADS

#### 4.4.2.2	Aircraft ATM Applications

Example for a Controller Pilot Communications (CPC) operated in a Lufthansa aircraft:

O = 	IATA

AO =	LH

AI =	46721005

AP =	CPC

#### 4.4.2.3	Other Ground-Based Applications

Example for a Flight Information Service (FIS) application operated by Lufthansa operational centre in Munich:

O = 	IATA

ADM =	Lufthansa

L =	Munich

AP =	FIS

## 4.5	Name and Address Relationship in Package 1

To date very little effort has been focused on the relationship between ATN application names and corresponding addresses outside of the scope of the Package-1 ATN air/ground applications, where the name-address-resolvement is achieved through the CM function [12]. For distributed air/ground applications such as ADS and CPC, a basic name-to-address resolver, CM, has already been defined.  CM is a critical function allowing systems onboard an aircraft and within a given ATC facility (or FIR) to identify their service capabilities (applications) and their corresponding addresses to each other.  It should be noted however that, as demonstrated in Chapter 6, CM provides limited capabilities in a limited context.

Based on the Package 1 upper layer architecture and the expected limited deployment of ATN systems during the Package 1 timeframe, the mapping between ground application names and corresponding addresses may be resolved a priori, i.e. based on static information configured into the ground ATN systems. This assumption is not unreasonable, especially for applications such as AIDC which will likely have few address and name changes.  However, the scalability of the ATN may be limited by requiring that all ATN applications individually maintain addressing information about all other applications with which they may need to communicate.  In the longer term, ICAO may pursue the definition of a naming tree allowing the development of ATN directory services to support the potential growth of ATN.

## 4.6	ATN Name and Address Cross-Reference

This section provides an overview of the individual components of ATN names and addresses, and gives reference to the detailed specification of these addressing/naming components in terms of syntax, semantics, ecoding and value range. References are provided with respect to both the ATN SARPs and relevant ISO/IEC standards.

|  |  |  | ATN | System | Providing |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Automatic Dependent Surveillance | Controller Pilot Data Link Com-munications | Automatic Terminal Information Services | Context Management | ATS Inter-Facility Data Communications | Type A Gateway | ATS Message Server | ATS Message User Agent | AFTN-ATN Gateway |
| AE-Qualifier | ADS | CPC | ATI | CMA | IDC | GWA | AMS | GWB | AUA |
| AP-Title |  |  | Specified in | ATN SARPs | Subvolume 4 | Section 3.2.2 |  |  |  |
| P-SEL |  |  | Null |  |  |  | Specified in | ISO/IEC 11188-1 | Section 7.2 |
| S-SEL |  |  | Null |  |  |  | Specified in | ISO/IEC 11188-1 | Section 9.3 |
| T-SEL |  |  | Specified in | ATN SARPs | Subvolume 5 | Section 5.4 |  |  |  |
| NSAP |  |  | Specified in | ATN SARPs | Subvolume 5 | Section 5.4 |  |  |  |
| SNPA |  |  | Specified in | ICAO Annex 10 | and relevant | ISO/IEC Standards |  |  |  |

Table 4.6-1: Overview of ATN Addresses and Names

# 5	ATN Name and Address Registration

Name/address registration is the mechanism through which a name/address is assigned to an object in a way which makes the assignment available to interested parties. It is carried out by a naming/addressing registration authority (called registration authority in the following).

## 5.1	Registration Authority

A registration authority may be an organisation, a document, an automated facility, or any other body capable of name/address assignment that performs registration of one or more types of objects within its jurisdiction. The set of name/address values administered by a single registration authority consitutes the naming/addressing domain of this authority.

Note - A registration authority is a specific instance of a naming/addressing authority (see section 2.3). A naming/addressing authority defines formats and/or values of names/addresses and assigns such values to entities.

Note - According to this definition, the ATN SARPs constitute an international naming/addressing authority, as they define formats and values of ATN names and addresses (or components of those respectively) for objects such as ATN network layer entities or ATN application processes, at the international level.

The role of a name/address registration authority is according to [9834-1]:

r	to assign and make available unambiguous names and addresses

r	to record definitions of the objects to which names and addresses are assigned.

## 5.2	General Procedures for Registration of ATN Objects

With regard to the initial assignment of names, addresses and definitions to objects and of subsequent additions to the register, each registration authority for ATN names/addresses should:

a)	receive proposals for register entries from applicants

b)	process received proposals for register entries, i.e. effect any necessary rationalisation or coordination of these proposals, such as check of responsibility of registration authority, verfication of the qualification of the applicant, verification of the compliance of the proposal with the relevant provisions in the ATN SARPs)

Note - There is a need to specify the method used to determine whether a request for registration or deletion is accepted or refused

c)	record names/addresses for each register entry that is accepted in accordance with the specified procedures for a register entry

Note - The contents of a register entry should include at least

-	the name/address assiogned to the object

-	the name of the organisation that proposed the entry

-	the dates of submission and registration respectively

-	the definition of the registered object.

d)	convey the results of the registration, i.e. the decision taken on the proposal, to the appropriate applicant, and

e)	promulgate the register entries within its sphere of responsibility according to specified procedures and in a specified form.

The procedures a) to e) should also apply with regard to deletions from the register, with item c) replaced by

c)	delete the relevant names/addresses from the register for each proposal that is accepted.

A user of a registration authority may request an allocation of names/addresses from a registration authority, leaving the choice of names to the registration authority. Alternatively, a user of a registration authority may request an allocation of particular names/addresses. The registration authority may grant that request if it chooses, provided the names/addresses have not previously been issued.

The use of a name/address can be terminated by a registration authority and then the name/address re-used at a later time. The precise rules and constraints related to the re-use of a name/address to ensure unambiguity are within the responsibility of the registration authority.

## 5.3	Internetwork Address Administration and Registration

### 5.3.1	Administration

ICAO is the administrative authority of the ATN internetwork addressing plan and administers this plan through Subvolume 5 of the ATN SARPs. Subvolume 5 defines and administers the ATN NSAP address syntax (i.e. field boundaries, field sizes and field formats), the ATN NSAP address semantics (i.e. the field content and interpretation), and the ATN NSAP address encoding procedures (i.e. the representation of the abstract field syntax and semantics). Subvolume 5 delegates authority for the definition of the semantic content and the encoding of particular fields as well as the assignment of address values for these fields to other State, aeronautical or telecommunication standards authorities.

### 5.3.2	Registration

Four of the nine address fields of an ATN NSAP address have been assigned fixed values by Subvolume 5 [11] and, consequently, do not need further registration. The values of the remaining five fields should be registered as follows:

ADM-Field

The ADM field shall be assigned a value representing either the IATA or ICAO organisation (depending on the value of the VER field) which is responsible for the identified ATN Administrative Domain.  This will effectively define the ATN administrative domain identifier.

In the case that it is an ATSC address, the ADM field value will be derived from the ISO country designator set defined in [3166] according to the encoding procedures for this field defined in [11]. The registration process for the ADM field thus is limited to the establishment of a repository listing all valid ADS field values.

In the case that it is an AINSC address, the ADM field value should be an IATA 3-letter code for airlines (e.g. the ”airline designator”) and other stakeholders.  IATA has already proposed a registration procedure through their Geneva offices based on the use of current alphanumeric ”airline designators” with extensions for ”other stakeholders” compatible with the IATA Passenger Services.

One (or more) registration document(s) should be established and maintained, where values for the ADM field of the ATN NSAP address will be registered and published. This could be similar to the ISO 6523 standard document, where International Code Designators (ICD) are listed for addressing authorities registered with ISO (e.g. the value "0027" for ICAO can be found there). The registration document for ADM values may for instance contain tables with the following layout:

| ICAO member / ATSC Administration | ISO Country Code (3 letter code) | ADM value (hexadecimal) |
| --- | --- | --- |
| ... |  |  |
| Finland | FIN | 46 49 4E |
| France | FRA | 46 52 41 |
| Germany | DEU | 44 45 55 |
| ... |  |  |

| IATA member / AISC Administration | IATA Airline Code (3 letter code) | ADM value (hexadecimal) |
| --- | --- | --- |
| ... |  |  |
| Air France | AFR | 41 46 52 |
| British Airways | BAE | 42 41 45 |
| Lufthansa | DLH | 44 4C 48 |
| South African Airlines | SAL | 53 41 4C |
| United Airlines | UAL | 55 41 4C |
| ... |  |  |

ARS-Field

ARS values should be assigned, administered and registered by the authority designated in the ADM field. A guideline for registration procedures and an outline for a registration document may be established by ICAO and/or IATA as the parent registration authorities.

Note - Administrations may decide to derive the ARS value for ground addresses from the IA-5 encoding of the IATA or ICAO location identifier [11].

Note - During the initial stage of Package-1, i.e. until appropriate registration authorities have been established by States, registration of ARS field values under the address space of ICAO will be within the responsibility of ATNP WG2 [13].

Note - A common ICAO/IATA database comprising all registered NSAP address prefixes including the ARS value may be maintained and published through the appropriate ICAO and IATA channels.  This database would allow to identify all registered ATN routing domains and the organisations responsible for each one.  This database may be the basis of generalised directory services for post-Package 1 ATN deployments.

LOC-, SYS-, N-SEL-Fields

Values for the LOC, SYS and N-SEL fields will be registered locally by a State’s authority, or airline’s or other aeronautical stakeholder’s registration authority. In general, there is little need for ATN-wide coordination and publication of these values.  However, in order to support ATN directory services in the long term, the registered NSAP addresses of ATSC applications should be recorded and maintained in a possibly distributed ATN directory under the auspices of ICAO.

## 5.4	Transport Address Administration and Registration

According to the hierarchical structure of ATN addresses, only the TSAP selector of the ATN address has to be administered and assigned in the process of defining and registering ATN transport addresses, i.e. ATN TSAP addresses.

Note - To register a TSAP address the underlying NSAP address must previously have been registered.

### 5.4.1	Administration

According to [11], the TSAP selector of an ATN Transport address is administered on a local basis. This means that TSAP selector values are assigned by the organisation responsible for a given ATN End System within the constraints defined in [11].

### 5.4.2	Registration

The value for the TSAP selector field will be registered locally by a State’s authority, or airline’s or other aeronautical stakeholder’s registration authority. In general, there is little need for ATN-wide coordination and publication of registered TSAP addresses.  However, in order to support ATN directory services in the long term, the registered TSAP addresses of ATSC applications should be recorded and maintained in a possibly distributed ATN directory under the auspices of ICAO.

Note - The recording and global publication of the TSAP addresses associated with ground CM applications is a requirement for Package-1.

## 5.5	Application Name Administration and Registration

ATN AP-titles should be registered based on the registration procedures in [9834-1] and [9834-6]. [9834-6] defines rules applicable to the registration of APs and AEs by internationally recognized organisations. As ICAO has been assigned an ICD, these rules are applicable for the assignment of ATN AP-titles and ATN AE-titles by ICAO. Consequently, ICAO is the international registration authority for the ATSC application names and exercises this authority in the form of the Subvolume 4 ATN SARPs. Likewise, IATA should act as the international registration authority for the naming of AINSC applications.

The registration responsibilities for application naming may be further delegated according to the administrative taxonomy of the ICAO and IATA organisations.

Note - Registration of Package-1 application names, in particular the AE-qualifier, will be performed under the responsibility of ATNP WG3 SG3, as defined in [13].

Both the object identifier form and directory name form described in [9834-1] may be registered, especially when considering the potential use of OSI directory services for the ATN in the longer run.

As both forms relate to the same naming tree, either nomenclature could be used to identify a given application.

Note - According to ISO/IEC 9834-6, a registration authority shall assign both an object identifier form and a directory name form of an AP-title or AE-qualifier when registering an AP or AE respectively. To be compliant which ISO/IEC 9834-6, the ATN SARPs have to define the directory names of ATN APs and AEs within their jurisdiction.

### 5.5.1	AP Registration

An AP is identified by an AP-title. The registration of an AP involves the assignment of an AP-title (in both an object identifier form and a directory name form) and the placement of these names (together with additional information, as appropriate) as the AP register entry for this AP.

In general, the assignment of the object identifier form of an AP-title involves the assignment of one (or more) object identifier components, which in combination with object identifier components assigned by superior registration authorities, form the object identifier for the AP-title. Likewise, the assignment of the directory name form of an AP-title involves the assignment of a relative distinguished name, which in combination with the set of relative distinguished names assigned by superior registration authorities, form the directory name for the AP-title.

Note - Subvolume 4 currently assigns only object identifiers to ATN AP-titles.

### 5.5.2	AE Registration

An AE is identified by an AE-title. As an AE-title consists of an AP-title and an AE-qualifier, the registration of an AE involves the assignment of an AE-qualifier only, if the AP that contains the AE has been previously registered. The AE-title can then be generated by a combination of the AP-title and the AE-qualifier. The registration authority assigns for the AE-qualifer an object identifier component (by assigning an integer) and a relative distinguished name which are unambiguous within the scope of the associated AP and places these names together with the previously registered AP-title (and additional information, as appropriate) as the AE register entry for this AE.

Subvolume 4 of the ATN SARPs is currently the registration authority for ATN AE-qualifiers associated with ATSC air/ground applications.

Note - Subvolume 4 assigns only object identifiers to ATN AE-titles.

# 6	The Use of Context Management as Basic Directory Service

This section introduces the use of the ATN naming and addressing concept for ATSC applications distributed between aircraft and ground-based systems.  Further and more detailed information about the context management application can be found in [5].

Within the Package 1 timeframe, the CM application will provide a basic directory service which provides the following capabilities:

1.	The ground-based CM application may determine the ATSC application capabilities of a given aircraft and determine the addresses of relevant aircraft ATSC applications.

2.	The aircraft CM applications may determine the ATSC application capabilities within the scope of the ground-based CM application (which may be for example a given FIR) and may determine the addresses of relevant ground ATSC applications.

3.	The ground-based CM function may advertise the update of relevant ground based ATSC application addresses to the aircraft CM application.

4.	The ground-based CM function may direct its aircraft peer CM to contact another ground-based CM application, perhaps in another FIR.

Once an aircraft has ”logged on” (see [5]) using CM the onboard systems are able to determine the application addresses of ATSC applications available from the logged-on ground CMA. The ground CMA may for example provide address information about all ”accessible” ATSC applications within an ATCC.  Correspondingly, the local control region’s ATCC will know the aircraft’s ATSC application capabilities and their respective ATN application addresses and names at completion of logon.  The logon process can be described as follows:

1.	Assuming one does not already exist, the aircraft’s CMA establishes a transport connection with the CMA in the control region currently responsible for the aircraft.  To do so, the aircraft is required to know which control region it is in and the corresponding address of the CMA application.

Note:  In the absence of generalised directory services, it may be worthwhile to establish default CMA application addresses within each ATN equipped control region.  This would simply mean defining, at the ICAO level, fixed ARS, LOC, SYS, NSEL, TSEL for the ground CM applications.  The RDI of a given control region can be derived from the name of the control region, presumably already known by the aircraft.  The aircraft CMA would then append the predefined values of the above-listed fields to the RDI identifier to produce the required CMA address.

2.	The aircraft CMA sends a CMLogonRequest message indicating the aircraft flight identification (its flight number), and an associative list of application names and addresses.  The application names are in fact currently defined to simply be 3-character Application IDs (from §5.3.2.2) It is assumed that the ATCC can derive the full application name from the associated addressing information and flight identification.  At this point, the ATCC will be able to associate the aircraft, identified in the flight plan by its flight identification, to the supported ATM applications and their corresponding addresses.

3.	If the ground CMA is able to support the logon request, it sends a CMLogonResponse message indicating that the aircraft logon request has been accepted along with an associative list of applications and their associated application addresses similar to the aircraft’s list. At this point, the aircraft will be able to associate the ATM applications supported by the ATCC with their corresponding applications addresses.

Following the logon procedure, peer application names and addresses can be distributed within the aircraft and ATCC domains by local means.  At this point, ATM applications may now communicate between the aircraft and ATCC.  If, for some reason, the ground application addresses change, the ATCC may send an updated associative name/address list to the aircraft.  Additionally, to support transfer of control to the aircraft’s next control region, the ATCC may send a ContactRequest message to the aircraft CMA indicating the next control centre’s CMA application address.

## 6.1	Resolvement of Address/Naming Information Provided By CMA

Following the establishment of the application context between an aircraft CMA and a ground-based CMA, the aircraft CMA provides pairs of information comprising

r 	an identifier for the type of ATSC application available onboard of the aircraft, and

r 	address fragments of the associated application.

The address fragments contain either the values of the VER, ADM, RDF, LOC, SYS and NSAP SEL and TSAP SEL fields of the ATN TSAP address (referred as ”long TSAP” in [12]) or the values of the LOC, SYS and NSAP SEL and TSAP SEL fields of the ATN TSAP address (referred as ”short TSAP” in [12]).

The full PSAP address of the associated aircraft application is generated by the ground by adding the fixed 3-bytes Initial Domain Part (IDP) of the ATN PSAP address, i.e. 47 00 27, as prefix to the received ”long TSAP” and by inserting the ICAO 24-bit aircraft address between the received VER and RDF field values, as illustrated in Figure 6.1-1.

Note - To restore the full PSAP address of the aircraft application,the ICAO 24-bit aircraft address must be known to the ground through a priori knowledge. This a priori knowledge may be, for example, information contained in the flight plan.

Note - The”long TSAP” structure defined in [12] may be reduced by 1 byte, if the RDF field value is omitted. This field has the fixed value ”00” which can be restored on the ground and inserted within the received TSAP fragments.

The full PSAP address of the associated aircraft application can be generated by the ground from a received ”short TSAP” fragment only if a long TSAP fragment has been received previously. In this case, the VER and ADM field values are restored from the last received ”long TSAP” fragment and added together with the fixed IDP (47 00 27) and the fixed ARS field value (ICAO 24-bit aircraft address) to the received ”short TSAP” fragment.

From the received aircraft application type identifier the AE-qualifier of the associated application can be derived on the ground by a simple mapping procedure.

Note - The application type identifiers for Package-1 still have to be specified by [12].

By adding the ICAO 24-bit aircraft address as prefix to the retrieved AE-qualifier, the full AP-title of the aircraft application can be generated on the ground.

Consequently, the ground CMA can completely resolve the addresses and names of the aircraft applications from the received CM information, following the procedures described above.

Figure 6.1-1: Generation of ATN Application Address From Received CM Information

Figure 6.1-2: Generation of ATN AE-Title From Received CM Information

On request of the aircraft CMA or triggered by a change in its local information repository, the ground-based CMA transmits to the aircraft CMA information pairs comprising:

r 	an identifier for the type of ATSC application available on the ground, and

r 	address fragments of the associated application.

The name and address resolution can be performed onboard the aircraft in the same way as described above and illustrated in Figure 6.1-1.

The full AE-title of the ground applications is generated by concatenating the ICAO Facility Designator of the ground CMA to the AE-qualifier derived from the transmitted information concerning the application type.

The full PSAP address of the ground applications is restored from a priori knowledge about the PSAP IDP prefix (which is constant ”47 00 27”) and the received ”long or short TSAP” fragments respectively, as described above. However it is currently undefined from which information the ARS field value of the PSAP address of the ground application is restored. It may be derived from the destination TSAP address used by the aircraft system when establishing the initial transport connection with the ground CMA server. However, such an approach would limit the scope of the address information provided by the ground CMA. As the ADS field value of the CMA is used to construct the PSAP address of the ground ATSC applications, these applications must be hosted by end systems which are located in the same routing domain as the ground CMA server.

According to ISO 7498-3, network layer entities are both named and located by their NET.

Subvolume 5 of the ATN SARPs currently defines ICAO and IATA as the registration authorities for this address field, but allows further delegation to State authorities and aeronautical industry organisation. It is recommended that the ARS field values for ATSC addresses are registered at the State level in order to relieve ICAO from establishing and operating an own entity and procedures for ATN address registration. Until appropriate registration authorities have been established by States, initial registration of ARS field values for ATSC addresses will be the responsibility of ATNP WG2 [13].

An applicant may be an ATS authority, a supra-national ATS organisation, an airline, an aircraft operator, an aeronautical communication service provider or any other aeronautical industry organisation.