Changes between “Old Sub-Volume 1” version 0.10.2 (0800) and Baseline Version 1.0.

Recommend Baseline “Old Sub-Volume” as 1.0.

Aeronautical industry.  The aeronautical industry comprises aircraft operators, services providers, aircraft and engine manufacturers, suppliers, and other industry parties participating in the implementation of systems and services supporting the use of the civil air transportation system and distinguished from the civil aviation and national authorities.

Air traffic flow management (ATFM).  The objective of the ATFM is to ensure an optimum flow of air traffic to or through areas within which traffic demand exceeds the available capacity of the ATC system.

AMHS.  The symbol used to designate ATS message service.

ATSC class.  The ATSC class parameter enables the ATSSC user to specify the quality of service expected for the offered data.  The ATSC class value is specified in terms of ATN end-to-end transit delay at 95% probability.

ATS communication (ATSC).  Communication related to air traffic services including air traffic control, aeronautical and meteorological information, position reporting and services related to safety and regularity of flight.  This communication involves one or more air traffic service administrations.  This term is used for purposes of address administration.

ATS interfacility data communication (AIDC).  An ATN application dedicated to exchanges between ATS units (ATSUs) of critical tactical air traffic control (ATC) information in support of flight notification, flight coordination, transfer of control, transfer of communication, transfer of surveillance data and transfer of general data.

ATS message.  A unit of user-data, coded in binary form, which is conveyed from an originator of the data to one or more recipients of the data.  It is possible to associate a unique message identifier and a priority with each ATS message.

ATS message handling service (ATSMHS).  Procedures used to exchange ATS messages over the ATN such that the conveyance of an ATS message is in general not correlated with the conveyance of another ATS message by the service provider.  There are two ATS message handling services.  They are the ATS message service and the ATN pass-through service.

ATS message service (AMHS).  The set of computing and communication resources implemented by ATS organizations to provide the ATS message service.

CIDIN.  The symbol used to designate common ICAO data interchange network.

Common ICAO data interchange network (CIDIN).  An improved transmission medium for the aeronautical fixed telecommunications network (AFTN).

Inter-centre communication (ICC).  ICC is data communication between ATS units to support ATS, such as notification, coordination, transfer of control, flight planning, airspace management and air traffic flow management.

Naming plan.  A plan that provides common naming conventions and designations for the unambiguous identification of all end and intermediate systems in accordance with the rules prescribed in ISO/IEC 7498-3, ISO/IEC TR 10730 and ISO/IEC 9545.

Residual error rate (RER).  The ratio of messages mis-delivered, non-delivered or delivered with an error undetected by the system, to the total number of messages delivered to the system during a measurement period (adapted from ISO/IEC 8072).

Note.— For the ATN, detected mis-delivered and non-delivered messages are not included in the ratio.

CCITT Rec X.25 (1996).  Interface between DTE and DCE terminals Operating in Packet Mode.

ISO/IEC 8802–3:1990.  Carrier Sense Multiple Access with Collision Detect Access method and Physical Layer Spec-S.

Requirements

1.2.1  The aeronautical telecommunication network (ATN) shall provide communication services and application entities in support of:

a)	the delivery of air traffic services (ATS) to aircraft;

b)	the exchange of ATS information between ATS units; and

c)

1.2.4  Recommendation.— Civil aviation authorities should coordinate, with national authorities and the aeronatical industry, those implementation aspects of the ATN which will permit its world-wide safety, interoperability and efficient use, as appropriate.

Tables and Figures notes added see attached.