ATNP/WG 1

WP 6-
	26 July 1996

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL

Working Group 1

Halifax, Nova Scotia, Canada

30. 7. - 2. 8. 1996

Proposed Amendments for Subvolume I of the Draft ATN SARPs

Presented by K. Platz

Prepared by A. Herber

SUMMARY

This paper summarizes identified deviations between sub-volume 1 of the Draft ATN SARPs and the ATNP lexicon as agreed in Munich and proposes amendments for sub-volume 1.

1.	Background

During the review of the ATNP lexicon, a comparison between part I of the Draft ATN SARPs (Version 0.9) and the ATNP lexicon as agreed in Munich was made. The following differences were found:

| Definition in ATNP Lexicon, Munich Version | Definition in sub-volume 1, Version 0.9 |
| --- | --- |
| Fast Byte Efficiency Enhancements for the presentation and session layer of the OSI 7 layer reference model. These are achieved by minimal functionality in these two layers, each one applying a service mapping between the layer above it and the layer below it. | Fast byte. The capability at any layer of the OSI reference model to negotiate out the capabilities of the base protocol. |
| ATN Systems Management The ATN Systems Management provides mechanisms for monitoring, control and co-ordination of resources necessary to provide ATN services. ATN Systems Management is based on OSI System Management principles and may be distributed, centralized or local. | Systems management (SM). The set of functions related to the management of various OSI resources and their status across all layers of the OSI architecture. The monitoring, controlling, operating, supervising, coordination and administration of a communications network. |
| Operational Requirements This term refers to a set of requirements which define the operational needs and constraints within which a technical system has to operate in order to fulfil its anticipated role. The ATN operational requirements relate to ATN communications as seen from the user point of view. Operational requirements are composed of functional and non-functional requirements. | Operational requirement. A statement of the operational attributes of a system needed for the effective and/or efficient provision of air traffic services to users. |
| Application Software providing services to its users, in the guise of a consistent set of functionalities; example given, the ATC related functions implemented in the server(s) and/or controller work position host computers. (from EATCHIP Glossary of Terms / COPS/CWP Report) | Application. A function providing services to its users, for example, the ATS related functions implemented in the server(s) and/or controller work position host computers. |
| ATN Applications The term ATN applications refers to applications which support ATM or aeronautical industry functions and which are designed to operate across an OSI communications system. ATN applications are always distributed applications, i.e. peer processes are hosted by different end systems which are interconnected. | ATN application. Refers to an application that is designed to operate over the ATN internet. An ATN application is always a distributed data application, i.e. processes are hosted by different end systems which are interconnected. |
| System Level Requirements System Level Requirements are high-level technical requirements that relate to the system as a whole. System Level Requirements are formed from operational needs, safety objectives, technological and regulatory constraints (administrative and institutional). The requirements can be mapped to the technical requirements detailed in the Subvolumes 2 through 5 of the CNS/ATM-1 Package SARP’s. Examples of System Level Requirements are end-to-end performance and total system reliability. | System level requirement. The system level requirement is a high-level technical requirement that has been derived from operational requirements, technological constraints and regulatory constraints (administrative and institutional). The system-level requirements are the basis for the functional requirements, lower level requirements and derived requirements. |

2.	Recommendation

As agreed in Munich by WG1, the ATNP lexicon's definition should take precedence over other definitions. As conclusion, it is proposed to amend sub-volume 1 accordingly.