ATNP/WG1/WP- __
26/7/96

DRAFT FOR INFORMATION/COMMENT

Comments on and Proposed Amendments to “Sub Volume 1- Introduction and System Level Requirements, Version 0.7 June 19, 1996)

SUMMARY

Version 0.7 of the draft Sub Volume 1 SARPs were agreed at the 5th Meeting of WG1 in Munich. This paper includes comments and proposed amendments to the that document.

PROPOSAL

WG1 is invited to review, comment and, where considered appropriate,  approve the proposed amendments to Sub Volume 1 (Version 0.7) that follow prior to its submission to ICAO for language translation.

(Presented By Brian Dennis)

(Prepared by Akhil Sharma)

Rev. No.

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL

WORKING GROUP 1

Halifax, Nova Scotia, 30th July - 1st August 1996

CNS/ATM-1 Package

Sub-Volume 1

Introduction and 
System-Level Requirements

Version 0.7

19-June-1996

## Sub-Volume 1:  Introduction and System-Level Requirements

## Introduction

### The Aeronautical Telecommunication Network (ATN) shall be capable of providing communication services in support of:

- the delivery of air traffic services (ATS) to aircraft based on use of the ADS, CPDLC and the FIS air/ground applications;

- the exchange of air traffic management (ATM) information between ATS Units  and/or AOC Units, as appropriate, based on the use of the ICC and AMHS ground/ground applications and the ground/ground elements of the ADS, CPDLC and CM air/ground applications; and

- other applications supporting  Aeronautical Operational Control (AOC) and Aeronautical Administrative Communications (AAC).

### The conceptual model of the ATN is   illustrated in Figure 1.1-1:

Ed Note:  Amendment suggested in Figure 1.1-1: replace « ATN End-to-End » with « ATN Application Association » : the term end-to-end is not applicable to such a diagram in a store-and-forward environment

Figure 1.1-1:  Conceptual Model of the ATN.

### The ATN shall  consist of the following:

- Context Management (CM) application;

- Controller Pilot Data Link Communications (CPDLC) application;

- Automatic Dependent Surveillance (ADS) application;

- Automatic Terminal Information Services (ATIS) application as an initial  application from a set of  possible Flight Information Services (FIS) related applications;

- Air Traffic Services (ATS) Interfacility Data Communications (AIDC) application as an initial application from a set of  possible Inter Centre Communications (ICC) related applications;

- ATS Message Handling Services (AMHS) application;

- Upper Layers Communications Service;

- Internet Communications Service.

### Overview

Figure 1.1-2 provides an overview of  this Appendix..

Figure 1.1-2: Overveiew of Appendix A

## System Level Requirements

Note.—  The system level requirements are high-level technical requirements that have been derived from operational needs, technological constraints and regulatory constraints (administrative and institutional).  These system-level requirements are the basis for the functional requirements, lower level requirements, and derived requirements defined in Appendices A.2 through A.5.

### ATN System Level Requirements

Ed note:  The ATN system level requirements will be reviewed for their sequence and structure.

#### The ATN standards shall be based on the International Organization for Standardization (ISO) Open Systems Interconnection (OSI) Reference Model and associated ISO\OSI standards.

#### The ATN standards shall facilitate the backward compatible  evolution  of systems compliant with this document to meet future requirements..

#### The ATN shall enable the transition of existing AFTN users and systems into the ATN architecture.

#### The ATN shall enable only the authorized ATC authority to provide ATC instructions (Comment #2 -  is this specific to CPDLC or is it meant to address ADS & FIS as well ?  It needs to be clarified.) to aircraft operating in its airspace.

#### The ATN shall be capable of implementing and employing policy based routing at the network layer(i.e. layer 3 of the ISO/Reference Model).

#### The ATN shall enable data communications to be carried only over authorized paths for the Traffic Type specified by the User.

Note. – The Internet Communications Service defines four Traffic Types:

- ATN Operational Communications;

- ATN Administrative Communications;

- General Communications;

- ATN Systems Management Communications.

- The ATN Operational Communications traffic type is sub-divided into two categories:

- Air Traffic Services Communications;

- Aeronautical Operational Control.

#### The ATN shall be capable of  offering an  ATSC class based communications service in accordance with the criteria in Table 1.2-1.

Table 1.2-1:  ATSC  Classes

| One way End-to-End Transit Delay at 95% probability (seconds) | ATSC Class |
| --- | --- |
| Reserved | A |
| Reserved | B |
| 7.2 | C |
| 13.5 | D |
| 18 | E |
| 27 | F |
| 50 | G |
| 100 | H |
| No value specified | No preference |

#### The ATN shall operate in accordance with the communications protocol priorities as defined  in Table 1.2-2.

Table 1.2-2.  Mapping  of  Message Categories to the ATN applications  and the Internet Communications Service Protocols

| Message Categories | ATN Application |  | Corresponding Protocol Priority |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Transport Layer Priority |  | Internet Layer Priority |
|  |  |  |  | Transport Connection Priority | TSDU Priority | CLNP Priority |
| Network/Systems Management |  |  |  | 0 | 0 | 14 |
| Distress Communications |  |  |  | 1 | 1 | 13 |
| Urgent Communications |  |  |  | 2 | 2 | 12 |
| High Priority Flight Safety Messages |  |  |  | 3 | 3 | 11 |
| Normal Priority Flight Safety Messages |  | CPDLC ADS ICC (AIDC) |  | 4 | 4 | 10 |
| Meteorological Communications |  |  |  | 5 | 5 | 9 |
| Flight Regularity Communications |  | CM, AMHS |  | 6 | 6 | 8 |
| Aeronautical Information Service Messages |  | FIS (ATIS) |  | 7 | 7 | 7 |
| Network/Systems Administration |  |  |  | 8 | 8 | 6 |
| Aeronautical Administrative Messages |  |  |  | 9 | 9 | 5 |
| <unassigned> |  |  |  | 10 | 10 | 4 |
| Urgent Priority Administrative and U.N. Charter Communications |  |  |  | 11 | 11 | 3 |
| High Priority Administrative and State/Government Communications |  |  |  | 12 | 12 | 2 |
| Normal Priority Administrative |  |  |  | 13 | 13 | 1 |
| Low Priority Administrative |  |  |  | 14 | 14 | 0 |
| Note:— Priorities above double line are for communications related to safety and regularity of flight. |  |  |  |  |  |  |

#### The ATN shall enable peer to peer application exchange of information when an authorized path exists. (Comment#3 - Is this not a consequence of adopting ISO OSI ?)

#### The ATN- shall enable store-and-forward application exchange of information when authorized communications paths exist.

#### ATN shall notify the appropriate application processes when no authorized path exists i.e. when:

a) an application process attempts to establish communications with a remote application process and fails due to a lack of a suitable communications path being available, in this case the originating application process shall be informed; and

b) a communications path becomes unavailable during the period between  an application to application dialogue initiation and its intended termination, in this case both the originating and destination application processes shall be informed .

The ATN shall provide a means to unambiguously address all ATN End and Intermediate systems.

#### The ATN shall enable the recipient of a message to positively identify the originator of that message within a dialogue.

#### The ATN addressing and naming plans shall permit States and organisations to assign addresses and names within their own administrative domains.

#### The ATN shall support data communications to fixed and mobile (e.g. aircraft) systems.

#### The ATN design shall facilitate the integration of  the AMSS, VDL and Mode S mobile subnetworks as defined in this document.

#### The ATN design shall be such that an aircraft Intermediate System may  be connected to a ground Intermediate System via concurrent mobile subnetworks.

#### The ATN design shall be such that an aircraft Intermediate System may be connected to multiple ground Intermediate Systems.

#### The ATN shall be capable of exchanging application address information between two End Systems  connected via a suitable communications path.

#### The ATN shall be capable of establishing, maintaining, releasing, forwarding and aborting peer to peer application associations for Context Management (CM).

#### A context management  log-on and exchange of information indicating supported applications and associated addressing information shall be completed, in the absence of  the necessary information being available a-priori,  prior to initiation of an application dialogue for allof the air/ground applications defined below.

#### The ATN shall enable the   provision of  an operational ADS based air traffic service.

#### The ATN shall enable the provision of an operational CPDLC based air traffic service.

#### The ATN shall enable the provision of Flight Information Services/Automatic Terminal Information Service (FIS/ATIS) in an operational air traffic environment.

#### The ATN shall enable the provision of  an  ATS Message Handling Services in an operational air traffic environment.

#### The ATN shall enable the provision of an Inter-Centre Communications /ATS Interfacility Data Communications  (ICC/AIDC) based service in an operational air traffic environment.

#### The ATN shall reference time based on Co-ordinated Universal Time (UTC).

Ed note.  A “shall” will be added to address the integrity expected from the “ATN end-to-end.”  (See Figure 1.1-1).

## References

| ICAO ADSP/4 | Report of the ICAO ADSP, “Automatic Dependent Surveillance (ADS) and Air Traffic Services (ATS) Data Link Applications Guidance Material,” dated tbd 1996. ADS, CMA, CPDLC (Comment# - Is it appropriate to reference Panel Meeting Reports in Annex 10 ?) |
| --- | --- |
| NOCAR | ICAO NATSPG IMG/x, Appendix y, “nth Edition of the North Atlantic Oceanic Concept and Requirements Document (NOCAR),” tbd (Comment #6- is this document referenced from any of the SARPs ? It was a very high level document and was, it is believed, adopted as the basis of future work within the CADAG, it is certainly not a definitive document that should be referenced from Annex 10). |
| ICAO Annex 10, VII, 5th Ed: 1995 | ICAO Annex 10, Volume II; fifth Edition, July 1995. (Comment #7 - This will eventually be Annex 10 so is it necessary to reference it albeit another Volume ?) Note 2.- The fifth Edition of Annex 10, Volume II includes the provisions of Amendment 70. |