ATNP/WG1/WP- __
26/7/96

Comments on and Proposed Amendments to “Draft ATN SARPs - Part 1, Version 0.4, June 24, 1996

SUMMARY

Version 0.4 of the draft Part 1 ATN SARPs was agreed at the 5th Meeting of WG1 in Munich (WP5-10( c )).  This paper includes comments and proposed amendments to the that document.  Use has been made of  a revision mark facility.  However, where the revisions have resulted in the text difficult to read the text has been duplicated in a box with all proposed revisions accepted in order to facilitate the review.

PROPOSAL

WG1 is invited to review, comment and, where considered appropriate,  approve the proposed amendments to Part 1 prior to its submission to ICAO for language translation.

(Presented By Brian Dennis)

(Prepared by Akhil Sharma)

DRAFT FOR INFORMATION

Rev. No.

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL

WORKING GROUP 1

Halifax, Nova Scotia, 30th July - 1st August 1996

# WP5-10(c)

# Draft ATN SARPs - Part 1

Version 0.4

June 24, 1996

## Definitions

(Comment#1 - This section is attempting to address both “definitions” and “explanation of terms”. It therefore needs to be sub-divided or solely address definitions).

(Comment#2 - The terms “application”, “application process” and “application entity”  and their relationship to each other must be defined.)

(Comment#3 - Define “Aircraft Operator” & “Aircraft Operating Agency”)

Aeronautical Operational Control (AOC)	- Communications required for the exercise of authority over the initiation, continuation, diversion, or termination of a flight in the interest of the safety of the aircraft and the regularity and efficiency of flight.

# (Comment #4 - The ATN is defined by these SARPs - it is therefore not considered necessary to include a definition the agreement of which could take a very long time !)

Airline Administrative Communications (AAC)	- Communications used by aeronautical operating agencies related to the business aspects of operating their flights and transport services.  These communications are used for a variety of purposes, such as flight and ground transportation bookings, deployment of crew and aircraft,, or any other logistic purposes that maintains or enhances the efficiency of overall flight operation. (Comment #5 - What is the source of this and the definitions for AOC and  APC ?  It is understood that the only document in which such terms are defined is the first edition of the ATN Manual - this should be checked.)

Air/Ground Data-Link Application - A set of services and/or functions  involving an air user in communication via data-link, with one or more ground users, or a ground user in communication, via data link, with one or more air users, designed to meet specific operational requirements.

(Comment #6 - Sub Volume 5 defines four “Traffic Types”:

- “ATN Operational Communications”

- “ATN Administrative Communications”

- “General Communications” and

- “ATN Systems Management Communications”.

The traffic type “ATN Operational Communications” is further sub-divided into two categories:

- “Air Traffic Service Communications”

- “Aeronautical Operational Control”

In order to avoid confusion it is proposed that the above definitions are included in the Part 1 definition section within the overall definition of the term “Traffic Type”)

(Comment#7 - Furthermore in the Sub Volume 5 addressing Chapter the terms: “ATSC” (Air Traffic Service Communications) and “AINSC” (Aeronautical Industry Service Communications) are used to differentiate between air traffic service related and aeronautical industry related  NSAP addresses.  It is proposed that “ATSC address” and “AINSC address” definitions are included in the Part 1 definitions section.)

Air Traffic Services (ATS)	- Services provided by air traffic service providers  to ensure the safe and expeditious flow of air traffic.

Air Traffic Management (ATM)	- ATM consists of a ground and air part, both needed to ensure the safe and efficient movement of aircraft during all phases of operation. (Comment#8 This definition is not considered to be sufficient.  It is understood that a WG3\6 action will propose an alternative defintion).

Airline Passenger Communications (APC)	- Communications used for public correspondence from the aircraft to the ground and vice versa

Air Traffic Services (ATS) Unit - TBD

Aeronautical Operational Control (AOC) Unit - TBD

ATS Message Handling Services (AMHS)	- The ground/ground communications function which  enables ATS messages to be exchanged between  users through the provision of a  generic message service.

Authorised ATC Authority - needs to be defined, Sub Volume 1, para. 1.2.1.4 refers.

Automatic Dependent Surveillance (ADS)	- A technique in which aircraft automatically provide, via a data link, data derived from on-board navigation and position-fixing systems, including aircraft identification, four-dimensional position, and additional data as appropriate. ADS is a an air-ground data link application.

Automatic Terminal Information Service (ATIS)	- The air-ground ATIS application provides pertinent information to aircraft operations in the terminal area.  (Comment#9 - This is not a formal definition of ATIS, refer to Annex 3 or 11 ?)

(Comment #10 - Should there be a defintion for the term AIDC ?)

Context Management (CM)	- Refers to an air-ground ATN application.  This application implements an ATN logon service allowing initial aircraft introduction into the ATN.  The logon service also allows indication of all other data link applications on the aircraft.  CM also includes functionality to forward addresses between ATC centres.  Thus, CM is a logon and simple directory service.  Note: “Context Management” is a recognised OSI presentation layer term.  The OSI use and the ATN use have nothing in common.

Controller Pilot Data Link Communications (CPDLC)	- The CPDLC air-ground ATN application enables the capability for data link communications between air traffic controllers and pilots.  The CPDLC application has the capability to establish, manage, and terminate CPDLC dialogues for controller/pilot message exchange. (Comment#11 - This appears to be an explanation rather than a definition).

End System (ES)	- A functional entity  that implements  the seven layers and associated communications protocols and implements  one or more  application processes.

Flight Information Services (FIS)	- Flight Information Services (FIS) are provided to the pilot to increase situational awareness and to enhance safety of flight.

Inter-Centre Communications (ICC)	- The Inter-Centre Communications functions allow for the exchange of information between Air Traffic Service providers. (Comment#12  The type of information needs to be defined e.g. flight plans, advance boundary information).

Intermediate System (IS)	- A system comprising the lower three layers of the OSI reference model and performing relaying and routing functions.

International Organization for Standardization (ISO)		- International Organization that defines the communication standards that have been used as the basis for the ATN communication services. (Comment#13 - This is not a defintion and, in any case, should not really need defining).

Internet Communications Service	- The ATN Internet Communication Service consists of ISO OSI network and transport layer services and protocols and supports interprocess data communication between aeronautical host computers.   The ATN internet service access point is provided at the transport layer.

Open System Interconnection	- Based on a communications model providing a standard approach to network design introducing modularity by dividing the complex set of functions into seven more manageable, self-contained, functional layers.  By convention these are usually depicted as a vertical stack.

Upper Layers Communication Services	- The upper layer communications services consists of an application entity, OSI presentation layer and OSI session layer.  The upper layers reside in ATN end systems between an application process and the internet communication services.

References

ISO 7498-3	-  ISO standard for Open System Interconnection Basic Reference Model

# 1.  Introduction

1.1		The Aeronautical Telecommunications Network (ATN)  has been designed in support of the Communications, Navigation, Surveillance and Air Traffic Management (CNS/ATM) System  as defined by ICAO.  In support of these concepts the ATN:-

1.1	The Aeronautical Telecommunications Network (ATN)  has been designed in support of the Communications, Navigation, Surveillance and Air Traffic Management (CNS/ATM) System  as defined by ICAO.  In support of these concepts the ATN:-

a)		is specifically and exclusively intended to provide data communications services to Air Traffic Service provider organisations and Aircraft Operating agencies supporting the following types of communications traffic:

(i) Air Traffic Services (ATS) Communications;

(ii), Aeronautical Operational Control (AOC) Communications;

(iii) Airline Administrative Communications (AAC); and

(iv) Airline Passenger Communications (APC).

a)	is specifically and exclusively intended to provide data communications services to Air Traffic Service provider organisations and Aircraft Operating agencies supporting the following types of communications traffic:

(i) Air Traffic Services (ATS) Communications;

(ii), Aeronautical Operational Control (AOC) Communications;

(iii) Airline Administrative Communications (AAC); and

(iv) Airline Passenger Communications (APC).

b)	provides a reliable end-to-end data communications service, essential to support the provision of safe and efficient air traffic services,  between:

(i)airborne systems and ground based systems and

(ii)	between ground based systems;

in a manner that is transparent to the User”.

c)	 provides a data communication service which is capable of  meeting the security and safety requirements of the Users;

c)	 provides a data communication service which is capable of  meeting the security and safety requirements of the Users;

d) 	 is based the on International Standards Organisation/Open Systems Interconnection (ISO/OSI) standards which will facilitate the development of compliant systems and encourage the competitive provision of  ATN compliant network services;

d)	is based the on International Standards Organisation/Open Systems Interconnection (ISO/OSI) standards which will facilitate the development of compliant systems and encourage the competitive provision of  ATN compliant network services;

e)

f) 	capable of offering differing classes of data communications service (based on end-to-end transit delays or air/ground subnetwork preference selection) as required by the User.

f) 	capable of offering differing classes of data communications service (based on end-to-end transit delays or air/ground subnetwork preference selection) as required by the User.

g) 	defines an architecture within which the  air/ground subnetworks (e.g. Mode S, AMSS, VDL) , ground/ground subnetworks (e.g. CIDIN), ATS private ground/ground subnetworks and commercially available ground subnetworks may be integrated into a seamless whole, i.e. an internetwork.  This allows the use of existing/planned  infrastructure and network technologies, as well as giving implementors the freedom to  scale the network to meet the increasing needs of the users; and

g)	defines an architecture within which the  air/ground subnetworks (e.g. Mode S, AMSS, VDL) , ground/ground subnetworks (e.g. CIDIN), ATS private ground/ground subnetworks and commercially available ground subnetworks may be integrated into a seamless whole, i.e. an internetwork.  This allows the use of existing/planned  infrastructure and network technologies, as well as giving implementors the freedom to  scale the network to meet the increasing needs of the users; and

h) 	efficiently uses the air/ground subnetworks in order  to overcome the bandwidth limitations of these subnetworks and consequently reduce the associated data communications costs.

h) 	efficiently uses the air/ground subnetworks in order  to overcome the bandwidth limitations of these subnetworks and consequently reduce the associated data communications costs.

1.2		The ATS applications presently defined (i.e. CPDLC, ADS, FIS, ICC and AMHS), have been defined to provide communication, surveillance, and information services in support of  the ICAO CNS/ATM System.  These applications  in turn enable the provision of  at least the following data-link based Air Traffic Services:

1.2	The ATS applications presently defined (i.e. CPDLC, ADS, FIS, ICC and AMHS), have been defined to provide communication, surveillance, and information services in support of  the ICAO CNS/ATM System.  These applications  in turn enable the provision of  at least the following data-link based Air Traffic Services:

a)	Approach Control Service;

b)	Aerodrome Control Service;

d)	Area Control Service;

e)	Information Service; and

f)	Alerting Service.

## 1.3		The following paragraphs  define the high level provisions with which the ATN shall comply and invokes the detailed Standards and Recommended Practices for the ATN as provided in Appendices A.1, A.2, A.3, A.4 and A.5. The figure below illustrates the structure of Appendix A.

## 1.3	The following paragraphs  define the high level provisions with which the ATN shall comply and invokes the detailed Standards and Recommended Practices for the ATN as provided in Appendices A.1, A.2, A.3, A.4 and A.5. The figure below illustrates the structure of Appendix A.

## a)	Applications Entities;

## 1) System Applications

## i) Context Management (CM)

## 2) Air/Ground Applications Requirements

## i) Controller Pilot Data Link Communications (CPDLC)

## ii) Automatic Dependent Surveillance (ADS)

## iii) Flight Information Services (FIS)

## 3) Ground/Ground Applications Requirements

## i) Air Traffic Services (ATS) Inter-Centre Communications (ICC)

## ii) ATS Message Handling Services (AMHS)

## b)	Communication Service Requirements;

## 1) Upper Layers Communication Services

## 2) Internet Communication Services

## c)	System Management Requirements; and

## d)	 Security Requirements.

## 2.	General

2.1		The ATN shall be capable of providing data communications services to ATM applications to  support the exchange of  appropriate data between:

- ATS units and aircraft under their control  to support the provision of  air traffic services (ATS) to such aircraft; and

- ATS units and aircraft  which may shortly come under their control to support the provision of  air traffic services (ATS) to such aircraft;

- ATS units ; and

- ATS units and  AOC Units.

2.1	The ATN shall be capable of providing data communications services to ATM applications to  support the exchange of  appropriate data between:

- ATS units and aircraft under their control  to support the provision of  air traffic services (ATS) to such aircraft; and

- ATS units and aircraft  which may shortly come under their control to support the provision of  air traffic services (ATS) to such aircraft;

- ATS units; and

- ATS units and  AOC Units.

2.2		The ATN shall be capable of providing data communications services to Aeronautical Operational Control (AOC), Aeronautical Administrative Communications (AAC) and Airline Passenger Communications (APC) designated applications.

2.2		The ATN shall be capable of providing data communications services to Aeronautical Operational Control (AOC), Aeronautical Administrative Communications (AAC) and Airline Passenger Communications (APC) designated applications.

## 2.3		The conceptual model of the ATN shall be as depicted below:

## Note:  ¾  Shading indicates elements outside the scope of this document.

## 3.	System Level Requirements

EDITOR’S NOTE:  INSERT SYSTEM LEVEL REQUIREMENTS FROM SV-1 HERE

- (Comment#14 - Is it really necessary to reproduce the System level requirements in Part 1 ?  This will undoubtedly lead to document maintenance problems in the future when it will be necessary to ensure that both sets of SLRs are identical.

If it is finally decided to insert these requirements then some introductory text is in order which should explain that the SLRs have been mostly derived from high level Operational Requirements and that they are in turn satisfied by the detailed SARPs defined in Appendices A.2 (Sub Vol 2) through A.5 (Sub Vol 5).)

# 4.	Applications Requirements

## 4.1. 	System Applications

Note. – System applications provide services that are necessary for the  reliable, efficient and correct operation of the applications and the ATN Communication Services..

Note. – System applications provide services that are necessary for the  reliable, efficient and correct operation of the applications and the ATN Communication Services.

### 4.1.1 	Context management (CM) application

Note.  ¾ The CM application provides the capability for an aircraft to log-on with an ATS ground system.  Once an appropriate communications connection is established, CM provides for the exchange of addressing  information of each supported application resident in the aircraft and ATS ground system. CM additionally provides the capability to update log-on information and the capability for a ATS ground system to forward log-on information  to adjacent ATS ground system(s).

Note.  ¾ The CM application provides the capability for an aircraft to log-on with an ATS ground system.  Once an appropriate communications connection is established, CM provides for the exchange of addressing  information of each supported application resident in the aircraft and ATS ground system. CM additionally provides the capability to update log-on information and the capability for a ATS ground system to forward log-on information  to adjacent ATS ground system(s).

The following CM application functions shall be in accordance with the requirements of Appendix A.2.1 :

a)	Logon;

b)	Contact;

c)	Update;

d)	Ground forwarding; and

e)	Registration.

4.1.2	Future System Applications

(to be developed)

## 4.2	Air-ground applications

4.2.1	A context management logon and exchange of information indicating supported applications and associated addressesing information shall be completed, in the absence of  the necessary information being available a-priori,  prior to initiation of an application dialogue for any of the air-ground applications defined below.

4.2.1	A context management logon and exchange of information indicating supported applications and associated addressesing information shall be completed, in the absence of  the necessary information being available a-priori,  prior to initiation of an application dialogue for any of the air-ground applications defined below.

Note.  ¾  The ground components of air/ground applications include functionality supports the forwarding of  the contents of air to ground messages along ground/ground communications paths.

Note.  ¾  The ground components of air/ground applications include functionality supports the forwarding of  the contents of air to ground messages along ground/ground communications paths.

### 4.2.2 	Automatic Dependent Surveillance (ADS) application

Note.  ¾ The ADS   application comprises an airborne and ground component. The airborne ADS component is capable of  automatically providing, via the underlying communications services, to the ground component data derived from on-board navigation and position-fixing systems e.g.  aircraft identification, four-dimensional position, and additional data as appropriate. The  ADS application provides service based on contracts established between its air and ground components (i.e. demand contact, periodic contract, event contract and emergency contract) and between two ADS ground components  (i.e. forward contract).

Note.  ¾ The ADS   application comprises an airborne and ground component. The airborne ADS component is capable of  automatically providing, via the underlying communications services, to the ground component data derived from on-board navigation and position-fixing systems e.g.  aircraft identification, four-dimensional position, and additional data as appropriate. The  ADS application provides service based on contracts established between its air and ground components (i.e. demand contact, periodic contract, event contract and emergency contract) and between two ADS ground components  (i.e. forward contract).

The following ADS application functions shall be in accordance with the requirements of Appendix A.2.2 :

a)	demand contracts;

b)	periodic contracts;

c)	event contracts ;

d)	emergency contracts and

e)	forward contracts .

### 4.2.3	Controller Pilot Data Link Communications (CPDLC) application

Note.  ¾ The CPDLC application, comprising an airborne and ground component,  enables the capability for data link communications between ATS Units and aircraft under their control and/or aircraft about to come under their control.  The CPDLC application has the capability to establish, manage, and terminate CPDLC dialogues for controller/pilot message exchange.

Note.  ¾ The CPDLC application, comprising an airborne and ground component,  enables the capability for data link communications between ATS Units and aircraft under their control and/or aircraft about to come under their control.  The CPDLC application has the capability to establish, manage, and terminate CPDLC dialogues for controller/pilot message exchange.

The following CPDLC application functions shall be in accordance with the requirements of Appendix A.2.3 :

a)	controller-pilot message exchange;

b)	transfer of data authority;

c)	down stream clearance; and

d)	ground forward.

### 4.2.4 	Flight Information Service (FIS) applications

Note.  ¾  The FIS application set allows a pilot to request and receive flight information services from ground FIS systems.  This document currently  defines the requirements for the automatic terminal information service (ATIS) application as one member of the FIS application set.

Note.  ¾  The FIS application set allows a pilot to request and receive flight information services from ground FIS systems.  This document currently  defines the requirements for the automatic terminal information service (ATIS) application as one member of the FIS application set.

4.2.4.1 	Automatic Terminal Information Service (ATIS) application

The following ATIS application functions shall be in accordance with the requirements of Appendix A.2.4 :

a)	aircraft initiated ATIS demand contracts;

b)	aircraft initiated ATIS update contracts; and

c)	both an aircraft and ground initiated ATIS cancellation of contracts.

4.2.5	Future  air-ground applications

(to be developed)

## 4.3	Ground/Ground Application Requirements

Note: ¾  ground/ground applications are defined as those ATN applications resident in ground based systems that exchange information  with peer applications also resident in ground based systems

#### 4.3.1	Inter-Centre Communications applications

Note: ¾  The Inter-Centre Communications applications set enables the exchange of information between Air Traffic Service Units over the underlying communications service, i.e the Upper Layers Communications Service. This document currently  defines the requirements for the ATS Inter-Facility Data Communication (AIDC) application as one member of the ICC application set.

Note: ¾  The Inter-Centre Communications applications set enables the exchange of information between Air Traffic Service Units over the underlying communications service, i.e the Upper Layers Communications Service. This document currently  defines the requirements for the ATS Inter-Facility Data Communication (AIDC) application as one member of the ICC application set.

#### 4.3.1.1	ATS Interfacility Data Communication (AIDC)

#### Note: ¾  AIDC is an ATN application which is used by two Air Traffic Service (ATS) units to enable the exchange of Air Traffic Control (ATC) information for an active flight related to flight notification, flight co-ordination, transfer of control, surveillance data and free (i.e. unstructured) text data over the underlying communications service.

#### Note: ¾  AIDC is an ATN application which is used by two Air Traffic Service (ATS) units to enable the exchange of Air Traffic Control (ATC) information for an active flight related to flight notification, flight co-ordination, transfer of control, surveillance data and free (i.e. unstructured) text data over the underlying communications service

The following AIDC application functions shall be in accordance with the requirements of Appendix A.3.2:

a)	Flight Notification;

b)	Flight Coordination;

c)	Transfer of Control;

d)	Transfer of Communications;

e)	Transfer of Surveillance Data; and

f)	Transfer of General Data.

#### 4.3.2	ATS Message Handling Service (MHS) application

Note: ¾  The ATS Message Handling Service (AMHS) application comprises two main functions, the ATS Message Service function and the ATN Pass-Through Service function.  The ATN Message Service function enables ATS Messages to be exchanged between service users, over the underlying communications service, through the provision  of generic message services. The ATN Pass-Through Service function is the ATS Message Handling Service offered over the ATN by the use of the Dialogue Service and of the associated upper layer architecture to exchange AFTN Messages formatted in IA-5 in compliance with the provisions of Volume II.   The AMHS application includes the definition of AFTN/ATN gateways.

Note: ¾  The ATS Message Handling Service (AMHS) application comprises two main functions, the ATS Message Service function and the ATN Pass-Through Service function.  The ATN Message Service function enables ATS Messages to be exchanged between service users, over the underlying communications service, through the provision  of generic message services. The ATN Pass-Through Service function is the ATS Message Handling Service offered over the ATN by the use of the Dialogue Service and of the associated upper layer architecture to exchange AFTN Messages formatted in IA-5 in compliance with the provisions of Volume II.   The AMHS application includes the definition of AFTN/ATN gateways.

The following ATS Message Handling Service (MHS) application functions shall be in accordance with the requirements of Appendix A.3.1:

a)	ATN message service;

- ATN pass-through Service.

4.3.3	Future ground/ground applications

(to be developed)

# 5.	Communication services requirements

Note: ¾  The ATN communications service requirements define the requirements for layers 3 through 6 of the ISO/OSI  Reference Model (ISO 7498-3).  This service provides  an  end-to-end communications service to the applications defined above in standardized formats. The communications service requirements are divided into two parts.  The Upper Layer Communications Service defines the standards for the layers 5 through 6 of the OSI Reference Model.  The Internet Communications Service defines standards for layers 3 and 4 of the OSI Reference Model.  The requirements for layers 1 and 2 are outside the scope of this chapter and its Appendix.

Note: ¾  The ATN communications service requirements define the requirements for layers 3 through 6 of the ISO/OSI  Reference Model (ISO 7498-3).  This service provides  an  end-to-end communications service to the applications defined above in standardized formats. The communications service requirements are divided into two parts.  The Upper Layer Communications Service defines the standards for the layers 5 through 6 of the OSI Reference Model.  The Internet Communications Service defines standards for layers 3 and 4 of the OSI Reference Model.  The requirements for layers 1 and 2 are outside the scope of this chapter and its Appendix.

## 5.1	Upper Layer Communications Service Requirements

Note:  ¾ The Upper Layer  Communications Service consists of an OSI compliant presentation layer and session layer.  The Upper Layer functionality resides in  End Systems providing a communications service to the application processes  defined in paragraph 4 and relying on the service provided by the underlying Internet Communications Service defined in paragraph 5.2.

Note:  ¾ The Upper Layer  Communications Service consists of an OSI compliant presentation layer and session layer.  The Upper Layer functionality resides in  End Systems providing a communications service to the application processes  defined in paragraph 4 and relying on the service provided by the underlying Internet Communications Service defined in paragraph 5.2.

5.1.1		The Upper Layer  Communications Service shall be implemented in End Systems in accordance with Appendix A.4 in support of  the ATN applications (with the exception of  the AMHS  Application the services for which are defined in Appendix A3.1) for:

5.1.1		The Upper Layer  Communications Service shall be implemented in End Systems in accordance with Appendix A.4 in support of  the ATN applications (with the exception of  the AMHS  Application the services for which are defined in Appendix A3.1) for:

a)	The Session Layer;

b)	The Presentation Layer;

c)	The Application Entity structure;

d)	The Association Control Service Element (ACSE);

e)	The Application Service Element (ASE) and Application Service Object (ASO) as specified in Appendix A

f)	The Control Function.

## 5.2	Internet Communication Service Requirements

Note: ¾  The requirements for the Internet Communication Service are defined in terms of  the Network and Transport layers of the ISO/OSI Reference Model.  These requirements are applicable to  the End System and Intermediate System  functional entities which together provide the Internet Communications Service.  End Systems functions will be  typically housed in Host Computer Systems e.g. Flight Data Processing Systems whilst Intermediate System functions will be typically housed in Routers.  However,it should be noted that, in certain situations it may be advantageous to house both End System and Intermediate System functionality in a single physical unit, i.e. a combined ES/IS. t The Internet Communications Service is provided to its user (i.e. the upper layers) via  its transport layer service interface.

Note: ¾  The requirements for the Internet Communication Service are defined in terms of  the Network and Transport layers of the ISO/OSI Reference Model.  These requirements are applicable to  the End System and Intermediate System  functional entities which together provide the Internet Communications Service.  End Systems functions will be  typically housed in Host Computer Systems e.g. Flight Data Processing Systems whilst Intermediate System functions will be typically housed in Routers.  However ,it should be noted that, in certain situations it may be advantageous to house both End System and Intermediate System functionality in a single physical unit, i.e. a combined ES/IS.  The Internet Communications Service is provided to its user (i.e. the upper layers) via  its transport layer service interface.

5.2.1		An End System shall comply with all applicable provisions as defined in Appendix A.5.

5.2.1		An End System shall comply with all applicable provisions as defined in Appendix A.5.

5.2.2		An Intermediate System  shall comply with all applicable provisions Appendix A.5

5.2.2		An Intermediate System  shall comply with all applicable provisions Appendix A.5.

Note: ¾  Appendix A.5 defines a number of different classes of ATN Router for which Network Layer profiles are defined.

5.3	ATN Naming and Addressing

5.3.1		The ATN naming and addressing shall comply with the requirements of Application Entity Naming, Appendix A (4.3) and Network/Transport addressing, Appendix A (5.4).

Note:  ¾ The ATN naming and addressing scheme supports the principles of unique and unambiguous identification of information objects and global address standardization which are essential features for an international, mixed-user communications system such as the ATN.

# 6.	Systems Management requirements

(to be developed)

Note: ¾   The definition of global ATN Systems Management requirements  are not currently defined within this document.  Until such time this document defines the provisions necessary for global systems management, States/Organisations are advised to implement systems management on a regional/local basis whilst ensuring that the safety and regularity of flight is not compromised as a result of  the operation of any regional and/or local systems management system.

Note: ¾   The definition of global ATN Systems Management requirements  are not currently defined within this document.  Until such time this document defines the provisions necessary for global systems management, States/Organisations are advised to implement systems management on a regional/local basis whilst ensuring that the safety and regularity of flight is not compromised as a result of  the operation of any regional and/or local systems management system.

## 7.	Security requirements

(to be developed)

Note:  ¾  This document does not mandate any technical security features for security.   States and Organizations will be expected to provide physical and/or procedural security for ATN components in accordance with their existing practices.