ATNP/2-WP/___

11/4/96

AERONAUTICAL TELECOMMUNICATION NETWORK PANEL (ATNP)

SECOND MEETING

Montreal, 4-15 November 1996

Overview of Introductory SARPs Material

(Presented by the ATNP WG 1 Rapporteur)

WORKING PAPER

Summary

At the initial ATNP/1 meeting in 1994, three Working Groups were formed.  These working groups were tasked with the development of ATN SARPs and Guidance Material for final review at ATNP/2. WG2 was formed to create the Internet SARPs and Guidance Material for the ATNP.  The following is a summary of the proposed material for Sub-Volume 1.

References

- Report of the first meeting of the Aeronautical Telecommunication Network Panel

- Proposed ATN SARPs Sub-Volume 5

References:

1.  Report of the first meeting of the Aeronautical Telecommunication Panel.

2.  Proposed ATN SARPs

Attachments

1.  Proposed ATN SARPs Sub-Volume 1

# BACKGROUND

The structure of the proposed ATN SARPs (Annex 10, Volume 3, Part 1, Chapter 3) is composed of a core part and an Appendix.  The following describes this structure and components:

- Core Part

- Appendix A

- Sub-Volume 1 of the Appendix contains the definitions and system level requirements for the ATN.

- Sub-Volume 2 contains the functional and interoperability requirements for the ATN air-ground applications.

- Sub-Volume 3 contains the functional and interoperability requirements for the ATN ground-ground applications

- Sub-Volume 4 contains the functional and interoperability requirements for the ATN Upper Layer architecture.

- Sub-Volume 5 contains the functional and interoperability requirements for the ATN Internet Communications Services.

This working paper summarizes the proposed Sub-Volume 1 of Appendix A.  The document is an introduction to the requirements that are satisfied through the use of the ATN.  It includes the following components:

- Definitions

- References

- System Level Requirements

The definitions and references in Sub-Volume 1 is the compilation of definitions and references all of the Sub-Volumes.  The following sections of this working paper highlight the main concepts within the Sub-Volume.

# GENERAL

The aeronautical telecommunication network (ATN) shall provide communication services and application entities in support of:

a)	the delivery of air traffic services (ATS) to aircraft;

b)	the exchange of ATS information between ATS units; and

c)	other applications such as aeronautical operational control (AOC) and aeronautical administrative communication (AAC).

Provisions have been made to accommodate the exchange of information between aircraft operator ground based systems and ATS units, such as weather, flight plans, notices to airmen and dynamic real time air traffic flow management.  Provisions have also been made to accommodate airline passenger communication (APC).

Requirements for use of the ATN shall be made on the basis of regional air navigation agreements. The regional air navigation agreements include means to address the operational capability of the system and air traffic services, the requirements for mandatory equipage, the airspace of operation and the time scales for implementation.

# SYSTEM LEVEL REQUIREMENTS

The system level requirements are high-level technical requirements that have been derived from operational requirements, technological constraints and regulatory constraints (administrative and institutional).  These system level requirements are the basis for the functional requirements and lower level requirements.  The following are the system level requirements for the ATN that fulfill the operational requirements imposed on the ATNP.

- The ATN shall use International Organization for Standardization (ISO) communication standards for open systems interconnection (OSI).

- The ATN shall provide a means to facilitate migration to future versions of application entities and/or the communication services. It is an objective that the evolution towards future versions facilitates the backward compatibility with previous versions.

- The ATN shall enable the transition of existing AFTN users and systems into the ATN architecture.

- The ATN shall make provisions whereby only the controlling ATS unit may provide ATC instructions to aircraft operating in its airspace.  This is achieved through the current and next data authority aspects of the CPDLC application entity.

- The ATN shall accommodate routing based on a pre-defined routing policy.

- The ATN shall provide means to define data communication that can be carried only over authorized paths for the traffic type and category specified by the user.

- The ATN shall offer ATSC classes in accordance with the criteria in Table 1.3-1 of Sub-Volume 1.  (Note—  When ATSC class is specified, packets will be forwarded in the ATN Internet communications service on a best effort basis.  Best effort basis means that when a route is available of the requested ATSC class the packet is forwarded on that route.  When no such route is available, the packet will be forwarded on the first known route of ATSC class higher than that requested, or if there is no such route, first known route of lower ATSC class than that requested).

- The ATN shall operate in accordance with the communication priorities defined in Table 1.3-2 of Sub-Volume 1.

- The ATN shall enable application exchange of information when one or more authorized paths exist.

- The ATN shall notify the appropriate application processes when no authorized path exists.

- The ATN shall provide a means to unambiguously address all ATN end and intermediate systems.

- The ATN shall enable the recipient of a message to identify the originator of that message.

- The ATN addressing and naming plans shall allow States and organizations to assign addresses and names within their own administrative domains.

- The ATN shall support data communication to fixed and mobile systems.

- The ATN shall accommodate ATN mobile subnetworks as defined in this annex.

- The ATN shall make provisions for the efficient use of limited bandwidth subnetworks.

- The ATN shall enable an aircraft intermediate system to be connected to a ground intermediate system via concurrent mobile subnetworks.

- The ATN shall enable an aircraft intermediate system to be connected to multiple ground intermediate systems.

- The ATN shall enable the exchange of address information between application entities.

- A context management application shall be supported if any of the other air-ground applications are supported.

- The ATN shall be capable of establishing, maintaining, releasing and aborting peer to peer application associations for the context management (CM) application.

- The ATN shall be capable of establishing, maintaining, releasing and aborting peer to peer application associations for the automatic dependent surveillance (ADS) application.

- The ATN shall be capable of establishing, maintaining, releasing and aborting peer to peer application associations for the controller pilot data link communication (CPDLC) application.

- The ATN shall be capable of establishing, maintaining, releasing and aborting peer to peer application associations for the automatic terminal information system (ATIS) application.

- The ATN shall be capable of establishing, maintaining, releasing and aborting application associations for the ATS message handling services (ATSMHS) application.

- The ATN shall be capable of establishing, maintaining, releasing and aborting peer to peer application associations for the ATS interfacility data communication (AIDC) application.

- Where the absolute time of day is used within the ATN, it shall be based on coordinated universal time (UTC).

- The ATN end system shall make provisions to ensure that the probability of not detecting a 255-octet message being mis-delivered, non-delivered or corrupted by the Internet communications service is less than or equal to 10-8 per message.  It is assumed that ATN subnetworks will ensure data integrity consistent with this system level requirement.

# PROPOSAL

It is proposed that the ATN Panel accept Attachment 1 (the proposed Sub-Volume 1 of the ATN SARPs) as completion of the tasking to develop same.