ATNP/WG 1

WP 6-
	25 July 1996

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL

Working Group 1

Halifax, Nova Scotia, Canada

30. 7. - 2. 8. 1996

Proposed ATNP/2 Working Paper on the

ATNP Lexicon

Presented by K. Platz

Prepared by A. Herber

SUMMARY

This paper presents in Appendix A a draft working paper which introduces the ATNP lexicon. The significantly revised ATNP lexicon itself is contained in Appendix B; amendments cover taking into account the definitions etc. contained in sub-volume 1 and part 1 of the Draft ATN SARPs. As result, a number of definitions (now called "explanations") and the list of acronyms were removed. The possibly slight deviation from the structure agreed in Munich (i.e. separating it now into two papers inc. a summary paper) was chosen in order to reduce the translation effort for ICAO.

The working group is invited to review the proposed draft working papers for ATNP/2, amend it, as appropriate, and agree its submission to ATNP/2 as a WG 1 paper.

Appendix A	ATNP/2-WP/

25/7/96

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL (ATNP)

SECOND MEETING

Montreal, 4 to 15 November 1996

AGENDA ITEM 2: 	Development of a planning document for the world-wide evolutionary implementation of the ATN

ATNP Lexicon Summary

Working Paper

(DRAFT)

(Presented by Tom Calow, Rapporteur WG 1)

SUMMARY

This paper introduces the ATNP lexicon developed by Working Group 1 . This lexicon is serving as a working aid for discussions within the ATN Panel and its working groups. It is therefore proposed to be published as part of the meeting report.

1.	Introduction

During previous discussions within the meetings of the ATNP working groups, occasionally confusions arose from the obviously different understanding of terms used. As a consequence, working group 1 developed a list of terms often used within the working groups, but leading to different understandings about their exact meaning in the specific context. This list was referred to as "ATNP lexicon".

2.	Discussion

During a number of meetings of WG-1, the ATNP lexicon was compiled and amended accordingly. Already on ist first meeting, WG1 agreed to continue on ist development and to submit it as a deliverable to ATNP/2. This lexicon reflects the understanding of the ATNP when developing the ATN SARPs. Therefore, it is important to note that the purpose of this lexicon is not to reiterate or generally specify terms in a generally usable way; rather, this lexicon is

- a working tool for the work in the ATNP working groups

- exists for clarification purposes within ATNP

- exists as a resting place for definitions and terms until an existing ICAO ATN document publishes the term at which time the term will be removed

3.	Proposal

The meeting is invited to

a)	note the ATNP Lexicon as developed and approved by Working Group 1 and to accept it as a working basis for further meetings;

b)	publish the ATNP Lexicon as attachment to the report of the second meeting of the ATN Panel.

Appendix B	ATNP/2-WP/

25/7/96

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL (ATNP)

SECOND MEETING

Montreal, 4 to 15 November 1996

AGENDA ITEM 2: 	Development of a planning document for the world-wide evolutionary implementation of the ATN

ATNP Lexicon

Deliverable WG1-15

Working Paper

(DRAFT)

(Presented by Tom Calow, Rapporteur WG 1)

SUMMARY

This paper presents an updated version of the ATNP lexicon developed by Working Group 1 and intended to serve as a working aid for discussions within the ATN Panel and its working groups.

ATNP Lexicon

Explanations of Terms in Support of ATNP Work

Purpose of Document This ATNP lexicon provides explanations of terms often used within the working groups of the ATN Panel, which are sometimes creating different understanding concerning their use. The ATNP lexicon recognizes the existence of a glossary and a list of acronyms and abbreviations related to the ATN which are contained in a number of reference documents, like ISO standards and Draft ATN SARPs. The purpose of this ATNP lexicon is not to re-iterate these terms and definitions, but rather to concentrate on the explanation of ambiguous respectively differently used terms and additional expressions. In particular, this document is intended to serve as a working aid to ensure common understanding within the working groups, but not as a generally applicable glossary of ATN/ATM terms.

List of Contents											Page

Addressing (logical)

Addressing (physical)

ATM / ATS Applications

ATN Manual Edition 2

ATN Network Operating Concept

ATN Communication Services

ATN System Applications

CNS/ATM-1 Package

ATN SARPs (Draft Version n.m)

Congestion

Congestion Avoidance

Congestion Management

Congestion Recovery / Congestion Control

Directory Service

Engineering Trials

Functional Requirements

Institutional Issues

Operating Concept

Operational Concept [of ATN]

Operational Trials

Performance Requirements

Explanations

Addressing (logical) Logical addressing means that the address defined in the addresing plan and used to locate the addressed object is a virtual address which is a substitute of the actual (physical) address of an object. Address mapping functions have to fulfill this substitution, carefully maintaining unambiguity of identification of objects.

Addressing (physical) Physical addressing means that the address defined in the addressing plan and used to locate the addressed object is the physical, i.e. hardwired, hard-coded, or configured address of the object. An example of a physical address is the ICAO 24-bit Aircraft Address used for the SSR Mode S Transponder.

ATM / ATS Applications These are applications supporting ATM or other ATS functions and do not necessarily correspond to ATN applications. The term is usually used to distinguish between ATM functions and other non-ATM functions using the same communication service.

ATN Manual Edition 2 The second edition of the ATN Manual approved at SICASP/5 is the ICAO document 9578-AN/935 not expected to be published by ICAO. The ATN Manual Edition 2 is derived from the material developed by the SICASP in the form of the ATN Manual (Version 2) and recommended for publication at the fifth meeting of this panel.

ATN Network Operating Concept An ATN Network Operating Concept will address the administrative, operational, institutional, and policy issues and additional (non-SARPs relevant) technical aspects to enable the efficient and correct operation of the ATN.

ATN Communication Services The ATN communication services are provided to ATN users that require ground-ground or air-ground data communication. The ATN accomodates different grades of services which can be expressed by Quality of Service parametersand by communication priorities.

ATN System Applications System Applications support the operation of the ATN communication services and are either not directly or not at all used by ATN users but rather by the service providers, operators or other ATN applications. Typical examples of ATN system applications are the ATN directory service, ATN context management or ATN systems management.

CNS/ATM-1 Package The term CNS/ATM-1 Package describes the ATN SARPs defining the initial set of ATN internet and upper layer functions and protocol features as well as the initial ATN applications which will be recommended by the ATNP for the first operational use of the ATN. The purpose of this package is the availability of early operational implementations and trials in the 1997 time frame. The applications comprising CNS/ATM-1 Package are ADS, Controller-Pilot Communication, Context Management, Flight Information Services, Message handling Services and Inter-Centre Coordination. In line with the incremental development and validation strategy of the ATN, CNS/ATM-1 Package will be followed by subsequent packages containing a more complete set of ATN functions and applications.

ATN SARPs (Draft Version n.m) Each Version of the SARPs subvolumes and Guidance Material is explicitely referenced as "n.m". In this usage, "n" represents the major revision level associated with agreements reached during a meeting of the ATNP Working Groups, and where "m" represents revisions applied between WG meetings based on interim agreements by appropriate experts, subject to the WG approval. Therefore, draft SARPs approved by the working groups carry the version number "n.0".

Congestion In the ATN Internet sense, congestion describes the state where the network is overloaded. Typical effects of congestion are extended transit delays, drastically reduced throughput, and the loss of data packets

Congestion Avoidance Techniques that regulate the data flow into the network in order to prevent the network from getting overloaded. These encompass both open-loop techniques which ensure that a traffic contract specified by the source is respected, and closed-loop techniques which monitor signals generated by the network and adapt the traffic generated by the sources accordingly.

Congestion Management This term refers to a set of rules and techniques which prevent congestion , e.g. by monitoring actual network load. Co-operative interaction of all end systems is required in order to prevent individual end-systems taking up the throughput saved by well-behaving systems.

Congestion Recovery / Congestion Control This term refers to a mechanism which reacts to congestion after it has occurred in order to remove the overload condition. Congestion Recovery can be initiated only after congestion has been experienced, and is not able to safely prevent congestion in the network.

Directory Service The ATN Directory Service provides the ATN user with the addressing information which is associated with the application process title or application entity title used as input to the directory. The addressing information provided by the directory service includes the network address as well as further technical addresses on the layers above, as required or applicable. Furthermore, the ATN Directory Service resolves generic application process titles or application entity titles, i.e. names which may be incomplete or contain "don't care" elements, into the corresponding (list of) non-generic application process titles or application entity titles.

Engineering Trials In contrast to operational trials, engineering trials may be based on pre-operational, prototype or experimental equipment. Aim is to demonstrate the technical feasibility and correctness of applied techniques, concepts and specifications.

Functional Requirements Requirements that determine what function a system should perform. They can usually be expressed by a verb applying to a type of data, e.g. display aircraft position.

Institutional Issues Issues related to ownership, control and responsibility for correct implementation and operation of systems which involve more than one state or organization.

Operating Concept The Operating Concept describes the technical functionality of a system and its inherent capabilities regarded from the system operator's point of view. This includes the interaction between user and system, the services provided by the system as well as the internal operation of the system.

Operational Concept [of ATN] Top level description of the infrastructure capable of meeting an approved set of user requirements. The ATN operational concept is composed of general system capabilities as well as expected level of (minimum) performance requirements (including the system's prerequisites and limitations/ constraints). The ATN Operational Concept addresses issues of the services to be delivered to the user by the system and interactions between the user and the system (alternatively, the ATN service provider).

Operational Trials Operational trials are based on operational environment. This includes operational systems and operational equipments, e.g. routinely scheduled flights in an operational ATS environment. Aim is to demonstrate the operational acceptance and correctness of applied mechanisms, applications and concepts.

Performance Requirements Performance Requirements are requirements with respect to the performance of a system (e.g. reliability, availability, response time, processing delay, etc.) and are derived from Operational Requirements. In general, they describe the minimum performance figures which a system must provide in order to fulfill the operationally required functions.