Working Group 1

8th Meeting

March 07 an March 10 1997

Phuket

CORE SARPs Validation Report

Prepared by: WG1-DG1

Presented by: Benoit Gosselin

Appendix A

Annex 10, volume 3, part 1, chapter 3

validation report

Document Control Log

| Section(s) |  | Date |  | New Version |  | Reason for Change |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| All |  | 17/10/96 |  | 1.0 |  | Document creation |  |  |
| All |  | 22/10/96 |  | 1.0 |  | Editorial changes from JWG |  |  |
|  | All |  | 07/-3/97 |  | 1.1 |  | Document update for WGW |  |

Appendix a : Annex 10, Volume 3, Part 1, Chapter 3 Validation Report

# Introduction

## Scope

Annex 10, Volume 3, Part 1, Chapter 3 , further called Core SARPs, is a compilation of the high level requirements of the ATN SARPs.  The purpose of this document is to report on the validation of the draft cor e SARPs, based on the monitoring of the validation results of the various initiatives of the other sections in chapter 3 that composed the ATN SARPs.

## Background

The ATN SARPs were placed under configuration control at the 5th meeting of WG1 (Munich, June 17th to 20th, 1996), and since that time a detailed change record has been maintained.

The baseline version of the core SARPs was agreed at the 6th meeting of WG1 (Halifax, July 29 to August 1st, 1996) and submitted to ICAO at the second meeting of ATNP in November 1996 in Montreal.

At the end of ATNP2, version 1.0 of the first part if ATN SARPs was agreed on by the panel members.

The following table describes the change history of the core SARPs.

| Date | Version | Comments |
| --- | --- | --- |
| August 1st, 1996 | 1.0 | Submission to ICAO |
| November 15, 1996 | ICAO 1.0 | Output version from ATNP2 |

# System Level Requirements and High Level Validation Objectives

Validation Objectives (VOs) are statements which express the analysis, tests and evaluations required in order to declare the SARPs validated.  There are 2 types of validation objectives to be considered for this appendix.  They are static and dynamic.  The static requirements can be achieved through the direct correlation of validation objectives to all high level requirements.  The dynamic objectives are performance oriented and are therefore not directly related to specific validation objectives. The static VOs are mapped to the systems level requirements in this report.

The following table reflects the sections of the ATN SARPs that wholely achieve the referenced System Level Requirements.

| core SARPS paragraph number | Relevant Documents and/or References |
| --- | --- |
| 2.1 | ULA Database,SECTION4, SECTION-5, and individual applications |
| 2.2 | Test Bed, Validation Reports |
| 2.3 | Individual Applications |
| 2.5 | Individual Applications |
| 3.1 | Individual Applications |
| 3.2 | Individual Applications |
| 3.3 | AMHS |
| 3.4 | ICS, CM, ADS, CPDLC, and FIS |
| 3.5 | ICS, CM, AMHS, AIDC |
| 3.6 | ICS and AMHS |
| 3.7 | ICS |
| 3.8 | ICS and AMHS |
| 3.9 | ICS and Individual Applications |
| 3.10 | ICS and AMHS |
| 3.11 | Individual Applications |
| 3.12 | ULA, ICS and Individual Applications |
| 3.13 | ICS and AMHS |
| 3.14 | ICS |
| 3.15 | ICS |
| 3.16 | ULA and ICS |
| 3.17 | ICS |
| 3.18 | ICS |
| 3.19 | ULA, CM, CPDLC and AIDC |
| 3.20 | CM |
| 3.21 | CM |
| 3.22 | ADS |
| 3.23 | CPDLC |
| 3.24 | FIS |
| 3.25 | AMHS |
| 3.26 | AIDC |
| 3.27 | ICS and Individual Applications |
| 3.28 | ICS and others |
| 4.1 | CM |
| 4.2.1 | ADS |
| 4.2.2 | CPDLC |
| 4.2.3 | FIS |
| 4.3.1 | AIDC |
| 4.3.2 | AMHS |
| 5.1 | ULA |
| 5.2 | ICS |
| 6 | ULA and ICS |

# Validation Means

The various validation efforts has occurred on several levels.  The air/ground applications, the ground/ground applications and the Internet portion of the ATN SARPs have been validated separately.  These validation initiatives have been categorized by different validation objectives (VOs).  A resulting status of “pass”, “fail” or “inconclusive” was obtained for the System Level Requirement, based on the result of each associated VO.

# Validation achieved

The validation achievements for this validation report rely on every other validation efforts carried out by several States/Organisations for each section of the SARPs.   A summary of all other validation currently being carried-out, as well as status and results, can be found in the validation reports made available by other sub-groups.

The role of the  Section drafting group is in two parts:

- For the static validation objectives, to review all other validation reports, and to extract the appropriate validation objectives and map them to System Level Requirements.  Upon the results of VOs, extract the appropriate result and complete the validation table.

- For dynamic validation objectives, to elaborate the VOs, and perform validation efforts in order to achieve the objectives.

# Defect Report Summary

Inspection and analysis process conducted by several parties, have generated comments and defect reports which have been analysed.

| Paragraph reference | Description | Proposed change | Status |
| --- | --- | --- | --- |
| Definitions | definition of “ATS message handling service (ATSMHS)” last 2 sentences | last 2 sentences should read “There are two ATS message handling services. They are the ATS message service and the ATN pass-through service.” | closed |
| 1.4 | second paragraph | “The remainder of this chapter…” replace “This” with “The” | closed |
| 3. | Table 3-1 Transit delay for ATSC Classes | ATSC B should be valued at 4.5 instead of “reserved” | closed |
| 3.28 | system level requirement | remove “ATN” from first sentence | closed |
| 5.1.3 | reference | last line - change “Appendix A, (3).” To “Appendix A, (3.1.2)” | closed |

# Analysis and Conclusions

## Static Validation Objectives

The following table is a list of the mapping between the System Level Requirements and Validation Objectives of all the SARPs.  In the last column is a result.  A pass or fail lable is put only when all the validation objectives have been tagged with a result.  If not all the VOs have been validated, an Inconclusive label is then tagged to the System Level Requirement meaning that ongoing validation activitie are perform on the associated SARPs.  A pass, fail or inconclusive label has been given to the High Level requirement when all associated VOs were validated by March 1997.

| Section 1 - system level requirements | Keyword | Section | SARPs | validation objectives associated | status pass/fail/inconclusive |
| --- | --- | --- | --- | --- | --- |
| 1.2.1 | Provide support communication services and application entities | 2 | CM | all VOs from the CM SARPs support this requirement | P |
|  |  |  | ADS | all VOs from the ADS SARPs support this requirement | P |
|  |  |  | CPDLC | all VOs from the CPDLC SARPs support this requirement | P |
|  |  |  | FIS | all VOs from the FIS SARPs support this requirement | P |
|  |  | 3 | AIDC | all VOs from the AIDC SARPs support this requirement | P |
|  |  |  | AMHS | all VOs from the AMHS SARPs support this requirement | P |
|  |  | 4 | ULA | all VOs from the ULA SARPs support this requirement | P |
|  |  | 5 | ICS | all AVOs from the ICS SARPs support this requirement | P |
| 1.2.2 | When aid to air traffic services, conform with provision of this appendix | 2 | CM | all VOs from the CM SARPs support this requirement | P |
|  |  |  | ADS | all VOs from the ADS SARPs support this requirement | P |
|  |  |  | CPDLC | all VOs from the CPDLC SARPs support this requirement | P |
|  |  |  | FIS | all VOs from the FIS SARPs support this requirement | P |
|  |  | 3 | AIDC | all VOs from the AIDC SARPs support this requirement | P |
|  |  |  | AMHS | all VOs from the AMHS SARPs support this requirement | P |
|  |  | 4 | ULA | all VOs from the ULA SARPs support this requirement | P |
|  |  | 5 | ICS | all AVOs from the ICS SARPs support this requirement | P |
| 1.2.3 | Use on the basis of regional air navigation agreements | 2 | CM | all VOs from the CM SARPs support this requirement | P |
|  |  |  | ADS | all VOs from the ADS SARPs support this requirement | P |
|  |  |  | CPDLC | all VOs from the CPDLC SARPs support this requirement | P |
|  |  |  | FIS | all VOs from the FIS SARPs support this requirement | P |
|  |  | 3 | AIDC | all VOs from the AIDC SARPs support this requirement | P |
|  |  |  | AMHS | all VOs from the AMHS SARPs support this requirement | P |
|  |  | 4 | ULA | all VOs from the ULA SARPs support this requirement | P |
|  |  | 5 | ICS | all AVOs from the ICS SARPs support this requirement | P |
| 1.2.5 | Model of the ATN | 2 | CM | all VOs from the CM SARPs support this requirement | P |
|  |  |  | ADS | all VOs from the ADS SARPs support this requirement | P |
|  |  |  | CPDLC | all VOs from the CPDLC SARPs support this requirement | P |
|  |  |  | FIS | all VOs from the FIS SARPs support this requirement | P |
|  |  | 3 | AIDC | all VOs from the AIDC SARPs support this requirement | P |
|  |  |  | AMHS | all VOs from the AMHS SARPs support this requirement | P |
|  |  | 4 | ULA | all VOs from the ULA SARPs support this requirement | P |
|  |  | 5 | ICS | all AVOs from the ICS SARPs support this requirement | P |
| 1.3.1 | OSI Standards | 2 | CM | all VOs from the CM SARPs support this requirement | P |
|  |  |  | ADS | all VOs from the ADS SARPs support this requirement | P |
|  |  |  | CPDLC | all VOs from the CPDLC SARPs support this requirement | P |
|  |  |  | FIS | all VOs from the FIS SARPs support this requirement | P |
|  |  | 3 | AIDC | all VOs from the AIDC SARPs support this requirement | P |
|  |  |  | AMHS | all VOs from the AMHS SARPs support this requirement | P |
|  |  | 4 | ULA | all VOs from the ULA SARPs support this requirement | P |
|  |  | 5 | ICS | all AVOs from the ICS SARPs support this requirement | P |
| 1.3.2 | Future Migration | 2 | CM | TV08 | P |
|  |  |  | ADS | TV08 | P |
|  |  |  | CPDLC | TV08 | P |
|  |  |  | FIS | TV08 | P |
|  |  | 3 | AIDC | TV08 | P |
|  |  |  | AMHS | TV08 | P |
|  |  | 4 | ULA | TV08 | P |
| 1.3.3 | AFTN Transition to ATN | 3 | AMHS | all VOs from the AMHS SARPs support this requirement | P |
| 1.3.4 | ATC instructions by ATC authority | 2 | CM | SVO1, FVO1, FVO2, FVO3, FVO4, FVO5, TVO1, TVO2, TVO3, TVO4, TVO6, TVO7, TVO10, TVO11 | P |
|  |  |  | ADS | SVO1, FVO1, FVO2, FVO3, FVO4, FVO5, TVO1, TVO2, TVO3, TVO4, TVO6, TVO7, TVO10, TVO11 | P |
|  |  |  | CPDLC | SVO1, FVO1, FVO2, FVO3, FVO4, FVO5, TVO1, TVO2, TVO3, TVO4, TVO6, TVO7, TVO10, TVO11 | P |
|  |  |  | FIS | SVO1, FVO1, FVO2, FVO3, FVO4, FVO5, TVO1, TVO2, TVO3, TVO4, TVO6, TVO7, TVO10, TVO11 | P |
|  |  | 3 | AIDC | SVO1, FVO1, FVO2, FVO3, FVO4, FVO5, TVO1, TVO2, TVO3, TVO4, TVO6, TVO7, TVO10, TVO11 | P |
| 1.3.5 | Policy Based Routing | 2 | CM | SVO1, FVO1, FVO2, FVO3, FVO4, FVO5, FVO6, TVO1, TVO2, TVO3, TVO4, TVO6, TVO7, TVO10, TVO11 | P |
|  |  |  | AIDC | SVO1, FVO1, FVO2, FVO3, FVO4, FVO5, FVO6, TVO1, TVO2, TVO3, TVO4, TVO6, TVO7, TVO10, TVO11 | P |
|  |  | 3 | AHMS | SVO1, FVO1, FVO2, FVO3, FVO4, FVO5, FVO6, TVO1, TVO2, TVO3, TVO4, TVO6, TVO7, TVO10, TVO11 | P |
|  |  | 5 | ICS | VO_101, AVO_102, AVO_103, AVO_104, AVO_105, AVO_106, AVO_121, AVO_122, AVO_201, AVO_205 | P |
| 1.3.6 | Authorized Paths | 3 | AMHS | SVO1, FVO1, FVO2, FVO3, FVO4, FVO5, FVO6, TVO1, TVO2, TVO3, TVO4, TV05, TVO6, TVO7, TVO10, TVO11 | P |
|  |  | 5 | ICS | AVO_101, AVO_102, AVO_103, AVO_201, AVO_240, AVO_242, AVO_303, AVO_310, AVO_313 | P |
| 1.3.7 | ASTC Traffic Classes | 5 | ICS | AVO_240, AVO_303, AVO_310, AVO_313 | P |
| 1.3.8 | Communications Priorities | 3 | AMHS | SVO1, FVO1, FVO2, FVO3, FVO4, FVO5, FVO6, TVO1, TVO2, TVO3, TVO4, TV05, TVO6, TVO7, TVO10, TVO11 | P |
|  |  | 5 | ICS | AVO_312 | P |
| 1.3.9 | Application exchange | 2 | CM | SVO1, FVO1, FVO2, FVO3, FVO4, FVO5, FVO6, TVO1, TVO2, TVO4, TVO7, TVO9, TVO10, TVO11 | P |
|  |  |  | ADS | SVO1, FVO1, FVO2, FVO3, FVO4, FVO5, FVO6, TVO1, TVO2, TVO4, TVO7, TVO9, TVO10, TVO11 | P |
|  |  |  | CPDLC | SVO1, FVO1, FVO2, FVO3, FVO4, FVO5, FVO6, TVO1, TVO2, TVO4, TVO7, TVO9, TVO10, TVO11 | P |
|  |  |  | FIS | SVO1, FVO1, FVO2, FVO3, FVO4, FVO5, FVO6, TVO1, TVO2, TVO4, TVO7, TVO9, TVO10, TVO11 | P |
|  |  |  | AMHS | SVO1, FVO1, FVO2, FVO3, FVO4, FVO5, FVO6, TVO1, TVO2, TVO4, TVO7, TVO9, TVO10, TVO11 | P |
|  |  |  | AIDC | SVO1, FVO1, FVO2, FVO3, FVO4, FVO5, FVO6, TVO1, TVO2, TVO4, TVO7, TVO9, TVO10, TVO11 | P |
|  |  |  | ICS | AVO_240 | P |
| 1.3.10 | Lack of Path Notification | 3 | AHMS | SVO1, FVO1, FVO2, FVO3, FVO4, FVO5, FVO6, TVO1, TVO2, TVO4, TVO7, TVO9, TVO10, TVO11 | P |
|  |  | 5 | ICS | AVO_249, AVO_301, AVO_303 | P |
| 1.3.11 | Unambiguous Addressing | 2 | CM | SVO1, FVO1, FVO2, FVO3, FVO4, FVO5, TVO1, TVO2, TVO4, TVO7, TVO9, TVO10, TVO11 | P |
| 1.3.12 | Originator Identification | 2 | CM | SVO1, FVO1, FVO2, FVO3, FVO4, FVO5, FVO6, TVO1, TVO2, TVO4, TVO7, TVO9, TVO10, TVO11 | P |
|  |  |  | ADS | SVO1, FVO1, FVO2, FVO3, FVO4, FVO5, FVO6, TVO1, TVO2, TVO4, TVO7, TVO9, TVO10, TVO11 | P |
|  |  |  | CPDLC | SVO1, FVO1, FVO2, FVO3, FVO4, FVO5, FVO6, TVO1, TVO2, TVO4, TVO7, TVO9, TVO10, TVO11 | P |
|  |  |  | FIS | SVO1, FVO1, FVO2, FVO3, FVO4, FVO5, FVO6, TVO1, TVO2, TVO4, TVO7, TVO9, TVO10, TVO11 | P |
|  |  | 3 | AMHS | SVO1, FVO1, FVO2, FVO3, FVO4, FVO5, FVO6, TVO1, TVO2, TVO4, TVO7, TVO9, TVO10, TVO11 | P |
|  |  |  | AIDC | SVO1, FVO1, FVO2, FVO3, FVO4, FVO5, FVO6, TVO1, TVO2, TVO4, TVO7, TVO9, TVO10, TVO11 |  |
| 1.3.13 | Addressing and name Assignments | 2 | AMHS | SVO1, FVO1, FVO2, FVO3, FVO4, FVO5, FVO6, TVO1, TVO2, TVO4, TVO7, TVO9, TVO10, TVO11 | P |
|  |  | 5 | ICS | AVO_101, AVO_102, AVO_121, AVO_248 | P |
| 1.3.14 | Fixed and Mobile Systems | 5 | ICS | AVO_201, AVO_202, AVO_203, AVO_204, AVO_311, AVO_421 | P |
| 1.3.15 | Standardized Mobile Networks | 5 | ICS | AVO_111, AVO_112, AVO_113, AVO_114 | P |
| 1.3.16 | Limited Bandwidth | 2 | CM | TVO9 | P |
|  |  |  | ADS | TVO9 | P |
|  |  |  | CPDLC | TVO9 | P |
|  |  |  | FIS | TVO9 | P |
|  |  | 4 | ULA | TVO9 | P |
|  |  | 5 | ICS | AVO_242, AVO_311, AVO_408, AVO_446, AVO_447, AVO_448, AVO_449, AVO_450, AVO_454, AVO_455, AVO_460, AVO_470, AVO_471, AVO_472, AVO_473 | P |
| 1.3.17 | Aircraft IS to Concurrent Subnetworks | 5 | ICS | AVO_104, AVO_105, AVO_111, AVO_112, AVO_113, AVO_114, AVO_241, AVO_303, AVO_429 | P |
| 1.3.18 | Aircraft IS to Multiple Ground IS | 5 | ICS | AVO_103, AVO_104, AVO_105, AVO_106, AVO_122, AVO_231, AVO_240 | P |
| 1.3.19 | Exchange of address | 2 | CM | SVO1, FVO1, FVO2, FVO3, FVO4, FVO5, TVO1, TVO2, TVO4, TVO7, TVO9, TVO10, TVO11 | P |
|  |  |  | CPDLC | SVO1, FVO1, FVO2, FVO3, FVO4, FVO5, TVO1, TVO2, TVO4, TVO7, TVO9, TVO10, TVO11 | P |
|  |  | 3 | AIDC | SVO1, FVO1, FVO2, FVO3, FVO4, FVO5, TVO1, TVO2, TVO4, TVO7, TVO9, TVO10, TVO11 | P |
|  |  | 4 | ULA | SVO1, FVO1, FVO2, FVO3, FVO4, FVO5, TVO1, TVO2, TVO4, TVO7, TVO9, TVO10, TVO11 | P |
| 1.3.20 | CM Primacy | 2 | CM | all VOs from the CM SARPs support this requirement | P |
| 1.3.21 | CM Association | 2 | CM | all VOs from the CM SARPs support this requirement | P |
| 1.3.22 | ADS Association | 2 | ADS | all VOs from the ADS SARPs support this requirement | P |
| 1.3.23 | CPDLC Association | 2 | CPDLC | all VOs from the CPDLC SARPs support this requirement | P |
| 1.3.24 | ATIS Association | 2 | FIS | all VOs from the FIS SARPs support this requirement | P |
| 1.3.25 | AMHS Association | 3 | AMHS | all VOs from the AMHS SARPs support this requirement | P |
| 1.3.26 | AIDC Association | 3 | AIDC | all VOs from the AIDC SARPs support this requirement | P |
| 1.3.27 | UTC Reference | 2 | CM | all VOs from the CM SARPs support this requirement | P |
|  |  |  | ADS | all VOs from the ADS SARPs support this requirement | P |
|  |  |  | CPDLC | all VOs from the CPDLC SARPs support this requirement | P |
|  |  |  | FIS | all VOs from the FIS SARPs support this requirement | P |
|  |  | 3 | AIDC | all VOs from the AIDC SARPs support this requirement | P |
|  |  |  | AMHS | all VOs from the AMHS SARPs support this requirement | P |
|  |  | 4 | ULA | all VOs from the ULA SARPs support this requirement | P |
|  |  | 5 | ICS | all AVOs from the ICS SARPs support this requirement | P |
| 1.3.28 | Integrity |  |  | DYNAMIC VO |  |
| 4.1 | CM Application | 2 | CM | all VOs from CM SARPs support this requirement | P |
| 4.2.1 | ADS Application | 2 | ADS | all VOs from ADS SARPs support this requirement | P |
| 4.2.2 | CPDLC Application | 2 | CPLDC | all VOs from CPDLC SARPs support this requirement | P |
| 4.2.3 | FIS Application1 | 2 | FIS | all VOs from FIS SARPs support this requirement | P |
| 4.3.1 | AIDC Application | 3 | AIDC | all VOs from AIDC SARPs support this requirement | P |
| 4.3.2 | AMHS Application | 3 | AMHS | all VOs from AMHS SARPs support this requirement | P |
| 5.1 | ULA Communication Service | 4 | ULA | all VOs from ULA SARPs support this requirement | P |
| 5.2 | ICS Communication Service | 5 | ICS | all AVOs from ICS SARPs support this requirement | P |
| 6. | Naming and Addressing | 4 | ULA | all VOs from ULA SARPs support this requirement | P |
|  |  | 5 | ICS | all AVOs from ICS SARPs support this requirement | P |

6.2	Dynamic Validation Objectives

Certain validation objectives can only be achieved on the basis of static validation objectives previously achieved.  The dynamic validation objectives can only be achieved on the basis of interoperability testing.

6.2.1	System Level Requirement 1.3.7 for ATSC Traffic Classes

This dynamic system level requirement has two aspects that have been evaluated in the course of the ATN SARPs validation.  First, ATSC traffic classes A through H are defined by the ATN SARPs in terms of the ATN end-to-end transit delay at 95% probability.  Transit delay values are specified for ATSC traffic classes B through H while the definition of the value for ATSC traffic class A is reserved.  The specific values used within the SARPs for ATSC traffic classes B through H were based on operational requirements developed by the ADS Panel.  The validation of this system level requirement has included an analysis, based on simulation results as well as test results, that has quantified the expected ATN end-to-end transit delays for ATSC traffic when each of the ICAO specified mobile air-ground subnetworks is used.  Specifically, the overall ATN end-to-end transit delays were evaluated when VHF Data Link (VDL - Mode 2), AMSS, and Mode S subnetworks were used within any overall ATN environment.  This analysis has concluded that the range of values specified within the ATN SARPs for the transit delays associated with ATSC traffic classes B through H are appropriate as there are specific configurations of the ATN that when used in certain operational environments are predicted to offer end-to-end performance consistent with specified ATSC traffic class requirements.

The second aspect of this system level requirement relates to the ability of the Internet Communications Service (ICS) to make routing decisions based on the specified ATSC traffic class.  The validation activities relative to the ICS area of the ATN SARPs have validated the ability ATN intermediate systems to correctly select a routing path consistent with a user specified ATSC traffic class.  The success of this aspect of the ICS validation is documented in the validation report for the ICS.

6.2.2	System Level Requirement 1.3.28: Integrity

TBD