ADSP WG A/B

WP__________

AUTOMATIC DEPENDENT SURVEILLANCE PANEL

JOINT MEETING of WORKING GROUPS A & B

Gold Coast, Australia  4-7 July 1997

Agenda Item 2:  Operational Requirements Considerations for Accommodating Differing Message Sets in Ground and Airborne Systems

GROUND ACCOMMODATION OF FANS-1/A ADS

USING THE SARPs APPROACH

Prepared by G. Anderson, J. Hamelink

Presented by D. Cherry

Summary This paper presents the results of the work done on ADS accommodation as a result of the Special Joint Transition Meeting held in Toulouse, France 5-9 May 1997.

# Introduction

## This paper presents the results of the work done by the US FAA on ground accommodation of FANS-1/A ADS using the SARPs approach.  This approach was agreed as the choice to be analyzed as an action from the Special Joint Transition Meeting held in Toulouse, France from 5-9 May 1997.

Note:  Information contained in this paper should not be interpreted as an 	endorsement of the SARPs Approach.  Decisions to accommodate, or not, should 	be based all aspects of implementation (e.g. cost benefit, operational and 	certification issues, equipage rates, etc.)

## The assumptions used in compiling this paper remain the same as the input to the Toulouse Meeting.  These assumptions are:

- the goal is to transition to uniform SARPs compliance,

- the ground system handles both FANS-1/A aircraft and SARPs compliant aircraft,

- the differences in operating or handling FANS-1/A and/or SARPs compliant aircraft should be minimized for both the pilot and controller (operational procedures and HCI),

- the outlined approach mitigates aircraft equipage changes,

- the ground system is capable of processing ALL downlink messages from either a FANS-1/A or a SARPs aircraft,

- the accommodation restrictions apply only to ground initiated uplink messages (contract establishment, modification, and cancellation),

- all uplink messages leave the ground as either a FANS-1/A message or a SARPs message, and

- the communication infrastructure is inherently different, only the messaging capabilities are addressed in this paper.

Note: “Messaging Capability” includes the sending and canceling of ADS 	contracts and the receipt of acknowledgments and ADS Reports as well as ADS 	message range and resolution.

# Discussion

## The Appendix to this paper details the ground system automation requirements for accommodation of FANS-1/A ADS by a SARPs compliant ground system

## For the purpose of this paper, the distinction between a FANS-1/A and SARPs aircraft is based on the messaging capability, not on its communication infrastructure.  For example, if an aircraft used the FANS-1/A (ACARS based) communication system, but had implemented the SARPs messaging capability it is considered a SARPs aircraft.

## Although there are (perhaps significant) implications on certification based on where the ground accommodation occurs (i.e., in-house or gateway) this area is not covered in this paper. The Appendix just describes the “what’s required” impact.

## The impact columns in the Appendix should be thought of as “lines of code” needed for a ground system to accommodate FANS-1/A using the SARPs Approach.

## Scenarios involving ground systems that handle only FANS-1/A, or that handle only SARPs aircraft are not considered here.  In this case, aircraft could “dual stack” and the converse of the scenarios presented here could be examined.

## As a reminder of what the SARPs Approach entails, the implications from the previous paper at Toulouse are:

- the SARPs application makes maximum use of messaging,

- the automation will have to tailor for FANS-1/A and SARPs differences (e.g., range and resolution, event, etc.), and

- when the ground system no longer supports FANS-1/A, the ground system application need not be further modified.

# Recommendation

## The meeting is invited to:

- consider the information contained in this paper in developing recommendations for transition strategies, operational concepts, and requirements,

- use this information, as modified by the meeting, to conduct cost-benefit analyses from which further recommendations may be established,

- determine if an alternative accommodation approach is more cost beneficial,

- consider how long accommodation of FANS-1/A is required in light of institutional issues (e.g., dual system operation and maintenance costs), and

- schedule another Special Joint Meeting of Working Groups A & B for the purposes of progressing this issue as the only agenda item.

Appendix

Accommodating FANS-1/A ADS Using the SARPs Approach

# ADS Demand Contract

| ADS SARPs Demand Contract PDU | FANS-1/A Processing/Impact Periodic Contract Request (Rate 0) | FANS-1/A |
| --- | --- | --- |
| (ADSGroundPDU [3] Demand Contract) | Set for FANS-1/A - ADS-Request Tag - ADS Contract Request Number - Reporting Rate Tag (0) | - Basically same - No Extended Projected Profile in FANS-1/A - Units of measurement same, range and resolution differences (in reports) |
| - Airframe-id | Set for FANS-1/A - Airframe ID Group Tag - Modulus = 0 |  |
| Projected-profile | Set for FANS-1/A - Predicted Route Group Tag - Modulus = 0 |  |
| Ground-vector | Set for FANS-1/A - Earth Reference Group Tag - Modulus = 0 |  |
| Air-vector | Set for FANS-1/A - Air Reference Group Tag - Modulus = 0 |  |
| Weather | Set for FANS-1/A - Meteorological Group Tag - Modulus = 0 |  |
| Projection time | Set for FANS-1/A - Aircraft Intent Group Tag - Modulus = 0 |  |
| Extended-projected-profile-request | Locally Generate an NCN for this data type |  |

# Periodic Contract (Uplink)

| ADS SARPs | FANS-1/A Processing/Impact | FANS-1/A |
| --- | --- | --- |
| (ADSGroundPDU [6] Periodic Contract) | Set for FANS-1/A - ADS-Request Tag - ADS Contract Request Number | - Flight -ID would never be requested |
| - Reporting-interval | Set for FANS-1/A - Reporting Rate Tag - Scale Factor - Rate based on reporting interval set by SARPs - For reporting interval 70-120 locally generate a NCN and set interval 69 minutes for FANS | - Reporting rate for FANS-1/A be set to match SARPs for values up to 69 minutes. FANS-1/A does not permit whole minute interval > 69 minutes |
| - Airframe-id-modulus | Set for FANS-1/A - Airframe ID Group Tag - Modulus | - Modulus same, result based on actual report rate. |
| - Projected-profile-modulus | Set for FANS-1/A - Predicted Route Group Tag - Modulus | - No extended projected profile available from FANS-1/A |
| - Ground-vector modulus | Set for FANS-1/A - Earth Reference Group Tag - Modulus |  |
| Air-vector-modulus | Set for FANS-1/A - Air Reference Group Tag - Modulus |  |
| Weather-modulus | Set for FANS-1/A - Meteorological Group Tag - Modulus |  |
| Short-term-intent (projection time) | Set for FANS-1/A - Aircraft Intent Group Tag - Modulus |  |
| Extended-projected-profile modulus(modulus, extended-projected-profile-request (time-interval, or number of way points)) | Cannot request Locally generate NCN |  |

# ADS Event Contract (Uplink)

| ADS SARPs Event Contract | FANS-1/A Processing/Impact ADS Event Contract Request | FANS-1/A |
| --- | --- | --- |
| (ADSGroundPDU [4] Event Contract) | Set for FANS-1/A - ADS-Request Tag - ADS Contract Request Number | - SARPs has more events than FANS-1/A so locally NCN non FANS-1/A events |
| - Lateral deviation change | Set for FANS-1/A - Lateral Deviation Tag - Lateral Deviation Threshold from Lateral deviation change - For values <0.125 set to 0.125 and locally notify - Re-arm each time event occurs | - Except for Lateral Deviation Change and Vertical Rate Change SARPs events request range and resolution within FANS-1/A - FANS-1/A must be re-armed when event occurs |
| - Vertical rate change | Set for FANS-1/A - Vertical rate change tag - Vertical rate change threshold from vertical rate change - Locally generate NCN for event if vertical rate change - <-8192 ft/min or >8192 ft/min - Re-arm each time event occurs | - Lateral Deviation and Level/Altitude range change event generate only Basic data in FANS-1/A instead of Basic and Ground Vector in SARPs. |
| - Level range: ceiling, floor | Set for FANS-1/A - Altitude Range Tag - Ceiling from level range ceiling - Floor from level range floor - Re-arm each time event occurs |  |
| Way-point-change | Set for FANS-1/A - Waypoint Change Tag |  |
| Air-speed-change: | - Locally generate NCN for event |  |
| Ground-speed-change | - Locally generate NCN for event |  |
| Heading-change | - Locally generate NCN for event |  |
| Extended-projected-profile-change: time-interval, number-of-way-points | - Locally generate NCN for event |  |
| FOM-change | - Locally generate NCN for event |  |
| Track-angle-change | - Locally generate NCN for event |  |
| Level-change | - Locally generate NCN for event |  |

# ADS Cancel Contract  (Uplink)

| ADS SARPs Cancel Contract | FANS-1/A Processing/Impact Cancel Contract Request | FANS-1/A |
| --- | --- | --- |
| ADS-Cancel-Contact PDU [1] Cancel Contract | Set for FANS-1/A - ADS-Request Tag (Cancel Contract) | - Emergency cancellation is defined differently for SARPs and FANS-1/A |
| - event-contract period-contract | Set for FANS-1/A - ADS Contract Request Number as event or periodic as appropriate |  |
| Cancel Contact Indication (local) | Set for FANS-1/A - ADS Contract Request Number as emergency |  |

# ADS Cancel All Contracts  (Uplink)

| ADS SARPs Cancel All Contracts | FANS-1/A Processing/Impact Cancel All Contracts and Terminate Connection Request | FANS-1/A |
| --- | --- | --- |
| ADS-Cancel-All-Contracts PDU [0] | Set for FANS-1/A - ADS-Request Tag to Cancel all contracts | Same |

# Modify Emergency Contract (Uplink)

| ADS SARPs Modify Emergency Contract PDU | FANS-1/A Processing/Impact Emergency Periodic Contract Request | FANS-1/A |
| --- | --- | --- |
| (ADSGroundPDU [5] Modify Emergency) | Set for FANS-1/A - ADS-Request Tag (modify emergency) - ADS Contract Request Number | - Contents no longer modifiable for FANS-1/A |
| - Reporting-interval | Set for FANS-1/A - Reporting Rate Tag - Scale Factor - Rate |  |

# ADS Cancel Emergency Acknowledgment (Uplink)

| ADS SARPs Cancel Emergency Acknowledgment | FANS-1/A Processing/Impact | FANS-1/A |
| --- | --- | --- |
| ADSGroundPDU [2] | not available |  |

# Uplink Parameters Accommodation

| ADS SARPs | FANS-1/A | FANS-1/A Processing/Impact |
| --- | --- | --- |
| ETA - time Hours(0..23) Minutes(0..59) Seconds (0..59) | ETA - Range: 0 - 16382 - Resolution: 1 second | - SARPs gives ETA as a time of day - What does FANS do, give time from now? - Range about 4.55 hours |
| - Extended Projected Profile Time - Range: 15 - 1200 - Resolution: 15 minutes Number of Waypoints - Range: 1 - 128 - Resolution: 1 | not provided | - Locally generate NCN as part of Demand or Periodic Contract response |
| - Ground speed change - Range: . 0 - 500 - Resolution: 1 knot | not provided | - Locally generate NCN as part of Event Contract Response |
| - Heading (degrees direction) - Range: 0.1 to 360 - Resolution: 0.1 degrees | True Heading - Range: -180 -+ 179.912 - Resolution: 0.001 degrees | - SARPs resolution restricts - Basically same |
| - Heading Change (degrees direction) - Range: 0.1 to 360 - Resolution: 0.1 degrees | not provided | - Locally generate NCN as part of Event Contract Response |
| - IAS - Range: 0 - 1100 - Resolution: 1 knot | not provided | - Locally generate NCN as part of Event Contract Response |
| - IAS change 1-700 1 speed unit not yet changed in SARPs - defect | not provided | - Locally generate NCN as part of Event Contract Response |
| - Lateral Change - Range: 0 - 31.875 - Resolution: 0.125 NM | Lateral Deviation Change Threshold - Range: 0.125 to 31.875 - Resolution: 0.125 NM | none |
| Level - Range: -750 to +100000 - Resolution: 10 feet | Altitude - Range: -131,072 to + 131,068 - Resolution: feet | - SARPs range restricts operationally rather than FANS-1/A bit boundaries - SARPs resolution restrictive |
| - Level - Range: -750 to +100000 - Resolution: 10 feet | Ceiling Altitude - Range: -131,072 to + 131,068 - Resolution: feet | - SARPs range restricts both upper and lower - SARPs resolution restrictive |
| - Level - Range: -750 to +100000 - Resolution: 10 feet | Floor Altitude - Range: -131,072 to + 131,068 - Resolution: feet | - SARPs range restricts operationally rather than FANS-1/A bit boundaries - SARPs resolution restrictive |
| - Level change (level) - Range: 1. - 5000 - Resolution:10 feet | not provided | - Locally generate NCN as part of Event Contract Response |
| - Mach - Range: 0.5. - 4 - Resolution: 0.001 Mach | Mach - Range: 0 - 4.095 - Resolution: 0.001 Mach | - SARPs range restrictive |
| - Mach number change - Range: .005 - 1.275 - Resolution: 0.005 | not provided | - Locally generate NCN as part of Event Contract Response |
| - Modulus - Range: 1 - 255 - Resolution: 1 | Modulus - Range: 1 - 255 - Resolution: 1 | same - Can no longer send modulus of 0 to cancel as in FANS-1/A |
| - Projection Time - Range: 1 - 240 - Resolution: 1 minute | Aircraft Intent Projection Time - Range: 1-255 - Resolution: 1 minute | - SARPs upper range restrictive - SARPs and FANS-1/A relative to current time stamp |
| - Reporting Interval Seconds - Range: 1 - 59 - Resolution: 1 second Minutes - Range: 1 - 120 - Resolution: 1 minute | Report interval - Range: 1 - 4160 - Resolution: 1 second | - Above 1 minute SARPs resolution restrictive - Largest interval that can be accepted by FANS-1/A is 69 minutes - For reporting interval 70-120 locally generate a NCN and set interval 69 minutes for FANS |
| - TimeStamp - date year (1996..2095) month(1..12) day(1..31) - time Hours(0..23) Minutes(0..59) Seconds (0..59) | Time Stamp - Range: 0 - 3599.875 - Resolution: seconds | - SARPs time stamp consists of date and time - FANS/1-A timestamp provides just time minutes and seconds relative to current hour |
| - Track Angle Change (degrees direction) - Range: 0.1 to 360 - Resolution: 0.1 degrees | True Track Angle - Range: -180 -+ 179.912 - Resolution: 0.001 degrees | - SARPs resolution restrictive |
| - Vertical rate change - Range:-30000 -+ 30000 - Resolution:10 ft/min | Vertical Rate - Range: -32,752 -+ 32,752 - Resolution: 16 ft/min | - For resolution < 16 provide local NCN? |
| - VerticalRateChange - Range: -30000 to +30000 - Resolution: 10 feet per minute | Vertical Rate Change Threshold - Range: -8192 to + 8128 - Resolution: 64 feet per minute | - For thresholds < -8192 or > 8128 generate local NCN? - For resolution < 64 generate local NCN? |

ADSP WG A/B

WP__________