ATNP WG1SG2

WP1012

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL (ATNP)

WG1 – Systems planning and concept working group

SG2 - SECURITY

7-9 December 1998

Phoenix, AZ, USA

Agenda Item 3.5

WG1 SG2 / WG3 SG2

Co-ordination

Prepared by Frederic Picard

Presented by M. Bigelow

SUMMARY This paper identifies several items that require co-ordination between WG1SG2 and WG3SG2.

There is an action item on WG3 sub-groups, and in particular on SG2 (air-ground applications), to review the SARPs and GM on ATN Security.

In order to allow us to do that, we need the latest version of both SARPs and GM documents. I was told that the document distributed in Bordeaux (WP905) has been reviewed in detail by your sub-group and has been partially modified, based mainly on Ian's comments. Could you send to us or make available on the CENA server the amended version ? Thanks a lot. Next SG2 meeting is 8 to 10 december in the US and we need to circulate the documents before if we want to have comments and constructed discussion.

Based on the Bordeaux version, I think co-ordination is needed between WG1 SG2 and WG3 SG2 at least for the following topics:

1) Security scenario 1 (item 9 of section 4.3.1) proposes that CPDLC sends security information related to the NDA with the CPLDC NDA message. My view on that is that CM is the appropriate application to exchange this information (during the CM-contact/CM-logon exchange) and not CPDLC. Even if in early ATN implementations this would help a lot...

2) Impact on CM. One of the proposal is to use the CM-logon/CM-update to uplink the public keys of the ground applications. This would require a change of the current CM message defition (ASN.1). If this solution is accepted, a formal definition of the security information to be sent by CM need to be sent to WG3 SG2. Another solution would be to replace CM by a X500 DUA/DSA solution. The decision to go for this drastic upgrade should be taken before WG3 SG2 define a version 2 CM application.

3) Impact on the current applications (excepted CM). Our understanding today of the way security is handled by applications is that adding security in the applications do not impact at all the current application protocols. The application entities could be modified in such a way to "pass-through" the security parameters between the secured dialogue service provider and the application-users. Application messages and procedures are not impacted by security. Item 1) above proves that this assumption is not true!