# DEFINITION OF THE SSO

## SSO Funtions

Note 1. – All ATN Security Primitives referenced within this section are defined in Section 8.5.

Note 2. – The term ‘Initiator’ is used to describe the entity that initiates a request for a secured dialogue.  In most cases, this will be the Calling Peer for a D-Start request/indication.  The term ‘Responder’ is used to describe the entity that responds to a request for a secured dialogue.  In most cases, this will be the Called Peer for a D-Start request/indication.

Editor’s Note. – The terms Calling Peer and Called Peer are used confusingly throughout and will be cleaned up.  In some instances, the roles of Initiator and Responder are unimportant, but in most, they are important.  This needs to be clarified.  Some functions are meant to be called by both the Initiator and Responder.  We need a mechanism to handle the Initiator and Responder in a generic way for these functions.

### SSO-Sign function

Note 1.— This function is activated with the atnSignatureST parameter described in 8.6.2.  The inputs are the STATIC-UNPROT-PARM and DYNAMIC-UNPROT-PARM fields.  When SSO-Sign is successfully completed, the X-FORMED-DATA-TYPE field will contain a valid signature.

Note 2. -- The atnSignatureST is reproduced below.

atnSignatureST	SECURITY-TRANSFORMATION ::= {

IDENTIFIER	{iso(1) identified-organization(3) icao(27) atn-security-requirements(4) modules(2) atnSecurityTransformations(2) atn-signature(1)}

INITIAL-ENCODING-RULES	{joint-iso-itu-t asn(1) packed-encoding(3) canonical(1) unaligned(1)}

STATIC-UNPROT-PARM	ATNSignatureStaticParameters

DYNAMIC-UNPROT-PARM	ATNSignatureDynamicParameters

X-FORMED-DATA-TYPE	ATNSignature

}

ATNSignatureStaticParameters ::= SEQUENCE {

intiatorId	ATNPeerId,

responderId	ATNPeerId

}

ATNSignatureDynamicParameters ::= SEQUENCE {

unprotectedData	EXTERNAL

}

ATNSignature ::= SEQUENCE {

AlgorithmId	AlgorithmIdentifier	OPTIONAL,

validity		GeneralizedTime,

appendix	BIT STRING (CONSTRAINED BY {

-- The ATN signature is generated following the specification

-- of the atnSignature security transformation defined in 8.6

-- and is the XFormedDataType resulting from this transformation

})

}

Note 3. – For the purposes of this section, the following shorthand notation is used:

- Intiator refers to ATNSignatureStaticParameters.initiatorId,

- Responder refers to ATNSignatureStaticParameters.responderId,

- User Data refers to ATNSignatureDynamicParameters.unprotectedData, and

- ATN Signature refers to atnSignatureST.X-FORMED-DATA-TYPE.

#### Upon activation of the SSO-Sign function, the SSO shall perform the sequence of steps identified in this section.

##### The SSO shall delete the shared public value, the session key, and the 16-bit unsigned counters for the Initiator and Responder.

##### The SSO shall retrieve the Initiator’s signature private key (dsig).

##### The SSO shall generate a Time Field using the current system time.

##### The Time Field shall conform to the abstract syntax GeneralizedTime.

##### The SSO shall construct the SignData from the Initiator, Responder, Time Field generated above and the User Data.

##### The SignData shall conform to the Canonical PER encoding of the abstract syntax ASPSignData.

Note. – The ASN.1 for ASPSignData is reproduced below.

ASPSignData ::= SEQUENCE {

initiatorId	ATNPeerId,

responderId	ATNPeerId,

timeField	GeneralizedTime,

userData	EXTERNAL

}

##### The SSO shall generate a signature over the SignData by invoking ASP as defined in Section 8.5.

##### When the output of ASP is ‘invalid’, the SSO shall output ‘invalid’.

Note. – The method of outputing ‘invalid’ is a local matter.

##### When the output of ASP is a valid signature, the SSO shall:

- Set ATNSignature.validity to the Time Field generated above, and

- Set ATNSignature.appendix to the ASP returned signature.

##### The SSO shall retain the ATNSignature associated with the Initiator and Responder for use later in generating the shared public value for use with AKDF.

##### The SSO shall output the completed atnSignatureST.

### SSO-SignCheck function

Note 1.— This function is activated with the atnSignCheckST parameter described in 8.6.2.  The inputs are the STATIC-UNPROT-PARM and DYNAMIC-UNPROT-PARM fields.  When SSO-SignCheck is successfully completed, the X-FORMED-DATA-TYPE field be set to the abstract value SSOPositive.  An error with the input is indicated by setting the X-FORMED-DATA-TYPE field to the abstract value SSONegative.  Indication of local processing errors is a local matter.

Note 2. -- The atnSignCheckST is reproduced below.

atnSignCheckST	SECURITY-TRANSFORMATION ::= {

IDENTIFIER	{iso(1) identified-organization(3) icao(27) atn-security-requirements(4) modules(2) atnSecurityTransformations(2) atn-sign-check(2)}

INITIAL-ENCODING-RULES	{joint-iso-itu-t asn(1) packed-encoding(3) canonical(1) unaligned(1)}

STATIC-UNPROT-PARM	ATNSignCheckStaticParameters

DYNAMIC-UNPROT-PARM	ATNSignCheckDynamicParameters

X-FORMED-DATA-TYPE	ATNSSOResult

}

ATNSignCheckStaticParameters ::= SEQUENCE {

initiatorId	ATNPeerId,

responderId	ATNPeerId

}

ATNSignCheckDynamicParameters ::= SEQUENCE {

securityItem		ATNEstablish,

unprotectedData	EXTERNAL

}

ATNEstablish ::= SEQUENCE {

timeStamp		GeneralizedTime,

callingCertificate	CertificatePath	OPTIONAL,

atnSignature		ATNSignature

}

Note 3. – For the purposes of this section, the following shorthand notation is used:

- Initiator refers to ATNSignCheckStaticParameters.initiatorId,

- Responder refers to ATNSignCheckStaticParameters.responderId,

- User Data refers to ATNSignCheckDynamicParameters.unprotectedData, and

- Security Item refers to ATNSignCheckDynamicParameters.securityItem, and

- Time Field refers to ATNEstablish.timeStamp, and

- Certificate Path refers to ATNEstablish.callingCertificate, and

- ATN Signature refers to ATNEstablish.atnSignature, and

- Result refers to the atnSignCheckST.X-FORMED-DATA-TYPE.

#### Upon activation of the SSO-SignCheck function, the SSO shall perform the sequence of steps identified in this section.

##### The SSO shall verify that the Time Field is valid.

Editor’s Note. – How verify the time stamp?  I guess we can specify this as local implementation.

##### When the Security Item does not contain a Certificate Path, the SSO shall retrieve the Initiator’s signing public key certificate path.

Note. – The term Certificate Path is used to refer to the entity’s certificate as well as any required cross-certification certificate.

Editor’s Note. – Make sure the term cross-certification makes it to the SARPs definitions.

##### The SSO shall invoke SSO-CertificateCheck to verify the Initiator’s signing public key Certificate Path.

##### When the verification of the signing public key certificate path fails, the SSO shall output the completed atnSignCheckST with the Result set to abstract value Negative.

##### The SSO shall reconstruct the SignData from the Initiator, Responder, Time Field, and the User Data.

##### The SignData shall conform to the Canonical PER encoding of the abstract syntax ASPSignData.

Note. – The ASN.1 for SignData is reproduced below.

SignData ::= SEQUENCE {

callingPeerId	ATNPeerId,

calledPeerId	ATNPeerId,

timeField	GeneralizedTime,

userData	EXTERNAL

}

##### The SSO shall retrieve the received ATN Signature from the Security Item.

##### The SSO shall retain the received ATN Signature associated with the Initiator and Responder for use later in generating the shared public value X for use with AKDF.

##### The SSO shall retrieve the signing public key from the Initiator’s signing public key certificate.

##### The SSO shall verify the received ATN Signature by invoking AVP.

##### When the verification of the received ATN Signature fails, the SSO shall output the completed atnSignCheckST with the Result set to the abstract value ‘Negative’.

##### When the verification of the received ATN Signature succeeds, the SSO shall output the completed atnSignCheckST with the Result set to the abstract value ‘Positive’.

### SSO-ProtectSign function

Note 1.— This function is activated with the atnProtectSignST parameter described in 8.6.2.  The inputs are the STATIC-UNPROT-PARM and DYNAMIC-UNPROT-PARM fields, and the unprotected parameter of the X-FORMED-DATA-TYPE field.  When SSO-ProtectSign is successfully completed, the appendix parameter of the X-FORMED-DATA-TYPE field will contain a valid message authentication code.

Note 2. – The DYNAMIC-UNPROT-PARM field is used to indicate if the SSO should generate a random, non-repeating value so that SSO-ProtectSign can be applied in all cases.  The S-ASO control function will need to set this field to True when calling SSO-ProtectSign for use with SA-START.  In all other cases, this field should be set to False.

Note 3. -- The atnProtectSignST is reproduced below.

atnProtectSignST	SECURITY-TRANSFORMATION ::= {

IDENTIFIER	{iso(1) identified-organization(3) icao(27) atn-security-requirements(4) modules(2) atnSecurityTransformations(2) atn-protect-sign(4)}

INITIAL-ENCODING-RULES	{joint-iso-itu-t asn(1) packed-encoding(3) canonical(1) unaligned(1)}

STATIC-UNPROT-PARM	ATNProtectSignStaticParameters

DYNAMIC-UNPROT-PARM	ATNProtectSignDynamicParameters

X-FORMED-DATA-TYPE	ATNProtectSign

}

ATNProtectSignStaticParameters ::= SEQUENCE {

callingPeerId	ATNPeerId,

calledPeerId	ATNPeerId

}

ATNProtectSignDynamicParameters ::= SEQUENCE {

generateRandom	Boolean

}

Boolean ::= ENUMERATED {

True	(0),

False	(1)

}

ATNProtectSign ::= SEQUENCE {

algorithmId	AlgorithmIdentifier	OPTIONAL,

unprotected	EXTERNAL,

appendix	BIT STRING (CONSTRAINED BY {

-- The ATN signed data is the XFormedDataType generated

-- following the specification of the atnProtectSign security

-- transformation defined in 8.6

})

}

Note 4. – For the purposes of this section, the following shorthand notation is used:

- Calling Peer refers to ATNProtectSignStaticParameters.callingPeerId,

- Called Peer refers to ATNProtectSignStaticParameters.calledPeerId,

- User Data refers to ATNProtectSign.unprotected, and

- Generate Random Flag refers to ATNProtectSignDynamicParameters.generateRandom, and

- Appendix refers to ATNProtectSign.appendix.

#### Upon activation of the SSO-ProtectSign function, the SSO shall perform the sequence of steps identified in this section.

##### The SSO shall retrieve the session key for the Calling Peer and Called Peer.

##### The SSO shall retrieve the 16-bit unsigned counter for the ordered pair of Calling Peer and Called Peer.

Note. – a 16-bit unsigned counter is maintained for messages sent from Calling Peer to Called Peer and a separate 16-bit unsigned counter is maintained for messages sent from Called Peer to Calling Peer.

##### The SSO shall increment the retrieved 16-bit unsigned counter by 1.

Note. – An 16-bit unsigned counter allows for use of values 1 through 65,535, inclusive.  The value 0 is not used.

Editor’s Note. – We need to address counter rollover even though It is not likely that a given entity will initiate 65,535 exchanges to another given entity.

##### When the Generate Random Flag is set to the abstract value ‘True’, the SSO shall generate a Random Value.

Note. – This may be achieved using the ARVP from Section 8.5 if a true random value cannot be generated.

##### When the Generate Random Flag is set to the abstract value ‘True’, the SSO shall retain the Random Value for use later in generating the shared public value X for the Calling Peer and Called Peer.

##### When the Generate Random Flag is set to the abstract value ‘True’, the SSO shall retrieve the ATN Signature corresponding to the Calling Peer and Called Peer.

##### When the Generate Random Flag is set to the abstract value ‘True’, the SSO shall construct the MacData for the AMACP from the Calling Peer, Called Peer, the 16-bit unsigned counter from above, the User Data, the random value generated above, and the ATN Signature from above.

##### When the Generate Random Flag is set to the abstract value ‘False’, the SSO shall construct the MacData for the AMACP from the Calling Peer, Called Peer, the 16-bit unsigned counter from above, and the User Data.

##### The MacData shall conform to the Canonical PER encoding of the abstract syntax MacData.

Note. – The ASN.1 for MacData is reproduced below.

MacData ::= SEQUENCE {

callingPeerId	ATNPeerId,

calledPeerId	ATNPeerId,

counter	INTEGER,

userData	EXTERNAL,

random	INTEGER	OPTIONAL,

atnSignature	ATNSignature	OPTIONAL

}

Editor’s Note. – I changed the way the above fields are concatenated so that SSO-ProtectSign could be used generically.  (See W1S2W1406a, Section 3.3.1, Steps 5 and 8.)

##### The SSO shall generate a MAC over MacData by invoking AMACP.

##### When the output of AMACP is ‘invalid’, the SSO shall output ‘invalid’.

Note. – The method of returning ‘invalid’ is a local matter.

##### When the the output of AMACP is a valid MAC, the SSO shall:

- Set ATNProtectSign.appendix to the AMACP returned MAC, and

- When the default algorith for AMACP is not used, set ATNProtectSign.algorithmId to the Algorithm Identifier of the algorithm used.

Editor’s Note. – We need to specify how and when to set the optional algorithm identifier.

##### The SSO shall output the completed atnProtectSignST.

### SSO-ProtectSignCheck function

Note 1.— This function is activated with the atnProtectSignCheckST parameter described in 8.6.2.  The inputs are the STATIC-UNPROT-PARM and DYNAMIC-UNPROT-PARM fields.  When SSO-ProtectSignCheck is successfully completed, the X-FORMED-DATA-TYPE field be set to the abstract value SSOPositive.  An error with the input is indicated by setting the X-FORMED-DATA-TYPE field to the abstract value SSONegative.  Indication of local processing errors is a local matter.

Note 2. -- The atnProtectSignCheckST is reproduced below.

atnProtectSignCheckST	SECURITY-TRANSFORMATION ::= {

IDENTIFIER	{iso(1) identified-organization(3) icao(27) atn-security-requirements(4) modules(2) atnSecurityTransformations(2) atn-sign-check(2)}

INITIAL-ENCODING-RULES	{joint-iso-itu-t asn(1) packed-encoding(3) canonical(1) unaligned(1)}

STATIC-UNPROT-PARM	ATNProtectSignCheckStaticParameters

DYNAMIC-UNPROT-PARM	ATNProtectSignCheckDynamicParameters

X-FORMED-DATA-TYPE	ATNSSOResult

}

ATNProtectSignCheckStaticParameters ::= SEQUENCE {

callingPeerId	ATNPeerId,

calledPeerId	ATNPeerId

}

ATNProtectSignCheckDynamicParameters ::= SEQUENCE {

securityItem		ATNProtectSign,

receivedRandom	INTEGER	OPTIONAL

}

ATNProtectSign ::= SEQUENCE {

algorithmId	AlgorithmIdentifier	OPTIONAL,

unprotected	EXTERNAL,

appendix	BIT STRING (CONSTRAINED BY {

-- The ATN signed data is the XFormedDataType generated

-- following the specification of the atnProtectSign security

-- transformation defined in 8.6

})

}

Note 3. – For the purposes of this section, the following shorthand notation is used:

- Calling Peer refers to ATNProtectSignCheckStaticParameters.callingPeerId,

- Called Peer refers to ATNProtectSignCheckStaticParameters.calledPeerId,

- User Data refers to ATNProtectSign.unprotected, and

- Generate Random Flag refers to ATNProtectSignDynamicParameters.generateRandom, and

- Appendix refers to ATNProtectSign.appendix, and

- Random Value refers to ATNProtectSignCheckDynamicParameters.receivedRandom, and

- Result refers to the atnProtectSignCheckST.X-FORMED-DATA-TYPE.

#### Upon activation of the SSO-ProtectSignCheck function, the SSO shall perform the sequence of steps identified in this section.

##### When the SSO does not already have a session key for the Calling Peer and Called Peer, the SSO shall invoke the SSO-SessionKey function to generate the Session Key shared by the two peers.

##### When the Result of the SSO-SessionKey function is set to the abstract value ‘Negative’, the SSO shall output ‘invalid’.

Note. – The method of outputing ‘invalid’ is a local matter.

##### The SSO shall retrieve the 16-bit unsigned counter for the ordered pair of Calling Peer and Called Peer.

Note. – a 16-bit unsigned counter is maintained for messages sent from Calling Peer to Called Peer and a separate 16-bit unsigned counter is maintained for messages sent from Called Peer to Calling Peer.

##### The SSO shall increment the retrieved 16-bit unsigned counter by 1.

Note. – An 16-bit unsigned counter allows for use of values 1 through 65,535, inclusive.  The value 0 is not used.

Editor’s Note. – We need to address counter rollover even though it is not likely that a given entity will initiate 65,535 exchanges to another given entity.

##### When the Random Value is present in the input, the SSO shall retain the Random Value for use later in generating the shared public value X for the Calling Peer and Called Peer.

##### When the Random Value is present in the input, the SSO shall retrieve the ATN Signature corresponding to the Calling Peer and Called Peer.

Note. – The retrieved signature is associated with the ordered pair Calling Peer and Called Peer  of the atnSignatureST parameter to the SSO function SSO-Signature.  When atnProtectSignST is created the Calling Peer from the atnSignatureST becomes the Called Peer and the Called Peer from the atnSignatureST becomes the Calling Peer.

##### When the Random Value is present in the input, the SSO shall reconstruct the MacData for the AMACP from the Calling Peer, Called Peer, the 16-bit unsigned counter from above, the User Data, the Random Value, and the ATN Signature from above.

##### When the Random Value is not present in the input, the SSO shall construct the MacData for the AMACP from the Calling Peer, Called Peer, the 16-bit unsigned counter from above, and the User Data.

##### The MacData shall conform to the Canonical PER encoding of the abstract syntax MacData.

Note. – The ASN.1 for MacData is reproduced below.

MacData ::= SEQUENCE {

callingPeerId	ATNPeerId,

calledPeerId	ATNPeerId,

counter	INTEGER,

userData	EXTERNAL,

random	INTEGER	OPTIONAL,

atnSignature	ATNSignature	OPTIONAL

}

Editor’s Note. – I changed the way the above fields are concatenated so that SSO-ProtectSign could be used generically.  (See W1S2W1406a, Section 3.3.1, Steps 5 and 8.)

##### The SSO shall verify that the received message authentication code by invoking AMACVP with Session Key.

##### When the verification of the received MAC fails, the SSO shall set the Result to the abstract value ‘Negative’.

##### When the verification of the received MAC succeeds, the SSO shall set the Result to the abstract value ‘Positive’.

##### The SSO shall output the completed atnProtectSignCheckST.

### SSO-SessionKey function

Note 1.— This function is activated with the atnSessionKeyST parameter described in 8.6.2.  The inputs are the STATIC-UNPROT-PARM and DYNAMIC-UNPROT-PARM fields.  When SSO-SessionKey is successfully completed, the X-FORMED-DATA-TYPE field be set to the abstract value SSOPositive.  An error is indicated by setting the X-FORMED-DATA-TYPE field to the abstract value SSONegative.

Note 2. – The generated session key is maintained local to the SSO.  The Result indicates whether or not the SSO was able to successfully generate the session key.

Note 3. -- The atnSessionKeyST is reproduced below.

atnSessionKeyST		SECURITY-TRANSFORMATION ::= {

IDENTIFIER	{iso(1) identified-organization(3) icao(27) atn-security-requirements(4) modules(2) atnSecurityTransformations(2) atn-session-key(3)}

STATIC-UNPROT-PARM	ATNSessionKeyStaticParameters

DYNAMIC-UNPROT-PARM	ATNSessionKeyDynamicParameters

X-FORMED-DATA-TYPE	ATNSSOResult

}

ATNSessionKeyStaticParameters ::= SEQUENCE {

callingPeerId	ATNPeerId,

calledPeerId	ATNPeerId

}

ATNSessionKeyDynamicParameters ::= SEQUENCE {

calledCertificatePath	ATNCredentials		OPTIONAL

}

Note 4. – For the purposes of this section, the following shorthand notation is used:

- Calling Peer refers to ATNSessionKeyStaticParameters.callingPeerId, and

- Called Peer refers to ATNSessionKeyStaticParameters.calledPeerId, and

- Result refers to the atnProtectSignCheckST.X-FORMED-DATA-TYPE.

#### Upon activation of the SSO-SessionKey function, the SSO shall perform the sequence of steps identified in this section.

##### The SSO shall retrieve the Called Peer’s key agreement public key certificate path.

##### The SSO shall invoke SSO-CertificateCheck to verify the Called Peer’s key agreement public key certificate path.

##### When the verification of the signing public key certificate path fails, the SSO shall output the completed atnSessionKeyST with the Result set to abstract value Negative.

##### The SSO shall retrieve the Calling Peer’s key agreement private key.

##### The SSO shall retrieve the Called Peer’s key agreement public key from the Called Peer’s key agreement public key certificate path.

##### The SSO shall compute the shared secret value for the Calling Peer and Called Peer by invoking ASVDP.

##### When the output of ASVDP is ‘invalid’, the SSO shall output the completed atnSessionKeyST with the Result set to abstract value Negative.

##### The SSO shall retrieve the ATN Signature for the Calling Peer and Called Peer.

##### The SSO shall retrieve the Random Value for the Calling Peer and Called Peer.

##### The SSO shall construct the SharedInfo for the shared public value for the Calling Peer and Called Peer as the concatenation of a single octet with value 0016, the ATN Signature from above, and the Random Value from above.

##### The SSO shall construct the shared public value for the Calling Peer and Called Peer by invoking AKDF.

##### The SSO shall construct the SharedInfo for the Session Key as the concatenation of a single octet with value 0116, the shared public value for the Calling Peer and the Called Peer, the Calling Peer, and the Called Peer.

##### The SSO shall compute Session Key for the Calling Peer and Called Peer by invoking AKDF.

##### When the output of AKDF is ‘invalid’, the SSO shall output the completed atnSessionKeyST with the Result set to abstract value ‘Negative’.

##### When the output of AKDF is a valid Session Key, the SSO shall initialize a 16-bit unsigned counter to zero for messages sent from the Calling Peer to the Called Peer.

##### When the output of AKDF is a valid Session Key, the SSO shall output the completed atnSessionKeyST with the Result set to abstract value ‘Positive’.

### SSO-CertificateCheck function

Note 1.— This function is activated with the atnCertificateCheckST parameter described in 8.6.2.  The inputs are the STATIC-UNPROT-PARM and DYNAMIC-UNPROT-PARM fields.  When SSO-CertificateCheck is successfully completed, the X-FORMED-DATA-TYPE field be set to the abstract value SSOPositive.  An error is indicated by setting the X-FORMED-DATA-TYPE field to the abstract value SSONegative.

Note 2. -- The atnCertificateCheckST is reproduced below.

atnCertificateCheckST	SECURITY-TRANSFORMATION ::= {

IDENTIFIER	{iso(1) identified-organization(3) icao(27) atn-security-requirements(4) modules(2) atnSecurityTransformations(2) atn-sign-check(2)}

INITIAL-ENCODING-RULES	{joint-iso-itu-t asn(1) packed-encoding(3) canonical(1) unaligned(1)}

STATIC-UNPROT-PARM	ATNCertificateCheckStaticParameters

DYNAMIC-UNPROT-PARM	ATNCertificateCheckDynamicParameters

X-FORMED-DATA-TYPE	ATNSSOResult

}

ATNCertificateCheckStaticParameters ::= SEQUENCE {

callingPeerId	ATNPeerId,

calledPeerId	ATNPeerId

}

ATNCertificateCheckDynamicParameters ::= SEQUENCE {

checkCertificatePath	ATNCredentials

}

Note 3. – For the purposes of this section, the following shorthand notation is used:

- Calling Peer refers to atnCertificateCheckStaticParameters.callingPeerId, and

- Called Peer refers to atnCertificateCheckStaticParameters.calledPeerId, and

- Result refers to the atnCertificateCheckST.X-FORMED-DATA-TYPE.

#### Upon activation of the SSO-CertificateCheck function, the SSO shall perform the sequence of steps identified in this section.

##### The SSO shall retrieve the current CRL.

Note. – The method of retrieval of the CRL and how frequently the CRL is retrieved is a local matter.

##### The SSO shall verify that the Certificate is not revoked by using the CRL.

##### When the Certificate is determined to be revoked, the SSO shall output the completed atnSignCheckST with the Result set to the abstract value ‘Negative’.

##### The SSO shall verify the validity of the Certificate Path according to the TBD procedure of section 8.5.

##### When the output of the verification of the validity of the Certificate Path is ‘invalid’, the SSO shall output the completed atnSignCheckST with the Result set to the abstract value ‘Negative’.

##### When the output of the verification of the validity of the Certificate Path is ‘valid’, the SSO shall output the completed atnSignCheckST with the Result set to the abstract value ‘Positive’.

### SSO-GetPublicKeyCertificate function

Note.— This function is activated with the atnGetPublicKeyCertificateST parameter described in 8.6.2.  The inputs are the STATIC-UNPROT-PARM and DYNAMIC-UNPROT-PARM fields.  When SSO-GetPublicKeyCertificate is successfully completed, the X-FORMED-DATA-TYPE field will contain a valid certificate path for the Calling Peer.

Editor’s Note. – Input should specify key agreement or signing so we don’ t need 2 functions here.

#### Upon activation of the SSO-GetPublicKeyCertificate function, the SSO shall perform the sequence of steps identified in this section.

##### The SSO shall TBD.

### SSO-GetPublicKey function

Note.— This function is activated with the atnGetPublicKeyST parameter described in 8.6.2.  The inputs are the STATIC-UNPROT-PARM and DYNAMIC-UNPROT-PARM fields.  When SSO-GetPublicKey is successfully completed, the X-FORMED-DATA-TYPE field will contain a valid public key for the Calling Peer.

Editor’s Note. – Input should specify key agreement or signing so we don’ t need 2 functions here.

#### Upon activation of the SSO-GetPublicKey function, the SSO shall perform the sequence of steps identified in this section.

##### The SSO shall TBD.

### SSO-Stop function

Note 1.— This function is activated with the atnStopST parameter described in 8.6.2.  The inputs are the STATIC-UNPROT-PARM and DYNAMIC-UNPROT-PARM fields.  When SSO-ProtectSign is successfully completed, the X-FORMED-DATA-TYPE field will contain a valid message authentication code.

Note 2. -- The atnStopST is reproduced below.

atnStopST		SECURITY-TRANSFORMATION ::= {

IDENTIFIER	{iso(1) identified-organization(3) icao(27) atn-security-requirements(4) modules(2) atnSecurityTransformations(2) atn-stop(5)}

STATIC-UNPROT-PARM	ATNStopStaticParameters

DYNAMIC-UNPROT-PARM	ATNStopDynamicParameters

X-FORMED-DATA-TYPE	ATNSSOResult

}

ATNSStopStaticParameters ::= SEQUENCE {

callingPeerId	ATNPeerId,

calledPeerId	ATNPeerId

}

ATNStopDynamicParameters ::= NULL

#### Upon activation of the SSO-Stop function, the SSO shall perform the sequence of steps identified in this section.

##### The SSO shall actively destroy the following state data for the Calling Peer and Called Peer:

- Shared secret value, and

- Share public value, and

- Both 16-bit unsigned counters, and

- ATN Signature, and

- Random Value.

##### Recommendation.  Retain fact that session key is revoked.

##### The SSO shall output the atnStopST with the Result set to the abstract value ‘Positive’.

## Formal definitions of security transformations

Note.— This chapter is contain a description of the interfaces to the Security Transformations in ASN.1 and a textual description of the action wich will take place in the SSO when the SSO-functions are activated.

ATNSecurityTransformations  {iso(1) identified-organization(3) icao(27) atn-security-requirements(4) modules(2) atnSecurityTransformations(2)}

DEFINITION AUTOMATIC TAGS ::= BEGIN

-- EXPORTS ALL --

IMPORTS

notation FROM

ObjectIdentifiers {joint-iso-ccitt genericULS(20) modules(1) objectIdentifiers(0)}

SECURITY-TRANSFORMATION FROM

Notation notation

AlgorithmIdentifier FROM

AuthenticationFramework

{joint-iso-itu-t ds(5) module(1) authenticationFramework(7) 2}

ATNSignature, ATNPeerId, ATNEstablish, ATNProtectSign FROM

ATNSecurityUsefull

{iso(1) identified-organization(3) icao(27) atn-security-requirements(4) modules(2) atnUsefull(3)}

-- Editor’s Note.

-- The specific security transformations are being defined within the text above.

-- Once stable, they will be copied into this module.  I’d also like this module

-- to be self-contained as much as possible, without reference to other SARPs

-- sections.

ATNSSOResult ::= ENUMERATED {

SSOPositive	(0),

SSONegative	(1)

}

END