8.4		ATN Public Key Infrastructure

This section specifies the requirements for the ATN  Public Key Infrastructure (PKI).

It specifies required certificate policy and certificate practice statements, the format and content of certificates and CRLs, and the requirements for validation of these certificates and CRLs .

8.4.1		Certificate Policy

8.4.1.1		CAs shall develop a Certificate Policy that defines the creation, management, and use of public key certificates that they issue.

8.4.1.2		The Certificate Policy shall conform to in Internet X.509 Public Key Infrastructure Certificate Policy and Certification Practices Framework, IETF PKIX Internet X.509 Public Key Infrastructure Certificate Policy and Certification Practices Framework, IETF PKIX RFC2527

8.4.2		Certificate Practice Statement

8.4.2.1		CAs shall publish a Certificate Practice Statement that describes the expected use of public key certificates that they issue.

8.4.2.2		The Certificate Practice Statement conform to as specified in Internet X.509 Public Key Infrastructure Certificate Policy and Certification Practices Framework, IETF PKIX Internet X.509 Public Key Infrastructure Certificate Policy and Certification Practices Framework, IETF PKIX RFC2527

Note. - The Certificate Policy and Certificate Practice Statements of a given state could be used by other states in establishing their trust relatioships and operating policies such as cross certification.

8.4.3.	ATN PKI Certificate Format

Note. - The general certificate format used for ATN PKI certificates is based on the PKIX profile of  the X.509 certificate format. Additional constraints are placed on the PKIX format when it is used in the ATN to enable the effective operation of the ATN.

8.4.3.1  General Format

All certificates in the ATN shall be an encoding of the following syntax using the canonical form of the Packed Encoding Rules (PER) for ASN.1.

Certificate ::= SEQUENCE {

tbsCertificate 		TBSCertificate,

signatureAlgorithm 	AlgorithmIdentifier,

signatureValue 	BIT STRING

}

8.4.3.1.1	The value tbsCertificate shall contain the information relating to the entity being certified which is signed by the CA.

Note. - The syntax TBSCertificate is described in more detail in Section 8.4.3.2.

8.4.3.1.2	The value signatureAlgorithm shall indicate which algorithm the CA uses to sign certificates.

8.4.3.1.3	The syntax AlgorithmIdentifier shall be defined as:

AlgorithmIdentifier ::= SEQUENCE {

algorithm 		OBJECT IDENTIFIER,

parameters 		ANY DEFINED BY algorithm OPTIONAL

}

8.4.3.1.3.1		When used in signatureAlgorithm, the syntax AlgorithmIdentifier shall contain an indication algorithm of the algorithm being used by the CA as well as an indication parameters of any associated parameters being used.

8.4.3.1.3.2		ATN CAs sign certificates using the ATN Signature Primitive (ASP) which shall be indicated by assigning to algorithm the value ecdsa-with-SHA1 and assigning to parameters the value NULL.

Note. - The ATN Signature Primitive is ECDSA.

8.4.3.1.3.3	The OID ecdsa-with-SHA1 shall be defined as:

ecdsa-with-SHA1 OBJECT IDENTIFIER ::= { id-ecSigType 1 }

where:

id-ecSigType OBJECT IDENTIFIER ::= { ansi-X9-62 signatures(4) }

and:

ansi-X9-62 OBJECT IDENTIFIER ::= { iso(1) member-body(2) us(840) 10045 }

8.4.3.1.4	The value signatureValue shall contain the actual signature of the CA on tbsCertificate.

8.4.3.1.5	When signatureAlgorithm indicates that the CA is signing the certificate with ECDSA, then signatureValue shall contain an encoding of ECDSA-Sig-Value containing the ECDSA signature consisting of two integers r and s.

ECDSA-Sig-Value ::= SEQUENCE {

r	INTEGER,

s	INTEGER

}

8.4.3.2		ATN To Be Signed Certificates

The syntax TBSCertificate which is used to convey the information about the entity being certified that is actually signed by the CA shall be as follows:

TBSCertificate ::= SEQUENCE {

version [0] 		EXPLICIT Version DEFAULT v1,

serialNumber 		CertificateSerialNumber,

signature 		AlgorithmIdentifier,

issuer 			Name,

validity 		Validity,

subject 		Name,

subjectPublicKeyInfo 	SubjectPublicKeyInfo,

issuerUniqueID 	[1] IMPLICIT UniqueIdentifier OPTIONAL,

subjectUniqueID 	[2] IMPLICIT UniqueIdentifier OPTIONAL,

extensions 		[3] EXPLICIT Extensions OPTIONAL

}

### 8.4.3.2.1	The value version shall indicate the version number of X.509 certificate format being used.

### 8.4.3.2.2	All ATN certificates shall be version 3 certificates (indicated by the integer 2).

Version ::= INTEGER { v1(0), v2(1), v3(2) }

8.4.3.2.3	The value serialNumber shall contain the serial number assigned to the certificate by the CA.

CertificateSerialNumber ::= INTEGER

Note. - The serial number is used to identify the certificate in, for example, the CRLs issued by the CA.

8.4.3.2.4	The value signature shall repeat the information contained in signatureAlgorithm about which algorithm the CA is using to sign certificates.

8.4.3.2.5	The value issuer shall contains the X.501 distinguished name of the CA issuing the certificate.

Note. - PKIX mandates that this field is not empty – thus CAs must be identified by a distinguished name as well as an AE-title.

8.4.3.2.6	The syntax for distinguished names shall be:

Name ::= CHOICE {

RDNSequence

}

RDNSequence ::= SEQUENCE OF RelativeDistinguishedName

RelativeDistinguishedName ::= SET OF AttributeTypeAndValue

AttributeTypeAndValue ::= SEQUENCE {

type 			AttributeType,

value 			AttributeValue

}

AttributeType ::= OBJECT IDENTIFIER

AttributeValue ::= ANY DEFINED BY AttributeType

8.4.3.2.7	The value validity shall contain information about the validity period of the certificate – specifically when the certificate becomes valid and when the certificate expires.

8.4.3.2.8	All ATN certificates shall represent times using the universal time type UTCTime and include minutes in values of this type.

Validity ::= SEQUENCE {

notBefore 		Time,

notAfter 		Time

}

Time ::= CHOICE

utcTime 		UTCTime,

generalTime 		GeneralizedTime

}

8.4.3.2.9	The value subject shall contain the X.501 distinguished name of the entity to whom the certificate is being issued.

8.4.3.2.9.1	If the subject itself is a CA, subject shall contain the X.501 name of the subject.

8.4.3.2.9.2	If the subject is not a CA, subject shall be empty.

8.4.3.2.10	The value subjectPublicKeyInfo shall contain information about the public key of the entity being certified.

8.4.3.2.11	The type SubjectPublicKeyInfo shall be defined as:

SubjectPublicKeyInfo ::= SEQUENCE {

algorithm 		AlgorithmIdentifier,

subjectPublicKey 	BIT STRING

}

8.4.3.2.11.1	subjectPublicKeyInfo shall contain an OID indicating which algorithm type the public key is, any parameters associated with the key, and the key itself.

8.4.3.2.11.2	The OID shall be id-ecPublicKey:

id-ecPublicKey OBJECT IDENTIFIER ::= { id-publicKeyType 1 }

id-publicKeyType OBJECT IDENTIFIER ::= { ansi-X9-62 keyType(2) }

Note. - In general elliptic curve domain parameters may be represented in several different ways.  ATN entities will use the ATN default elliptic curve domain parameters sect233r1 (for CAs) or sect163r1 (for other entities) and will represent these parameters in subjectPublicKeyInfo using the OIDs assigned to them in SEC1.

8.4.3.2.11.2.1		If ATN user elliptic curve domain parameters are used, the following OID shall be indicated:

sect163r2 OBJECT IDENTIFIER ::= { ellipticCurve 15 }

8.4.3.2.11.2.2		If ATN CA elliptic curve domain parameters are used, the following OID shall be indicated:

sect233r1 OBJECT IDENTIFIER ::= { ellipticCurve 27 }

8.4.3.2.11.2.3		In either case, OBJECT IDENTIFIER shall be defined by the following arc:

ellipticCurve OBJECT IDENTIFIER ::= {  certicom-arc   curve(0) }

certicom-arc OBJECT IDENTIFER ::= {

iso(1) identified-organization(3) certicom(132)

}

8.4.3.2.11.3	Elliptic curve public keys shall be represented in subjectPublicKeyInfo in the subjectPublicKey component of subjectPublicKeyInfo.

8.4.3.2.11.3.1	subjectPublicKeyInfo which shall contain an encoding of the subject’s public key represented using the type ECPoint.

ECPoint ::= OCTET STRING

8.4.3.2.12	The value issuerUniqueID shall be omitted.

8.4.3.2.13	The value subjectUniqueID shall be omitted

Note. - Additional unique identifiers for the issuer and subject of the certificate are placed in the more flexible subject alternative name and issuer alternative name extensions.

8.4.3.2.14	The value extensions contains shall contain the key usage extension, the subject alternative name extension, and the issuer alternative name extension.

8.4.3.2.15	The type Extensions shall be specified as follows.

Extensions ::= SEQUENCE SIZE (1..MAX) OF Extension

Extension ::= SEQUENCE {

extnID 			OBJECT IDENTIFIER,

critical 			BOOLEAN DEFAULT FALSE,

extnValue 		OCTET STRING

}

8.4.3.2.15.1	Key usage extension

8.4.3.2.15.1.1	The key usage extension shall indicate what the certified key is going to be used for.

8.4.3.2.15.1.2	The key usage extension shall be identified by the OID id-ce-keyUsage.

id-ce-keyUsage OBJECT IDENTIFIER ::= { id-ce 15 }

id-ce OBJECT IDENTIFIER ::= {joint-iso-ccitt(2) ds(5) 29}

8.4.3.2.15.1.3	The key usage extension shall be marked critical in all ATN certificates.

8.4.3.2.15.1.4	Information in the key usage extension shall be a value of the type KeyUsage, defined as follows:

KeyUsage ::= BIT STRING {

digitalSignature (0),

nonRepudiation (1),

keyEncipherment (2),

dataEncipherment (3),

keyAgreement (4),

keyCertSign (5),

cRLSign (6),

encipherOnly (7),

decipherOnly (8)

}

8.4.3.2.15.1.5	KeyUsage in ATN certificates shall only be assigned one of the values digitalSignature, keyAgreement, or keyCertSign.

8.4.3.2.15.2	Subject alternative name extension

8.4.3.2.15.2.1	The subject alternative name extension shall contain an alternative name for the subject of the certificate.

8.4.3.2.15.2.2	The subject alternative name extension shall be identified by the OID id-ce-subjectAltName.

id-ce-subjectAltName OBJECT IDENTIFIER ::= { id-ce 17 }

8.4.3.2.15.2.3	The subject alternative name extension shall be marked critical in all ATN certificates.

8.4.3.2.15.2.4	Information in the subject alternative name extension shall be a value of type SubjectAltName defined as follows:

SubjectAltName ::= GeneralNames

GeneralNames ::= SEQUENCE SIZE (1..MAX) OF GeneralName

GeneralName ::= CHOICE {

otherName 		[0] OtherName,

rfc822Name 		[1] IA5String,

dNSName 		[2] IA5String,

x400Address 		[3] ORAddress,

directoryName 	[4] Name,

ediPartyName 		[5] EDIPartyName,

uniformResourceIdentifier [6] IA5String,

iPAddress 		[7] OCTET STRING,

registeredID 		[8] OBJECT IDENTIFIER

}

OtherName ::= SEQUENCE {

type-id 			OBJECT IDENTIFIER,

value 			[0] EXPLICIT ANY DEFINED BY type-id

}

8.4.3.2.15.2.5	The subject alternative name extension in ATN certificates shall contain a single alternative name (which will be either the entity’s AE-title or the entity’s NSAP).

8.4.3.2.15.2.6	When the subject alternative name extension contains an entity’s AE-title, this shall be placed in the extension as the value encoded as a registeredID.

Editor's Note. - Add reference to ICAO assigned OIDs for AE-titles (SV 9?)

8.4.3.2.15.2.7	When the subject alternative name extension contains an entity’s NSAP, this shall be encoded as the value of otherName.

8.4.3.2.15.2.8	ICAO assigned OIDs shall be placed in the type-id field,

Editor's Note. - Add reference to ICAO assigned OIDs for NSAPs (SV 9?)

8.4.3.2.15.2.9	The entity's NSAP, encoded as specified in Sub-Volume V, Section 5.4 shall be placed in the value field.

8.4.3.2.15.3	Issuer alternative name extension

8.4.3.2.15.3.1	The issuer alternative name extension shall contain an alternative name for the issuer of the certificate.

8.4.3.2.15.3.2	The issuer alternative name extension shall be identified by the OID id-ce-issuerAltName.

id-ce-issuerAltName OBJECT IDENTIFIER ::= { id-ce 18 }

8.4.3.2.15.3.3	The issuer alternative name extension shall be marked critical in all ATN certificates.

8.4.3.2.15.3.4	Information in the issuer alternative name extension shall be a value of type IssuerAltName.

IssuerAltName ::= GeneralNames

8.4.3.2.15.3.5	The issuer alternative name extension in ATN certificates shall contain a single alternative name (which will be issuer’s AE-title)

8.4.3.2.15.3.6	The issuing entitiy's AE-title shall be placed in the extension as the value encoded as a registeredID.

Editor's Note. - Add reference to ICAO assigned OIDs for AE-titles (SV 9?)

8.4.3.3		Compressed certificates

ATN certificates transmitted over air/ground subnetworks shall be sent in a compressed form.

Note. - Compressed certificates are formed by omitting fields from the full certificate which can be pre-determined by the communicating entities.  The information in the omitted fields is pre-stored by the receiving entity which can recover the full certificate using the pre-stored values and the values in the compressed certificate. After recovering the full certificate from the compressed certificate, the receiving entity validates the full certificate.

8.4.3.3.1	Compressed CA certificates

CA Certificates transmitted over air/ground subnetworks shall be compressed using the following syntax.

CompressedATNStateCertificate ::= SEQUENCE {

serialNumber		CertificateSerialNumber,

validity 		Validity,

subjectPublicKey	BIT STRING,

subjectAETitle		OBJECT IDENTIFIER,

issuerAETitle 		OBJECT IDENTIFIER,

signatureValue		BIT STRING

}

8.4.3.3.2	Compressed Ground CMA certificates

Ground CMA Certificates transmitted over air/ground subnetworks shall be compressed using the following syntax.

CompressedATNGroundCMACertificate ::= SEQUENCE {

serialNumber		CertificateSerialNumber,

validity 		Validity,

subjectPublicKey	BIT STRING,

subjectAETitle		OBJECT IDENTIFIER,

issuerAETitle 		OBJECT IDENTIFIER,

signatureValue		BIT STRING

}

8.4.3.3.3	Compressed Router certificates

Air/Ground and Airborne Router Certificates transmitted over air/ground subnetworks shall be compressed using the following syntax.

CompressedATNRouterCertificate ::= SEQUENCE {

serialNumber		CertificateSerialNumber,

validity 		Validity,

subjectPublicKey	BIT STRING,

subjectNSAP		NSAP,

issuerAETitle 		OBJECT IDENTIFIER,

signatureValue		BIT STRING

}

8.4.4.	ATN PKI CRL Format

Note. - The general certificate format used for ATN PKI CRLs is based on the PKIX profile of  the X.509 CRL format. Additional constraints are placed on the PKIX format when it is used in the ATN to enable the effective operation of the ATN.

8.4.4.1  General Format

All CRLs in the ATN shall be an encoding of the following syntax using the canonical form of the Packed Encoding Rules (PER) for ASN.1.

CertificateList ::= SEQUENCE {

tbsCertList 		TBSCertList,

signatureAlgorithm 	AlgorithmIdentifier,

signatureValue 	BIT STRING

}

8.4.4.1.1	The value tbsCertList shall contain the list of revoked certificates and other relevant information which is signed by the CA.

Note. - The syntax TBSCertList is described in more detail in Section 8.4.4.2.

8.4.4.1.2	The value signatureAlgorithm shall indicate which algorithm the CA uses to sign CRLs.

8.4.4.1.3	The use of AlgorithmIdentifier shall be as defined in Section 8.4.3.1.5

8.4.4.1.4	The value signatureValue shall contain the actual signature of the CA on tbsCertList.

8.4.4.1.5	The encoding of signatureValue shall be as defined in Section 8.4.3.1.5

8.4.4.2		ATN To Be Signed Certificate Revocation Lists

The syntax TBSCertList  which contains the list of revoked certificates and which is actually signed by the CA shall be as follows:

TBSCertList ::= SEQUENCE {

version 		Version OPTIONAL,

signature 		AlgorithmIdentifier,

issuer 			Name,

thisUpdate 		Time,

nextUpdate 		Time OPTIONAL,

revokedCertificates 	SEQUENCE OF SEQUENCE {

userCertificate 	CertificateSerialNumber,

revocationDate 	Time,

crlEntryExtensions 	Extensions OPTIONAL

} OPTIONAL,

crlExtensions 		[0] EXPLICIT Extensions OPTIONAL

}

### 8.4.4.2.1	The value version shall indicate the version number of X.509 CRL format being issued.

### 8.4.4.2.2	All ATN CRLs should be version 2 CRLs (indicated by the integer 1).

Version ::= INTEGER { v1(0), v2(1), v3(2) }

8.4.4.2.3	The value signature shall repeat the information contained in signatureAlgorithm about which algorithm the CA is using to sign CRLs.

8.4.4.2.4	The value issuer shall contain the X.501 distinguished name of the CA issuing the CRL.

8.4.4.2.5	The syntax for distinguished names shall be as defined in Section 8.4.3.2.6.

8.4.4.2.6	The value thisUpdate shall contain the time when the CRL was issued.

8.4.4.2.7	All ATN CRLs shall represent times using the universal time type UTCTime as defined in Section 8.4.3.2.8.

8.4.4.2.8	The value nextUpdate shall contain the time when the next CRL will be issued.

8.4.4.2.9	The value revokedCertificates shall contains information about the certificates that have been revoked.

8.4.4.2.9.1	Each component of the revokedCertificates sequence shall contain only the certificate serial number of the revoked certificate and the revocationDate (using the type UTCTime and including minutes).

8.4.4.2.9.2	The optional field crlEntryExtensions, which can be used to convey information such as the reason for revocation, shall not be present in ATN CRLs.

8.4.4.2.10	The optional field crlExtensions shall contain a single CRL extension in this field – the issuer alternative name extension.

8.4.4.2.10.1	The issuer alternative name shall contain the AE-title of the issuing CA, encoded as the certificate issuer alternative name extension described in Section 8.4.3.2.15.3.

8.4.5		ATN PKI Certificate and CRL Validation

This section specifies how ATN entities should validate ATN certificates and CRLs.

8.4.5.1		Certificate Validtion

When an ATN entity validates a certificate, the ATN entity shall perform, at a minimum, the following checks:

- The ATN entity shall retrieve an authentic copy of the issuing State CA’s public key.

- The ATN entity shall check that that the certificate is an X.509 certificate with the key usage, subject alternative name, and issuer alternative name extensions present, and should check that the subject alternative name and issuer alternative name extensions each contain a single name.

- The ATN entity shall check that the certificate is signed using ECDSA by examining the signatureAlgorithm and signature fields of the certificate.

- The ATN entity shall check that the certificate is a version 3 certificate by examining the version field of the certificate.

- The ATN entity shall check that the certificate was issued by the appropriate CA by examining the issuer field and the issuer alternative name extension.

- The ATN entity shall check that the certificate is fresh by examining the validity field.

- The ATN entity shall check that the certificate was issued to the appropriate CA by examining the subject field and the subject alternative name extension.

- The ATN entity shall check that the certificate contains an elliptic curve public key by examining the subjectPublicKeyInfo field.

- The ATN entity shall check that the certificate certifies a certificate signing key by examining the key usage extension.

- The ATN entity shall check that the certificate is valid by verifying the signature contained in the signatureValue field using the issuing State CA’s public key.

8.4.5.2		CRL Processing

ATN entities shall check that certificates have not been revoked by performing the following checks:

- The ATN entity shall retrieve the most recent CRL  issued by the issuing State CA.

- The ATN entity shall check that the CRL is an X.509 CRL with the nextUpdate field present, the issuer alternative name CRL extension present, and a single name in the issuer alternative name extension.

- The ATN entity shall check that the CRL is signed using ECDSA by examining the signatureAlgorithm and signature fields of the CRL.

- The ATN entity shall check that the CRL is a version 2 CRL by examining the version field of the CRL.

- The ATN entity shall check that the CRL was issued by the appropriate CA by examining the issuer field and the issuer alternative name extension.

- The ATN entity shall check that the CRL is fresh by examining the thisUpdate field and the nextUpdate field.

- The ATN entity shall check that the State CA’s certificate is not revoked by checking that the certificate’s serial number does not appear in the list of revoked cerificates’ serial numbers in the revokedCertificates field.

- The ATN entity shall check that the CRL is valid by verifying the signature in the signatureValue field using the issuing State CA’s public key.

8.4.5.3		Additional Certificate Path Validation

In addition to the certificate and CRL validation procedures already described in this section, a number of other checks may be necessary in specific circumstances.

8.4.5.3.1	When validating a certificate path, ATN entities shall check that the certificate path contains at most one certificate issued by a State CA to another State CA.

Note. - Trust among States is not transitive – a certificate issued to State B’s CA from State A’s CA together with a certificate to State C’s CA from State B’s CA does not indicate that State A trusts State C.

8.4.5.3.2	When validating a certificate path, ATN aircraft entities shall check that the ground entity certified is in the domain of the State CA certifying the ground entity, and shall indicate to the pilot which State the ground entity he or she is communicating with lies in.

Note. - State A may wish to allow State B to control State A’s aircraft only when the aircraft are in State B’s airspace but to prevent State B from controlling State A’s aircraft when State A’s aircraft are in State A’s airspace.