FAA_IDRP_FVE_101

Scenario 4

Air-Ground Secured Exchange

Single Entity Authentication

Access to Delivery Service not available

Conditions:

- Airborne and Air-Ground Routers support Type 2 Authentication

- Airborne Router has not pre-stored Air-Ground Router certificate

- Airborne Router policy is for single entity authentication

- Air-Ground Router does not have access to a certificate delivery service

- Certificate path validation by the Air-Ground Router is successful

| Item | Scenario Step | Requirement |
| --- | --- | --- |
|  | During the ISH exchange, the airborne and air-ground routers signal type 2 authentication in the ATN Data Link Capability Parameter in the ISH PDU | 5.3.5.2.16.1 |
|  | The airborne router signals public key certificate not required in the ATN Data Link Capability Parameter in the ISH PDU | 5.3.5.2.16.2.1 |
|  | The air-ground router signals public key certificate required in the ATN Data Link Capability Parameter in the ISH PDU | 5.3.5.2.16.3.1 |
|  | The air-ground router generates a 32-bit random value, RandV | Derived from 5.8.3.2.10.2.1.3 |
|  | The air-ground router sends an OPEN BISPDU with: Code 2 in the Authentication Code (AC) field | 5.8.3.2.10.2.1.1 5.8.3.2.10.2.1.2 |
|  | The air-ground router sends an OPEN BISPDU with: a Certificate Required Indicator set to indicate public-key certificate required required. | 5.8.3.2.10.2.1.1 5.8.3.2.10.2.3.4 |
|  | The air-ground router sends an OPEN BISPDU with: A random number RandV to protect against interception and manipulation attacks. | 5.8.3.2.10.2.1.1 5.8.3.2.10.2.1.3 |
|  | The air-ground router sends an OPEN BISPDU with: Since public-key certificate required was not signaled by the airborne router in the ISH exchange, the air-ground router’s public key certificate path, CertV, is placed in the Authentication Data field | 5.8.3.2.10.2.1.1 5.8.3.2.10.2.3.3 |
|  | The air-ground router sends an OPEN BISPDU with: a Type-1 authenticator in the Validation Pattern (VP) field. | 5.8.3.2.10.2.1.1 5.8.3.2.10.2.1.4 |
|  | Upon receipt of the OPEN BISPDU, the airborne router validates the MD4 checksum | No explicit requirement |
|  | Upon receipt of the OPEN BISPDU, the airborne router checks to see if the air-ground router's public-key certificate path, CertV is in the authentication data field. If present, the certificate path is validated. The certificate contains the air-ground router's public key agreement key (Qs,V). | 5.8.3.2.10.2.6.1 |
|  | The airborne router generates a 32-bit random value, RandU. | Derived from 5.8.3.2.10.2.1.3 |
|  | The airborne router computes the Diffie-Hellman shared secret value, ZU,V, using the ATN Secret Value Derivation Primitive (ASVDP) with its pre-stored private key, ds,U, and the received public key, Qs,V. | 5.8.3.2.10.2.6.2 |
|  | The airborne router derives an 160-bit session key, MacKeyU,V, using the ATN Key Derivation Function (AKDF) with a concatenation of random values, i.e., the connection initiator's value concatenated with the peer-BIS's value (RandV¦¦RandU), as shared data. | 5.8.3.2.10.2.6.2 |
|  | The airborne router sends an OPEN BISPDU with: Code 2 in the Authentication Code (AC) field, | 5.8.3.2.10.2.1.1 5.8.3.2.10.2.1.2 |
|  | The airborne router sends an OPEN BISPDU with: a Certificate Required Indicator set to indicate public-key certificate not required in the Authentication Data (AD) field | 5.8.3.2.10.2.1.1 Derived from 5.3.5.2.16.2.1 |
|  | The airborne router sends an OPEN BISPDU with: A random number RandU to protect against interception and manipulation attacks in the Authentication Data (AD) field | 5.8.3.2.10.2.1.1 5.8.3.2.10.2.1.3 |
|  | The airborne router sends an OPEN BISPDU with: Since public key certificate required was signalled by the Air-Ground Router, the airborne router sends it certificate path in the Authentication Data (AD) field | 5.8.3.2.10.2.1.1 5.8.3.2.10.2.4.1 |
|  | The airborne router sends an OPEN BISPDU with: a Type-1 authenticator in the Validation Pattern (VP) field | 5.8.3.2.10.2.1.1 5.8.3.2.10.2.1.4 |
|  | Upon receipt of the OPEN BISPDU, the air-ground router validates the MD4 checksum | No explicit requirement |
|  | Upon receipt of the OPEN BISPDU, the air-ground router validates the certificate path of the airborne router | 5.8.3.2.10.2.6.1 |
|  | The air-ground router computes the Diffie-Hellman shared secret value, ZU,V, using the ATN Secret Value Derivation Primitive (ASVDP) with its pre-stored private key, ds,U, and the received public key, Qs,V. | 5.8.3.2.10.2.6.2 |
|  | The air-ground and airborne routers tag subsequent BISPDUs with an HMAC seal over the BISPDU using the ATN Keyed Message Authentication Code Generation Primitive (AMACP). | 5.8.3.2.10.2.6.4 |
|  | The air-ground and airborne routers check the purported tags over received BISPDUs using the ATN Keyed Message Authentication Code Verification Primitive (AMACVP). | 5.8.3.2.10.2.6.5 |