FAA_IDRP_FVE_101

Scenario 2

Air-Ground Unsecured Exchange

Conditions:

- Airborne Router supports Type 2 Authentication

- Air-Ground Router does not  support Type 2 Authentication

| Item | Scenario Step | Requirement |
| --- | --- | --- |
|  | During the ISH exchange, the airborne router signals type 2 authentication in the ATN Data Link Capability Parameter in the ISH PDU | 5.3.5.2.16.1 |
|  | During the ISH exchange, the air-ground router signals type 1 authentication in the ATN Data Link Capability Parameter in the ISH PDU | Derived from 5.3.5.2.16.1 |
|  | The airborne router signals public key certificate required in the ATN Data Link Capability Parameter in the ISH PDU | 5.3.5.2.16.2.1 |
|  | The air-ground router sends an OPEN BISPDU with: Code 1 in the Authentication Code (AC) field | 5.8.3.2.10.1.2 |
|  | The air-ground router sends an OPEN BISPDU with: a Type-1 authenticator in the Validation Pattern (VP) field. | 5.8.3.2.10.1.4 |
|  | Upon receipt of the OPEN BISPDU, the airborne router validates the MD4 checksum | 5.8.3.2.10.1.3 5.8.3.2.10.1.4 |
|  | The airborne router sends an OPEN BISPDU with: Code 1 in the Authentication Code (AC) field, | 5.8.3.2.10.2.2.1 5.8.3.2.10.1.1 |
|  | Upon receipt of the OPEN BISPDU, the air-ground router validates the MD4 checksum | 5.8.3.2.10.1.4 |
|  | The air-ground and airborne routers tag subsequent BISPDUs with an MD4 checksums. | 5.8.3.2.10.1.4 |
|  | The air-ground and airborne routers check MD4 checksums on subsequent BISPDUs | 5.8.3.2.10.1.4 |