FAA_IDRP_FVE_101

Scenario 5

Air-Ground Secured Exchange

Certificate Path Validation Failure

Conditions:

- Airborne and Air-Ground Routers support Type 2 Authentication

- Airborne Router has not pre-stored Air-Ground Router certificate

- Airborne Router policy is for mutual authentication

- Air-Ground Router has access to a certificate delivery service

- Certificate path validation by the Air-Ground Router is not successful

| Item | Scenario Step | Requirement |
| --- | --- | --- |
|  | During the ISH exchange, the airborne and air-ground routers signal type 2 authentication in the ATN Data Link Capability Parameter in the ISH PDU | 5.3.5.2.16.1 |
|  | The airborne router signals public key certificate required in the ATN Data Link Capability Parameter in the ISH PDU | 5.3.5.2.16.2.1 |
|  | The air-ground router retrieves the aircraft’s public-key certificate, CertU from a supporting directory service and validates the certificate path. | 5.3.5.2.16.3.2 |
|  | If validation fails, the air-ground does not establish the IDRP connection. | 5.8.3.2.16.3.6 |