EUROCONTROL

ATN PROJECT

Systems Management SARPs - 'FastMIP' Profile Proposals

ATN Ref.	: DED6/ATNCT/SYSMAN/DCI/L7_007

Author	: Tony Kerr

Rev. No.	: 0.1

Date	: 13/01/98

DOCUMENT CONTROL LOG

| SECTION | DATE | REV. NO. | REASON FOR CHANGE OR REFERENCE TO CHANGE |
| --- | --- | --- | --- |
| All | 13/01/98 | 0.1 | First working draft |

TABLE OF CONTENTS

1. Introduction

1.1 Background

1.2 References

2. Efficient CMIP Profile for General ATN Systems Management

2.1 Extensibility and Encoding

2.1.1 Encoding of CMIP/ROSE APDUs

2.1.2 Encoding of Embedded Management Information

2.2 Suitability of Existing International Standardised Profiles

2.3 Changes Required to AOM 12

3. Summary of Requirements

4. Draft SARPs and GM for Fast MIP Profile

4.1 The SM Application Model

4.2 Profile for SM Stack

4.3 Encoding Requirements

4.4 Mapping to Dialogue Service

4.4.1 Mapping to Dialogue Service Parameters

# Introduction

The aim of this document is to consider the requirements for an efficient communications profile for general ATN systems management, and to define the provisions necessary for inclusion in ATNP SARPs and guidance material.

## Background

The ICAO ATN Panel has instructed its Working Groups to identify and define standards and recommended practices (SARPs) necessary for the management and administration of the ATN and its hosted applications.

An ATN System Management convergence process has been initiated by Eurocontrol, with active participation from ProATN and ACI implementation projects and STNA to ensure:

- that managers and agents developed by the projects will interwork successfully,

- that implementations will be SARPs-compliant, to the extent possible given the project constraints,

- that ATN SM SARPs will capture the requirements identified by the implementations.

To address the protocol stack and encoding issues, a SM Stack Task Team (Stack-TT) was set up.  The Task Team has confirmed that the proposed ATNP/WG3 approach of defining a "FastMIP" protocol stack is the most appropriate way of providing system management communication standards for ATN mobile subnetworks and inter-domain management.

The "FastMIP" stack is described in detail elsewhere [4].  Briefly, it consists of utilising the "Fast Byte" stack already specified in the ULCS SARPs.  This requires the short connect / null encoding options of Session and Presentation layers, PER-encoded ACSE edition 2, and PER-encoded application information.

The rationale for this decision is based on a number of factors, including:

- bit-efficient protocol for limited bandwidth air-ground datalinks,

- avoidance of multiple parallel protocol stacks in ATN systems,

- simplified certification requirements,

- compatibility with the existing application SARPs,

- lightweight ATN upper layers in ISs.

## References

- ISO/IEC 9596-1 (X.711) - Information Technology - Open Systems Interconnection - Common Management Information Protocol - Part 1: Specification

- ISO/IEC ISP 11183-1 - Information Technology - International Standardized Profiles AOM1n, OSI Management - Management Communications - Part 1: Specification of ACSE, presentation and session protocols for use by ROSE and CMISE

- ISO/IEC ISP 11183-2 - Information Technology - International Standardized Profiles AOM1n, OSI Management - Management Communications - Part 2: CMISE/ROSE for AOM 12: Enhanced Management Communications.

- ATNP/WG3/SG3/WPxxx - System Management Application (SMA) - The Fast MIP Option - draft paper by F Picard, February 1998

- ISO/IEC 9072-2 (X.229) - Information Processing Systems - Text Communication - Remote Operations - Part 2: Protocol Specification

- ISO/IEC 8825-2 (X.691) - Information Technology - ASN.1 Encoding Rules: Specification of Packed Encoding Rules (PER)

# Efficient CMIP Profile for General ATN Systems Management

ATN applications in general are specified to utilise the protocol stack defined in the Internet Communications Service (ICS) and Upper Layer Communications Service (ULCS) SARPs. There are requirements:

- to avoid multiple protocol stacks in ATN systems, and

- to use highly bit-efficient protocols over mobile subnetworks.

Therefore, for ATN systems management, a CMIP profile is required which is based on the ULCS and ICS SARPs.

Existing international standardised profiles (ISPs) for the OSI systems management protocol CMIP (e.g. references [2] and [3]) assume that:

- basic session and presentation layer functionality is present, and

- CMIP and ACSE APDUs are encoded for transfer using the Basic Encoding Rules of ASN.1.

Neither of these assumptions is true for the profile defined in the ULCS SARPs.

The ISPs also assume a standard connection-oriented Transport service, and as such there is no provision for ATN-specific parameters defined in the ICS SARPs such as traffic type and class.

The ULCS SARPs specify that all data transferred between applications uses the Packed Encoding Rules (PER) for information which is defined using the ASN.1 notation.  The CMIP standard assumes that ASN.1 Basic Encoding Rules (BER) will be used.  Hence, the CMIP abstract syntax definitions are not currently specified to take full advantage of ASN.1 extensibility features and optimal PER encoding.  To this end, it may be necessary to augment CMIP and ROSE ASN.1 definitions to add PER-visible constraints and extensibility markers.

The following steps are necessary:

- If major efficiency gains can be made, augment the CMIP abstract syntax specification in ISO/IEC 9596-1 with PER-visible constraints and extensibility markers, in order to allow optimal encoding efficiency and provision for backwards compatibility in the future.

- Specify that CMISE APDUs are to be encoded for interchange using PER.

- Specify MO attribute syntaxes using PER-visible constraints and extensibility markers

- Specify that attribute values are to be encoded for interchange using PER

- As ULCS supports only a single presentation context, ensure that a mechanism is in place to distinguish PDUs from different abstract syntax modules.

## Extensibility and Encoding

The ATN profile requires the following features:

- The abstract syntax module defined in clause 9 of the ACSE protocol specification is augmented with the ASN.1 extensibility notation, as specified in ISO/IEC 8650-1 Amendment 1.

- The system supports that encoding which results from applying the ASN.1 packed encoding rules (basic, unaligned variant), as specified in ISO/IEC 8825-2, to the abstract syntax module specified in ISO/IEC 8650-1 Amendment 1.

- Packed encoding rules (basic, unaligned) are used for encoding all ACSE Protocol Control Information (PCI) for interchange.

The encoding issue thus applies at two levels:

- the encoding of CMIP/ROSE/ACSE APDUs, and

- the encoding of the MIB information.

### Encoding of CMIP/ROSE APDUs

Given the existing ULCS architecture, the "Fast Byte" presentation profile requires all presentation data to be taken from a single presentation context.  This implies that all application data must be treated as part of a single abstract syntax definition.  The ULCS SARPs contain mechanisms for routing received application data to the correct ASE, which are independent of the presentation context.

### Encoding of Embedded Management Information

The CMIP syntax encapsulates MO information by means of definitions such as:

Attribute ::= SEQUENCE {

attributeId		AttributeId,

attributeValue	ANY DEFINED BY attributeId

}

The actual type used in the ANY DEFINED BY will depend upon the definition of the attribute.  For example, a counter attribute may be defined as an INTEGER type, in which case the transfer syntax would be a PER-encoded unconstrained INTEGER value.

In principle it would be possible to re-define all attributes to have EXTERNAL syntax at the CMISE level.  This could then encapsulate the attribute value encoded using BER.  In this way, standard BER could be used by the Manager and Agent applications.  However, there are a number of drawbacks:

- non-standard attribute definitions,

- loss of encoding efficiency,

- dual encoder/decoders per SM. stack.

This approach is therefore rejected, and it is assumed that the management information in the MIB will belong to the same presentation context as used for ACSE and other ATN applications.

## Suitability of Existing International Standardised Profiles

The candidate ISPs which could potentially be adopted for ATN system management are ISO/IEC ISO 11183 parts 1 and 2.

ISO/IEC ISP 11183-1 specifies how the Association Control Service Element (ACSE), the Presentation layer and the Session layer are used to provide the required upper layer functions for the CMISE/ROSE functions.  In addition, requirements for abstract and transfer syntax handling are specified.

ISP 11183-1 is based on a full OSI upper layer stack.  It does not take into account the efficiency option of the presentation and the session protocols, nor does it take into account the second edition of the ACSE protocol.  ISP 11183-1 section 8 requires ISP conforming systems to encode PDUs with the Basic Encoding Rules.  ISP 11183-1 does not require the compliance to ISO/IEC 8650-1 Amendment 1.

The ISP requires conforming implementation to support at least 2 simultaneous presentation contexts.  The ATN specification is non-conformant to the ISP proforma, in that only one presentation context is supported.  In the case of the System Management Application, the presentation context should identify a single abstract syntax formed by ACSE and ROSE/CMISE abstract syntaxes and a single transfer syntax.

Part 2 of ISP 11183 specifies how the OSI Common Management Information Service Element (CMISE) combined with the OSI Remote Operation Service Element (ROSE) shall be used to provide the complete set of operation and notification services to the CMISE-service-users of two end systems.  It specifies the CMIP / ROSE protocol features for the definition of the Enhanced Management Communications profile, identified as AOM 12.

Reference [4] compares ISP Part 1 with the functionality specified in the ULCS SARPs and concludes that the efficient ULCS profile is capable of supporting the AOM 11 and AOM 12 CMIP profiles.  It also proposes an extension to the current ISP 11183-1 which would include this new protocol functionality.

It has thus been shown in [4] that the ULCS SARPs fully support ISP 11183-2 (AOM 12) for CMIP support.

Part 2 of ISP 11183 defines the support level of all of the OSI management communication features needed by implementations.  It specifies general purpose management communication capabilities by requiring the support of all CMIP Function Units except the extended services FU.

The support of the complete set of operation and notification services, and of the corresponding protocol elements does not imply that all these features must be used in all instances of communications.  The selection of the features depends on the needs and dynamic requirements of the CMISE-service-users who may choose between:

- application entity roles

- functional units

- operation / notification services

- optional parameters.

## Changes Required to AOM 12

Having selected an ISP, the specification of the FastMIP stack still needs some additional detailed work.  It has been shown [4] that the internationally standardised AOM 12 profile can be used, if references to ISP 11183-1 (ACSE, presentation and session for use by CMISE/ROSE) are replaced by references to the ICAO ULCS stack.

Some optimisation of AOM 12 will also be required.  For example, the CMIP standard defines the following type to identify MO attributes:

AttributeId ::= CHOICE {

globalForm 	[0] IMPLICIT OBJECT IDENTIFIER

localForm	[1] IMPLICIT INTEGER

}

AOM 12 specifies that all MO attributes are identified using the global Object Identifier form.  This entails a very large overhead for systems management operations which involve several separate attributes.  For a specific application context, such as ATN systems management, it is permissible to define INTEGER values instead of Object Identifiers, and to use the localForm of identification, which is much more bit-efficient.  However, the local form is defined in AOM 12 as "out of scope" for that profile.

The communication profile for ATN system management will therefore be as specified in ISO/IEC ISP 11183-2 (CMISE/ROSE for AOM 12) [3], with the following general modifications:

- All references to ISP 11183-1 to be replaced by references to the ULCS SARPs

- Local forms of identification to be allowed where this will make a large difference to the overhead.

# Summary of Requirements

It is necessary to produce key SARPs and GM which will:

- Define a CMIP communications profile based on identified functional requirements for ATN systems management, bearing in mind the need for future extensibility and the potential for interworking both with existing CMIP products and with other systems management technologies.  It is probable that one of the existing internal standardised profiles AOM 1x will satisfy these requirements.

- Define a mapping of the selected profile to the efficient upper layer protocol stack specified in the ULCS SARPs, which in turn maps to the lower layer stack specified in the ICS SARPs.

- Specify a presentation context based on PER encoding of all systems management and ACSE APDUs.

# Draft SARPs and GM for Fast MIP Profile

## The SM Application Model

Most of the existing ATN application SARPs were specified to make use of the Dialogue Service, which is defined in the ULCS SARPs.  The Dialogue Service hides the ACSE and Presentation services from the application ASEs, and is provided by the CF.  The "Lower CF," which supports the Dialogue Service, is fully specified in the ULCS SARPs.

The architecture, as applied to ATN systems management, is illustrated in Figure 1.

Figure 1: Use of the CNS/ATM-1 Dialogue Service

The CF provides a "pass-through" service to the SM Application User, allowing the SM Application User to invoke CMISE services and also to establish and release associations.  For modelling purposes, a "conceptual SM ASO" envelopes CMISE and ROSE and invokes association establishment and termination services on behalf of the Application User.  Thus, the Dialogue service is used to establish, release and abort the application-association with the peer SMAE and to exchange SM information when requested by CMISE and ROSE.  The CMIS service is provided unchanged to the SM-users as part of the SM service.

The actions of the "conceptual SM ASO" need to be specified in Sub-Volume VI of the ATN SARPs, in particular the actions to handle the primitive exchanges:

- between the SM-users and CMISE,

- between the SM-users and the Dialogue Service Provider, and

- between ROSE and Dialogue Service Provider.

In particular, the specification is responsible for re-mapping the Presentation service primitives (P-DATA request and indication) used by ROSE at its lower interface to the Dialogue service interface.

The provisions in section 4.4 equate to the specification of the "conceptual SM ASO".

The definition of the "conceptual SM ASO" is required only for modelling purposes.  It avoids any need to modify the existing ULCS SARPs, which assume a one-to-one service mapping between the application ASE and the AE.  The SM-User is provided with a service consisting of all the CMIS service primitives, plus a pass-through to the D-START, D-END and D-ABORT services.  There is no requirement to implement any physical entity corresponding to the "conceptual SM ASO."

Note.— The main problem of this architecture when used for the ATN air-ground application specifications was the induced complexity of the App-ASE protocol, because the states of the underlying dialogue (e.g. pending establishment, established, pending release, collision) should be handled by the ASE protocol itself.  For the SMAE, CMISE and ROSE assume the association established and invoke only a data transfer primitive.  This problem is therefore not encountered for the specification of the SM application.

## Profile for SM Stack

The complete communication requirements between Manager and Agent for ATN systems management shall be as specified here, taken together with the profile defined in the ULCS SARPs and the connection-mode Transport service defined in the ICS SARPs.

The complementary communications interactions between CMISE-service-users within two end Management Information systems, with scope as shown in Figure 2, shall comply with the provisions specified here.

Figure 2: Scope of the SM Communications Profile

The supporting stack shall be as specified in the international standards indicated in Table 1, subject to the constraints and options specified in this profile.

Table 1 - Profile supporting stack

| Application Layer | ISO 9595, 9596-1 (CMIS, CMIP v2) ISO 9072-1, 9072-2 (ROSE) ISO 8649, 8650-1 Amd.1 (ACSE ed.2) ULCS SARPs (CF, encoding) |
| --- | --- |
| Presentation Layer | ISO 8822, 8823-1 Amd.1 (Service, "Fast Byte" protocol) ISO 8824, 8825-2 (ASN.1, PER) |
| Session Layer | ISO 8826, 8327-1 Amd.1 (Service, "Fast Byte" protocol) |

The communication profile for ATN system management shall be as specified in ISO/IEC ISP 11183-2, with modifications as specified in Table 2.

Table 2 - Modifications to ISP 11183-2

| ISP 11183-1 clause | Modification for ATN Profile |
| --- | --- |
| 1.5, Table 1 | replace the table with Table 1 in this document |
| 5 (Conformance to AOM 12) | replace "and also in ISO/IEC ISP 11183-1" with "and also in the ULCS SARPs profile" |
| 5.1 | delete "ISO/IEC ISP 11183-1 and". |
| 5.3 | delete Note 2. |
| 5.3 | delete final paragraph. |
| 5.4 | replace "shall support the mapping of ROSE APDUs only onto the P-DATA Presentation service" with "shall support the mapping of ROSE APDUs only onto the D-DATA Dialogue service". |
| A.1 | This clause allows non-conformant implementations to list the non-supported mandatory capabilities. For the SM SARPs, non-compliance is not permitted. Therefore the following provision is required: "All mandatory capabilities of ISO/IEC ISP 11183-2 as modified here shall be implemented." |
| A.2.1 | replace "association" with "dialogue" throughout this clause, as the CMISE services are mapped to the ULCS Dialogue service, and not directly to ACSE. |
| A.2.3, Table A.3 | replace "(in AARQapdu)" with "(in D-START Request and Indication User-Data)". |
| A.2.3, Table A.4 | replace "(in AAREapdu)" with "(in D-START Response and Confirmation User-Data)". |
| A.2.3, Table A.3 and A.4 | redefine profile support for userInfo parameter in CMIPUserInfo as "out of scope". |
| Clause A.2.4, Table A.5 | profile support for userInfo parameter in CMIPAbortInfo is changed to "out of scope". |
| Clause A.3.2, Table A.13a and A.13b, index A.13a.1, A.13a.2, A.13b.1, A.13b.2 | replace "See ISP 11183-1, 8.3" with "(3)", and insert new note after table: (3) A sender shall not encode values of greater than 2**31-1 or less than -2**31. A receiver shall be able to decode at least values in the range -2**31 to 2**31-1. |
| Clause A.3.2, Table A.13a, index A.13a.10, A.13a.11, A.13a.12 | profile support of the INTEGER form of actionType, attributeId and eventType is changed from "i" to "m". |
| Table A.123 and A.124 | delete note referring to ISP 11183-1. |

Note 1.— accessControl parameters are outside the scope of this profile.

Note 2.— the format of the information contained in the CMIP PDUs "userInfo" in CMIPUserInfo and CMIPAbortIInfo definitions is outside the scope of AOM 12.

## Encoding Requirements

The abstract syntax of the management information conveyed in CMIP PDUs shall be as defined in the ATN MIB specification.

The encoding of management information for interchange shall be realised using the basic, unaligned variant ASN.1 Packed Encoding Rules.

Implementations shall support the transfer syntax derived from the encoding rules specified in ISO/IEC 8825-2 and named { joint-iso-itu-t asn1 (1) packed-encoding (3) basic (0) unaligned (1) } for the purpose of generating and interpreting CMIP PDUs as defined in ISO/IEC 9596-1 by the abstract syntax "CMIP-PCI".

Note.— The above requirement is equivalent to specifying that all CMISE and ROSE APDUs are encoded using the basic, unaligned variant ASN.1 Packed Encoding Rules.  It replaces the requirement in clause 8.1 of ISO/IEC 9596-1 that "the implementation shall support the transfer syntax derived from the encoding rules specified in ISO/IEC 8825 and named { joint-iso-ccitt asn1(1) basic-encoding(1) } for the purpose of generating and interpreting CMIP PDUs as defined by the abstract syntax "CMIP-PCI".

## Mapping to Dialogue Service

ROSE service primitives shall map to the D-DATA request / indication primitives of the Dialogue Service defined in the ULCS SARPs.

Note.— The above requirement replaces the ISO 9072-2 mapping to P-DATA request / indication primitives.

When a CMISE-service-user requires to open an association for the exchange of CMISE / ROSE APDUs, the following sequence of events shall occur:

- A connection is established using the D-START service (and not the A-ASSOCIATE service as specified in ISO/IEC 9596-1).  The CMIPUserInfo maps to the D-START request User Data.

- On receiving a D-START indication containing User Data, the peer CMIPM and CMISE-service-user analyse the CMIPUserInfo as specified in ISO/IEC 9596-1 A.2.2.

- If the dialogue parameters are acceptable, the receiving CMISE-service-user and CMIPM construct the CMIPUserInfo required for the response and invoke a positive D-START response primitive, with the CMIPUserInfo as User-Data.

- If the dialogue parameters are not acceptable, the receiving CMISE-service-user and/or CMIPM invoke a negative D-START response primitive, with the constructed CMIPUserInfo, if any, as User-Data.

- If the initiating CMISE-service-user receives a negative D-START confirmation, no association has been established.

- If the initiating CMISE-service-user receives a positive D-START confirmation, an association has been established and the peer CMISE-service-users can exchange management protocol data units.

When a CMISE-service-user requires to close an association, the following sequence of events shall occur:

- A D-END request primitive is invoked by the release initiator.

- On receiving a D-END indication, the release responder invokes a positive D-END response, which will close the connection.

- On receiving a positive D-END confirmation, the association ceases to exist.

### Mapping to Dialogue Service Parameters

The D-START Routing Class QoS parameter shall be set as specified in Table 3.

Table 3

| Abstract Class of Communication | Routing Class Value (Hex) |
| --- | --- |
| ATN Systems Management Communications | 60 |

The D-START Priority QoS parameter shall be set as specified in Table 4.

Table 4

| Abstract Priority Value | QoS Priority Value (Decimal) |
| --- | --- |
| Network / Systems Management | 14 |