# Action 7/5

# Change Proposal Material in response to DR44

# 1. Result of discussions

The DR was accepted at WG2/7 with the following amendment: reason code g) appears to be required since some compressed PDU types seems to be unassigned and may therefore lead to unrecognised PDUs.

Reason code f) will be replaced by “reserved”

# 2. Detailed Change Proposal

#### 7.5.4.8.  SNDCF Error Report

7.5.4.8.a. The SNDCF Error Report is a packet format unique to the  Mobile SNDCF, and shall be used to report errors in the use of local references as specified below [a10 t 1930]. The SNDCF Error Report PDU shall be constructed as follows:

1.	The most significant four bits (PDU Type) of the first octet are set to binary 1110, while the least significant four bits are set to 0000.

2.	The second octet is a discard reason encoded as an unsigned integer, with the following reason codes defined:

- reason = [0000 0000] - compressed NPDU with unrecognized local reference

- reason = [0000 0001] - creation of directory entry outside of sender's permitted range

- reason = [0000 0010] - directory entry exists

- reason = [0000 0011] - local reference greater than maximum value accepted.

- reason = [0000 0100] - Data Unit Identifier missing when SP=1.

- reason = [0000 0101] - reserved

- reason = [0000 0111] - Compressed ISO 8473 PDU with unrecognized Type.

- reason = [0000 1000] - local reference cancellation error