#### Timers and Protocol Parameters

Although the implementation of most of the timers and protocol parameters is mandatory, there are no mandatory values for them, other than the minimum and maximum values which may be defined for each.

It is recommended for ground systems that timers be configurable on a per TC basis.

In general, the assignment of values for timers and parameters must be optimised based on operational testing of the applications. In such testing, incompatible timer values and optimum combinations can be identified. Implementations of the transport protocol should support configurable values for all timers and protocol parameters, rather than having fixed values. This allows modification as operational experience is gained.

Note 1.— Refer to Table 0-1 for the complete listing of timers and parameters.

Note 2.— Refer also to 12.2.1.1 of ISO/IEC 8073 for more details on the timers.

Note 3.— In Table 0-1, the subscripts "R" and "L" refer to "remote" and "local", respectively. The variable ERL, for example, refers to the maximum transit delay from the remote entity to the local entity. The variable ELR is the maximum transit delay from the local entity to the remote entity. It is assumed that these values may be different.

Note 4- The example values in Table 0-1 have not been subject to validation and are for illustrative purposes only.

Several of the timers and variables listed in Table 0-1 are not directly configurable, but may be determined based on the values of other timers and variables. That is:

| Name | Description | Minimum value | Nominal value | Maximum value |
| --- | --- | --- | --- | --- |
| MRL, MLR | NSDU Lifetime, seconds | 26 | 400 | 600 |
| ERL, ELR | Maximum Round-trip Transit Delay, seconds | 1 | 100 | 150 |
| AL, AR | Acknowledgement Time, seconds | 1 | 20 | 400 |
| T1 | Local Retransmission Time, seconds | 12 | 221 | 300 |
| R | Persistence Time, seconds | 1 | 443 | 2710 |
| N | Maximum Number of Transmissions | 1 | 3 | 10 |
| L | Time bound on reference and/or sequence numbers, seconds | 160 | 1263 | 3000 |
| I | Inactivity Time, seconds | 600 | 4500 | 6000 |
| W | Window Time, seconds | 160 | 4000 | 6000 |

Table 0-1 Example Timer and Parameter Values and Ranges

- The NSDU lifetime variables, MRL and MLR, may have a general estimate, based on the lifetime values used for NPDUs. The NSDU lifetime value is the value used to delete aged packets from the ATN. It should be over three times the expected end-to-end time. The expected air-to-ground end-to-end time can be up to 30-40 seconds.

- The end-to-end delay variables, ERL and ELR, may be estimated only, or some mechanism may be available to determine these dynamically.

- The value for the local acknowledgement timer, AL, may be determined based on application requirements. For example, applications supporting ATC may require immediate acknowledgement of TPDUs so that the uncertainty about delivery is minimised. The remote acknowledgement time variable AR, for example, may not be known or it may be provided by the remote transport entity explicitly during the connection establishment phase. The value for AL should be dynamically configurable.

- The local retransmission time, T1, is defined by ISO as:

T1 = ELR + ERL + AR + x,

where x is the local processing time for a TPDU.

- The persistence time, R, is the maximum time a transport entity will attempt to retransmit a TPDU. The persistence time is larger, in general, than the maximum number of retransmissions, N-1, times the local retransmission time, T1.

- The maximum number of transmissions, N, is related to the expected transmission reliability of the end-to-end path, since exceeding N results in the termination of a transport connection. Too high a value, however, may result in wasted retransmissions if end-to-end communication is no longer possible.

- The maximum time to receive an acknowledgement of a given TPDU, L, is bounded by ISO as:

L = MLR + MRL + R + AR

- In general, a reference or sequence number should not be re-used for the time period L. The value of L, in combination with the expected traffic, may be used to determine if extended TPDU numbering is required.

- The inactivity timer, I, is set based on network delays and the expected QoS. Specification of this parameter is related to the use of the maximum number of transmissions parameter, N, since it is used to terminate transport connections.

- The window timer, W, determines when acknowledgements are sent in the case of no activity. Up-to-date window information is sent when W expires. It should be set smaller than the expected value of the remote value of I.