ATNP/WG-2

Flimsy # 7-9

9 February, 1996

Aeronautical Telecommunications Network Panel
Working Group 2

Brisbane 5-9 February 1996

Review of AMCP Responses to WG2’s questions regarding VDL Priority Handling and ATN Priority Mapping

Summary
This Flimsy is a draft report on WG2’s review of the responses provided by AMCP , WG-D, to the WG2’s questions listed in WG2’s paper “Definition of Priority Categories within the ATN Internetwork”

# I. Introduction

During the ATNP WG meeting held in October 1995, the discussion relating to ATNP WG2 WP184 “Priority Definitions within Annex 10 and the Relationship to the ATN SARPS” raised several questions which have been forwarded to the AMCP WGD and WGC .

AMCP WG-D has then provided a response drafted at AMCP WG2 January 1996 meeting held in Sweden , requesting some changes in the guidance material proposed in the ATN internet SARPs for Mapping Message Categories and Associated Priority Levels.

In Brisbane, WG2 has reviewed this AMCP Working Group-D  “ Response to Questions Regarding VDL Priority Handling and ATN Priority Mapping”, and and has the following comments :

# II. Analysis

## AMCP item a :

WG2 has understood the AMCP requirement to associate only messages of the ITU category “Flight regularity “  ,or of higher ITU priority, to safety and regularity of flight.

In subvolume 5 of the CNS/ATM-1 Package Internet Draft SARPS,(version 3.0)  - this ITU category “Flight regularity” has been subdivided in three different ATNP types of messages : “Flight regularity communications” , “Aeronautical Information Services ”, “Network/System administration” which have respectively CLNP priority 8,7,6. Despite of this division, all these three message types are ITU “flight regularity” messages and therefore should be associated to regularity of flight.

This means that all messages of CLNP priorities 6 and greater are associated to safety and regularity of flight as proposed in Working paper WG2-184.

## AMCP item b

WG2 has noted the absence of prioritisation of messages in VDL modes 1 and 2 .

## AMCP item c

When available, the proposed VDL Mode-3 priority mapping will be incorporated in the CNS/ATM Package Internet guidance documentation. As the CNS/ATM-1 Package Internet guidance is scheduled to be provided to I.C.A.O for translation end of June 1996 , the proposed mapping information should be provided as an input to the ATNP Working Group 2 meeting which will be held in June 1996 in Munich ( 21 -27 June )

## AMCP general comment

The ATN 8473 (CLNP) priority scheme is the one selected by the International Standards Organization in ISO 8473 . Changing this scheme would mean a deviation from the standard . For this reason, the change proposed by the AMCP cannot be adopted by ATNP WG2.