Flimsy #10

Generation of the “Leave Event” upon call cleared due to Idle timer expiration

# Introduction

Based on WP221, section 4, WG2 discussed whether or not the generation of a “Leave Event” by the mobile SNDCF is appropriate when a subnetwork connection is closed due to the expiration of the X.25 Idle timer.

The group agreed that, because the expiration of the Idle timer does not always ensure that subnetwork connectivity has effectively been lost (e.g. in case of multiple subnetwork connections), it is not desirable to systematically generate a “Leave Event” which would then result in the termination of the BIS-BIS connection.

But when a subnetwork connection which was previously closed due to the expiration of the Idle timer fails to be re-established, the probability that subnetwork connectivity has effectively been lost is greater and the generation of the “Leave Event” becomes then desirable in order to avoid long lived routing black holes.

# Recommendation

In order to record these considerations in the CNS/ATM-1 SARPs, this flimsy proposes to add the following recommendation in section 3.5.2.12 (Air/Ground Route Termination), just following Note 2

Recommendation. A “Leave Event” should not be generated by the mobile SNDCF when a subnetwork connection is closed due to the expiration of the X.25 Idle timer, except if this subnetwork connection fails to be re-established.