ATNP WG2-16

Flimsy 2

Issue 1.1

1 Oct 1998

Proposed Resolution for PDR 98090003

1. Introduction

PDR 98090003 (entitled Downgrading of ATSC Class) reports about a potential deadlock situation between a pair of BISs when forwarding NPDUs containing an ATSC Class Security Tag within the CLNP header. This PDR has been accepted by the CCB/7 as a potential defect and given to the WG2 SARPs Development Mechanism (SDM) for resolution.

A full description of the identified problem has been presented to the Working Group in Attachment B of WP 473. The working group reviewed the reported problem and noted that section 5.3.2.2.3.2.2.3 of the ICS SARPs specifies the tie-breaking rule in order to avoid the reported deadlock situation. Moreover, the working group noted that the specification in section 5.3.2.2.3.2.2.3 lacks a definition of the semantics of the “lowest relative cost” which is to be used for route selection. Consequently the working group agreed to amend the ICS SARPs as follows:

2. Proposed SARPs Amendment

Modify section 5.3.2.2.3.2.2.3 to read:

5.3.2.2.3.2.2.3  If multiple routes are available which meet or exceed the required ATSC Class, then the route with the lowest relative cost, i.e.actual monetary cost, shall be selected.

Subsequent to section 5.3.2.2.3.2.2.3 add the following Note and new section 5.3.2.2.3.2.2.4:

Note. -- The actual monetary cost is determined through means outside the scope of this specification.

5.3.2.2.3.2.2.4   If the monetary cost is the same or unknown, then the hop count shall be used as the relative cost metric.