ATNP/WG2-17

Flimsy 2

Revision a/

Aeronautical Telecommunication Network Panel (ATNP)

Working Group2 – 17th Meeting

Honolulu, Hawaii, USA

19 – 22 January 1999

## ATNP WG2 response to "Issues on SNDCF or LANs" (WP490), raised by the ATN Panel Secretary

Background

During its 17th meeting of ATNP/WG2, Working Paper 490 was discussed. This working paper was produced by the ATN Panel Secretary, Mr Paydar. It raises two issues related to references, in Doc. 9705, to the ISO/IEC 8802-2 standard:

First Issue: In SubVolume V, section 5.7.3.1 refers to “ISO/IEC 8802-2 Broadcast Subnetworks” whereas the ISO/IEC 8802-2 standard is entitled “Telecommunications and information exchange between systems - Local and metropolitan area networks - Specific requirements - Part 2: Logical link control”

Second Issue: In SV I, Section 1.1.2 (References), only ISO/IEC 8802-3 (on CSMA/CD type of LAN) has been mentioned. The ISO/IEC 8802-2 hasn't been identified.    Furthermore, the title is incomplete and thus inconsistent with other standards being elaborated in that Section.

Analysis

1. First Issue:

ISO/IEC 8802-2 defines the common logical link control (LLC) sublayer for use by a number of different LAN and MAN types, such as CSMA/CD, Token Bus, Token Ring, Distributed Queue Dual Bus, etc.

This common LLC sublayer offers a common service interface to higher layer entities (such as an    SNDCF) independent of the particular LAN/MAN type, i.e. the MAC sublayer of the LAN/MAN.

All these LAN and MAN types use in the MAC sublayer (OSI layer 2a) a certain type of broadcast mechanism. Although this mechanism is not the classical broadcast mechanism in all cases (for example Token Ring LANs pass frames to all stations attached to the ring in a sequential order), it is fair to say that the LANs/MANs operating under LLC (i.e. providing the ISO/IEC 8802-2 service) are broadcast LANs.

This is confirmed, by the definition of broadcast subnetworks, which is found in ISO/IEC 9542 (ES-IS), clause 5.4.2:

A broadcast subnetwork supports an arbitrary number of End Systems and Intermediate systems, and additionally is capable of transmitting a single SNPDU to all or a subset of these systems in response to a single SN_UNITDATA..-Request. An example of a broadcast subnetwork is a LAN (local area network) conforming to ISO 8802-2, type 1 operation.

Section 5.7.3.1 of SV 1 simply refers to ISO/IEC 8473-2. However, in ISO/IEC 8473-2 the term 'broadcast subnetwork' is not used nor defined. Instead the terms 'ISO/IEC 8802 subnetwork' and 'ISO/IEC 8802 subnetwork that provides the LLC sublayer interface    service defined by ISO/IEC 8802-2' are used.

To address the first issue and to be closer to the ISO/IEC 8473-2 terminology, it is proposed to delete the word 'broadcast' from the title of section 5.7.3 and from the text in section 5.7.3.1.

Furthermore a note, explaining that ISO/IEC 8802-2 subnetworks are subnetworks that    provide the logical link control sublayer service defined by ISO/IEC 8802-2, is proposed to be added to section 5.7.3.1.

2. Second Issue:

WG2 considers that it is appropriate to add ISO/IEC 8802-2 to the list of international standards in section 1.1.2 of SubVolume 1.

WG2 also confirms that in section 1.1.2,  the title in the reference to ISO/IEC 8802-3 is not the exact title of the ISO/IEC 8802-3 standard.

In summary, WG2 recognizes the validity of the editorial defects described in WP490 and proposes that these defects be resolved through the standard CCB process, by submission to the CCB of a PDR on the Subvolume V, and of another PDR on the Subvolume I.

Proposed text for the PDR on Subvolume V:

Title:                                                    References to ISO/IEC 8802-2 Broadcast Subnetworks

PDR Reference:

Originator Reference:

SARPs Document Reference:              SubVolume 5, section 5.7.3

Status:

PDR Revision Date:

PDR Submission Date:                                        20/1/99

Submitting State/Organization:                ATNP WG2

Submitting Author Name:

Submitting Author E-mail Address:

Supplemental Contact Information:

SARPs Date:                                          IV2.3 (Doc 9705 Ed1)

SARPs Language:                                                  English

Summary of Defect:

In SubVolume V, section 5.7.3.1 refers to “ISO/IEC 8802-2 Broadcast Subnetworks” whereas the ISO/IEC 8802-2 standard is entitled “Telecommunications and information exchange between systems - Local and metropolitan area networks - Specific requirements - Part 2: Logical link control”.

Note: ATNP WG2/17 Flimsy #2 includes a more detailed description of this defect.

Assigned SME: Sub-Volume V SME (K.-P. Graf)

Proposed SARPs Amendment:

1) Delete the word ‘broadcast’ from the title of section 5.7.3 and from the text of paragraph 5.7.3.1.

2) In section 5.7.3, add the following note before the paragraph 5.7.3.1:

# Note.- ISO/IEC 8802-2 subnetworks are subnetworks that    provide the logical link control sublayer service defined by ISO/IEC 8802-2.

SME Recommendation to CCB:

CCB Decision:

Proposed text for the PDR on Subvolume I:

Title:                                                    Reference to ISO/IEC 8802-2 is missing in SubVolume I

PDR Reference:

Originator Reference:

SARPs Document Reference:              SubVolume 1, section 1.1.2

Status:

PDR Revision Date:

PDR Submission Date:                                        20/1/99

Submitting State/Organization:                ATNP WG2

Submitting Author Name:

Submitting Author E-mail Address:

Supplemental Contact Information:

SARPs Date:                                          IV2.3 (Doc 9705 Ed1)

SARPs Language:                                                  English

Summary of Defect:

1) A reference to the ISO/IEC 8802-2 standard is missing in SubVolume 1 section 1.1.2.

2) The title in the reference to ISO/IEC 8802-3 is not the exact title of the ISO/IEC 8802-3 standard

Note: ATNP WG2/17 Flimsy #2 includes a more detailed description of this defect.

Assigned SME: Sub-Volume I SME

Proposed SARPs Amendment:

1) In section 1.1.2, add the following reference:

ISO/IEC 8802-2 “Telecommunications and information exchange between systems - Local and metropolitan area networks - Specific requirements - Part 2: Logical Link Control”

2) Replace the title in the reference to ISO/IEC 8802-3 with the exact title of the ISO/IEC 8802-3 standard

SME Recommendation to CCB:

CCB Decision: