Future Work Plan Beyond ATNP/3

In response to WG1/14 (Honolulu) meeting report, paragraph 10.1

"WG2 will prepare a draft work plan for deliverables beyond ATNP/3. WG1 will consolidate the three drafts (WG1, WG2 & WG3) during the second week in Naples, and agree on a draft paper for ATNP/3,"

the following high level panel work (not down to working group level) is proposed as the WG2 input to WG1:

|  | Title | Sample Description | Reference |  |
| --- | --- | --- | --- | --- |
| 01 | Enhancements to the ATN SARPs and Guidance Material based on new and/or revised user requirements | Additional requirements for mobile subnetworks issuing Join and Leave events (e.g., latency requirements, handling of link fading conditions, etc.). Also potential enhancements to the mobile SNDCF for the handling of changing mobile subnetwork connectivity | WG2/18 WP510 No. 1 |  |
| 02 | Enhancements to the ATN SARPs and Guidance Material based on operational experience | Enhancements to the connection oriented transport service to better cope with the loss of TPDUs that will result from changes in mobile subnet connectivity | WG2/18 WP510 No. 2 |  |
| 03 | Provision of subnetwork dependent convergence functions (SNDCFs) within the ATN SARPs and Guidance Material for additional subnetwork types | Coordination with AMCP on needed support for new mobile subnetworks (e.g., VDL Modes 3 & 4, asynchronous transfer mode, frame relay, broadcast subnetworks, etc.) | WG2/18 WP510 No. 4 |  |
| 04 | Investigate the provision of quality of service (QoS) management functions service | QoS routing and performance management enhancements (beyond CNS/ATM-2 Package) | WG2/18 WP510 No. 5 |  |
| 05 | Provide Guidance Material on financial accounting mechanisms, including the identification of network cost parameters | ATN accounting management elements and requirements are closely tied with systems management provisions | WG2/18 WP510 No. 7 |  |
| 06 | New ATN SARPs and Guidance Material based on the multicast architecture adopted at ATNP/3 | Following the ATNP/3 adoption of the multicast architecture, prepare Doc 9705 changes for ATNP/4 | WG2/18 WP510 No. 8 |  |
| 07 | Address issues related to backwards compatibility of SARPs enhancements with existing ATN SARPs | Working paper needed for ATNP/4 | WG2/18 WP510 No. 13 |  |
| 08 | Aid for global ATN implementations |  |  |  |