Communiqué

IATA Requirement for Subnetwork Selection

At the ATN Panel, WG2/18 meeting in Naples, Italy (18-21 May 1999), it was determined that the complete IATA requirement for AOC/AAC/APC air/ground subnetwork routing preference was necessary in order to design suitable ATN SARPs and Guidance Material enhancements for the CNS/ATM-2 Package.

Subject matter experts in attendance agreed that, to the best of their knowledge, the following represents this IATA requirement, and WG2 requests that the ICAO ATN Panel Secretary obtains an IATA confirmation before September 1999.

"AOC/AAC/APC routing must be able to specify on a by-message basis,

in both the uplink or downlink direction by specific aircraft,

no preference (select based on other criteria e.g., policy) OR

a single physical subnetwork preference to ONLY be used, OR

an ordered list from a total list of all available subnetworks,

noting that subnetworks are unique by technology and service providers,

and noting that no more than eight (8) subnetworks are ever active on any

one aircraft."

Note that the uplink route selection process affects all routers on the ground, whereas the downlink route selection process affects only the airborne routers.