ATNP/WG2/WP.......

10 July 2000

AERONAUTICAL TELECOMMUNICATION NETWORK PANEL

Working Group 2

21st Meeting

Limerick, Ireland

11 – 14 July 2000

SME 5 (Internet Communications Service)

Status Report

Working Paper

Presented by Klaus-Peter Graf (Sub-Volume 5 SME)

Summary

This paper provides a summary report on the status of the PDRs which have been submitted against the ATN ICS SARPs (Sub-Volume 5 of Doc 9705) since ATNP/3 and other CCB-related matters.

Furthermore it presents the proposed solution of PDR M0040002 for review by the meeting.

# Introduction

This paper provides a summary on the status of the Proposed Defect Reports (PDRs) raised against the ATN Internet Communications Service (ICS) SARPs since ATNP/3 in February 2000 for information of the WG2 members.

Furthermore it presents 14 PDRs which have been raised against Draft Edition 3 of the ICS SARPs as a result of validation activities on enhancements made in Draft Edition 3. These PDRs have been labelled as “P3DRs” and distributed on the WG2_SDM mailing list for discussion and resolution. They have also been reviewed by IDG/4 in April 2000 and most of them have been resolved and appropriate corrective actions made to Draft Edition 3 of the ICS SARPs.

# PDR Status

Table 1 presents the list of those PDRs which have been submitted to the ATNP Configuration Control Board (CCB) since its last meeting in December 1999 and which apply to the Internet Communications Service (ICS) SARPs.

It should be noted that this table only contains PDRs raised against Edition 1 and/or Edition 2 of the ICS SARPs as Draft Edition 3 is not yet under configuration control by the ATNP CCB and consequently no CCB-controlled defect resolution mechanism is available for this last edition up to now.

| PDR Number | PDR Title | CCB Status | SDM Status |
| --- | --- | --- | --- |
| M0040001 | Incorrect/Duplicated ATSC Class Security Tag Requirement | PROPOSED | RESOLVED |
| M0040002 | Extended Transport Checksum | PROPOSED | RESOLVED |

Table 1: Status of ICS PDRs in the ATNP CCB Process

Whereas the resolution for PDR M0040001 is straightforward, PDR M0040002 is attached to this paper as Appendix A for review and approval by WG 2 members. Appendix B contains a comment received on this PDR for consideration by the meeting.

# Potential Draft Edition 3 Defects

As a result of comprehensive validation exercises performed by STNA, the following potential defects have been identified in Draft Edition 3 of the ICS SARPs since the last WG2 meeting in December 1999. These potential defects relate to enhancements introduced in Draft Edition 3 and have no impact on Editions 1 and 2 of the ICS SARPs. Resolution of these potential defects is not under control by the CCB, but within the responsibility of the WG2 SDM process. These PDRs have been registered by the SME 5 in the form of P3DRs and distributed on the WG2_SDM mailing list for discussion and resolution.

| P3DR Number | P3DR Title | SDM Status | Resolved in Draft Edition 3 |
| --- | --- | --- | --- |
| M0020010 | Processing of received Deflate Maintenance Parameter | RESOLVED | Yes |
| M0020011 | Issues on the concept of Subnetwork Connection Group | RESOLVED | Yes |
| M0020012 | Bit 0 of the ISH Data Link Capability Parameter | RESOLVED | Yes |
| M0020013 | TP4 retransmission timer on the first RTT sample | RESOLVED | Yes |
| M0020014 | Valid/invalid round trip time sample in TP4 | RESOLVED | Yes |
| M0020015 | Error condition for Deflate decompressor window | SUBMITTED |  |
| M0020016 | Use of received security info by Airborne BIS | RESOLVED | Yes |
| M0020017 | Interoperability problem due to the suppression of ACA | RESOLVED | Yes |
| M0020018 | Interoperability with a peer BIS that does not support authentication type 2 | SUBMITTED |  |
| M0020019 | BIS behaviour in case of certificate path validation failure | RESOLVED | Yes |
| M0020020 | A/G BIS access to delivery service | RESOLVED | No (GM) |
| M0020021 | Encoding of Random Variable Parameter value | RESOLVED | Yes |
| M0020022 | Length of Certificate Path Parameter | RESOLVED | Yes |
| M0020023 | Alignment of Reset Request and Reset Indication Procedures | SUBMITTED |  |

Table 2: Identified Defects in Draft Edition 3 of the ICS SARPs

All of the above defects are classified as minor/clarification, i.e. no major deficiency has been identified on the ICS enhancements so far.

# Recommendation

The meeting is invited to

- note the above reported status; and

- review and approve the proposed solution for PDR M0040002 attached to this report.

Appendix A

Title:	Potential Misdelivery of CLNP Packets

PDR Reference:	M0040002

Originator Reference:

SARPs Document Reference:	ICS SARPs, Section 5.5.2.4

Status:	ACCEPTED

Impact:	A (Critical)

PDR Revision Date:	SUBMITTED  ACCEPTED(23/5/00)

ACCEPTED  PROPOSED (26/6/00)

PDR Submission Date:	13/04/00

Submitting State/Organisation:	Eurocontrol

Submitting Author Name:	Tony Whyman

Submitting Author E-mail Address:	tony.whyman@fans-is.com

Submitting Author Supplemental

Contact Information:

SARPs Date:	SV 5 Edition 2

SARPs Language:	English

Summary of Defect:

In ICAO Doc 9705 Subvolume one, it is required that:

"1.3.28 The end system shall make provisions to ensure that

the probability of not detecting a 255-octet message being

mis-delivered, non-delivered or corrupted by the internet

communications service is less than or equal to 10-8 per

message."

Currently there is no ensurance that this requirement is met by

systems which implement the technical provisions of the ATN

SARPs.

It seems that validation of this SV1 requirement has not been

done to a level which clearly indicates that the risk of either

mis-delivery of CLNP packets by the ATN ICS or not detecting

mis-delivered packets is less than or equal to 10-8.

There are at least three possible methods of resolution:

a) Further validation material is submitted which indicates that

the possibility of mis-delivery or non-detection of mis-

delivered packets is less than 10-8.

b) Additional measures are taken in the ATN applications to

ensure that mis-delivered messages are detected and not

operated upon.

c) Additional measures are added to the ATN ICS to ensure that

mis-directed CLNP packets are discarded by the receiver and

packet recovery mechanisms are initiated.

The discussion part of this PDR and the proposed SARPs amendement

below relate to the third option. The presented solution is

favoured by the IDG in the case that resolution if this PDR is

aimed at the ICS level. However, it is not intended to exclude

any proposals for resolution falling into category a) or b)

above.

Discussion:

If a TPDU is mis-delivered to the wrong destination then this

can be detected through:

a) an incorrect Destination NSAP Address in the CLNP header;

b) an inconsistency in the TPDU header information; or

c) an inconsistency in the application data.

Unfortunately, it is not possible to rely upon any of the above

to detect mis-delivery to the required level.

The ATN ICS SARPs currently do not include a genuine end-to-end

check on the integrity of the CLNP Destination NSAP Address; the

CLNP Header Checksum is not an end-to-end checksum as it is

manipulated by intermediate systems and is anyway discarded by

LREF. However, it is possible to extend the transport checksum

to include the destination NSAP Address without having to include

the NSAP Address itself in the transport protocol header. This

will provide an end-to-end check on NSAP Address integrity and

hence provide the receiving transport entity with a means of

detecting and rejecting TPDU mis-delivered due to an undetected

error in the destination NSAP Address or an LREF problem.

A means to achieve this may be found in TCP. The TCP/IP world

has traditionally been less concerned with layer independence

than the OSI world and the TCP checksum includes the source and

destination IP addresses within its scope by constructing a

“pseudo header” including these fields. This pseudo header is

never transmitted but is assumed to the part of the TCP header

for checksum computation purposes. When a packet is received,

the receiving system reconstructs the pseudo header from the

information contained in the IP header and verifies the

checksum and hence IP Address integrity.

An extended transport checksum could readily be defined for TP4

to include the source and destination NSAP Addresses in a similar

pseudo header or trailer. This may then be used to detect

mis-delivery and hence avoid the identified hazards.

However, in order to meet the requirement for better than 1 in

10E8 such errors to remain undetected, a better checksum

algorithm is also required. This is because the existing 16 bit

checksum has an undetected error rate of the order of 1 in 10E5.

A 32-bit ones complement checksum will have an undetected error

rate of the order of 1 in 10E10 and hence this is preferred for

an extended checksum.

Proposed SARPs Amendment:

1) Add the following new para 5.5.2.2.15 and subsequent

sub-paras:

"5.5.2.2.15  Negotiation of the Use of the Extended Transport

Checksum

5.5.2.2.15.1  When supported, the use of the Extended Transport

Checksum shall be proposed by an ATN End System during

connection establishment by including an Extended Transport

Checksum parameter in a CR TPDU as specified in 5.5.2.4.3.

Note.- The ISO/IEC 8073 checksum parameter is also included in

the CR TPDU in order to comply with the ISO/IEC standard and to

support backwards compatibility.

5.5.2.2.15.2  The use of the 16-bit checksum shall be proposed

in the ”Additional Option Selection” parameter of the CR TPDU.

5.5.2.2.15.3  When a CR TPDU is received that includes the

Extended Transport Checksum parameter, then the connection

responder shall

a) include an Extended Transport Checksum parameter, as

specified in 5.5.2.4.3, in the responding CC TPDU;

Note.- The connection responder may select the 2-octet

long or 4-octet long Extended Transport Checksum parameter

in the responding CC TPDU.

b) ommit the ISO/IEC 8073 checksum parameter from the

responding CC TPDU;

c) reject the use of the 16-bit checksum in the

”Additional Option Selection” parameter of the

responding CC TPDU.

5.5.2.2.15.4  All other TPDUs exchanged on the same transport

connection shall also include the Extended Transport Checksum

parameter with the same length as selected in the CC TPDU and

shall not include the ISO/IEC 8073 checksum parameter.

5.5.2.2.15.5  A CR TPDU that does not contain an Extended

Transport Checksum parameter shall be processed in compliance

with ISO/IEC 8073.

5.5.2.2.15.6  If a CR TPDU without an Extended Transport

Checksum parameter is received, then an Extended Transport

Checksum parameter shall not be included in any TPDUs sent on

that transport connection.

5.5.2.2.15.7  If a CC TPDU is received that includes an

Extended Transport Checksum parameter, then all other TPDUs

exchanged on the same transport connection shall

a) also include the Extended Transport Checksum parameter

with the same length as specified in the CC TPDU,and

b) not include the ISO/IEC 8073 checksum parameter.

5.5.2.2.15.8  If a CC TPDU is received that does not include an

Extended Transport Checksum parameter, then

a) the CC TPDU shall be processed in compliance with

ISO/IEC 8073, and

b) an Extended Transport Checksum parameter shall not be

included in any TPDUs sent on the transport connection.

5.5.2.2.15.9 If both an Extended Transport Checksum parameter

and either the ISO/IEC 8073 checksum parameter are present, or

16-bit checksums are accepted by the connection responder, or

both, then this shall be treated as a protocol error."

2) Add the following new para 5.5.2.4.3 and subsequent sub-paras:

"5.5.2.4.3 Encoding of the Extended Transport Checksum Parameter

5.5.2.4.3.1 The Extended Transport Checksum Parameter shall be

encoded as a variable part TPDU parameter using one of the two

following formats:

Parameter Code:   0000 1000,indicating Extended Transport

Checksum parameter with 4 octets length

Parameter Length: 4

Parameter Value:  Result of the checksum algorithm as

specified in 5.5.2.7.3.4

or

Parameter Code:   0000 1001,indicating Extended Transport

Checksum parameter with 2 octets length

Parameter Length: 2

Parameter Value:  Result of the checksum algorithm as

specified in 5.5.2.7.3.3

Note.— When supported, the parameter is included in a CR TPDU

and is thereafter included in all other TPDUs except when the

connection responder indicates non-support of the parameter by

omitting it from the variable part of the CC TPDU.

5.5.2.4.3.2 An Extended Transport Checksum parameter shall

only be used when both connection initiator and receiver have

been assigned NSAP Addresses in the ATN Network Addressing

Subdomain.

5.5.2.4.3.3 An Extended Transport Checksum Parameter received

in a TPDU other than a CR TPDU and from a source NSAP with an

NSAP Address in any other Addressing Subdomain shall be treated

as a protocol error; in a CR TPDU it shall be ignored.

Note 1.— Use of this parameter is not in compliance with ISO/IEC

8073. However, it is not a protocol error to use it in a CR

TPDU. Hence,as long as it is only used when both initiator and

receiver indicate support by including it in the CR TPDU, and

in a CC TPDU in response to such a CR TPDU, its implementation

will not result in interoperability problems.

Note 2.— Parameter codes with bits 7 and 8 are explicitly not

assigned by ISO/IEC 8073, nor is their use precluded.

Note 3.— Requiring that both sender and receiver are ATN Systems

ensures that the correct semantics of the parameter are  observed.

5.5.2.4.3.4 In a CR TPDU, the Extended Transport Checksum shall

be calculated first and the resulting check digits inserted into

the TPDU before the 16-bit checksum is calculated.

5.5.2.4.3.5 The value of the 16-bit checksum parameter shall be

set to zero before the Extended Transport Checksum is computed.

5.5.2.4.3.6 When present in a TPDU, the Extended Transport

Checksum parameter shall be validated using the algorithm

specified in 5.5.2.7.

5.5.2.4.3.7 If the validation fails, the TPDU shall be discarded

without further processing.

5.5.2.4.3.8 When a CR TPDU is received, the 16-bit checksum

shall be verified as correct before the Extended Transport

Checksum is verified."

3) Add the following new para 5.5.2.7 and subsequent sub-paras:

"5.5.2.7 Extended Transport Checksum Computation

Note.— The style of Appendix B of ISO/IEC 8073 is followed in

the definition of the extended transport checksum algorithm.

5.5.2.7.1 Symbols

Note.- The following symbols are used:

a) C0, C1, C2, C3 are variables used by the algorithm

b) i is the number (i.e. position) of an octet within the TPDU

c) n is the number (i.e. position) of the first octet of the

Extended Transport Checksum parameter

d) L is the length of the complete TPDU including the "pseudo

trailer"

e) Xj is the value of the jth octet of the Extended Transport

Checksum parameter (in transmission order).

5.5.2.7.2 Arithmetic Conventions

5.5.2.7.2.1 Addition shall be performed in one of the two

following modes:

a) modulo 255

b) ones complement arithmetic in which if any of the variables

has the value minus zero (i.e. 0xFFFF) shall be regarded as

though it were plus zero (i.e. 0).

5.5.2.7.3 Algorithm for Generating the Checksum Parameters

5.5.2.7.3.1 The complete TPDU with the Extended Transport

Checksum Parameter value field set to zero shall be set up.

5.5.2.7.3.2 A "pseudo trailer" created from:

a) the length of the destination NSAP Address

b) the destination NSAP Address

c) the length of the source NSAP Address

d) the source NSAP Address,

encoded identically to their encoding in the CLNP header shall

be appended to the TPDU.

Note 1.— This pseudo trailer is not part of the TPDU and is

never transmitted to the destination end system.

Note 2.— A pseudo trailer rather than a pseudo header is used

because the check digits have to be moved to the end of the TPDU

by the receiver and hence a trailer will have to be constructed

anyway.

5.5.2.7.3.3 The 2-octet long Extended Transport Checksum shall

be created by the following algorithm:

1) Initialise C0 and C1 to zero

2) Process each octet in the combined TPDU and pseudo trailer

sequentially from i = 1 to L by

a) adding the value of the octet to C0; then

b) adding the value of C0 to C1.

3) Set the octets of the Extended Transport Checksum parameter

as follows:

a) X0 = -C1 + (L-n)*C0

b) X1 = C1 - (L-n+1)*C0.

4) Discard the pseudo trailer octets.

5.5.2.7.3.4 The 4-octet long Extended Transport Checksum shall

be created by the following algorithm:

1) Initialise C0 , C1, C2 and C3 to zero

2) Process each octet in the combined TPDU and pseudo trailer

sequentially from i = 1 to L by

a) adding the value of the octet to C0; then

b) adding the value of C0 to C1, C1 to C2, and C2 to C3

3) Set the octets of the Extended Transport Checksum parameter

as follows:

a) X0 = - (C0 + C1 + C2 + C3)

b) X1 = C1 + 2*C2 + 3*C3

c) X2 = - (C2 + 3*C3)

d) X3 = C3

4) Discard the pseudo trailer octets.

5.5.2.7.4 Algorithm for Checking the Checksum Parameters

5.5.2.7.4.1 The transport entity shall appended to the received

TPDU a "pseudo trailer" which is created from the source and

destination NSAP Addresses associated with the incoming TPDU

and the value of the received Extended Transport Checksum

parameter in the following order:

a) the length of the destination NSAP Address

b) the destination NSAP Address

c) the length of the source NSAP Address

d) the source NSAP Address,

encoded identically to their encoding in the CLNP Header,

e) the octets of the Extended Transport Checksum parameter

in the same order in which they appear in the checksum

parameter.

5.5.2.7.4.2 The value of the Extended Transport Checksum

Parameter shall be set to zero.

5.5.2.7.4.3 If the received TPDU is a CR TPDU, then the value of

the 16-bit checksum parameter shall be set to zero.

5.5.2.7.4.4 If the received TPDU contains a 4-octet long Extended

Transport Checksum, this checksum shall be validated as follows:

1) Initialise C0, C1, C2 and C3 to zero

2) Process each octet in the combined TPDU and pseudo trailer

sequentially from i = 1 to L by

a) adding the value of the octet to C0; then

b) adding the value of C0 to C1, C1 to C2, and C2 to C3

3) Discard the pseudo trailer.

4) If, when all the octets have been processed, one or more

of the variables C0, C1, C2 or C3 do not have the value

zero,then the checksum validation has failed.

5.5.2.7.4.5 If the received TPDU contains a 2-octet long Extended

Transport Checksum, this checksum shall be validated as follows:

1) Initialise C0 and C1 to zero

2) Process each octet in the combined TPDU and pseudo trailer

sequentially from i = 1 to L by

a) adding the value of the octet to C0; then

b) adding the value of C0 to C1

3) Discard the pseudo trailer.

4) If, when all the octets have been processed, one or more

of the variables C0 and C1do not have the value zero,

then the checksum validation has failed."

5) Add the following new APRL item to para 5.5.2.8.1.2.2:

"ATN7   Use of Extended Transport Checksum  5.5.2.2.15.1,

5.5.2.4.3.2   O"

6) Add the following new para 5.5.2.8.1.13:

"5.5.2.8.1.13 Extended Transport Checksum

ETC1  Extended Transport Checksum in CR TPDU  5.5.2.2.15.1

ATN7:M

ETC2  Proposal of 16-bit Checksum in CR TPDU  5.5.2.2.15.2

ATN7:M

ETC3  Extended Transport Checksum in CC TPDU  5.5.2.2.15.3

ETC1:M

ETC4  ISO/IEC 8073 Checksum Parameter in CC TPDU

5.5.2.2.15.3  ETC1:X

ETC5  Rejection of Use of 16-bit Checksum in CC TPDU

5.5.2.2.15.3  ETC1:M

ETC6  Extended Transport Checksum all subsequent TPDUs

5.5.2.2.15.4, 5.5.2.2.15.6, 5.5.2.2.15.7, 5.5.2.2.15.8

(ETC1 and ETC3):M

ETC7  Encoding of Extended Transport Checksum  5.5.2.4.3  ATN7:M

ETC8  Computation of Extended Transport Checksum  5.5.2.7.3

ATN7:M

ETC9  Validation of Extended Transport Checksum  5.5.2.7.4

ATN7:M"

7) Add the following new para 5.5.3.4.2:

"5.5.3.4.2  Extended Transport Checksum

5.5.3.4.2.1  An Extended Transport Checksum parameter shall be

included in the varible part of an UD TPDU when both the sending

and receiving end system have been assigned NSAP addresses in

the ATN Network Addressing Subdomain.

Note 1.— Requiring that both sender and receiver are ATN

Systems ensures that the correct semantics of the parameter

are observed.

Note 2.- The ISO/IEC 8602 16-bit checksum parameter may also

be included in the UD TPDU.

5.5.3.4.2.2  The Extended Transport Checksum parameter shall be

encoded as follows:

Parameter Code:   0000 1000

Parameter Length: 4

Parameter Value:  Result of the algorithm for the 4-octets

long Extended Transport Checksum as

specified in 5.5.2.7.3

Note 1.— Use of this parameter is not in compliance with ISO/IEC

8602. However, as long as it is only used by communicating ATN

End Systems,its implementation will not result in interopera-

bility problems.

Note 2.— Parameter codes with bits 7 and 8 set to zero are

explicitly  not assigned by ISO/IEC 8602, nor is their use

precluded.

5.5.3.4.2.3 If both the Extended Transport Checksum parameter

and the ISO/IEC 8602 16-bit checksum parameter are present in

an UD TPDU, then the Extended Transport Checksum shall be

calculated first and the resulting check digits inserted into

the UD TPDU before the 16-bit checksum is calculated.

5.5.3.4.2.4  An UD TPDU that does not contain an Extended

Transport Checksum parameter shall be processed in compliance

with ISO/IEC 8602.

5.5.3.4.2.5  When present in a received UD TPDU, the Extended

Transport Checksum parameter shall be validated using the

algorithm for the 4-octets long Extended Transport Checksum

parameter as specified in 5.5.2.7.4.

5.5.3.4.2.6  When the validation fails, the TPDU shall be

discarded without further processing."

8) In the APRL table of para 5.5.3.6.2 modify the entry in

line 7 as follows:

"TpTc  <t> TPDU UD Checksum   6.2.4.1  O    O"

9) Add the following new APRL items to para 5.5.3.6.2:

"TpTetc  <t> TPDU UD Extended Transport Checksum  5.5.3.4.2

(ATN SARPs)  --   M"

"TpRetc  <r> TPDU UD Extended Transport Checksum  5.5.3.4.2

(ATN SARPs)  --   M"

Impact on Interoperability:

The solution proposed for the COTP is backwards compatible, as the

use of the Extended Transport Checksum is negotiated during

connection establishment. The additional checksum is not included

in subsequent TPDUs if the negotiation reveals that one of the

communicating entities does not support this feature.

The solution proposed for the CLTP is not backwards compatible

and there is no way to introduce this feature in a way that

ensures interoperability with previous editions. Therefore,

the impact category of this PDR has been upgraded to category A

(Critical). However, it is assumed that there is no operational

system implementing the Edition 1&2 short stack CLTP and

consequently the impact on existing implementations should be

minimal.

SME Recommendation to CCB:  Accept proposed SARPs Amendment

CCB Decision: PDR Accepted (23/05/99)

Appendix B

RE: P1DR M0040002 PROPOSED

Date:    Mon, 3 Jul 2000 11:40:26 +0100

From:  "Fieldhouse, Dirk" <Fieldhouse@logica.com>

To:      "'ATNP WG/2 Mailing List'" <atnp_wg2@tls.cena.fr>

All,

Sorry to be a layer pedant, but assuming for the moment that there is a need

for an extended checksum as proposed below, should it not be a CLNP option

(say, "CLNP extended checksum")? Although the local NSAP address is a

mandatory parameter of the CLNS N-UNITDATA.req primitive, there may be

implementations that do not make the local NSAP address explicitly available

when a TPDU is being constructed. With any luck these do not include any

extant ATN CLNP/TP4 implementations.

Note that IP6 has a clear concept of end-end options vs hop-by-hop options,

or did when I last checked.

It would then be necessary to consider:

-       should the transport checksum option, when selected, be suppressed

if the CLNP extended checksum is selected, and if so how?

-       how would this CLNP extended checksum option interact with LREF

compression (in particular, be preserved end-to-end without disabling the

use of LREF)?

-       would it only be carried in the initial NPDU in case of

fragmentation?

/df