# Details of outstanding Change Proposals

## Unresolved CPs

### 960900071.CP

Title                           : Routes under empty RIBAtt are non-ATSC

Change Proposal Reference       : 96090071.CP

Defect Report or Change Request Reference: 96080108.DR

Status                          : SUBMITTED

Change Proposal Revision Date   : 24/09/96

Change Proposal Format Revision Date:      20 January 1995

Change Proposal Submission Date : 24/09/96

Submitting State/Organization: France (STNA)

Submitting Author Name: Tamalet Stephane

Submitting Author E-mail Address: TAMALET_Stephane@ccmail.dgac.fr

Submitting Author Supplemental Contact Information:

STNA - 1, avenue Grynfogel

BP 1085 - 31035 Toulouse Cedex

Fax: (33) 62.14.53.53

Tel: (33) 62.14.54.83

ATN SARPs & Guidance Material Draft Version: 6.0

SARPs/GM Document Reference: section 5.8.3.2.12

Summary of Proposal:

In the paragraph 5.8.3.2.12.2 replace the item c):

"c) have been classed as ATSC Class H"

by:

"c) are not available to ATSC traffic"

Discussion:

The item c) was wrong. An ATSC Route is a route containing an ATSC Class

Security tag in its Security Path attribute. Routes advertised under the

empty RIB_Att are non-ATSC routes.

### 960900072.CP

WG2 WP 335

### 96100073.CP

Title                           : Inconsistent SNDCF parameter block length

Change Proposal Reference       : 96100073.CP

DR or CR Reference              : 96090109.DR

Status                          : SUBMITTED

Change Proposal Revision Date   : 03/10/96

Change Proposal Format Revision Date: 20 January 1995

Change Proposal Submission Date : 03/10/96

Submitting State/Organization: Eurocontrol

Submitting Author Name: Jean-Pierre Briand

Submitting Author E-mail Address: bri@eurocontrol.fr

Submitting Author Supplemental Contact Information:

Tel: +33 1 69 88 76 19

Fax: +33 1 69 88 73 33

ATN SARPs & Guidance Material Draft Version: 6.0

SARPs/GM Document Reference: 5.7.6.2.1.5.3 and Fig. 5.7-2

Category: Minor Technical

Discussion:

Some concerns were raised during DR vote on which way the inconsistency

should be fixed. The general opinion seems to be in favor of aligning the

text.

Summary of Proposal:

Clause 5.7.6.2.1.5.3 should be rewritten as follows:

"5.7.6.2.1.5.3 The value of the second octet (length indicator)

shall be an unsigned binary number giving the number of octets

in the SNDCF parameter block (from version number field up to and

including (if present) the maximum number of directory entries field)."

Impact on requirements database: none

### 96100074.CP

Title                           : Air-Ground Route termination defect

Change Proposal Reference       : 96100074.CP

DR or CR Reference: 96090118.DR

Status                          : SUBMITTED

Change Proposal Revision Date   : 03/10/96

Change Proposal Format Revision Date:      20 January 1995

Change Proposal Submission Date : 03/10/96

Submitting State/Organization: France (STNA)

Submitting Author Name: Tamalet Stephane

Submitting Author E-mail Address: TAMALET_Stephane@ccmail.dgac.fr

Submitting Author Supplemental Contact Information:

STNA - 1, avenue Grynfogel

BP 1085 - 31035 Toulouse Cedex

Fax: (33) 62.14.53.53

Tel: (33) 62.14.54.83

ATN SARPs & Guidance Material Draft Version: 6.0

SARPs/GM Document Reference: section 5.3.5.2.13

Summary of Proposal:

The proposed changes to section 5.3.5.2.13 are described in the

attached document. This includes change bars to show where changes

have been made to original text.

Discussion:

Comments 1), 2) and 3) from Klaus-Peter on the DR have been taken into

account in this proposal. Comment 4) (" Shouldn't the new paragraph

5.3.5.2.13.7 also include an ATN Airborne Router having established

an BIS-BIS connection ? ") has been considered, but has not modified

the initial proposal, because it is believed that the answer to this

comment is "No".

Compared to the initial draft change proposal included in the DR, this

CP proposes also that the air/ground route termination actions to be

performed by the IS_SME be not only initiated in case of Leave Event

but also in case of ISH Holding Timer expiration. (Quote :"If, as a

result of this procedure or subsequent to the execution of the

ISO/IEC 9542 "Flush Hold Configuration" function, ...")

Impact on Requirements: