Flimsy #2
25 June 1996

Rev. No.

Use of the ISO/IEC 9542 Configuration Timer

# Introduction

In  5.3.5.2.10.8, it is recommended that the ISH PDU Holding Timer is set to 65535 in order to effectvely suppress the generation of ISH PDUs once an IDRP BIS-BIS connection has been established. However, this is one step removed from the actual requirement which is to stop generating ISH PDUs; the generation of ISH PDUs being on expiry of the Configuration Timer.

The recommendation needs to be re-cast to focus on the real requirement.

# Proposed Change

It is proposed that 5.3.5.2.10.8 is changed to:

Recommendation.— On subnetwork connections supporting an active BIS-BIS connection, ISH PDUs should not be generated , except when a watchdog timer is applied to the subnetwork connection (see 5.3.5.2.13).

On subnetwork connections supporting an active BIS-BIS connection, non-receipt of an ISH PDU shall not be a reason for terminating a subnetwork connection, except as a consequence of the expiry of a watchdog timer.

The subsequent note also should be deleted, as should notes 1,2 and 3 following table  5.8-1.