Flimsy # 6
25 June 1996

Rev. No.

Amending the Definition of the Default Route to all Mobilestic indicating “LREF Compression not Supported”

# Introduction

During the review of the proposed revisions to chapter 7 (WP292), it was agreed that the mobile SNDCF call setup procedures should anticipate a future possible withdrawal of the requirement to always use the LREF compression on mobile subnetwork connections.

The new text for chapter 7 effectively considers the case where the calling DTE would not offer the LREF compression in the call user data of the ISO 8208 call request packet (e.g. in paragraph 5.7.6.2.1.5.12).

But the text for chapter 7 omits to consider the case where the called DTE would not support the LREF compression mechanism and would clear a call indication for this reason.

# Proposed Modifications

Add the following paragraph at the beginning of section  5.7.6.2.1.7.3:

When a DTE originated ISO/IEC 8208 Call Clearing Packet is received with a diagnostic indicating LREF compression not supported (see Table 5.7-3), the call shall be re-attempted without offering LREF compression.

Modify the Table 5.7-3 as follows:

|  | Hexadecimal value | Decimal value | Clearing Cause |
| --- | --- | --- | --- |
| 1 | 1111 1001 | 249 | Connection Rejection - unrecognized protocol identifier in user data |
| 2 | 1000 0000 | 128 | Version number not supported |
| 3 | 1000 0001 | 129 | Length field invalid |
| 4 | 1000 0010 | 130 | Call Collision Resolution |
| 5 | 1000 0011 | 131 | Proposed Directory Size too large |
| 6 | 1000 0100 | 132 | Local Reference Cancellation Not Supported |
| 7 | 1000 0101 | 133 | Received DTE refused, received NET refused or invalid NET selector |
| 8 | 1000 0110 | 134 | Invalid SNCR field |
| 9 | 1000 0111 | 135 | ACA compression not supported |
| 10 | 1000 1000 | 136 | LREF compression not supported |
| 11 | 1000 1111 | 143 | V42bis compression not supported |
| 12 | 1111 0000 | 240 | System lack of resources |
| 13 | 0000 0000 | 0 | Cleared by System Management |
| 14 | 1001 0000 | 144 | Idle Timer expiration |
| 15 | 1001 0001 | 145 | Need to re-use the circuit |
| 16 | 1001 0010 | 146 | By local means (to be used for system local error) |
| 17 | 1001 0011 | 147 | Invalid SEL field value in received NET |