ATNP/WG2-WP/291

AERONAUTICAL TELECOMMUNICATION NETWORK PANEL

Working Group 2

Munich, Germany  24 - 28 June 1996

Proposed Changes to Sub-Volume V Chapter 4

Presented by HENK J. HOF

Prepared by Tony Whyman

SUMMARY

Flimsy #1 generated by the ATN SARPs editorial committee recognised a number of agreed problems with chapter 4 of the ATN SARPs. This working paper provides a revised version of chapter 4 that has been edited in line with the proposed resolution of Flimsy #1 and it is recommended that the Working Group reviews and adopts the proposed replacement text.

DOCUMENT CONTROL LOG

| SECTION | DATE | REV. NO. | REASON FOR CHANGE OR REFERENCE TO CHANGE |
| --- | --- | --- | --- |
|  | 21-June 96 | Issue 1.0 |  |

TABLE OF CONTENTS

1. Introduction

2. Editorial Meeting Flimsy #1

2.1 Problem Statement

2.2 Resolution

3. Proposed Revisions to Chapter 4

# Introduction

The ATNP/WG2 Editorial Committee reviewed Chapter 4 (Addressing) of the draft ATN Internet SARPs and  found a number of editorial and technical problems with the specification. Flimsy #1 was then produced in order to record the problems found, and to suggest how a solution may be found. The contents of Flimsy #1 are repeated below.

# Editorial Meeting Flimsy #1

## Problem Statement

The following problems were identified:

- 5.4.1.10 misleadingly re-defines Administrative Domains as addressing domains. This hides a greater problem, that is that the notion of a Network Addressing Domain (as defined in ISO 8348) is not properly introduced and related to the ATN Addressing Plan. In particular, it is necessary to introduce the idea that there is an ATN Network Addressing Domain that is a subset of the Global NSAP Addressing Domain, and this is sub-divided  into a number of nested Addressing Domain,  each administered by a different authority i.e. that the administration of NSAP Addressing is iteratively devolved, to states and organisations and possible to subordinate organisations.

- The ICAO secretariat has already objected to ICAO being responsible for administering NSAP Addresses. In many places, ICAO is required to be responsible for address administration that is fully defined in the draft SARPs. This is clearly unnecessary. Furthermore, even when administration of addresses is devolved to (e.g.) Administrations, ICAO is still specified as the administrative authority.

- AINSC organisations are only recommended as to how the ADM field is assigned. However, for unambiguous address assignment,  it is necessary for either the draft SARPs, or a separate IATA specification, to definitively specify how this field is administered. A recommendation does not appear to be appropriate here.

- The encoding of the AFI and IDI fields is specified without reference to ISO 8348 and the preferred binary encoding. Instead, it is specified as “BCD” without a clear statement of whether a big or little endian approach is required. Furthermore, this is not consistent with the modified encoding specified in ISO 10589 and used by ISO 10747 for the encoding of NSAP Address Prefixes that end within the IDP.

- Alphanumeric and “Identifier” encodings for addressing fields are defined. However, neither is referenced by Table 5-4.1, which defines which encodings are used. The Alphanumeric encoding does appear to be used for the ADM field. However, no use is made of the identifier encoding.

- There is confusion in several fields of the DSP (e.g. the ADM field, which is unclear as to whether the hexadecimal or alphanumeric encoding is being used.

- The ADM field does not provides values for International Organisations (e.g. Eurocontrol) and ignores the need for efficient NSAP Address Allocation to the members of an ATN Island.

## Resolution

There appear to be three major changes required to this chapter:

- The first is to clearly define the nested addressing domains associated with the ATN NSAP Addressing Plan and revise which organisation, or type of organisation, is responsible for each such addressing domain.

- The second is to separate out the encoding of the AFI and IDI (which should follow the International standards) from the encoding of the DSP fields and to make clear which encoding is used for each field.

- The third is to introduce additional addressing domains (i.e. further ADM field allocations) for International Organisations and regions (e.g. ATN Islands). This may require the establishment of international addressing authorities.

# Proposed Revisions to Chapter 4

The following attachment provides replacement text for chapter 4. The following changes have been carried out:

- The specification of the TSAP Selector has been moved up to be before the NSAP Address specification. This was a change discussed by the editorial meeting and appears to be a more logical presentation. The encoding specification is changed to a reference to ISO 8073 as this provides the definitive specification.

- I have placed the definitions of “reserved” and character encoding in the introduction section, so that these are clearly defined “up front”.

- The Network Layer Addressing now starts with definitions of NSAP Address, NET, NSAP Address Prefix and the Network Addressing Domain.

- The NSAP Address Syntax is now first introduced with reference to ISO 10589. This is  after all, the only structure that is known to Routers and makes clear the derivation of the SYS and SEL fields in context.

- The IDP fields (AFI and IDI) are then introduced and defined with reference to the ISO 8348 addressing plan. I have specified their values at this point as decimal numbers, as these are essentially constants for the ATN Addressing Plan.

- The Reference Publication Format is then defined. This is part of the ISO standard, but was not previously part of the ATN Addressing Plan. I have introduced it here so that example NSAP Addresses can be readily referred to in the remainder of the chapter. A lack of examples had been noted by the editorial meeting.

- The full format of the ATN NSAP Address is then introduced and related back to ISO 8348 and ISO 10589 through the new figure (5.4-3).

- A technical defect is then fixed by making it clear that encoding of the NSAP Address is outside of the scope of this chapter. Indeed, many of the subsequent changes relate to removing the word “encoding” from the specification.

- The DSP fields are then specified. Although the field sizes and prescribed values are unchanged, the approach to the specification has had to be changed very significantly in order to provide examples, make clear administration responsibilities, and to concentrate on specifying the binary values of the fields instead of the misleading references to encoding.

- An additional VER field value and ADM encoding is added for Regional Addressing Plans (e.g. ATN Islands).

- The Default NSAP Address Prefixes for all Mobiles and to “Home” providers are now defined here.

- Given the replace all nature of the changes to field definitions, I have not included the struck out text in this part of the revised text.

# Recommendation

It is recommended that WG2 conducts a line by line review of the attachment prior to considering it as replacement text for Chapter 4 of the draft ATN Internet SARPs.