Clarification of ICS SARPs security label requirements

Prepared by Dirk Fieldhouse

# 1	Introduction

This flimsy describes a problem in the current specification of 5.6.2.2.4 of the ATN Internet Communication Service (ICS) SARPs and proposes replacement text.

# 2	Problem statement

Defect Report 113 on the Upper Layer Communications Service (ULCS) SARPs revises the example of the security label following informal discussions between WG2 and WG3/SG3 experts. In particular, this DR removes two octets from the format previously shown.

This specification is unsatisfactory:

a) the specification is in one place and the example is in another;

b) 	the specification in 5.6.2.2.4.1 is apparently ambiguous since experts have independently produced two different and incompatible interpretations.

It is essential that the encoding of the security parameter value is unambiguous since the encoded value will be used as a bit-string for routing decisions.

In addition, the specification calls up the Basic Encoding Rules for the only time in the ATN SARPs and this should be made an explicit reference.

# 3	Analysis

The encoding of an object identifier using the ASN.1 Basic Encoding Rules consists of

tag	[UNIVERSAL 6]

length	(of encoded value in octets)

value	(using a special compact encoding defined in BER)

The effect of this is that the ATN security registration object identifier {1 3 27 0 0} is encoded as '06 04 2B1B0000'H.

The ICS specification does not make it clear  whether the tag and length should be included in the Security Registration ID field. They are not needed since the length of the field is already known and the value is only allowed to be an object identifier. The previous ULCS example included the tag and length, in accordance with what appears to be the stated requirement in 5.6.2.2.4, but these have now been removed.

# 4	Proposal

The text below shows with double-underline and strikeout the proposed revision of ICS SARPs clause 5.6.2.2.4.

#### 5.6.2.2.4	Security Registration IDtc "5.0§ Security Registration ID" \l 4§

##### 5.6.2.2.4.1	This variable field shall contain the encoded value only of a Security Type object identifier, encoded using ASN.1 Basic Encoding Rules [ISO/IEC 8825-1], and omitting the tag and length fields.

##### 5.6.2.2.4.2	The Security Type object identifier shall bewith the following sequence of integer values:

{1 3 27 0 0}	The ATN Security Registration Identifier

Note.— The ATN Security Registration Identifier identifies the ATN Security Authority. ICAO has been assigned an International Code Designator (ICD) decimal value [0027] in accordance with the dictates of ISO/IEC 6523. According to ISO/IEC 6523 and ISO/IEC 8824 this value identifies an arc of the identified organisation of ISO. ICAO object identifiers designate an ICAO defined hierarchy starting with {1 3 27}. Under this arc, {0} has been designated as ATN, and the flat address space under ATN starts with object identifiers {0,1,2,3,4, ...}. Value {0} has been assigned as the Traffic Type and Routing Policy identifier.