# Internetwork Service and Protocol Specification

## Introduction

Note 1. — The ATN Internet comprises a number of interconnected ATN routers and constituent subnetworks supporting  data communication among host computers operating the ATN Internet protocols.

Note 2.— All ATN NPDUs (Network Protocol Data Units) are encapsulated within appropriate subnetwork protocol data units for transfer among ATN network entities using the connectionless ISO OSI Network Layer service provided by the ATN Internet. As the ATN Internet protocol is connectionless, any information required to process a particular NPDU is carried within the header of that network protocol data unit for processing by ATN routers and host computers.

### Scope

Note 1.— This chapter provides requirements and recommendations pertaining to the use of the ISO/IEC 8473 by ATN End System and Intermediate System Network entities. This Chapter is concerned with the use of ISO/IEC 8473 in the context of the internetworking protocol approach to the provision of CLNS as defined in ISO/IEC 8348. This Chapter contains ATN-specific protocol implementations and is concerned with the interoperability of protocol implementations. It therefore provides appropriate compliance statements and APRLs for this purpose.

Note 2. — The ATN Network Layer Connectionless-Mode Network Service supports the transfer of a connectionless network service data unit (NSDU) from a source NSAP to a destination NSAP within the ATN network. Each such NSDU transfer is the result of a single invocation of the connectionless-mode Network Service encompassed within the ATN.

### Applicability of Requirements

#### All ATN Intermediate System and End System Network entities shall comply with the provisions contained in 5.6.2 and 5.6.3, in addition to all APRLs specified in 5.6.5.

## ATN Specific Features

### Purpose of ATN Specific Features

Note 1.— The ATN infrastructure, referred to as an Internet, comprises the interconnection of computers with gateways and routers via real subnetworks. This internetworking infrastructure, allows for the incorporation of differing Air/Ground and Ground/Ground subnetworks servicing differing user groups, i.e., Air Traffic Services (ATS), Aeronautical Operational Control Services (AOC), and others.

Note 2.— The CLNP (Connectionless Network Protocol) protocol used to operate this internetworking infrastructure is based on ISO/IEC 8473 with ATN-specific additions to reflect the unique communications environment of the ATN.

Note 3.— The ATN specific functions listed in this chapter reflect responses to the additional functional needs of ATN Network entities in order to support user requirements concerned with:

a)	Ensuring that information is conveyed about Traffic Type and Routing Policy requirements pertaining to user data in NPDUs;

b)	Ensuring that a priority scheme can be applied for management of End Systems and Intermediate Systems output queues and buffers;

c)	 Ensuring that specific policies and procedures are available to handle congestion avoidance and congestion control requirements within the ATN.

### The Security Function

#### General

##### The SECURITY Function of ISO/IEC 8473, as defined in this specification, shall be supported by ATN End System or Intermediate System Network entities receiving or transmitting inter-domain traffic other than Traffic Type as General Communications.

##### ATN Network entities shall therefore provide the Globally Unique Security format for all created NPDUs.

##### The sole exception to this requirement is for General Communications traffic where no Security parameter information is required to be encoded in created NPDUs.

#### Encoding of the Security Parameter

##### The CLNP Options Security Parameter shall be used in the ATN to convey information about the Traffic Type and Routing Policy Requirements pertaining to the user data of the NPDU (other than General Communications).

Note .— The CLNP Options Security Parameter may also be used to convey a security classification.

##### The value component of the CLNP Options Security Parameter (in the NPDU header) shall be encoded as follows:

- The first octet shall always be encoded as [1100 0000] to indicate the Globally Unique Security Format;

- The remaining octets shall contain the ATN Security Label encoded as the four fields illustrated in Figure 5.6-1, and defined below.

|  | Security Registration ID Length | Security Registration ID (variable) | Security Information Length |  | Security Information (optional) |
| --- | --- | --- | --- | --- | --- |
| Octet | 0 | 1 | n |  | n+1 |
|  | Figure 5.6-1: The ATN Security Label |  |  |  |  |

#### Security Registration ID Length

##### This field shall be one octet long and contain the length in octets of the Security Authority's Security Registration Identifier.

Note .— The Security Registration ID identifies the authority that has specified the associated security policy.

#### Security Registration ID

##### This variable field shall contain a Security Type object identifier encoded using ASN.1 Basic Encoding Rules with the following sequence of integer values:

{1 3 27 0 0}	The ATN Security Registration Identifier

Note.— The ATN Security Registration Identifier identifies the ATN Security Authority. ICAO has been assigned an International Code Designator (ICD) decimal value [0027] in accordance with the dictates of ISO/IEC 6523. According to ISO/IEC 6523 and ISO/IEC 8824 this value identifies an arc of the identified organisation of ISO. ICAO object identifiers designate an ICAO defined hierarchy starting with {1 3 27}. Under this arc, {0} has been designated as ATN, and the flat address space under ATN starts with object identifiers {0,1,2,3,4, ...}. Value {0} has been assigned as the Traffic Type and Routing Policy identifier.

#### Security Information Length

##### This field shall be one octet in length and shall define the length in octets of the Security Information.

##### If there is no security information, this field shall be set to zero.

#### Security Information

##### General

###### When present, the Security Information field of the ATN Security Label shall be used to convey, as separate Tag Sets:

- The Traffic Type and Routing Policy Requirements, if any, applicable to the transfer of the user data through the ATN.

- The Security Classification

###### When no traffic type is identified then the General Communications traffic type shall be assumed, with a routing policy requirement of “no preference”. When no security classification is specified then “unclassified” shall be assumed.

##### Encoding of the Security Information Field

###### The Security Information Field shall comprise zero, one or more Security Tag Sets. A Security Tag with the same Tag Set Name shall not occur more than once in the options Security Parameter of the CLNP NPDU

| Each Security Tag Set shall consist of four fields, as illustrated in Figure 5.6-2, and shall be as defined in the following sections: | Tag Set Name Length | Tag Set Name | Tag Set Length | Security Tag |
| --- | --- | --- | --- | --- |
| Octet | 0 | 1 | n | n+1 |
|  |  | Figure 5.6-2: Security Tag Set Format |  |  |

Note.— This format has been chosen to provide for an extensible type-length-value encoding method for security related information placed in the CLNP Header under rules specified by the ATN Security Authority.

##### Security Tag Set Name Length

###### The Security Tag Set Name Length shall contain the length in octets of the Tag Set Name field.

##### Security Tag Set Name

###### The Security Tag Set Name shall be used to uniquely identify the Tag Set.

##### Tag Set Length

###### The Tag Set Length Field shall contain the length in octets of the Security Tag field.

##### Security Tag

###### The Security Tag field shall be used to convey security related information for which the syntax and semantics are identified by the preceding Tag Set Name.

##### Encoding of the Tag Set for Traffic Type and Associated Routing Policies

###### The Tag Set Name shall be set to [0000 1111].

###### When present in the CLNP options Security Parameter, this Tag Set shall always be the first Tag Set to be encoded in the Security Information field of the ATN Security Label.

Note.— This Tag Set is used to identify the traffic type of the data, whether it is for ATC or Airline communications, and, for Operational Communications, any Routing Policy requirements that apply.

###### The Security Tag shall indicate the Routing Policy Requirements for the data contained in the same NPDU, according to Table 5.6-1.

Note . — See 5.2.7 for detailed information on the ATN Security Policy.

Encoding of the Tag Set for Security Classification

Table 5.6-1 Encoding of Traffic Type Security Tag

| Traffic Type | Category | Security Tag Value | Semantics |
| --- | --- | --- | --- |
| ATN Operational Communications | Air Traffic Service Communications (ATSC) | 000 00001 | No Traffic Type Policy Preference. |
|  |  | 000 10000 | Traffic preference for Class A ATSC route(s). |
|  |  | 000 10001 | Traffic preference for Class B ATSC route(s). |
|  |  | 000 10010 | Traffic preference for Class C ATSC route(s). |
|  |  | 000 10011 | Traffic preference for Class D ATSC route(s). |
|  |  | 000 10100 | Traffic preference for Class E ATSC route(s). |
|  |  | 000 10101 | Traffic preference for Class F ATSC route(s). |
|  |  | 000 10110 | Traffic preference for Class G ATSC route(s). |
|  |  | 000 10111 | Traffic preference for Class H ATSC route(s). |
|  | Aeronautical Operational Control (AOC) | 001 00001 | No Traffic Type Policy Preference. |
|  |  | 001 00010 | Route Traffic only via Gatelink. |
|  |  | 001 00011 | Route Traffic only via VHF Data Link. |
|  |  | 001 00100 | Route Traffic only via Satellite Data Link. |
|  |  | 001 00101 | Route Traffic only via HF Data Link. |
|  |  | 001 00110 | Route Traffic only via Mode S Data Link. |
|  |  | 001 00111 | Route Traffic using an ordered preference of Gatelink first, then VHF Data Link. |
|  |  | 001 01000 | Route Traffic using an ordered preference of Gatelink first, then VHF Data Link, then Satellite. |
|  |  | 001 01001 | Route Traffic using an ordered preference of Gatelink first, then VHF Data Link, then HF Data Link, then Satellite Data Link. |
| ATN Administrative Communications | No category defined | 001 10000 | No Traffic Type Policy preference. |
| General Communications | No category defined | N/A | Note. — General Communications traffic does not require encoding of security parameters within created NPDUs. Specification of a Security Tag Value for such General communications is therefore not applicable. |
| ATN Systems Management Communications | No category defined | 011 00000 | No Traffic Type Policy preference. |

###### The Tag Set Name shall be set to [0000 0011].

###### When present in the security parameter, this Tag Set shall always follow the Tag Set for Traffic Type and Associated Routing Policies (see 5.) if present, but otherwise shall be the first Tag Set to be encoded in the field.

Note.— The purpose of this field is to permit the later extension of the ATN to handle classified data.

###### The Security Tag shall indicate the security classification of the NPDU according to the following table:

Table 5.6-2 Encoding of the Security Classification Tag

| Value | Security Classification |
| --- | --- |
| 0000 0001 | unclassified |
| 0000 0010 | restricted |
| 0000 0011 | confidential |
| 0000 0100 | secret |
| 0000 0101 | top secret |
| 0000 0110 to 1111 1111 | unassigned |

### Management of Network Priority

Note.— Network priority handling provisions are specified in 5.2.8.

### Congestion Management

Note 1.— The congestion management provisions in the Network Layer are intended to guarantee the notification to the Transport Layer of potential risks of congestion via the CE bit conveyed in the QoS Maintenance parameter of an ISO/IEC 8473 NPDU. 5.5.2.5 defines the measures that the Transport Layer implements upon receipt of NPDUs with the CE bit set.

Note 2.— The above requirement is applicable to all types of ISO/IEC 8473 NPDUs.

#### Setting of the congestion experienced flag

##### The congestion experienced flag (CE-flag) in the QoS maintenance parameter in the options part of an NPDU header shall initially be set to zero by the originator of the NPDU.

##### When a NPDU is being forwarded by an ATN Intermediate System, the Intermediate System shall examine the depth of the output queue selected for that NPDU.

##### If the depth of the selected output queue exceeds a threshold a (see Table 5.6-3), the ATN Intermediate System shall set the CE-flag in the NPDU header.

Note.— The above assumes a single output queue per network (CLNP) priority.  If mixed priority queues are implemented, which is valid provided that priority order is always maintained, then the CE-flag is set only when the number of NPDUs on the queue of the same or a higher priority exceeds alpha.

##### Once the CE-flag is set, it shall not be reset by any ATN Intermediate System traversed by the NPDU further along to the path towards the destination.

#### Forwarding congestion information to the receiving NS-User

##### For each sequence of NPDUs that together form an NSDU, the destination network entity shall keep two counters:

- the first one, n-total, shall reflect the length of that sequence.

- the second one, n-CE, shall reflect the number of those NPDUs of this sequence, that had the CE-flag set on reception by the destination network entity.

Note.— Each NSDU is forwarded through the network as a sequence consisting of one or more NPDUs.

##### When the destination network entity passes an NSDU to the receiving NS-User, it shall convey the associated counters n-total and n-CE to the NS-User.

Note.— The conveyance of the congestion information associated with an NSDU to the NS-User is a local matter.

#### Required algorithm values

##### The value settings defined in the following table shall be implemented:

Table 5.6-3 Required Value for a

| Name | Description | Required range |
| --- | --- | --- |
| a | Output queue threshold | 1 packet |

## ATN Specific Requirements for ISO/IEC 8473

Note . — This section defines ATN specific requirements on the ISO/IEC 8473 Protocol.

### Segmentation Function.

#### ATN Intermediate Systems (ISs) shall support both the segmenting and the non-segmenting subsets of ISO/IEC 8473.

#### ATN End Systems shall support the ISO/IEC 8473 segmenting subset.

#### Recommendation. — ATN End Systems should use the non-segmenting ISO/IEC 8473 subset for NSDUs of up to 1024 octets.

### Security Function

Note . — The ATN Specific use of the ISO/IEC 8473 Security Function is specified in 5.6.2.2.

### Echo Request Function

#### Recommendation. —— ATN End System and Intermediate System Network Entities (NEs) should support the ECHO REQUEST Function as invoked by Network Layer management.

Note.— The Echo Request Function is invoked to obtain information on the reachability of specific network entities and the path characteristics between NEs through the operation of Network Layer routing functions.

### Network Priority

Note . — The ATN Specific use of the ISO/IEC 8473 Priority is specified in 5.2.8.4.

## APRLs

### General

#### An implementation of the ISO/IEC 8473 Protocol shall be used in an ATN System if and only if its PICS is in compliance with the APRL provided in these SARPs.

Note.—The CLNP requirements list is a statement of which capabilities and options of the protocol at minimum are required to be implemented for the ATN environment. The requirements list may be used by the protocol implementor as a check list to conform to this standard; by the supplier and procurer to provide a detailed indication of the capabilities of an implementation; by the user to check the possibility of interworking between two different implementations; and by the protocol tester, as the basis for selecting appropriate tests against which to assess the claim for conformance to the protocol.

### Support of ATN-Specific Network Layer features

| Index | Item | ATN SARPs Reference | ATN Support |
| --- | --- | --- | --- |
| ATN CLNP1 | Encoding and use of the Security Parameter | 5.6.2.2 | M |
| ATN CLNP2 | Management of Network Priority | 5.6.2.3, 5.2.8.4 | M |
| ATN CLNP3 | Use of Non-Segmenting Subset | 5.6.3.1.3 | O |
| ATN CLNP4 | Echo Request Function | 5.6.3.3 | O |
| ATN CLNP5 | Congestion Management | 5.6.2.4 | M |

### Major Capabilities

| Item | Capability | ISO/IEC 8473 Reference | Status | ATN Support |
| --- | --- | --- | --- | --- |
| ES | End System |  | O.1 | O.1 |
| IS | Intermediate System |  | O.1 | O.1 |
| FL-r | <r> Full protocol | 8473-1: 6 | M | M |
| FL-s | <s> Full protocol | 8473-1: 6 | M | M |
| NSS-r | <r> Non-segmenting subset | 8473-1: 5.2 | M | M |
| NSS-s | <s> Non segmenting subset | 8473-1: 5.2 | IS:M ^IS:O | IS:M ^IS:O |
| IAS-r | <r> Inactive subset | 8473-1: 5.2 | ES:O | ES:O |
| IAS-s | <s> Inactive subset | 8473-1: 5.2 | IAS-r:M ^IAS-r:X | IAS-r:M ^IAS-r:X |
| S802 | SNDCF for ISO 8802 | 8473-2: 5.4 | O.2 | O |
| SCLL | SNDCF for CL Link Service | 8473-4: 5.3.1 | O.2 | O |
| SCOL | SNDCF for CO Link Service | 8473-4: 5.3.2 | O.2 | O |
| SX25 | SNDCF for ISO 8208 | 8473-3: 5.4 | O.2 | O |
| ATN SNDCF | SNDCF for Mobile Subnetworks | ATN SARPs Ref: 5.7 | N/A | ISMOB:M ISGRD:O ^IS:O |

ISMOB: If ISO/IEC 8473 is used over Mobile Subnetworks, then ISMOB is true, else ISMOB is false.

ISGRD: If ISO/IEC 8473 is used over Ground Subnetworks, then ISGRD is true, else ISGRD is false.

O.1: The supported functions, NPDUs, associated parameters and timers required for End Systems are provided in APRLs 5.6.4.4 through 5.6.5.11. The supported functions, NPDUs, associated parameters and timers required for Intermediate Systems are provided in APRLs 5.6.5.12 through 5.6.4.18.

O.2: APRLs for the SNDCF for use with ISO/IEC 8802-2 subnetworks are provided in 5.7.7.2 and 5.7.7.3. APRLs for the SNDCF for use with ISO/IEC 8208 subnetworks are provided in 5.7.7.4 through 5.7.7.7.

### End Systems - Supported Functions

| Item | Function | ISO/IEC 8473-1 Reference | Status | ATN Support |
| --- | --- | --- | --- | --- |
| ePDUC | PDU Composition | 6.1 | M | M |
| ePDUD | PDU Decomposition | 6.2 | M | M |
| eHFA | Header Format Analysis | 6.3 | M | M |
| ePDUL-s | <s> PDU Lifetime Control | 6.4 | M | M |
| ePDUL-r | <r> PDU Lifetime Control | 6.4 | O | M |
| eRout | Route PDU | 6.5 | M | M |
| eForw | Forward PDU | 6.6 | M | M |
| eSegm | Segment PDU | 6.7 | M | M |
| eReas | Reassemble PDU | 6.8 | M | M |
| eDisc | Discard PDU | 6.9 | M | M |
| eErep | Error Reporting | 6.10 | M | M |
| eEdec-s | <s> Header Error Detection | 6.11 | M | M |
| eEdec-r | <r> Header Error Detection | 6.11 | M | M |
| eSecu-s | <s> Security | 6.13, ATN SARPs Ref: 5.6.2.2 | O | M |
| eSecu-r | <r> Security | 6.13, ATN SARPs Ref. 5.6.2.2 | O | M |
| eCRR-s | <s> Complete Route Recording | 6.15 | O | OX |
| eCRR-r | <r> Complete Route Recording | 6.15 | O | O |
| ePRR-s | <s> Partial Route Recording | 6.15 | O | M |
| ePRR-r | <r> Partial Route Recording | 6.15 | O | M |
| eCSR | Complete Source Routing | 6.14 | O | OX |
| ePSR | Partial Source Routing | 6.14 | O | OX |
| ePRI-s | <s> Priority | 6.17, ATN SARPs Ref. 5.6.3.4 | O | M |
| ePRI-r | <r> Priority | 6.17, ATN SARPs Ref. 5.6.3.4 | O | M |

| Item | Function | ISO/IEC 8473-1 Reference | Status | ATN Support |
| --- | --- | --- | --- | --- |
| eQOSM-s | <s> QOS Maintenance | 6.16 | O | M |
| eQOSM-r | <r> QOS Maintenance | 6.16 | O | M |
| eCong-s | <s> Congestion Notification | 6.18 | eQOSM-s:M | eQOSM-s:M |
| eCong-r | <r> Congestion Notification | 6.18 | O | M |
| ePadd-s | <s> Padding | 6.12 | O | OX |
| ePadd-r | <r> Padding | 6.12 | M | M |
| eEreq | Echo request | 6.19, ATN SARPs Ref. 5.6.3.3 | O | O |
| eErsp | Echo response | 6.20 | O | M |
| eSegS | Create segments smaller than necessary | 6.8 | O | O |

### End Systems - Supported NPDUs

| Item | NPDU | ISO/IEC 8473-1 Reference | Status | ATN Support |
| --- | --- | --- | --- | --- |
| eDT-t | DT (full protocol) transmit | 7.7 | M | M |
| eDT-r | DT (full protocol) receive | 7.7 | M | M |
| eDTNS-t | DT (non-segment) transmit | 7.7 | NSS-s:M | NSS-s:M |
| eDTNS-r | DT (non-segment) receive | 7.7 | M | M |
| eER-t | ER transmit | 7.9 | M | M |
| eER-r | ER receive | 7.9 | M | M |
| eIN-t | Inactive PDU transmit | 7.8 | IAS-s:M | IAS-s:M |
| eIN-r | Inactive PDU receive | 7.8 | IAS-r:M | IAS-r:M |
| eERQ-t | ERQ transmit | 7.10 | eEreq:M | eEreq:M |
| eERQ-r | ERQ receive | 7.10 | M | M |
| eERP-t | ERP transmit | 7.11 | eErsp:M | eErsp:M |
| eERP-r | ERP receive | 7.11 | M | M |

### End Systems - Supported DT Parameters

| Item | Parameter | ISO/IEC 8473-1 Reference | Status | ATN Support |
| --- | --- | --- | --- | --- |
| edFxPt-s | <s> Fixed Part | 7.2 | M | M |
| edFxPt-r | <r> Fixed Part | 7.2 | M | M |
| edAddr-s | <s> Address | 7.3 | M | M |
| edAddr-r | <r> Address | 7.3 | M | M |
| edSeg-s | <s> Segmentation Part | 7.4 | M | M |
| edSeg-r | <r> Segmentation Part | 7.4 | M | M |
| edPadd-s | <s> Padding | 7.5.2 | ePadd-s:M | - |
| edPadd-r | <r> Padding | 7.5.2 | M | M |
| edSecu-s | <s> Security | 7.5.3 | eSecu-s:M | eSecu-s:M |
| edSecu-r | <r> Security | 7.5.3 | eSecu-r:M | eSecu-r:M |
| edCRR-s | <s> Complete Route Recording | 7.5.5 | eCRR-s:M | - |
| edCRR-r | <r> Complete Route Recording | 7.5.5 | eCRR-r:M | eCRR-r:M |
| edPRR-s | <s> Partial Route Recording | 7.5.5 | ePRR-s:M | ePRR-s:M |
| edPRR-r | <r> Partial Route Recording | 7.5.5 | ePRR-r:M | ePRR-r:M |
| edCSR-s | <s> Complete Source Routing | 7.5.4 | eCSR:M | - |
| edPSR-s | <s> Partial Source Routing | 7.5.4 | ePSR:M | - |
| edQOSM-s | <s> QOS Maintenance | 7.5.6 | eQOSM-s or eCong-s:M | eQOSM-s:M |
| edQOSM-r | <r> QOS Maintenance | 7.5.6 | eQOSM-r or eCong-r :M | eQOSM-r or eCong-r:M |
| edPri-s | <s> Priority | 7.5.7 | ePRI-s:M | ePRI-s:M |
| edPri-r | <r> Priority | 7.5.7 | ePRI-r:M | ePRI-r:M |
| edData-s | <s> Data | 7.6 | M | M |
| edData-r | <r> Data | 7.6 | M | M |
| edUnSup2 | Are received PDUs containing parameters selecting unsupported type 2 functions discarded and where appropriate an Error Report PDU generated ? | 6.21 | M | M |
| edUnSup3 | Are parameters selecting unsupported Type 3 functions ignored ? | 6.21 | M | M |

### End Systems - Supported ER Parameters

| Item | Parameter | ISO/IEC 8473-1 Reference | Status | ATN Support |
| --- | --- | --- | --- | --- |
| eeFxPt-s | <s> Fixed Part | 7.2 | M | M |
| eeFxPt-r | <r> Fixed Part | 7.2 | M | M |
| eeAddr-s | <s> Address | 7.3 | M | M |
| eeAddr-r | <r> Address | 7.3 | M | M |
| eePadd-s | <s> Padding | 7.5.2 | ePadd-s:M | - |
| eePadd-r | <r> Padding | 7.5.2 | M | M |
| eeSecu-s | <s> Security | 7.5.3 | eSecu-s:M | eSecu-s:M |
| eeSecu-r | <r> Security | 7.5.3 | eSecu-r:M | eSecu-r:M |
| eeCRR-s | <s> Complete Route Recording | 7.5.5 | eCRR-s:M | - |
| eeCRR-r | <r> Complete Route Recording | 7.5.5 | eCRR-r:M | eCRR-r:M |
| eePRR-s | <s> Partial Route Recording | 7.5.5 | ePRR-s:M | ePRR-s:M |
| eePRR-r | <r> Partial Route Recording | 7.5.5 | ePRR-r:M | ePRR-r:M |
| eeCSR-s | <s> Complete Source Routing | 7.5.4 | eCSR:M | - |
| eePSR-s | <s> Partial Source Routing | 7.5.4 | ePSR:M | - |
| eeQOSM-s | <s> QOS Maintenance | 7.5.6 | eQOSM-s or eCong-s:M | eQOSM-s or eCong-s:M |
| eeQOSM-r | <r> QOS Maintenance | 7.5.6 | eQOSM-r or eCong-r:M | eQOSM-r or eCong-r:M |
| eePri-s | <s> Priority | 7.5.7 | ePri-s:M | ePri-s:M |
| eePri-r | <r> Priority | 7.5.7 | ePri-r:M | ePri-r:M |
| eeDisc-s | <s> Reason for discard | 7.9.5 | M | M |
| eeDisc-r | <r> Reason for discard | 7.9.5 | M | M |
| eeData-s | <s> Data | 7.9.6 | M | M |
| eeData-r | <r> Data | 7.9.6 | M | M |
| eeUnSup2 | Are received PDUs containing parameters selecting unsupported type 2 functions discarded and where appropriate an Error Report PDU generated ? | 6.21 | M | M |
| eeUnSup3 | Are parameters selecting unsupported Type 3 functions ignored ? | 6.21 | M | M |

### End Systems - Inactive DT Parameters

| Item | Parameter | ISO/IEC 8473-1 Reference | Status | ATN Support |
| --- | --- | --- | --- | --- |
| eiNLPI-s | <s> Inactive Network Layer Protocol identifier | 7.8.2 | IAS-s:M | IAS-s:M |
| eiNLPI-r | <r> Inactive Network Layer Protocol Identifier | 7.8.2 | IAS-r:M | IAS-r:M |
| eiData-s | <s> Data | 7.8.3 | IAS-s:M | IAS-s:M |
| eiData-r | <r> Data | 7.8.3 | IAS-r:M | IAS-r:M |

### End Systems - Supported ERQ Parameters

| Item | Parameter | ISO/IEC 8473-1 Reference | Status | ATN Support |
| --- | --- | --- | --- | --- |
| eqFxPt-s | <s> Fixed Part | 7.2 | M | M |
| eqFxPt-r | <r> Fixed Part | 7.2 | M | M |
| eqAddr-s | <s> Address | 7.3 | M | M |
| eqAddr-r | <r> Address | 7.3 | M | M |
| eqSeg-s | <s> Segmentation Part | 7.4 | M | M |
| eqSeg-r | <r> Segmentation Part | 7.4 | M | M |
| eqPadd-s | <s> Padding | 7.5.2 | ePadd-s:M | - |
| eqPadd-r | <r> Padding | 7.5.2 | M | M |
| eqSecu-s | <s> Security | 7.5.3 | eSecu-s:M | eSecu-s:M |
| eqSecu-r | <r> Security | 7.5.3 | eSecu-r:M | eSecu-r:M |
| eqCRR-s | <s> Complete Route Recording | 7.5.5 | eCRR-s:M | - |
| eqCRR-r | <r> Complete Route Recording | 7.5.5 | eCRR-r:M | eCRR-r:M |
| eqPRR-s | <s> Partial Route Recording | 7.5.5 | ePrr-s:M | ePrr-s:M |
| eqPRR-r | <r> Partial Route Recording | 7.5.5 | ePrr-r:M | ePrr-r:M |
| eqCSR-s | <s> Complete Source Routing | 7.5.4 | eCSR:M | - |
| eqPSR-s | <s> Partial Source Routing | 7.5.4 | ePSR:M | - |
| eqQOSM-s | <s> QOS Maintenance | 7.5.6 | eQOSM-s or eCong-s:M | eQOSM-s:M |
| eqQOSM-r | <r> QOS Maintenance | 7.5.6 | eQOSM-r or eCong-r :M | eQOSM-r or eCong-r:M |
| eqPri-s | <s> Priority | 7.5.7 | ePri-s:M | ePri-s:M |
| eqPri-r | <r> Priority | 7.5.7 | ePri-r:M | ePri-r:M |
| eqData-s | <s> Data | 7.6 | M | M |
| eqData-r | <r> Data | 7.6 | M | M |
| eqUnSup2 | Are received PDUs containing parameters selecting unsupported type 2 functions discarded and where appropriate an Error Report PDU generated ? | 6.21 | M | M |
| eqUnSup3 | Are parameters selecting unsupported Type 3 functions ignored ? | 6.21 | M | M |

### End Systems - Supported ERP Parameters

| Item | Parameter | ISO/IEC 8473-1 Reference | Status | ATN Support |
| --- | --- | --- | --- | --- |
| epFxPt-s | <s> Fixed Part | 7.2 | M | M |
| epFxPt-r | <r> Fixed Part | 7.2 | M | M |
| epAddr-s | <s> Address | 7.3 | M | M |
| epAddr-r | <r> Address | 7.3 | M | M |
| epSeg-s | <s> Segmentation Part | 7.4 | M | M |
| epSeg-r | <r> Segmentation Part | 7.4 | M | M |
| epPadd-s | <s> Padding | 7.5.2 | ePadd-s:M | - |
| epPadd-r | <r> Padding | 7.5.2 | M | M |
| epSecu-s | <s> Security | 7.5.3 | eSecu-s:M | eSecu-s:M |
| epSecu-r | <r> Security | 7.5.3 | eSecu-r:M | eSecu-r:M |
| epCRR-s | <s> Complete Route Recording | 7.5.5 | eCRR-s:M | - |
| epCRR-r | <r> Complete Route Recording | 7.5.5 | eCRR-r:M | eCRR-r:M |
| epPRR-s | <s> Partial Route Recording | 7.5.5 | ePr-s:M | ePrr-s:M |
| epPRR-r | <r> Partial Route Recording | 7.5.5 | ePr-r:M | ePrr-r:M |
| epCSR-s | <s> Complete Source Routing | 7.5.4 | eCSR:M | - |
| epPSR-s | <s> Partial Source Routing | 7.5.4 | ePSR:M | - |
| epQOSM-s | <s> QOS Maintenance | 7.5.6 | eQOSM-s or eCong-s:M | eQOSM-s:M |
| epQOSM-r | <r> QOS Maintenance | 7.5.6 | eQOSM-r or eCong-r :M | eQOSM-r or eCong-r:M |
| epPri-s | <s> Priority | 7.5.7 | ePri-s:M | ePri-s:M |
| epPri-r | <r> Priority | 7.5.7 | ePri-r:M | ePri-r:M |
| epData-s | <s> Data | 7.6 | M | M |
| epData-r | <r> Data | 7.6 | M | M |
| epUnSup2 | Are received PDUs containing parameters selecting unsupported type 2 functions discarded and where appropriate an Error Report PDU generated ? | 6.21 | M | M |
| epUnSup3 | Are parameters selecting unsupported Type 3 functions ignored ? | 6.21 | M | M |

### End Systems - Timers

| Item | Timer | Ref | ISO Status | ISO Range | ATN Support | Values Supported |
| --- | --- | --- | --- | --- | --- | --- |
| ELifReas | Is reassembly timer <= received derived PDU lifetime? | 6.8 | M |  | M |  |
| eReasTim | Reassembly Timer | 6.8 | M | 500ms to 127.5s | M | <= lifetime |

### Intermediate Systems - Supported Functions

| Item | Function | ISO/IEC 8473-1 Reference | Status | ATN Support |
| --- | --- | --- | --- | --- |
| iPDUC | PDU Composition | 6.1 | M | M |
| iPDUD | PDU Decomposition | 6.2 | M | M |
| iHFA | Header Format Analysis | 6.3 | M | M |
| iPDUL | <s> PDU Lifetime Control | 6.4 | M | M |
| iRout | Route PDU | 6.5 | M | M |
| iForw | Forward PDU | 6.6 | M | M |
| iSegm | Segment PDU | 6.7 | iDSNS:M | iDSNS:M |
| iReas | Reassemble PDU | 6.8 | O | O |
| iDisc | Discard PDU | 6.9 | M | M |
| iErep | Error Reporting | 6.10 | M | M |
| iEdec | <s> Header Error Detection | 6.11 | M | M |
| iSecu | <s>Security | 6.13 ATN SARPs Ref: 5.6.2.2 | O | M |
| iCRR | <s> Complete Route Recording | 6.15 | O | OX |
| iPRR | <s> Partial Route Recording | 6.15 | O | M |
| iCSR | Complete Source Routing | 6.14 | O | OX |
| iPSR | Partial Source Routing | 6.14 | O | OX |
| iPri | <s> Priority | 6.17, ATN SARPs Ref: 5.6.3.4 | O | M |
| iQOSM | <s> QOS Maintenance | 6.16 | O | M |
| iCong | <s> Congestion Notification | 6.18, ATN SARPs Ref: 5.6.2.4 | O | M |
| iPadd | <s> Padding | 6.12 | M | M |
| iEreq | Echo request | 6.19, ATN SARPs Ref: 5.6.3.3 | O | O |
| iErsp | Echo response | 6.20 | O | M |
| iSegS | Create segments smaller than necessary | 6.8 | O | O |
| iDSNS | Simultaneous support of subnetworks with different SN-User data sizes | 6.7 | O | O |

#### Supported Security Parameters

| Item | Function | ISO/IEC 8473-1 Reference | Status | ATN Support |
| --- | --- | --- | --- | --- |
| iSADSSEC | Source Address Specific Security | 7.5.3.1 | iSecu:O.5 | iSecu:O |
| iDADSSEC | Destination Address Specific Security | 7.5.3.2 | iSecu:O.5 | iSecu:O |
| iGUNSEC | Globally Unique Security | ATN SARPs Ref. 5.6.2.2 | iSecu:O.5 | iSecu:M |

O.5: The Security parameter within a single NPDU specifies a security format code indicating Source Address Specific, Destination Address Specific or Globally Unique Security.

#### Quality of Service Maintenance Function

| Item | Function | ISO/IEC 8473-1 Reference | Status | ATN Support |
| --- | --- | --- | --- | --- |
| iQOSNAVAIL | If requested QOS not available, deliver at different QOS | 6.16 | iQOSM:M | iQOSM:M |
| iQOSNOT | Notification of failure to meet requested QOS | 6.16 | iQOSM:O | iQOSM:O |
|  | Which of the following formats of QOS are implemented ? |  |  |  |
| iSADDQOS | Source Address Specific QOS | 7.5.6.1 | iQOSM:O.3 | iQOSM:O |
| iDADDQOS | Destination Address Specific QOS | 7.5.6.2 | iQOSM:O.3 | iQOSM:O |
| iGUNQOS | Globally Unique QOS | 7.5.6.3 | iQOSM:O.3 | iQOSM:M |
| iSvTD | Sequencing versus Transit Delay | 7.5.6.3 | iGUNQOS:O.4 | iGUNQOS:O.4 |
| iCongE | Congestion Experienced | 7.5.6.3 | iGUNQOS:O.4 | iGUNQOS:M |
| iTDvCst | Transit Delay versus Cost | 7.5.6.3 | iGUNQOS:O.4 | iGUNQOS:O.4 |
| iREPvTD | Residual Error Probability versus Transit Delay | 7.5.6.3 | iGUNQOS:O.4 | iGUNQOS:O.4 |
| iREPvCst | Residual Error Probability versus Cost | 7.5.6.3 | iGUNQOS:O.4 | iGUNQOS:O.4 |

O.3: The Quality of Service Maintenance parameter within a single NPDU specifies a QOS format code indicating Source Address Specific, Destination Address Specific or Globally Unique QOS.

O.4: If the QOS format code indicates that the Globally Unique QOS maintenance function is employed, then each bit in the associated parameter value may be set to indicate the order of intra and inter domain routing decisions based on QOS. The parameter values which apply to inter-domain routing are provided in Table 4 of ISO/IEC 10747.

### Intermediate Systems - Supported NPDUs

| Item | Function | ISO/IEC 8473-1 Reference | Status | ATN Support |
| --- | --- | --- | --- | --- |
| iDT-t | DT (full protocol) transmit | 7.7 | M | M |
| iDT-r | DT (full protocol) receive | 7.7 | M | M |
| iDTNS-t | DT (non-segment) transmit | 7.7 | M | M |
| iDTNS-r | DT (non-segment) receive | 7.7 | M | M |
| iER-t | ER transmit | 7.9 | M | M |
| iER-r | ER receive | 7.9 | M | M |
| iERQ-t | ERQ transmit | 7.10 | iEreq:M | O |
| iERQ-r | ERQ receive | 7.10 | M | M |
| iERP-t | ERP transmit | 7.11 | iErsp:M | O |
| iERP-r | ERP receive | 7.11 | M | M |

### Intermediate Systems - Supported DT Parameters

| Item | Parameter | ISO/IEC 8473-1 Reference | Status | ATN Support |
| --- | --- | --- | --- | --- |
| idFxPt-s | <s> Fixed Part | 7.2 | M | M |
| idFxPt-r | <r> Fixed Part | 7.2 | M | M |
| idAddr-s | <s> Addresses | 7.3 | M | M |
| idAddr-r | <r> Addresses | 7.3 | M | M |
| idSeg-s | <s> Segmentation Part | 7.4 | M | M |
| idSeg-r | <r> Segmentation Part | 7.4 | M | M |
| idPadd-s | <s> Padding | 7.5.2 | M | M |
| idPadd-r | <r> Padding | 7.5.2 | M | M |
| idSecu-s | <s> Security | 7.5.3 | iSecu:M | iSecu:M |
| idSecu-r | <r> Security | 7.5.3 | iSecu:M | iSecu:M |
| idCRR-s | <s> Complete Route Recording | 7.5.5 | iCRR:M | M |
| idCRR-r | <r> Complete Route Recording | 7.5.5 | iCRR:M | - |
| idPRR-s | <s> Partial Route Recording | 7.5.5 | M | M |
| idPRR-r | <r> Partial Route Recording | 7.5.5 | iPRR:M | iPRR:M |
| idCSR-s | <s> Complete Source Routing | 7.5.4 | iCSR:M | - |
| idCSR-r | <r> Complete Source Routing | 7.5.4 | iCSR:M | - |
| idPSR-s | <s> Partial Source Routing | 7.5.4 | M | M |
| idPSR-r | <r> Partial Source Routing | 7.5.4 | iPSR:M | - |
| idQOSM-s | <s> QOS Maintenance | 7.5.6 | M | M |
| idQOSM-r | <r> QOS Maintenance | 7.5.6 | iQOSM or iCong:M | iQOSM or iCong:M |
| idPri-s | <s> Priority | 7.5.7 | M | M |
| idPri-r | <r> Priority | 7.5.7 | iPRi:M | iPri:M |
| idData-s | <s> Data | 7.6 | M | M |
| idData-r | <r> Data | 7.6 | M | M |
| idUnSup2 | Are received PDUs containing parameters selecting unsupported type 2 functions discarded and where appropriate an Error Report PDU generated ? | 6.19 | M | M |
| idUnSup3 | Are parameters selecting unsupported Type 3 functions ignored ? | 6.19 | M | M |

### Intermediate Systems - Supported ER Parameters

| Item | Parameter | ISO/IEC 8473-1 Reference | Status | ATN Support |
| --- | --- | --- | --- | --- |
| ieFxPt-s | <s> Fixed Part | 7.2 | M | M |
| ieFxPt-r | <r> Fixed Part | 7.2 | M | M |
| ieAddr-s | <s> Address | 7.3 | M | M |
| ieAddr-r | <r> Address | 7.3 | M | M |
| ieSeg-s | <s> Seg Part | 7.4 | M | M |
| ieSeg-r | <r> Seg Part | 7.4 | M | M |
| iePadd-s | <s> Padding | 7.5.2 | M | M |
| iePadd-r | <r> Padding | 7.5.2 | M | M |
| ieSecu-s | <s> Security | 7.5.3 | iSecu:M | iSecu:M |
| ieSecu-r | <r> Security | 7.5.3 | iSecu:M | iSecu:M |
| ieCRR-s | <s> Complete Route Recording | 7.5.5 | M | M |
| ieCRR-r | <r> Complete Route Recording | 7.5.5 | iCRR:M | - |
| iePRR-s | <s> Partial Route Recording | 7.5.5 | M | M |
| iePRR-r | <r> Partial Route Recording | 7.5.5 | iPRR:M | iPRR:M |
| ieCSR-s | <s> Complete Source Routing | 7.5.4 | iCSR:M | - |
| ieCSR-r | <r> Complete Source Routing | 7.5.4 | iCSR:M | - |
| iePSR-s | <s> Partial Source Routing | 7.5.4 | M | M |
| iePSR-r | <r> Partial Source Routing | 7.5.4 | iPSR:M | - |
| ieQOSM-s | <s> QOS Maintenance | 7.5.6 | M | M |
| ieQOSM-r | <r> QOS Maintenance | 7.5.6 | iQOSM or iCong:M | iQOSM or iCong:M |
| iePri-s | <s> Priority | 7.5.7 | M | M |
| iePri-r | <r> Priority | 7.5.7 | iPri:M | iPri:M |
| ieDisc-s | <s> Reason for Discard | 7.9.5 | M | M |
| ieDisc-r | <r> Reason for Discard | 7.9.5 | M | M |
| ieData-s | <s> Data | 7.6 | M | M |
| ieData-r | <r> Data | 7.6 | M | M |
| ieUnsup2 | Are received PDUs containing parameters selecting unsupported type 2 functions discarded ? | 6.21 | M | M |
| ieUnsup3 | Are parameters selecting unsupported Type 3 functions ignored ? | 6.21 | M | M |

### Intermediate Systems - Supported ERQ Parameters

| Item | Parameter | ISO/IEC 8473-1 Reference | Status | ATN Support |
| --- | --- | --- | --- | --- |
| iqFxPt-s | <s> Fixed Part | 7.2 | M | M |
| iqFxPt-r | <r> Fixed Part | 7.2 | M | M |
| iqAddr-s | <s> Addresses | 7.3 | M | M |
| iqAddr-r | <r> Addresses | 7.3 | M | M |
| iqSeg-s | <s> Segmentation Part | 7.4 | M | M |
| iqSeg-r | <r> Segmentation Part | 7.4 | M | M |
| iqPadd-s | <s> Padding | 7.5.2 | M | M |
| iqPadd-r | <r> Padding | 7.5.2 | M | M |
| iqSecu-s | <s> Security | 7.5.3 | iSecu:M | iSecu:M |
| iqSecu-r | <r> Security | 7.5.3 | iSecu:M | iSecu:M |
| iqCRR-s | <s> Complete Route Recording | 7.5.5 | iCRR:M | M |
| iqCRR-r | <r> Complete Route Recording | 7.5.5 | iCRR:M | - |
| iqPRR-s | <s> Partial Route Recording | 7.5.5 | M | M |
| iqPRR-r | <r> Partial Route Recording | 7.5.5 | iPRR:M | iPRR:M |
| iqCSR-s | <s> Complete Source Routing | 7.5.4 | iCSR:M | - |
| iqCSR-r | <r> Complete Source Routing | 7.5.4 | iCSR:M | - |
| iqPSR-s | <s> Partial Source Routing | 7.5.4 | M | M |
| iqPSR-r | <r> Partial Source Routing | 7.5.4 | iPSR:M | - |
| iqQOSM-s | <s> QOS Maintenance | 7.5.6 | M | M |
| iqQOSM-r | <r> QOS Maintenance | 7.5.6 | iQOSM or iCong:M | iQOSM or ICong:M |
| iqPri-s | <s> Priority | 7.5.7 | M | M |
| iqPri-r | <r> Priority | 7.5.7 | iPRi:M | iPri:M |
| iqData-s | <s> Data | 7.6 | M | M |
| iqData-r | <r> Data | 7.6 | M | M |
| iqUnSup2 | Are received PDUs containing parameters selecting unsupported type 2 functions discarded and where appropriate an Error Report PDU generated ? | 6.19 | M | M |
| iqUnSup3 | Are parameters selecting unsupported Type 3 functions ignored ? | 6.19 | M | M |

### Intermediate Systems - Supported ERP Parameters

| Item | Parameter | ISO/IEC 8473-1 Reference | Status | ATN Support |
| --- | --- | --- | --- | --- |
| ipFxPt-s | <s> Fixed Part | 7.2 | M | M |
| ipFxPt-r | <r> Fixed Part | 7.2 | M | M |
| ipAddr-s | <s> Addresses | 7.3 | M | M |
| ipAddr-r | <r> Addresses | 7.3 | M | M |
| ipSeg-s | <s> Segmentation Part | 7.4 | M | M |
| ipSeg-r | <r> Segmentation Part | 7.4 | M | M |
| ipPadd-s | <s> Padding | 7.5.2 | M | M |
| ipPadd-r | <r> Padding | 7.5.2 | M | M |
| ipSecu-s | <s> Security | 7.5.3 | iSecu:M | iSecu:M |
| ipSecu-r | <r> Security | 7.5.3 | iSecu:M | iSecu:M |
| ipCRR-s | <s> Complete Route Recording | 7.5.5 | iCRR:M | M |
| ipCRR-r | <r> Complete Route Recording | 7.5.5 | iCRR:M | - |
| ipPRR-s | <s> Partial Route Recording | 7.5.5 | M | M |
| ipPRR-r | <r> Partial Route Recording | 7.5.5 | iPRR:M | iPRR:M |
| ipCSR-s | <s> Complete Source Routing | 7.5.4 | iCSR:M | - |
| ipCSR-r | <r> Complete Source Routing | 7.5.4 | iCSR:M | - |
| ipPSR-s | <s> Partial Source Routing | 7.5.4 | M | M |
| ipPSR-r | <r> Partial Source Routing | 7.5.4 | iPSR:M | - |
| ipQOSM-s | <s> QOS Maintenance | 7.5.6 | M | M |
| ipQOSM-r | <r> QOS Maintenance | 7.5.6 | iQOSM or iCong:M | iQOSM or ICong:M |
| ipPri-s | <s> Priority | 7.5.7 | M | M |
| ipPri-r | <r> Priority | 7.5.7 | iPRi:M | iPri:M |
| ipData-s | <s> Data | 7.6 | M | M |
| ipData-r | <r> Data | 7.6 | M | M |
| ipUnsup2 | Are received PDUs containing parameters selecting unsupported type 2 functions discarded and where appropriate an Error Report PDU generated ? | 6.19 | M | M |
| ipUnsup3 | Are parameters selecting unsupported Type 3 functions ignored ? | 6.19 | M | M |

### Intermediate Systems - Timer and Parameter Values

| Item | Timer | ISO/IEC 8473-1 Reference | Status | ATN Support |
| --- | --- | --- | --- | --- |
| iReasTim | Reassembly Timer | 6.8 | iReas:M | M |