ATNP/WG2

WP 558

27 November 1999

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL

Working Group 2

20th Meeting

Tokyo, Japan

3 December 1999

SME 5 (Internet Communications Service)

Status Report

Working Paper

Presented by Klaus-Peter Graf (Sub-Volume 5 SME)

Summary

This paper provides a summary on the status of the PDRs which have been submitted against the ATN ICS SARPs (Sub-Volume 5). Furthermore, it presents those PDRs which are still awaiting final resolution by the CCB. Positive approval of the proposed SARPs amendment of these PDRs is expected at the upcoming CCB meeting, i.e. on 6 December 1999.

# Introduction

This paper provides a summary on the status of the Proposed Defect Reports (PDRs) raised against the ATN Internet Communications Service (ICS) SARPs for information of the WG 2 members.

Furthermore, it presents in the attachment those PDRs which have been accepted by the CCB but final approval of the proposed SARPs amendment of these PDRs is still outstanding. Positive approval of the proposed SARPs amendment is expected at the next CCB meeting, i.e. on 6 December 1999.

# PDR Status

Table 1 presents the list of those PDRs which have been submitted to the ATNP Configuration Control Board (CCB) since its establishment in spring 1997 and which apply to the Internet Communications Service (ICS) SARPs. Column 3 of Table 1 lists the status of these PDRs in the ATNP CCB process as of 27th November 1999. Column 4 indicates the version/edition of the ATN ICS SARPs in which the agreed technical solution of the resolved PDR has been included.

| PDR Number | PDR Title | CCB Status | Included (I) in ... Scheduled (S) for .. |
| --- | --- | --- | --- |
| 97060028 | Transport Timers Configuration | ADOPTED | ICAO Version 2.2 (I) |
| 97060029 | Various Editorial Defects (1) | ADOPTED | ICAO Version 2.2 (I) |
| 97060030 | IDRP Timers | ADOPTED | ICAO Version 2.2 (I) |
| 97100001 | Incomplete specification for use of V.42bis by Mobile SNDCF | ADOPTED | ICAO Version 2.2 (I) |
| 97100002 | SNDCF Call Request/Confirm User Data Length Indicator | ADOPTED | ICAO Version 2.2 (I) |
| 97100003 | Various Editorial Defects (2) | ADOPTED | ICAO Version 2.2 (I) |
| 97100048 | LREF Directory Management | ADOPTED | ICAO Version 2.2 (I) |
| 98040003 | X.25 Address Extension Facility | ADOPTED | Doc 9705 Edition 2 (I) |
| 98050001 | IDRP Update Receive Process | ADOPTED | Doc 9705 Edition 2 (I) |
| 98060003 | Predicates in ISO/IEC 8473 APRL | ADOPTED | Doc 9705 Edition 2 (I) |
| 98060004 | Support of IDRP by Airborne Router implementing optional non-use of IDRP | ADOPTED | Doc 9705 Edition 2 (I) |
| 98060005 | Air/Ground Route Initiation APRL | ADOPTED | Doc 9705 Edition 2 (I) |
| 98060006 | Correlation of ATSC Class with A/G Subnetwork Type in Airborne Router | ADOPTED | Doc 9705 Edition 2 (I) |
| 98060007 | Symmetry of Mobile SNDCF APRL and Route Initiation APRL | ADOPTED | Doc 9705 Edition 2 (I) |
| 98060008 | IDRP Traffic Typing | ADOPTED | Doc 9705 Edition 2 (I) |
| 98080001 | Segmentation of Error Report PDU | ADOPTED | Doc 9705 Edition 2 (I) |
| 98090002 | Incorrect term "24-bit ICAO Aircraft Identifier" | ADOPTED | Doc 9705 Edition 2 (I) |
| 98090003 | Downgrading of ATSC Class | ADOPTED | Doc 9705 Edition 2 (I) |
| 98090004 | Backbone Hides Optimal Route to Off-Back-bone BISs | REJECTED | ---- |
| 98090010 | Value of SNCR in X.25 Call Request Packets | ADOPTED | Doc 9705 Edition 2 (I) |
| 98100002 | Deflate Frame Checksum | ADOPTED | Doc 9705 Edition 2 (I) |
| 98100003 | End-of-Block Code in Deflate Data Block | ADOPTED | Doc 9705 Edition 2 (I) |
| 98100004 | Deletion of Trailing Zero-Octet | ADOPTED | Doc 9705 Edition 2 (I) |
| 98100005 | Deflate Backwards Window Size | ADOPTED | Doc 9705 Edition 2 (I) |
| 98100007 | Handoff Event | ADOPTED | Doc 9705 Edition 2 (I) |
| 99010001 | Over-specification of SNSDU Requirement | ADOPTED | Doc 9705 Edition 2 (I) |
| 99010005 | Loss of IDRP Connection | ADOPTED | Doc 9705 Edition 2 (I) |
| 99010008 | References to ISO/IEC 8802-2 Broadcast Subnetworks | ADOPTED | Doc 9705 Edition 2 (I) |
| 99030001 | Parameter Setting in CLNP Echo Response PDU | ADOPTED | Doc 9705 Edition 2 (I) |
| 99030002 | Emergency Use of a Mobile Subnetwork | ADOPTED | Doc 9705 Edition 2 (I) |
| 99050001 | Echo NPDUs Supported By Iss | ADOPTED | Doc 9705 Edition 2 (I) |
| 99070004 | Remove Jitter on IDRP Timers for Airborne BIS | RESOLVED | Final Draft Edition 3 (I) |
| 99070005 | ATSC Class of Locally Originated Routes | RESOLVED | Final Draft Edition 3 (I) |
| 99070006 | ATN NSAP Address Compression Algorithm (ACA) | RESOLVED | Final Draft Edition 3 (I) |
| 99090001 | Over-specification of ARS Address Field Assignment | RESOLVED | Final Draft Edition 3 (I) |
| 99090002 | Extension Capability of Mobile SNDCF Header | PROPOSED | Final Draft Edition 3 (I) |
| 99100003 | LREF Compression and CLNP Echo NPDUs | PROPOSED | Final Draft Edition 3 (I) |
| 99100004 | ISO/IEC 8208 Non-Standard Default Packet Size Facility | PROPOSED | Final Draft Edition 3 (I) |
| 99100005 | Reservation of Unassigned/Undefined Values | PROPOSED | Final Draft Edition 3 (I) |

Table 1: Status of ICS PDRs in the ATNP CCB Process

As illustrated in Table 1, a total of 39 PDRs have been raised against the ICS SARPs over the past 31 months.

WG2 members should note that there are currently no outstanding ICS PDRs, i.e. all raised defects since the closing date of Doc 9705 Edition 2 (i.e. end of WG2 Naples meeting) have been resolved and included in the Final Draft Edition 3 of Doc 9705.

## Resolved PDRs

34 of these PDRs have been resolved by the WG 2 SARPs Development Mechanism (SDM) and the proposed technical solutions approved by the CCB. One PDR has been rejected.

Concerning 7 of these 34 resolved PDRs the relevant technical modifications have been included in the ICAO Version 2.2 of the ATN SARPs and also brought forward to the Manual of Technical Provisions for the Aeronautical Telecommunication Network (ATN) - ICAO Doc 9705-AN/956 (1st Edition, 1998). The agreed technical solutions of another 23 resolved PDRs have been included in Edition 2 of ICAO Doc 9705 (November 1999). The remaining 4 resolved PDRs as well as 4 other PDRs which are currently in the “Proposed” status (see last four entries in Table 1) have been included in the Final Draft Edition 3 of ICAO Doc 9705 as presented in WP 557 to this meeting.

## Pending PDRs

There are currently four ICS PDRs which have been accepted by the ATNP CCB but which are still awaiting final approval of the proposed SARPs amendment by the CCB. These are PDR 99090002, PDR 99100003, PDR 99100004 and PDR 99100005 (see also Table 1). These PDRs are attached to this report for information of WG 2 members. The proposed SARPs amendment contained in these PDRs has already been approved by the WG2 SDM process; official approval by the CCB is expected at the upcoming CCB meeting on 6th December 1999.

## Editiorial PDRs

In addition to the PDRs listed in Table 1 a substantial number of editorial defects has been identified during the review of the ATN SARPs ICAO Version 2.0 (distributed at the Langen ATNP meetings), ICAO Version 2.1 (distributed at the Redondo Beach ATNP meetings), ICAO Version 2.2 (distributed at the Rio ATNP meetings), ICAO Doc 9705 1st Edition (distributed at the Utrecht ATNP meetings) and ICAO Doc 9705 2nd Edition (distributed to the CCB in August 1999). These defects have been documented in five editorial PDRs which apply to all Sub-Volumes of Doc 9705, including Sub-Volume 5. These PDRs and their status in the CCB process are listed in the following table:

| PDR Number | PDR Title | CCB Status | Included (I) in ... Scheduled (S) for .. |
| --- | --- | --- | --- |
| 97060001 | Corrections to ICAO V2.0 produced by ICAO secretariat | ADOPTED | ICAO Version 2.1 (I) |
| 97110001 | Corrections to ICAO V2.1 produced by ICAO secretariat | ADOPTED | ICAO Version 2.2 (I) |
| 98040005 | Corrections to ICAO V2.2 produced by ICAO secretariat | ADOPTED | Doc 9705 Edition 1 (I) |
| 98070003 | ICAO 9705 – Engineering Version Discrepancies and Editorial Errors | ADOPTED | Doc 9705 Edition 2 (I) |
| 99010004 | ICAO 9705 Edition 1 Editorial Errors | ADOPTED | Doc 9705 Edition 2 (I) |
| 99070001 | ICAO 9705 Edition 2 Editorial Errors | PROPOSED | Final Draft Edition 3 (I) |

Table 2: Status of PDRs Documenting Editorial Defects in Sub-Volume 5

PDR 9907001 is expected to be closed at the end of the series of WG meetings in Japan.

# Recommendation

The working group is invited to note the above reported status on the ICS PDRs and to note the attached PDRs which will be presented to the upcoming CCB meeting for final resolution.

Attachment A

Title:	Extension Capability of Mobile SNDCF Header

PDR Reference:	99090002

SARPs Document Reference:	ICS SARPs, Section 5.7.6.2.1 and 5.7.6.2.2

Status:	PROPOSED

Impact:	C (Clarrification)

PDR Revision Date:	SUBMITTED --> ACCEPTED (28/09/99)

ACCEPTED --> PROPOSED (10/11/99)

PDR Submission Date:	14 September 1999

Submitting State/Organisation:	Germany/DFS

Submitting Author Name:	Klaus-Peter Graf

Submitting Author E-mail Address:	klaus.graf@unibw-muenchen.de

Submitting Author Supplemental

Contact Information:

SARPs Date:	SV 5 Edition 1

SARPs Language:	English

Summary of Defect:

The current specification of the mobile SNDCF does not allow for octet extensions in the SNDCF header in a backwards compatible way. Such extensions will be required to signal new capabilities (e.g. maintenance of DEFLATE history window) which may be added to future versions of the ATN SARPs.

To accommodate additional mobile SNDCF options in future editions of Sub-Volume 5 in a backwards compatible way, the capability of extending the mobile SNDCF header is proposed in the following SARPs amendment.

Assigned SME: Sub-Volume V SME (K.-P. Graf)

Discussion:

The approach to provide for an extension capability to the Mobile SNDCF header has been agreed by IDG/2. A PDR has been chosen as method of promulgating this intended change in order to inform implementors early about the direction in which Sub-Volume 5 is intended to be progressed to provide backwards-compatible extensibility in the long term.

The proposed solution extends the parameter block of the Call Request and Call Accept User Data and has been specified along the following outline:

1) Format of the Call Request User Data:

a) The SNDCF Parameter block remains as specified in Editions 1 and 2. In particular the definition of the length indicator field is unchanged and continues to indicate the number of octets in the SNDCF parameter  block, from the version number field up to and including (if present)  the maximum number of directory entries field.

b) The version number of the SNDCF Parameter  block is used to indicate the presence or absence  of an additional SNDCF Parameter Extension Block. Version Number = 1 indicates that no SNDCF Parameter Extension Block is present and the format of the Call Request User Data remains as specified in Editions 1 and 2. Version Number = 2 indicates that an additional SNDCF Parameter Extension Block is following the existing SNDCF Parameter Block.

c) The SNDCF Parameter Extension Block consists of a (second) Length Indicator followed by a sequence of TLV (Type-Length-Value)encoded optional parameters.

d) The (second) Length Indicator is one octet-long, and indicates the total length of the SNDCF Parameter Extension Block.

e) The SNDCF Parameter Extension Block may be followed by a "user data field", that can be used to convey an ISH PDU.

When establishing a call, a Package 2 router may select to use Version 1 or Version 2 of the SNDCF protocol. In the case, additional compression parameters have to be conveyed, it will use Version 2. A Package 1 compliant  router that is called by a Package 2 compliant router, will process  the SNDCF Parameter Block and will not accept the call, if the Version Number is 2; it will respond with a Call Clear packet indicating "Version number not supported" in the diagnostic code. This will allow the calling Package 2 router to re-establish the call with Version Number set to one, but without the additional compression parameters.

2) Format of the Call Accept User Data:

a) One of the spare bits in octet 1 of the Call Accept user  data is used to indicate the presence (if the bit set) or absence (if the bit is not set) of a subsequent SNDCF Parameter Extension Block in the Call Accept user data.

b) An SNDCF Parameter Extension Block will be present in the Call Accept User Data only if a Call Requst packet with Version Number = 2 has been received. This insures backwards compatibility between a calling Package 1 router and a called Package 2 router.

c) When present, the SNDCF Parameter Extension Block consists of a Length Indicator followed by a sequence of TLV  (Type-Length-Value) encoded optional parameters.

d) The Length Indicator is one octet long and indicates the total length of the SNDCF Parameter Extension Block.

e) The SNDCF Parameter Extension Block may be followed by a "user data field", that can be used to convey an ISH PDU.

This approach allows for the extension of the SNDCF header to convey additional parameters in a backwards compatible way, but at the cost of a second call establishment sequence, if the calling router uses Version 2 of the SNDCF protocol and the calling router complies to Edition 1 or 2 of the ICS SARPs.

Proposed SARPs Amendment:

See file 99090002.zip in the directory atnp/ccb/sme5 on the CENA ATNP archive (http://www.tls.cena.fr/atnp/ccb/sme5/99090002.zip)

Impact on Interoperability:

The SARPs amendment proposed in this PDR will provide for the necessary long-term backwards compatibility mechanism when introducing new SNDCF capabilities in future SARPs versions. The SARPs amendment is specified in a way that it will be backwards compatible with implementations based on Edition 1 and 2 of Doc 9705. This backwards compatibility is achieved by a mechanism which requires routers implementing this PDR to fall back to version 1 of the SNDCF protocol (specified in Edition 1 and 2 of Doc 9705), when recognising (from the diagnostic code of a received call clear packet) that the peer router has not implemented this PDR.

SME Recommendation to CCB: Accept proposed SARPs amendment

CCB Decision:  PDR ACCEPTED (CCB-10)

Attachment B

Title:	LREF Compression and CLNP ECHO NPDUs

PDR Reference:	99100003

Originator Reference:

SARPs Document Reference:	ICS SARPs, Section 5.7.6.3.2.3

Status:	PROPOSED

Impact:	C (Clarification)

PDR Revision Date:	SUBMITTED --> ACCEPTED (10/11/99)

ACCEPTED --> PROPOSED (26/11/99)

PDR Submission Date:	29/10/99

Submitting State/Organisation:	France/STNA

Submitting Author Name:	Stephane Tamalet

Submitting Author E-mail Address:	Tamalet_stephane@stna.dgac.fr

Submitting Author Supplemental

Contact Information:

SARPs Date:	SV 5 Edition 2

SARPs Language:	English

Summary of Defect:

It appears that chapter 7 of Subvolume 5 is not very specific on the way ECHO REQUEST/RESPONSE PDUs have to be processed by the mobile SNDCF when the LREF compression is in use:

1) the section 5.7.6.3.2.2 explains that any PDUs other than ISO 8473 (CLNP), ISO 9542 (ES-IS), ISO 10589 (IS-IS) and ISO 11577 (NLSP) PDUs  sent over the A/G link, have to be discarded by the mobile SNDCF. According to this section, ECHO REQUEST/RESPONSE PDUs should not be discarded since they are standard ISO 8473 PDUs

2) the paragraph 5.7.6.3.2.3.1 lists the cases for which an ISO 8473 PDU must be sent unchanged (uncompressed) over the A/G link. Namely:

a) When the Source Routing option is present,

b) When the Recording of Route option is present,

c) When the QoS Maintenance option is anything other than the globally unique format,

d) When the padding option is present,

e) When the priority option is present with a value > 14,

f) When an unknown parameter is present.

According to this paragraph, an ECHO REQUEST/RESPONSE PDU that does not verify any of the above conditions, should be sent compressed over the A/G link

3) the section 5.7.6.3.3 specifies how to compress with LREF a PDU. However, the compression procedures specify only how to compress an ISO 8473 DATA and ERROR REPORT PDU. There is therefore an ambiguity on the way ECHO REQUEST/RESPONSE PDU must be processed by the mobile SNDCF when LREF is in use.

It is proposed to clarify the issue by removing the ambiguity that currently exists on this subject in the SARPs.

Assigned SME: Sub-Volume V SME (K.-P. Graf)

Discussion:

There are 3 possible options in the processing of an ECHO REQUEST/RESPONSE PDUs by the mobile SNDCF when the LREF compression is in use.

Option a): The ERQ/ERP PDUs are systematically discarded when LREF is used

Option b): The ERQ/ERP PDUs are sent uncompressed (the ProATN and TAR approach)

Option c): The ERQ/ERP PDUs are sent compressed

The option b) is proposed as the default to be implemented in all ATN systems.

The rationale is as follows:

1) The option a) can be of interest if it is commonly agreed that ERQ/ERP must not be exchanged over mobile subnetworks. However, in such a case, option a) would not be sufficient. This is because with option a)ERQ/ERP are only filtered if and only if LREF is used. If Deflate is used without LREF, or no compression mechanism is in use, then the ERQ/ERP PDU will not be filtered. So, if there is a requirement to prevent the transmission of ERP/ERQ PDU over mobile subnetworks, then a more general filtering mechanism (compression method independent) has to be specified within the SNDCF.

2) Option b) is proposed because it is believed that the use of ERP/ERQ over mobile subnetwork can be of interest in some cases for fault and performance management. This feature should therefore be supported by default by implementations.

3) Option c) would require major text amendments in chapter 7 (to specify how ERP and ERQ must be compressed. It would be costly for implementations and would not be backwards compatible. Furthermore, there is little to gain in compressing the ERQ and ERP PDUs since it is expected that these PDUs will be very rarely exchanged over mobile subnetworks.

Proposed SARPs Amendment:

Replace paragraph 5.7.6.3.2.3.1 by

"5.7.6.3.2.3.1  The ISO/IEC 8473 NPDU header contained in the SN-Userdata shall then be

inspected. If one of the following is true:

- the ISO/IEC 8473 NPDU is an Echo Request (ERQ) or Echo Response(ERP) NPDU,

- parameters other than the security, priority or QoS Maintenance parameters are present in the options part of the NPDU header,

- the QoS Maintenance parameter is anything other than the globally unique format,

- the priority option is present with a value greater than 14,

then the SN-Userdata shall be sent unchanged over the virtual circuit using M-bit segmentation procedures as appropriate."

Impact on Interoperability:  None

SME Recommendation to CCB: Accept proposed SARPs amendment

CCB Decision: PDR ACCEPTED (10/11/99)

Attachment C

Title:	ISO/IEC 8208 Non-Standard Default Packet Size Facility

PDR Reference:	99100004

Originator Reference:

SARPs Document Reference:	ICS SARPs, Section 5.7.6.2.1.3

Status:	PROPOSED

Impact:	C (Clarification)

PDR Revision Date:	SUBMITTED --> ACCEPTED (10/11/99)

ACCEPTED --> PROPOSED (26/11/99)

PDR Submission Date:	29/10/99

Submitting State/Organisation:	USA/FAA/MITRE

Submitting Author Name:	Linda Boan

Submitting Author E-mail Address:	lboan@mitre.org

Submitting Author Supplemental

Contact Information:

SARPs Date:	SV 5 Edition 2

SARPs Language:	English

Summary of Defect:

Para 5.7.6.2.1.3.1 mandates the use of the ISO/IEC 8208 non-standard default packet size facility to make full use of the maximum packet size supported by the a/g subnetwork. However, there are alternative methods (e.g. the ISO/IEC 8208 flow control negotiation facility) of achieving the same effect. So, the underlying requirement is correct, i.e. using the biggest packet size possible for each SVC, but the SARPs are over pre-screptive in mandating a particular procedure. Consequently they should be relaxed to let the implementor choose the most appropriate way of achieving the underlying requirement.

Assigned SME: Sub-Volume V SME (K.-P. Graf)

Proposed SARPs Amendment:

1) Amend the existing para 5.7.6.2.1.3 to read: "5.7.6.2.1.3 Non-Standard Default Packet Size Facility and Flow Control Parameter Negotiation Facility"

2) Amend the existing para 5.7.6.2.1.3.1 to read: "5.7.6.2.1.3.1 Either the Non-Standard Default Packet Size Facility or the Flow Control Parameter Negotiation Facility shall be used to request the maximum packet size supported by the subnetwork."

3) Add a new note under 5.7.6.2.1.3.1 to read: "Note. -- The selection of which facility to use is dependent on the facilities supported by the subnetwork."

Impact on Interoperability:  None

SME Recommendation to CCB: Accept proposed SARPs amendment

CCB Decision:  PDR ACCEPTED (10/11/99)

Attachment D

Title:	Reservation of Unassigned/Undefined Values

PDR Reference:	99100005

Originator Reference:

SARPs Document Reference:	ICS SARPs, Sections 5.6.2.2.6.7, 5.6.2.2.6.8,5.8.2.1.4.4, 5.8.3.2.8

Status:	PROPOSED

Impact:	C (Clarification)

PDR Revision Date:	SUBMITTED --> ACCEPTED (10/11/99)

ACCEPTED --> PROPOSED (26/11/99)

PDR Submission Date:	31/10/99

Submitting State/Organisation:	Germany/DFS

Submitting Author Name:	Klaus-Peter Graf

Submitting Author E-mail Address:	klaus.graf@unibw-muenchen.de

Submitting Author Supplemental

Contact Information:

SARPs Date:	SV 5 Edition 2

SARPs Language:	English

Summary of Defect:

The paragraphs 5.6.2.2.6.7, 5.6.2.2.6.8, 5.8.2.1.4.4, 5.8.3.2.8 assign specific values for the routing preferences, security classification, supported ATSC Class, and the capacity route metric respectively, to be used in the header of CLNP and IDRP PDUs respectively. However, the specification misses to reserve currently unassigned/undefined values for future use by future editions of SV5.

Assigned SME: Sub-Volume V SME (K.-P. Graf)

Proposed SARPs Amendment:

1.) Add the following new paragraph 5.6.2.2.6.7.4:

"5.6.2.2.6.7.4  Those security tag values which are not defined in Table 5.6-1 shall be reserved for future use by this specification."

2.) Add the following new paragraph 5.6.2.2.6.8.4:

"5.6.2.2.6.8.4  Those security classification tag values which are not assigned in Table 5.6-2 shall be reserved for future use by this specification."

3.) Add the following new paragraph 5.8.2.1.4.4.3.8:

"5.8.2.1.4.4.3.8  Those ATSC Class values which are not defined in Table 5.8-1 shall be reserved for future use by this specification."

4.) Add the following new paragraph 5.8.3.2.8.2:

"5.8.3.2.8.2  Those capacity route metric values which are not assigned in Table 5.8-6 shall be reserved for future use by this specification."

Impact on Interoperability:

Implementations compliant with Edition 1 or 2 of SV5 should not have used any undefined or unassigned values for parameters specified in SV5. Although their use is highly unlikely, it cannot be ruled out completely.

Interoperability is not affected by the proposed SARPs amendment. Interoperability problems with Package 1 systems may arise if the currently undefined or unassigned values will be allocated by future editions of SV5.

SME Recommendation to CCB: Accept proposed SARPs amendment

CCB Decision: PDR ACCEPTED (10/11/99)