FLIMSY  7

WG2/8 - Brussels, Belgium

April 25, 1996

## Changes to TP4 Timer Specification

Proposed changes agreed by the CCB and  presented in ver 4.1 relating to TP4 timer values are changed based upon input from WG2/8 as follows:

## 5.4 Implementation

Recommendation. - The following timers and variables should be configurable on a TC basis:

·	the local retransmission timer (T1),

·	the acknowledgment timer (AL),

·	the window update timer (W),

·	the inactivity timer (I),

·	the frozen reference time (L),

·	the maximum retransmission number (N),

·	the persistence timer (R)

For use in the ATN, the acknowledgment timer (AL) shall be modified according to the following:

Code: 	 	identical to ISO 8073 standard parameter

Length:   	2 or 3 octets (up to 24 bits);

Value:	  	acknowledgment timer value expressed in milliseconds (per ISO 8073 standard)

Note 1. - This change is in response to the unique requirements of the aeronautical environment which may require longer acknowledgment times.

Note 2. - Initial values of these timers will be dependent upon the subnetwork,  traffic type and routing policy requirements expressed in the associated ATN Security label.

Note 3. - In cases where the ALvalue is 2 (less than 65,565 milliseconds), the ATN implementation will behave in compliance with the ISO 8073 standard.

Note 4. - Implementors are advised to permit systems administrators to readily specify such initial values.