WG2/WP93

ROCKWELL-COLLINS

March 2, 1995

ATN Working Group 2 and Working Group 3

Rockwell-Collins is concerned about what has been going on

in the ATN panel and its three working groups. Currently ADI

(AVPAC DUTCH INITIATIVE); AA, ARINC, Rockwell Collins, SITA,

and UAL; and the ADS Europe team are developing systems

based on the current ATN manual (Second Edition) and ADS (DO-

212/AEEC 745-2) with IDRP as an option.   As an Avionics

vendor we have to meet customer needs with a cost effective

solution and make a profit.  In meeting the customer's

needs, we need to give the customer the power to control

cost.  We need to define the bare minimum as mandatory PICS

and let the airlines add options, if they are cost

justified.  Furthermore, all changes should be backward

compatible!  It is difficult for vendors to build to a

moving target when there is no backward compatibility.   For

example, we are currently building ADS  to use the services

of TP4 and CMA (DO-223).  If the applications were

allomorphic and had a standard API, venders/airlines could

invest with a reasonable return on their investment.

However, if the ATN panel keeps throwing out the old,  with

no backward compatibility and continues to define new

specifications , ATN will die.   Airlines/vendors will not

be able to afford to invest.  The specific issues we are

referring to are:  (1) mandatory IDRP and (2) overhaul of

ADS & TWDL - with no backwards compatibility.

Mandatory IDRP will cause many extra packets  to be

exchanged across the RF with limited benefit/pay-back for

the airlines.  If  we put on our "pure communication

academia hat", we agree that IDRP between the airborne

router makes the ATN more like a standard WAN.  However,

cost analysis makes the academia solution not viable to sell

as a product.  The cost of  having the airborne router do

IDRP updates over the RF is as follows:  (1) airborne

certified IDRP ($$$) and  (2) many packets exchanged over

the RF for IDRP ( who will pay for these packets?) .   If

the ground BISs are a little smarter, IDRP can be kept off

the aircraft.  The airborne router sends a ISH hello to the

attached BIS on the ground; the ISH hello allows the Ground

BIS to find out the domain identifier of the airborne

domain.  The ground BIS updates ground routers FIB to

indicate all traffic destined to the airborne domain

identifier should  be routed via this BIS, which has the

current connection to the aircraft.  If an airline wants to

use IDRP and is willing to pay for it, we will be glad to

supply a airborne router with IDRP.  However, Rockwell-

Collins feels  IDRP should be optional and the ground BIS

should support airborne routes with or without IDRP (i.e. as

defined by ADS Europe trials).

The overhaul of ADS to use the services of A2CSE may be

correct for the future; however,  making ATM applications

dependent on A2CSE will slow down implementation of ATN.  We

do not know any software vendors building or planning to

build A2CSE  in the near future.  If development on A2CSE

started today, it would be several years before a useable

product would be available.  We are not trying to kill

A2CSE. We think  the full stack implementation is a good

thing because things like security/authentication and

orderly initiation/termination of message exchange has to be

built into every application. However, we believe we must

walk before we can run.  If we build an airborne end system

with ADS  to operate over the transport layer using the

services of CMA and make it available as a product, the

airlines will need to get enough service out of the product

to justify the investment.  The airlines will not be able to

justify the investment  if there is no backward

compatibility.  Therefore, if the airlines cannot invest in

ATN for 3 years the vendors will wait. In 3 years maybe

A3CSE comes out; do we wait another 3 years after that

because there is no backward compatibility?  We believe with

the talent in WG2 & WG3 that the backward compatibility can

be addressed.  The  end system applications should be

allomorphic which allows the applications to receive

ADS/TWDL messages transparent to the communication stack.

This will allow airlines/vendors who invest now to make a

profit on their investment and allow A2CSE to progress.  We

suggest a design where applications can operate with a

standard API over  ATN (second edition [i.e. 10 year sunset]

) or ATN+ (A2CSE).

ATN Team Leader

Fred Mabe

Ph # 319-395-3006