ATNP/WG2/
 WP/292
20 June 1996

Proposed Revisions to Chapter 7

SUMMARY

A number of technical problems were identified by the ATN SARPs editorial committee in chapter 7. This working paper is the result of an action accepted at the editorial meeting to propose detailed revisions to correct the identified problems.

C&R Ref.

Presented By Henk Hof

Prepared by Tony Whyman

Rev. No.

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL

WORKING GROUP TWO

Munich 24.6.96-28.6.96

DOCUMENT CONTROL LOG

| SECTION | DATE | REV. NO. | REASON FOR CHANGE OR REFERENCE TO CHANGE |
| --- | --- | --- | --- |
|  | 20-june 96 | Issue 1.0 |  |

TABLE OF CONTENTS

1. Introduction

2. Proposed Resolution

3. Recommendation

# Introduction

The following problems and issues were identified during the editorial committee meeting:

- There are serious technical problems in 5.7.5.2 (On subnetwork QoS). The specified way in which subnetwork QoS is used is wrong and needs to be re-specified.

- Non-use of Fast Select procedures need to be clarified and the Call Request procedures for the Mobile SNDCF generally tidied up. We found it almost impossible to understand when reading through.

- The procedures that are common to all ISO 8208 SNDCFs and those which are specific to the fixed and mobile SNDCFs need to be clearly identified.

- The CIDIN SNDCF has been inserted in the wrong place and separates the ACA from the rest of the Mobile SNDCF.

- A proposal to move the SNDCF APRLs from chapter 6 to chapter 7 appears to be in order, as the SNDCFs are specified in chapter 7, not chapter 6.

# Proposed Resolution

The attachment to this working paper provides the proposed revised text for chapter 7. The following changes have been applied:

- The SN-Service interface is now presented as notes in line with the other chapters.

- ISO 8802 and CIDIN SNDCFs are presented before the Mobile SNDCF. This avoids the separation of the ACA from the other parts of the Mobile SNDCF and makes these other SNDCFs more visible.

- The text of ISO 8208 QoS is deleted. It was not relevant and contained technical errors.

- Call Setup procedures for the Mobile SNDCF have been fully revised in order to make them clearer and to bring in non-use of Fast Select.

- APRLs for 8802-2 and 8208 have been moved from chapter 6 into this chapter. If this change is accepted then they must be deleted from chapter 6.

# Recommendation

It is recommended that WG2 conducts a line by line review of the attachment prior to considering it as replacement text for Chapter 7 of the draft ATN Internet SARPs.